3GPP TSG RAN WG2 #129                                                                                  R2-2500239

Athens, Greece, Feb. 17th – 21st, 2025

Source:	CATT

Title:	Consideration on LCM for Positioning use case

Agenda Item:	8.1.2.3

Document for:	Discussion and Decision

- Introduction

In last RAN2 meeting [1], some agreements have been achieved for LCM for Positioning use case:

Agreements 1 For POS Case 1, RAN2 confirm that the existing unsolicited UE capability report mechanism in LPP can support UE to report the applicable functionality in both “proactive” and “reactive” as a baseline. - Proactive case: When the applicability change, UE can send an unsolicited LPP ProvideCapabilities message to LMF . - Reactive case: If the applicability changes based on the configuration in LPP ProvideAssistanceData message in step 3, UE can send an unsolicited LPP ProvideCapabilities message to LMF. Configuration details are FFS 2 As a baseline, If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause. FFS if other fallback options are considered

Since aspects related to data collection should be covered in other section, in this contribution, we continue to discuss the LCM procedure related issues on Positioning use case.

- Discussion

  - Positioning case priority

It is approved to focus on the first priority issues (case1/3a/3b) until end of 2024 in RAN#105 meeting [2]. And inside 3 first priority cases, RAN3 has already discussed on case 3a and 3b and some agreements have been achieved in last RAN3 meeting. Therefore, RAN2 should only focus on the PoS case 1.

Proposal 1: RAN2 only focuses on the PoS case 1 among all PoS cases.

  - Positioning method

In previous RAN2 meeting, it is agreed RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

In RAN1 agreements, many parameters in the existing positioning methods are reused, such as timing information of DL RSTD or UE Rx-Tx time difference, LOS/NLOS soft/hard indicator, .etc. In TS38.305, the existing positioning methods are listed as below. The 3 highlighted lines are the NR UE side positioning which parameters are mentioned by RAN1 agreements:

Table 4.3.1-1: Supported versions of UE positioning methods

| Method | UE-based | UE-assisted, LMF-based | NG-RAN node assisted | SUPL Note 8 |
| --- | --- | --- | --- | --- |
| A-GNSS | Yes | Yes | No | Yes |
| OTDOA Note1, Note 2 | No | Yes | No | Yes |
| E-CID Note 4, Note 7 | No | Yes | Yes | Yes for E-UTRA |
| Sensor | Yes | Yes | No | No |
| WLAN | Yes | Yes | No | Yes |
| BluetoothNote 9 | Yes | Yes | No | No |
| TBS Note 5 | Yes | Yes | No | Yes (MBS) |
| DL-TDOA | Yes | Yes | No | Yes |
| DL-AoD | Yes | Yes | No | Yes |
| Multi-RTT | No | Yes | Yes | Yes |
| NR E-CID | No | Yes | Yes | Yes (DL NR E-CID) |
| UL-TDOA | No | No | Yes | Yes |
| UL-AoA | No | No | Yes | Yes |
| NOTE 1: This includes TBS positioning based on PRS signals. NOTE 2: In this version of the specification only OTDOA based on LTE signals is supported. NOTE 3: Void NOTE 4: This includes Cell-ID for NR method when UE is served by gNB. NOTE 5: In this version of the specification only for TBS positioning based on MBS signals. NOTE 6: Void NOTE 7: Enhanced Cell ID based on LTE signals. NOTE 8: This shows whether the positioning method is supported by SUPL ULP [16]. NOTE 9: UE-based positioning mode is supported only for Bluetooth AoD positioning method. |  |  |  |  |

As we mentioned in the Email discussion [3], we do not think there is essential reason to introduce mew positioning method, since based on all the agreements provided currently, the potential specification enhancement based on legacy positioning method is limited:

- For assistance data

RAN1 agreed that all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. Therefore most assistance information of legacy UE-based DL-TDOA can be re-used. RAN1 only have such agreements on UE-based DL-TDOA in RAN1#119 meeting, another method of UE-based DL-AoD mentioned in RAN1#118bis meeting is still under RAN1 discussion.

Agreement For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study, if necessary, choose one alternative from the following: - Alternative 1. Info #7 is provided implicitly via associated ID. - Associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 2. Info #7 can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether info #7 is provided implicitly or explicitly, and the UE can request info #7 to be provided explicitly or implicitly. - If provided implicitly, associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 3. Info #7 is not be provided from LMF to UE. - If info #7 is not provided, UE may assume info #7 is consistent between training and inference. - Alternative 4. Info #7 is provided explicitly from LMF to UE.

For info #7, none of the Alternative#1~4 would cause major changes to the legacy specification.

Observation 1: Besides the assistance information from legacy UE-based DL-TDOA which can be reused for AI/ML, none of the Alternative#1~4 for info#7 would cause major changes to the legacy specification.

- For reference signal

At least for case1 PRS, RAN1 agreed both options are feasible by using legacy mechanisms:

Conclusion For training data collection of Case 1, in terms of DL PRS configuration for collecting training data, both options are feasible by using legacy mechanisms: Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. Option B. (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE. Note: the UE can be a PRU and/or a Non-PRU UE. Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

The RS should be the same for training and inference for consistency. Since current LPP could already support the Options above including on demand PRS and LMF determined PRS, no RS request or transmission related specification impact is foreseen.

Observation 2: For PoS case 1, legacy RS request and/or transmission is enough, and no specification impact related RS enhancement is foreseen based on current RAN1 agreements.

- For the measurement

RAN1 has agreed to use DP/PDP (CIR FFS) in previous meetings:

- Timing: Reuse of existing measurements (e.g. UL RTOA, DL RSTD, Rx-Tx, and some other parameters such as reference time, timing quality);

- Power: DL PRS-RSRPP and UL SRS-RSRPP, as starting points.

Since all parameters are exist in current spec, no new measurement or measurement type has been defined by RAN1.

Observation 3: For timing information and power information in RAN1 agreement, existing measurement is enough, and no new measurement or measurement type has been defined by RAN1.

- Sample based measurement

RAN1 has considered sample based measurement type for AI/ML data collection. This is different from the legacy positioning methods and it is the timing information based on the samples related to the first detected path. We think all the sample based measurement may need enhancement based on the legacy method, e.g. NR-DL-TDOA-MeasElement-r16, and no new positioning method is needed.

Observation 4: For sample based measurement, the sample based measurement may need enhancement based on the legacy method.

Base on all the analysis above, we think there is no essential reason to introduce new positioning method for AI/ML positioning. The AI/ML positioning related UE-side model cases could be considered as enhancements base on the existing positioning methods, e.g. DL-TDOA, DL-AoD, Multi-RTT. And some AI/ML specific parameters could be added into these methods which should depend on RAN1’s discussion.

Proposal 2: Consider AI/ML positioning of UE-side model as enhancements based on the existing positioning methods, e.g. DL-TDOA, DL-AoD, Multi-RTT.

  - Inference related spec impact

For case 1, the model output is the location of UE. Therefore the legacy LPP message of ProvideLocationInformation can be directly reused. We do not foresee specification impact based on the model output on its own, but the LMF needs to know whether the UE’s location is based on prediction or based on measurement. For BM use case, it is the NW to manage the applicable functionality:

Agreements

- When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

For PoS use case, it is better for NW to manage the functionality too. RAN1 agreed that all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. Therefore most assistance information of legacy UE-based DL-TDOA can be re-used in both the training configuration and inference configuration for consistent. The assistance information can be considered as the “NW-side additional condition” in the PoS use case.

Observation 5: Most legacy assistance information and one potential alternative of Info#7 agreed in RAN1 agreements can be considered as the “NW-side additional condition” in the PoS use case.

Proposal 3: For inference configuration of PoS case 1, RAN2 waits RAN1’s last decision about the Alternative#1~4 for info#7, and the potential alternative of Info#7 together with other assistance information from legacy UE-based DL-TDOA agreed by RAN1 is used in training configuration and inference configuration.

  - Monitoring related spec impact

For case 1, RAN1 has listed multiple options with regard to how to generate information on ground truth label:

- Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF.

Until RAN1#118 meeting, it is only concluded that Option A-4 could be realized by implementation and no further discussion is needed on Option A-4. All the other options are not discussed.

In RAN1#119 meeting, RAN1 only agreed that:

Agreement For model performance monitoring of AI/ML positioning Case 1, support at least: - Option A. The target UE side performs monitoring metric calculation. - The target UE may signal the monitoring outcome to the LMF. - FFS: content of monitoring outcome - FFS: Option B

Therefore, Option B is FFS, and only Option A-1/2/3 for Option A will be further discussed by RAN1. For these 3 options:

- Option A-1: It has spec impact. The current LMF could send location information to the UE through a higher layer (e.g. NAS layer) when the UE is act as a client, not through the LPP protocol. So if the target UE sends measurement (e.g., legacy measurement) to LMF, and the LMF provides the ground truth label to the UE, new signaling and procedure maybe needed in LPP protocol;

- Option A-2: It has no spec impact. The existing information of position calculation assistance data is provided from LMF to the target UE;

- Option A-3: It has no spec impact. In R18, NR-PRU-DL-Info-r18 has been introduced into assistance data transfer framework of TS37.355, and this information could be reused directly.

Observation 6: Besides the monitoring outcome transferred from UE to the LMF which apply to all options, Option A-1 needs the enhancement of providing the ground truth label of location information from LMF to UE in LPP protocol, and Option A-2 and Option A-3 have no spec impact.

Proposal 4: For performance monitoring of PoS case 1, if Option A-1 is agreed by RAN1, RAN2 to introduce the location information transmission from LMF to UE in LPP protocol as the ground truth label.

Proposal 5: RAN2 to introduce the monitoring outcome transfer from UE to the LMF in LPP protocol after RAN1 decides the content of monitoring outcome.

- Conclusion

In this contribution, we provide our views on the LCM related issues of for Positioning use case, and the observation and proposals are summarized as follows:

For priority:

Proposal 1: RAN2 only focuses on the PoS case 1 among all PoS cases.

For method definition:

Observation 1: Besides the assistance information from legacy UE-based DL-TDOA which can be reused for AI/ML, none of the Alternative#1~4 for info#7 would cause major changes to the legacy specification.

Observation 2: For PoS case 1, legacy RS request and/or transmission is enough, and no specification impact related RS enhancement is foreseen based on current RAN1 agreements.

Observation 3: For timing information and power information in RAN1 agreement, existing measurement is enough, and no new measurement or measurement type has been defined by RAN1.

Observation 4: For sample based measurement, the sample based measurement may need enhancement based on the legacy method.

Proposal 2: Consider AI/ML positioning of UE-side model as enhancements based on the existing positioning methods, e.g. DL-TDOA, DL-AoD, Multi-RTT.

For inference:

Observation 5: Most legacy assistance information and one potential alternative of Info#7 agreed in RAN1 agreements can be considered as the “NW-side additional condition” in the PoS use case.

Proposal 3: For inference configuration of PoS case 1, RAN2 waits RAN1’s last decision about the Alternative#1~4 for info#7, and the potential alternative of Info#7 together with other assistance information from legacy UE-based DL-TDOA agreed by RAN1 is used in training configuration and inference configuration.

For monitoring:

Observation 6: Besides the monitoring outcome transferred from UE to the LMF which apply to all options, Option A-1 needs the enhancement of providing the ground truth label of location information from LMF to UE in LPP protocol, and Option A-2 and Option A-3 have no spec impact.

Proposal 4: For performance monitoring of PoS case 1, if Option A-1 is agreed by RAN1, RAN2 to introduce the location information transmission from LMF to UE in LPP protocol as the ground truth label.

Proposal 5: RAN2 to introduce the monitoring outcome transfer from UE to the LMF in LPP protocol after RAN1 decides the content of monitoring outcome.

- References

- R2-2500002, RAN2#128 Meeting Report

- RP-242387	Moderator's summary on offline discussion about non-RAN2 aspects on REL-19 WI NR_AIML_Air	RAN1 vice-chair (CMCC)

- [POST128][026][AIML] LCM Procedure for Positioning Case1 (Ericsson)