3GPP TSG RAN WG2 #129                                                                         	  R2-2500241

Athens, Greece, Feb. 17th – 21st, 2025

Source:	CATT

Title:	Consideration on UE side data collection

Agenda Item:	8.1.4

Document for:	Discussion and Decision

- Introduction

In RAN#106 meeting, RAN has sent a LS to SA/SA2/SA5 about the AI/ML UE sided data collection [1] and asked TSG SA to start studying the transfer of data over UP for solution 2 in WG SA2 and transfer of data over UP for solution 3 in WG SA5:

TSG RAN respectfully thanks WG SA2/SA5 for their response LSs on AI/ML UE side data collection. TSG RAN continued further discussions in RAN#106 and has agreed to a way forward in RP-243307. More specifically, TSG RAN has agreed to the following: From RAN perspective, transfer of data over UP for Solution 2 in SA2 should be studied and provide feedback on fulfillment of visibility and controllability requirements. Transfer of data over UP for Solution 3 can be studied by SA5 RAN has already concluded that transfer of data over CP for Solution 2 and 3 can meet the visibility and controllability RAN requirements, even though some scalability concerns have been raised for high amount of data transfer. RAN is not expecting SA2 and SA5 to study the transfer of data over CP for now. TSG RAN has also agreed that RAN2 will continue to discuss and study UE sided data collection in WG RAN2 during Rel-19 time frame. TSG RAN will make a final decision on the way forward taking into account SA inputs when available. TSG RAN will have a checkpoint in Q2, 2025

In this contribution, we further discuss the UE side data collection on Solution 2/3.

- Discussion

  - RAN aspects related to data transfer over UP

Solution 2 UP

For data transfer Solution 2 over UP, RAN2 asks SA2 to provide feedback on fulfillment of visibility and controllability requirements. The legacy UP tunnel is between UE and UPF, therefore the data content visible to CN, and the visibility to the MNO can be discussed between SA2 and SA5. RAN could control and configure the UE side data collection, but the data visibility to the gNB seems not required.

Proposal 1: Data visibility to gNB is not required, and fulfillment of visibility and controllability for data transfer over UP for Solution 2 should be studied by SA2.

Solution 3 UP

In the legacy LS between RAN and SA5 [2], SA5 has provided their view on solution 3:

For option 3: CP tunnel: - The feasibility for the functionality of “Server for data collection for UE-side model training” to be carried by TCE as defined in SA5 MDT mechanism or MCE as defined in SA5 QoE mechanism for CP tunnel can be studied in SA5. SA5 will need to evaluate potential enhancement that may be needed based on the progress in RAN. UP tunnel: - The feasibility of UP tunnelling methods covering UE, gNB, OAM, and Server for data collection for UE-side model training needs to be studied, along with potential coordination with SA2.

For data transfer Solution 2 over UP, the feasibility needs to be studied by SA5.

From our point of view, the key point of UP and CP selection is the transmission on the air-interface. From RAN perspective, both SRB and DRB are limited by the PDCP SDU limit size of 9000bytes, and both could have integrity protection, security, similar transfer time (if assume same priority). The difference between CP and UP solution is the QoS Setting and the data termination point.

Observation 1: From RAN perspective, both SRB and DRB are limited by the PDCP SDU limit size of 9000bytes, the difference between CP and UP solution is the QoS Setting and the data termination point.

For the QoS setting, the transmission priority mechanism is mature by setting SRBs and DRBs, and for data termination point, currently there is no UP tunnel involving OAM in the traditional sense. Therefore considering the CP and the UP solutions:

  - RAN2 has no specific requirements that need to be met by the UP solution compared with the CP solution;

  - A bran-new protocol stack may need to be designed for introducing UP tunnel, since currently OAM does not even have the concept of protocol stack. The complexity is high and the spec impact is great;

  - Reuse of existing CP solutions (enhanced with MDT or QoE) is simple which could already fulfill the requirement of feasibility;

Observation 2: Data transfer over UP for Solution 3 may need to design brand-new protocol stack for UP tunnel which has great RAN and SA impact.

Based on the analyses above, we do not see the benefit to use the solution3 UP.

Proposal 2: If there is no significant benefit to use Solution 3 over UP for AI data transfer, we should not consider this solution which may introduce great RAN and SA impact.

  - RAN aspects related to data transfer over CP

Solution 2 CP

For solution2 over CP, NAS message will be used for AI training data transmission. RAN has already concluded that transfer of data for Solution 2 over CP can meet the visibility and controllability RAN requirements. NAS message will be carried in UL/DL direct transfer message in RRC, so the 9000 bytes PDCP SDU size limit is also applicable for NAS message transmission in the air-interface.

Observation 3: The 9000 bytes PDCP SDU size limit is also applicable for NAS message transmission in the air-interface.

NAS message will be transferred via SRB2 if SRB2 has been established, otherwise SRB1 will be used. Since RRC cannot see the content of the NAS message, it is hard to control the transmission priority for large scale of UE side AI training data by RAN.

Observation 4: RAN is hard to control the transmission priority for large scale of UE side AI training data transmission, if the solution2 over CP is adopted.

And even RAN has already concluded that transfer of data for Solution 2 over CP can meet the visibility and controllability RAN requirements, the applicability of solution2 over CP should be studied by SA2.

Proposal 3: The applicability of solution2 over CP to transfer large scale of UE side AI training data should be studied by SA2.

Solution 3 CP

To transmit large AI training data over CP, we think the TR about segmentation of UE capability information could be considered as background:

There are two options to split and transmit large data in Uu control plane based on study on optimizations of UE radio capability signaling [3]:

6.2.1.3.1 Segmentation at RRC level In the segmentation of UE capabilities in the RRC level, for each segment, a type (e.g. notLastSegment, lastSegment) as well as a segment number is added. The RRC segmentation can be done in different ways as described below. Option 1: "hard" split RRC Segmentation: This approach is similar to ETWS/CMAS in LTE and NR. The total UE radio capability information is encoded as one structure (message or information element) similar to non-segmented case. This is then split into multiple segments in bits. The network can only decode the complete capabilities after assembling all parts, rather than decoding each part. Option 2: "soft" split RRC Segmentation: UE radio capability information is encoded separately in parts and carried by multiple messages or information elements (smaller than 9000 bytes for NR or 8188 bytes for LTE). The network can decode and process every segment and combine the content after all the segments are received.

Considering that it is meaningless to decode each segment separately of the UE capability information, the UL UE capability transmission uses the “hard” split option similar as ETWS/CMAS at last. And network can only decode the complete capabilities after assembling all parts.

For AI training data, the differences from the UE capability information report are:

  - AI training data does not need to create packages based on NW request requirement, and it can directly slice content from UE memory, which does not need large RRC processing delay;

  - UE AS layer collects measurement data from PHY layer based on samples. Therefore, each sample entry may be small and can be stored in cleartext which is more like MDT measurement result.

As in reply LS from SA5[2], the feasibility for the functionality of “Server for data collection for UE-side model training” to be carried by TCE as defined in SA5 MDT mechanism or MCE as defined in SA5 QoE mechanism for CP tunnel can be studied in SA5. To discuss Solution 3 over CP on air-interface, we take reusing MDT mechanism as an example:

- The MDT report is in cleartext on the air-interface. gNB packages the received MDT report as a file and sent it to the OAM through the IP packet. The transmission mode on the cable network is not critical, how to organize the data transmission on the air-interface is more important;

- At least for BM use case, the AI training data is collected from the PHY layer periodically which is visible for RRC layer. If MDT mechanism is reused, the AI data transmitted on the air-interface can be cleartext.

Observation 5: At least for BM use case, the AI training data is collected from the PHY layer periodically and is visible to RRC layer which can be transmitted on the air-interface in cleartext.

Based on the analysis above, The AI training data collection solution over CP is more suitable to use “soft” split option which is similar to the logged MDT report. Each segment can be decoded independently, and then packet loss has no significant impact in the limit case.

For the CP solution scalability issue concerned by some companies, if  “soft” split option is used, the UE could send the AI training data segments one by one until all the stored data is transmitted successfully, each segment should include as many samples as possible, not more than the PDCP SDU limit size. The last segment should be marked with a 1 bit indicator similar as the logged MDT report. Based on this mechanism, the scalability does not need to be considered, since each segment could be decode individually.

Observation 6: To use a “soft” split RRC segmentation method similar as logged MDT report, the AI training data transmission does not need to consider the scalability.

Proposal 4: To use a “soft” split RRC segmentation method similar as logged MDT report for solution3 over CP to avoid the scalability problem.

For NW side data collection, it is agreed that:

Agreements on NW side data collection

- Low priority SRB will be used.    FFS new SRB or use of SRB4

For the transmission priority of large scale of UE side AI training data, the same solution used for NW side data collection could be used. It is, to use a low priority SRB, either a new SRB or legacy SRB4.

Proposal 5: The low transmission priority for NW side data collection is also used for UE side data collection, if the solution3 over CP is adopted.

- Conclusion

In this contribution, we provide our views on the UE side data collection, and the observation and proposals are summarized as follows:

For solution2 over UP:

Proposal 1: Data visibility to gNB is not required, and fulfillment of visibility and controllability for data transfer over UP for Solution 2 should be studied by SA2.

For solution3 over UP:

Observation 1: From RAN perspective, both SRB and DRB are limited by the PDCP SDU limit size of 9000bytes, the difference between CP and UP solution is the QoS Setting and the data termination point.

Observation 2: Data transfer over UP for Solution 3 may need to design brand-new protocol stack for UP tunnel which has great RAN and SA impact.

Proposal 2: If there is no significant benefit to use Solution 3 over UP for AI data transfer, we should not consider this solution which may introduce great RAN and SA impact.

For solution2 over CP:

Observation 3: The 9000 bytes PDCP SDU size limit is also applicable for NAS message transmission in the air-interface.

Observation 4: RAN is hard to control the transmission priority for large scale of UE side AI training data transmission, if the solution2 over CP is adopted.

Proposal 3: The applicability of solution2 over CP to transfer large scale of UE side AI training data should be studied by SA2.

For solution3 over CP:

Observation 5: At least for BM use case, the AI training data is collected from the PHY layer periodically and is visible to RRC layer which can be transmitted on the air-interface in cleartext.

Observation 6: To use a “soft” split RRC segmentation method similar as logged MDT report, the AI training data transmission does not need to consider the scalability.

Proposal 4: To use a “soft” split RRC segmentation method similar as logged MDT report  for solution3 over CP to avoid the scalability problem.

Proposal 5: The low transmission priority for NW side data collection is also used for UE side data collection, if the solution3 over CP is adopted.

- References

- RP-243316 LS on AI/ML UE sided data collection TSG RAN

- S5-247219 Reply LS on AIML Data Collection SA5

- TR37.873 Study on optimizations of UE radio capability signalling; (Release 16)