3GPP TSG-RAN WG2 Meeting #129                                                                   R2-2500246

Athens, Greece, Feb. 17th – 21st, 2025

Source:	CATT

Title:	LP-WUS in RRC_IDLE/INACTIVE

Agenda Item:	8.4.2

Document for:	Discussion and Decision

- Introduction

In this contribution, further considerations are provided on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE, including:

- LP-WUS separated band issues.

- UE_ID based LP-WUS Subgrouping.

- SA2 reply LS.

- Configurations for derivation of serving cell measurement quality from LR.

- Discussion

  - LP-WUS separated band issues

In RAN#106 meeting, Rel-19 LP-WUS separated band issues were discussed and the following way forward was achieved [1]:

- For idle/inactive mode, consider a standardized solution for intra-frequency case (e.g. solution 1, 2, and 3 in RP-242656). For Rel-19, inter-frequency solutions (e.g. solution 4) will not be considered. - Detailed discussions and final solution selection is left up to WGs.

- Solution 1: MR is prioritized (via SIBs) to camp on the same band as LR (System information, Paging, RACH on LR band) [2]

With solution 1, RAN2 need to discuss:

- How to prioritize LP-WUS capable UEs to camp on a band supporting LP-WUS.

For example: The network may provide a band/frequency list which supports LP-WUS via SIB. LP-WUS capable UEs consider bands/frequencies supporting LP-WUS with higher priority or the highest priority during cell reselection. Or the network may provide a new frequency priority list which is specific for LP-WUS capable UEs. LP-WUS capable UEs use the specific frequency priority list during cell reselection.

With solution 1, only LP-WUS capable UEs are prioritized to camp on bands/frequencies supporting LP-WUS. Considering the number of LP-WUS capable UEs will not be particularly high, the load on RACH resources on cells supporting LP-WUS will not be particularly heavy.

Observation 1: With solution 1, RAN2 need to discuss how to prioritize LP-WUS capable UEs to camp on a band supported LP-WUS. And the load on RACH resources on cells supporting LP-WUS is not particularly heavy considering the number of LP-WUS capable UEs is not particularly high.

- Solution 2: MR is prioritized to camp on the same band as LR (System information, Paging on LR band; and RACH on another MR band based on reselection criteria) [2]

With solution 2, RAN2 need to discuss the following issues:

- How to prioritize LP-WUS capable UEs to camp on a LR band cell, meanwhile how to avoid LP-WUS capable UEs camping on MR band cells but performing RACH on MR band cells.

Same as solution 1, how to prioritize a UE to camp on a LR band cell still needs to be considered to increase the usage of LP-WUS. As RACH selection is based on reselection criteria while cell reselection is based on reselection criteria as well, how to avoid LP-WUS capable UEs camping on MR band cells but performing RACH on MR band cells need to be considered. For example: During cell reselection procedure, MR band cells are considered as lower priority. But during RACH selection MR band cells are considered as higher or equal priority.

- How the UE obtains RACH configuration of another MR band.

The following solutions can be considered. E.g., the UE obtains RACH configuration of the MR band from camped cell if camped cell can provide related configurations via SI. Or the UE can obtain RACH configuration of the MR band after the UE obtains MIB and SIB1 from the MR band cell which is selected for RACH. But this introduces additional access delay.

- Whether/how Access barring check related procedure is performed if RACH on another MR band is performed.

Currently the UE performs access barring check for each access attempt based on access control parameters in SIB1. Firstly, RAN2 need discuss if the access control parameters in SIB1 of camped cell are applied to the access attempt on the MR band or the access control parameters of the MR band cell are applied to the access attempt on the MR band. If the access control parameters of the MR band cell are applied to the access attempt on the MR band, how the UE obtains related parameters of the MR band needs to be considered, e.g. from camped cell or from SIB1 of the MR band cell.

- How to handle measurement on MR bands which can provide RACH resources:

According to existing agreements on LP-WUS, the UE can stop MR serving cell measurement and MR neighboring cell measurement in fully offloading case. However, the UE may perform access attempt in fully offloading case. In order to assist RACH selection on another MR band and reduce the access delay, measurements on MR bands (neighboring cells) which can provide access attempt cannot be stopped in fully offloading case. Special handling of measurement on MR bands which provide access attempt needs to be considered.

With solution 2, as access attempts on another MR band are allowed, there is no overload issue on RACH resources. But as analyzed above, there are many issues need to be discussed in RAN2.

Observation 2: With solution 2, there is no overload issue on RACH resources but there are many issues need to be discussed in RAN2, including:

- How to prioritize LP-WUS capable UEs to camp on a LR band cell, meanwhile how to avoid LP-WUS capable UEs camping on MR band cells but performing RACH on MR band cells.

- How the UE obtains RACH configuration of another MR band.

- Whether/how Access barring check related procedure is performed if RACH on another MR band is performed.

- How to handle measurement on MR bands which can provide RACH resources.

- Solution 3: MR is not prioritized to camp on the same band as LR but can be redirected via dedicated signalling depending on LR band load (up to gNB implementation) [2]

If the load on RACH of LR band is not heavy, the gNB can redirect LP-WUS capable UEs to the bands supporting LP-WUS via the existing field redirectedCarrierInfo in RRCRelease message. Moreover, with suitable frequency priority configurations, e.g. equal priority between LP-WUS bands and other bands, or dedicated priorities for cell reselection for LP-WUS bands are configured via the existing field cellReselectionPriorities in RRCRelease message, LP-WUS capable UEs can camp on LP-WUS bands as much as possible. And if the network is aware that the load on RACH of LR band is heavy, the gNB can stop to redirect/prioritize LP-WUS capable UEs to LP-WUS bands. Hence, solution 3 can be achieved via gNB implementation without overload issue on LP-WUS bands.

Observation 3: Solution 3 can be achieved via gNB implementation without overload issue on RACH resources on LP-WUS bands.

Table 1 Comparison of three solutions

|  | RAN2 impact | Overload issue |
| --- | --- | --- |
| Solution 1 | RAN2 need to discuss how to prioritize LP-WUS capable UEs to camp on a band supported LP-WUS. | The load on RACH resources on cells supporting LP-WUS is not heavy. |
| Solution 2 | Many issues need to be discussed in RAN2, including: - How to prioritize LP-WUS capable UEs to camp on a LR band cell, meanwhile how to avoid LP-WUS capable UEs camping on MR band cells but performing RACH on MR band cells. - How the UE obtains RACH configuration of another MR band. - Whether/how Access barring check related procedure is performed if RACH on another MR band is performed. - How to handle measurement on MR bands which can provide RACH resources. | No |
| Solution 3 | Via gNB implementation. No RAN2 impact. | No. gNB can stop to redirect/prioritize LP-WUS capable UEs to LP-WUS bands via dedicated signaling if needed. |

With above observations, solution 1 and solution 3 are acceptable considering the complexity and the possible overload issue on LP-WUS bands. And we slightly prefer solution 3 considering the possible RAN2 impact.

Proposal 1: RAN2 consider solution 1 or solution 3 for LP-WUS separated band issues:

- Solution 1: MR is prioritized (via SIBs) to camp on the same band as LR (System information, Paging, RACH on LR band).

- Solution 3 (slight preference): MR is not prioritized to camp on the same band as LR but can be redirected via dedicated signalling depending on LR band load (up to gNB implementation).

  - UE_ID based LP-WUS Subgrouping

The following is the formula for UE_ID based PEI subgrouping in TS 38.304 [3]:

SubgroupID = (floor(UE_ID/(N*Ns)) mod subgroupsNumForUEID) + (subgroupsNumPerPO - subgroupsNumForUEID), where: N: number of total paging frames in T, which is the DRX cycle of RRC_IDLE state as specified in clause 7.1 Ns: number of paging occasions for a PF UE_ID: 5G-S-TMSI mod X, where X is 32768, if eDRX is applied; otherwise, X is 8192 subgroupsNumForUEID: number of subgroups for UE_ID based subgrouping in a PO, which is broadcasted in system information

    - Formula of UE ID based LP-WUS subgrouping

In RAN2#128 meeting, the following options for UE_ID based LP-WUS subgrouping were considered.

FFS on the following options - - Option 1: The subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping. - - Option 2 The subgrouping number for UE_ID LPWUS subgrouping includes an offset K which is configurable or fixed. - - Option 3: The formula for UE_ID based PEI subgrouping is reused.

Option 1: The subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping.

For example, SubgroupID_LP = (floor (UE_ID/(N*Ns*Npei)) mod subgroupsNumForUEID_LP) + (subgroupsNumPerPO_LP – subgroupsNumForUEID_LP), where Npei is equal to the subgrouping number for UE_ID based PEI subgrouping.

Option 2: The subgrouping number for UE_ID LPWUS subgrouping includes an offset K which is configurable or fixed.

For example, SubgroupID_LP = (floor (UE_ID/(N*Ns*K)) mod subgroupsNumForUEID_LP) + (subgroupsNumPerPO_LP – subgroupsNumForUEID_LP), where K is configurable or fixed.

Option 3: The formula for UE_ID based PEI subgrouping is reused.

For example, SubgroupID_LP = (floor (UE_ID/(N*Ns)) mod subgroupsNumForUEID_LP) + (subgroupsNumPerPO_LP – subgroupsNumForUEID_LP)

In Option 1, bits within UE_ID for UE_ID based PEI subgrouping are excluded when the UE_ID LP-WUS subgrouping is determined. As different bits within UE_ID are used for UE_ID subgrouping of PEI and LP-WUS, UEs belong to the same LP-WUS subgrouping can be assigned to different PEI subgrouping as much as possible.

In Option 3, if the maximum number for UE_ID based LP-WUS subgrouping is not equal to that for UE_ID based PEI subgrouping, UEs belong to the same ID of UE_ID based LP-WUS subgrouping may be assigned to different IDs for UE_ID based PEI subgrouping. But we should avoid subgroupsNumForUEID_LP (i.e. the maximum number of subgroups for UE_ID LP-WUS subgrouping) is an integer multiple of subgroupsNumForUEID (i.e. the maximum number of subgroups for UE_ID based PEI subgrouping), otherwise UEs belong to the same UE_ID LP-WUS subgrouping are assigned to the same UE_ID based PEI subgrouping. Considering the formula in Option 3 is similar as that for UE_ID based PEI subgrouping, the rate that UEs belong to the same LP-WUS subgrouping are assigned to different PEI subgrouping with Option 3 is lower than that using Option 1.

In option 2, K is configurable or fixed. If K=1, it is option 3. If K= the subgrouping number for UE_ID based PEI subgrouping, it is option 1. Therefore, with option 2, UEs belong to the same LP-WUS subgrouping can be assigned to different PEI subgrouping as much as possible with suitable configurations.

Option 2 is a compromised solution between Option 1 and Option 3. If the network wants to have separate bits within UE_ID for calculation of UE_ID based PEI subgrouping and UE_ID based LP-WUS subgrouping, it can configure K to the subgrouping number for UE_ID based PEI subgrouping. If the network thinks it is acceptable to have some overlapping bits within UE_ID for calculation of UE_ID based PEI subgrouping and UE_ID based LP-WUS subgrouping, e.g. the maximum number of subgroups for UE_ID LP-WUS subgrouping is 32 while the maximum number of subgroups for UE_ID based PEI subgrouping is 3, it can configure K to 1. Hence, we propose:

Proposal 2: The subgrouping number for UE_ID LP-WUS subgrouping includes an offset K which is configurable.

SubgroupID_LP = (floor (UE_ID/(N*Ns*K)) mod subgroupsNumForUEID_LP) + (subgroupsNumPerPO_LP – subgroupsNumForUEID_LP),

where:

- N: number of total paging frames in T,

- Ns: number of paging occasions for a PF,

- K: configurable integer, FFS the maximum value.

- UE_ID: 5G-S-TMSI mod X, X is FFS.

- subgroupsNumForUEID_LP: number of subgroups for UE_ID based LP-WUS subgrouping in a PO,

- subgroupsNumPerPO_LP: total number of subgroups per PO for LP-WUS.

    - Impact of UE ID

In RAN1#118bis meeting, the following work assumption was made. With this assumption, up to 5 bit within UE_ID will be used for UE_ID based LP-WUS subgrouping.

Working Assumption The maximum number of subgroups per PO supported in Rel-19 is 32.

If proposal 2 is adopted, the maximum number of K is 8 considering that the maximum number of subgroups for UE_ID based PEI subgrouping is 8. For DRX, the maximum number of Ns is 4 while the maximum number of N is 256. Then for DRX with LP-WUS, UE_ID in the formula of UE_ID based LP-WUS subgrouping should be at least 18bits with proposal 2, as shown in Figure 1. Otherwise, some values for LP-WUS subgrouping maybe never used.

For eDRX, the maximum number of Ns is 4 while the maximum number of N is 1024. Then for eDRX with LP-WUS, UE_ID in the formula of UE_ID based LP-WUS subgrouping should be at least 20bits, as shown in Figure 1.

Figure 1 Example of bits within UE_ID for UE_ID based LP-WUS subgrouping

Corresponding, for UE-ID based LP-WUS subgrouping, UE_ID = 5G-S-TMSI mod X, where X is 218 for DRX case and 220 for eDRX case if supported.

Observation 5: If the maximum value of K is 8, X for UE_ID needs to be 218 for DRX case and 220 for eDRX case if supported.

The size of 5G-S-TMSI is 48bit. Hence, there is no issue for the UE to calculate UE-ID based LP-WUS subgrouping with observation 5. However, for RAN paging, RAN node is provided with an extended IE of 16 LSBs of  5G-S-TMSI, not the entire 5G-S-TMSI, as shown below [4].

#### 9.3.3.52 Extended UE Identity Index Value This IE is used by the NG-RAN node to calculate the Paging Frame and Paging Occasion as specified in TS 36.304 [29], the Paging Frame and Paging Occasion for eDRX and the UE_ID based subgroup ID as specified in TS 38.304 [12]. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Extended UE Identity Index Value; M; BIT STRING (SIZE(16)).

With observation 5, the UE-ID in RAN node for RAN-originated paging, is not enough for UE-ID based LP-WUS subgrouping. There will be misalignment between the results of UE-ID based LP-WUS subgrouping in UE side and RAN node side for RAN-originated paging. Hence, the extended IE for RAN-originated paging in RAN node needs to be extended.

Observation 6: If the maximum value of K is 8, the Extended UE Identity Index Value for RAN-originated paging in RAN node needs to be extended to at least 18bits for DRX case and at least 20bits if eDRX with LP-WUS is supported.

RAN2 need to send LS to SA2/RAN3/CT1 to ask for the extended bits for extended IE for RAN-originated paging in RAN node with observation 6.

Proposal 3: Send LS to SA2/RAN3/CT1 to ask whether the bits for Extended UE Identity Index Value for RAN paging-originated can be extended, at least 18bits for DRX case and at least 20bits if eDRX with LP-WUS is supported.

  - SA2 reply LS

SA2 replied the LS on LP-WUS subgrouping [5]. In the LS, SA2 asked RAN2/3 on the terminology/name and lost issue of UE Radio Capability for Paging Information. We would like to discuss these aspects.

    - Terminology/name

In the LS [5], SA2 asked:

1) SA2 noted that the name “LP-WUS” can be mis-understood, as SA2 understands that the Wake Up Signal is transmitted with high power to enable a large coverage area, and low power refers to a low power wake up receiver used in the mobile. SA2 invites feedback on appropriate terminology and naming of the “LP-WUS” feature. SA2 will then align the terminology and name, based on the feedback.

It is true that low power refers to a low power wake up receiver used in mobile, as mentioned below in TR 38.869[6].

Currently, UEs need to periodically wake up once per DRX cycle, which dominates the power consumption in periods with no signalling or data traffic. If UEs are able to wake up only when they are triggered, e.g., paging, power consumption could be dramatically reduced. This can be achieved by using a wake-up signal to trigger the main radio and a separate receiver which has the ability to monitor wake-up signal with ultra-low power consumption. Main radio works for data transmission and reception, which can be turned off or set to deep sleep unless it is turned on.

We prefer to keep the terminology/name “LP-WUS” considering it has already been used during SI and WI. And if needed, we can update a general description in stage 2 to highlight low power refers to a low power wake up receiver in UE.

Proposal 4: The terminology/name “LP-WUS” is kept, and if needed a general description in stage 2 is introduced to highlight low power refers to a low power wake up receiver in UE.

    - Issues of UE Radio Capability for Paging Information

- LP-WUS feature

SA2 mentioned it will be important that LP-WUS related aspects of UE Radio Capability for Paging Information are not lost when Initial Registration is performed on a pre-Rel-19 gNB. If the LP-WUS aspects are lost, then there can be paging problems, when for example, paging is performed by a gNB that supports LP-WUS.

RAN2 have not discuss how to introduce LP-WUS related aspects of UE Radio Capability for Paging Information yet. Let’s assume the mechanism for PEI related aspects of UE Radio Capability for Paging Information is reused for LP-WUS. As shown below, LP-WUS related aspects of UE Radio Capability for Paging Information will be extended in the IE UE-RadioPagingInfo. And the IE UE-RadioPagingInfo is transferred as a container within the UERadioPagingInformation message as UE Radio Capability for Paging Information between gNB and 5GC. The IE UE-RadioPagingInfo is introduced in R17. As it is transferred as a container within the UERadioPagingInformation message, extended field within the IE UE-RadioPagingInfo will be not lost for CN paging when Initial Registration is performed on R17+ gNB. LP-WUS related aspects of UE Radio Capability for Paging Information for CN paging are not lost in this case. But when Initial Registration is performed on pre-R17 gNB, the whole IE UE-RadioPagingInfo is absent in the UERadioPagingInformation message when it is sent from gNB to 5GC. LP-WUS related aspects of UE Radio Capability for Paging Information for CN paging are lost in this case. Then there can be paging problems, e.g. legacy paging is performed by a R19 gNB that supports LP-WUS while UE monitors LP-WUS in the gNB.

Observation 7: If LP-WUS related aspects of UE Radio Capability for Paging Information are extended in the IE UE-RadioPagingInfo, LP-WUS related aspects of UE Radio Capability for Paging Information for CN paging are not lost when Initial Registration is performed on R17+ gNB.

Observation 8: If LP-WUS related aspects of UE Radio Capability for Paging Information are extended in the IE UE-RadioPagingInfo, LP-WUS related aspects of UE Radio Capability for Paging Information for CN paging are lost when Initial Registration is performed on pre-R17 gNB. Then there may be paging problems.

- UE-RadioPagingInfo The IE UE-RadioPagingInfo contains UE capability information needed for paging. UE-RadioPagingInfo information element -- ASN1START -- TAG-UE-RADIOPAGINGINFO-START UE-RadioPagingInfo-r17 ::= SEQUENCE { -- R1 29-1: Paging enhancement pei-SubgroupingSupportBandList-r17 SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR OPTIONAL, ... } -- TAG-UE-RADIOPAGINGINFO-STOP -- ASN1STOP #### – UERadioPagingInformation This message is used to transfer radio paging information, covering both upload to and download from the 5GC, and between gNBs. Direction: gNB to/ from 5GC and gNB to/from gNB // skip UERadioPagingInformation-v15e0-IEs ::= SEQUENCE { dl-SchedulingOffset-PDSCH-TypeA-FDD-FR1 ENUMERATED {supported} OPTIONAL, dl-SchedulingOffset-PDSCH-TypeA-TDD-FR1 ENUMERATED {supported} OPTIONAL, dl-SchedulingOffset-PDSCH-TypeA-TDD-FR2 ENUMERATED {supported} OPTIONAL, dl-SchedulingOffset-PDSCH-TypeB-FDD-FR1 ENUMERATED {supported} OPTIONAL, dl-SchedulingOffset-PDSCH-TypeB-TDD-FR1 ENUMERATED {supported} OPTIONAL, dl-SchedulingOffset-PDSCH-TypeB-TDD-FR2 ENUMERATED {supported} OPTIONAL, nonCriticalExtension UERadioPagingInformation-v1700-IEs OPTIONAL } UERadioPagingInformation-v1700-IEs ::= SEQUENCE { ue-RadioPagingInfo-r17 OCTET STRING (CONTAINING UE-RadioPagingInfo-r17) OPTIONAL, inactiveStatePO-Determination-r17 ENUMERATED {supported} OPTIONAL, numberOfRxRedCap-r17 ENUMERATED {one, two} OPTIONAL, halfDuplexFDD-TypeA-RedCap-r17 SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR OPTIONAL, nonCriticalExtension UERadioPagingInformation-v1800-IEs OPTIONAL } UERadioPagingInformation-v1800-IEs ::= SEQUENCE { numberOfRxERedCap-r18 ENUMERATED {one, two} OPTIONAL, supportOf2RxXR-r18 ENUMERATED {supported} OPTIONAL, nonCriticalExtension UERadioPagingInformation-v1840-IEs OPTIONAL } UERadioPagingInformation-v1840-IEs ::= SEQUENCE { dl-SchedulingOffset-PDSCH-TypeA-FDD-FR2-NTN-r18 ENUMERATED {supported} OPTIONAL, dl-SchedulingOffset-PDSCH-TypeB-FDD-FR2-NTN-r18 ENUMERATED {supported} OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL }

The similar issue exists for RAN paging. The UERadioPagingInformation message, which includes UE Radio Capability for Paging Information, may be transferred between gNBs for RAN paging. The IE UE-RadioPagingInfo is introduced in R17. When the UE is released into RRC_INACTIVE state on R17+ gNB, the IE UE-RadioPagingInfo can be included in the UERadioPagingInformation message in the RAN PAGING message. LP-WUS related aspects of UE Radio Capability for Paging Information for RAN paging can be transferred to the target gNB. However, when the UE is released into RRC_INACTIVE state on pre-R17 gNB, the IE UE-RadioPagingInfo cannot be included in the RAN PAGING message. LP-WUS related aspects of UE Radio Capability for Paging Information for RAN paging are lost in this case. Then there can be paging problems, e.g. legacy RAN paging is performed by a R19 gNB that supports LP-WUS while UE monitors LP-WUS in the gNB.

Observation 9: If LP-WUS related aspects of UE Radio Capability for Paging Information are extended in the IE UE-RadioPagingInfo, LP-WUS related aspects of UE Radio Capability for Paging Information for RAN paging are not lost when the UE is released into RRC_INACTIVE state on R17+ gNB.

Observation 10: If LP-WUS related aspects of UE Radio Capability for Paging Information are extended in the IE UE-RadioPagingInfo, LP-WUS related aspects of UE Radio Capability for Paging Information for RAN paging are lost when the UE is released into RRC_INACTIVE state on pre-R17 gNB. Then there may be paging problems.

- Other features in the UE radio capability for Paging Information

As shown above, there are other features extended within the UERadioPagingInformation message since R17, including:

- PEI related parameters;

- inactiveStatePO-Determination;

- Redcap/eRedCap related parameters, i.e. numberOfRxRedCap-r17, halfDuplexFDD-TypeA-RedCap-r17 and numberOfRxERedCap-r18;

- 2RxXR, i.e. supportOf2RxXR-r18;

- NTN related parameters, i.e. dl-SchedulingOffset-PDSCH-TypeA-FDD-FR2-NTN-r18 and dl-SchedulingOffset-PDSCH-TypeB-FDD-FR2-NTN-r18.

PEI

Similar as LP-WUS, PEI rated aspects of UE radio capability for Paging Information are lost for CN paging when Initial Registration is performed on pre-R17 gNB and for RAN paging the UE is released into RRC_INACTIVE state on pre-R17 gNB. Then there can be paging problems, e.g. legacy paging is performed by a R17 gNB that supports PEI while UE monitors PEI in the gNB.

Observation 11: PEI related aspects of UE Radio Capability for Paging Information are lost for CN paging when Initial Registration is performed on pre-R17 gNB and for RAN paging when the UE is released into RRC_INACTIVE state on pre-R17 gNB. Then there may be paging problems.

inactiveStatePO-Determination

Similarly, inactiveStatePO-Determination is lost for RAN paging the UE is released into RRC_INACTIVE state on pre-R17 gNB. Then there can be paging problems, e.g. a R17 gNB that broadcasts ranPagingInIdlePO with value "true" determines the i_s based on parameters of RRC_INACTIVE state while the RRC_INACTIVE UE in the gNB uses the same i_s as for RRC_IDLE state.

Observation 12: inactiveStatePO-Determination is lost for RAN paging the UE is released into RRC_INACTIVE state on pre-R17 gNB. Then there may be paging problems.

RedCap/eRedCap

Different from LP-WUS features, UEs support Redcap and/or eRedcap features are not allowed to access legacy gNBs, e.g, field intraFreqReselectionRedCap can be used to determine whether the cell can be considered as barred for a RedCap UE. Therefore, for UEs support Redcap and/or eRedcap features, Initial Registration is always performed on R17+ gNB and it is always released into RRC_INACTIVE state on R17+ gNB. (e)Redcap Related aspects of UE radio capability for Paging Information for RAN paging are not lost.

Observation 13: (e)Redcap Related aspects of UE radio capability for Paging Information for RAN paging are not lost as UEs support Redcap and/or eRedcap features are not allowed to access legacy gNBs.

2RxXR

supportOf2RxXR was introduced in R18 to indicate whether the UE is a 2Rx XR UE. UEs support 2RxXR are allowed to access legacy gNBs. Then 2RxXR related aspects of UE radio capability for Paging Information, i.e. supportOf2RxXR is lost for CN paging when Initial Registration is performed on pre-R18 gNB and for RAN paging the UE is released into RRC_INACTIVE state on pre-R18 gNB. But supportOf2RxXR is used for the network to avoid sending a paging message to the cell which the UE cannot camp on. If the IE is lost, the gNB may send redundant paging message to the 2RxXR UE even if the UE cannot camp on. But there is no paging problem.

Observation 14: supportOf2RxXR is lost for CN paging when Initial Registration is performed on pre-R18 gNB and for RAN paging when the UE is released into RRC_INACTIVE state on pre-R18 gNB. But there is no paging problem.

NTN

In R18, indicators to indicate whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type A/B in FDD FR2-NTN are introduced within the UERadioPagingInformation message. These indicators are lost for CN paging when Initial Registration is performed on pre-R18 gNB and for RAN paging the UE is released into RRC_INACTIVE state on pre-R18 gNB. Similar as indicators to indicate whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type A/B for Terrestrial Network, if these new capabilities for NTN are lost for CN paging or RAN paging in the network side, the network can paging the UE with K0=0. There is no paging problem.

Observation 15: NTN related aspects of UE Radio Capability for Paging Information are lost for CN paging when Initial Registration is performed on pre-R18 gNB and for RAN paging when the UE is released into RRC_INACTIVE state on pre-R18 gNB. But there is no paging problem.

We propose RAN2 confirm these observations (Observation 7- Observation 15) and consider a unified solution for these impacted features for both CN paging and RAN paging.

Proposal 5: RAN2 confirm these observations (Observation 7- Observation 15) and consider a unified solution for these impacted features for both CN paging and RAN paging.

  - Configurations for derivation of serving cell measurement quality from LR

In RAN2#126 meeting, it was agreed:

- The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - - LP-SS configuration - - LP-WUS configuration - - Entry/exit condition for LP-WUS monitoring (FFS if it always configured)

According to the agreements for entry/exit condition for LP-WUS monitoring, serving cell quality from LR needs to be defined. And multi-beam operation is supported for LP-WUR. Therefore, RAN2 needs to discuss how to derive serving cell measurement quantity from LR.

For cell reselection, how to derive a cell measurement quantity of MR in multi-beam operations is captured in TS 38.304 [3] as follows:

For cell selection in multi-beam operations, measurement quantity of a cell is up to UE implementation. For cell reselection in multi-beam operations, including inter-RAT reselection from E-UTRA to NR, the measurement quantity of this cell is derived amongst the beams corresponding to the same cell based on SS/PBCH block as follows: - if nrofSS-BlocksToAverage (maxRS-IndexCellQual in E-UTRA) is not configured in SIB2/SIB4 (SIB24 in E-UTRA); or - if absThreshSS-BlocksConsolidation (threshRS-Index in E-UTRA) is not configured in SIB2/SIB4 (SIB24 in E-UTRA); or - if the highest beam measurement quantity value is below or equal to absThreshSS-BlocksConsolidation (threshRS-Index in E-UTRA): - derive a cell measurement quantity as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [11]. - else: - derive a cell measurement quantity as the linear average of the power values of up to nrofSS-BlocksToAverage (maxRS-IndexCellQual in E-UTRA) of highest beam measurement quantity values above absThreshSS-BlocksConsolidation (threshRS-Index in E-UTRA).

The derivation of cell measurement quantity from MR with multi-beam operation can be reused for LR, e.g. derivation of cell measurement quality from LR as the linear average of the power values of up to N highest beam measurement quantity values above a threshold. And similar to derivation of cell measurement quantity from MR, parameters for derivation of serving cell from LR, e.g. the threshold for beams used for derivation of cell measurement quantity and the number of beams (e.g. N ) used for derivation of cell measurement quantity, need to be included in SIB for LP-WUS monitoring in RRC_IDLE/INACTIVE.

Proposal 6: Cell measurement quality from LR is derived from the linear average of the power values of up to N highest beam measurement quantity values above a threshold. Parameters for derivation of serving cell from LR in SIB for LP-WUS monitoring in RRC_IDLE/INACTIVE, including:

- the threshold for beams used for derivation of cell measurement quantity;

- N, the number of beams to average for cell measurement derivation.

- Conclusion

According to the analysis in section 2, we propose:

LP-WUS separated band issues:

Observation 1: With solution 1, RAN2 need to discuss how to prioritize LP-WUS capable UEs to camp on a band supported LP-WUS. And the load on RACH resources on cells supporting LP-WUS is not particularly heavy considering the number of LP-WUS capable UEs is not particularly high.

Observation 2: With solution 2, there is no overload issue on RACH resources but there are many issues need to be discussed in RAN2, including:

- How to prioritize LP-WUS capable UEs to camp on a LR band cell, meanwhile how to avoid LP-WUS capable UEs camping on MR band cells but performing RACH on MR band cells.

- How the UE obtains RACH configuration of another MR band.

- Whether/how Access barring check related procedure is performed if RACH on another MR band is performed.

- How to handle measurement on MR bands which can provide RACH resources.

Observation 3: Solution 3 can be achieved via gNB implementation without overload issue on RACH resources on LP-WUS bands.

Proposal 1: RAN2 consider solution 1 or solution 3 for LP-WUS separated band issues:

- Solution 1: MR is prioritized (via SIBs) to camp on the same band as LR (System information, Paging, RACH on LR band).

- Solution 3 (slight preference): MR is not prioritized to camp on the same band as LR but can be redirected via dedicated signalling depending on LR band load (up to gNB implementation).

UE_ID based LP-WUS Subgrouping:

Proposal 2: The subgrouping number for UE_ID LP-WUS subgrouping includes an offset K which is configurable.

SubgroupID_LP = (floor (UE_ID/(N*Ns*K)) mod subgroupsNumForUEID_LP) + (subgroupsNumPerPO_LP – subgroupsNumForUEID_LP),

where:

- N: number of total paging frames in T,

- Ns: number of paging occasions for a PF,

- K: configurable integer, FFS the maximum value.

- UE_ID: 5G-S-TMSI mod X, X is FFS.

- subgroupsNumForUEID_LP: number of subgroups for UE_ID based LP-WUS subgrouping in a PO,

- subgroupsNumPerPO_LP: total number of subgroups per PO for LP-WUS.

Observation 5: If the maximum value of K is 8, X for UE_ID needs to be 218 for DRX case and 220 for eDRX case if supported.

Observation 6: If the maximum value of K is 8, the Extended UE Identity Index Value for RAN-originated paging in RAN node needs to be extended to at least 18bits for DRX case and at least 20bits if eDRX with LP-WUS is supported.

Proposal 3: Send LS to SA2/RAN3/CT1 to ask whether the bits for Extended UE Identity Index Value for RAN paging-originated can be extended, at least 18bits for DRX case and at least 20bits if eDRX with LP-WUS is supported.

SA2 reply LS:

Proposal 4: The terminology/name “LP-WUS” is kept, and if needed a general description in stage 2 is introduced to highlight low power refers to a low power wake up receiver in UE.

LP-WUS

Observation 7: If LP-WUS related aspects of UE Radio Capability for Paging Information are extended in the IE UE-RadioPagingInfo, LP-WUS related aspects of UE Radio Capability for Paging Information for CN paging are not lost when Initial Registration is performed on R17+ gNB.

Observation 8: If LP-WUS related aspects of UE Radio Capability for Paging Information are extended in the IE UE-RadioPagingInfo, LP-WUS related aspects of UE Radio Capability for Paging Information for CN paging are lost when Initial Registration is performed on pre-R17 gNB. Then there may be paging problems.

Observation 9: If LP-WUS related aspects of UE Radio Capability for Paging Information are extended in the IE UE-RadioPagingInfo, LP-WUS related aspects of UE Radio Capability for Paging Information for RAN paging are not lost when the UE is released into RRC_INACTIVE state on R17+ gNB.

Observation 10: If LP-WUS related aspects of UE Radio Capability for Paging Information are extended in the IE UE-RadioPagingInfo, LP-WUS related aspects of UE Radio Capability for Paging Information for RAN paging are lost when the UE is released into RRC_INACTIVE state on pre-R17 gNB. Then there may be paging problems.

PEI

Observation 11: PEI related aspects of UE Radio Capability for Paging Information are lost for CN paging when Initial Registration is performed on pre-R17 gNB and for RAN paging when the UE is released into RRC_INACTIVE state on pre-R17 gNB. Then there may be paging problems.

inactiveStatePO-Determination

Observation 12: inactiveStatePO-Determination is lost for RAN paging the UE is released into RRC_INACTIVE state on pre-R17 gNB. Then there may be paging problems.

RedCap/eRedCap

Observation 13: (e)Redcap Related aspects of UE radio capability for Paging Information for RAN paging are not lost as UEs support Redcap and/or eRedcap features are not allowed to access legacy gNBs.

2RxXR

Observation 14: supportOf2RxXR is lost for CN paging when Initial Registration is performed on pre-R18 gNB and for RAN paging when the UE is released into RRC_INACTIVE state on pre-R18 gNB. But there is no paging problem.

NTN

Observation 15: NTN related aspects of UE Radio Capability for Paging Information are lost for CN paging when Initial Registration is performed on pre-R18 gNB and for RAN paging when the UE is released into RRC_INACTIVE state on pre-R18 gNB. But there is no paging problem.

Proposal 5: RAN2 confirm these observations (Observation 7- Observation 15) and consider a unified solution for these impacted features for both CN paging and RAN paging.

Configurations for derivation of serving cell measurement quality from LR:

Proposal 6: Cell measurement quality from LR is derived from the linear average of the power values of up to N highest beam measurement quantity values above a threshold. Parameters for derivation of serving cell from LR in SIB for LP-WUS monitoring in RRC_IDLE/INACTIVE, including:

- the threshold for beams used for derivation of cell measurement quantity;

- N, the number of beams to average for cell measurement derivation.

- Reference

- RP-243266, Way forward on Rel-19 LP-WUS separate band issues, RAN2 chair (Interdigital),RAN#106.

- RP-242656, On LR and MR operating in different bands in Release 19, Vodafone,Vivo,Huawei,HiSilicon, RAN#106.

- 3GPP TS 38.304 V18.2.0, User Equipment (UE) procedures in Idle mode and RRC Inactive state.

- 3GPP TS 38.413 V18.2.0, NG-RAN, NG Application Protocol (NGAP).

- R2-2500050, LS Reply on LP-WUS subgrouping

- 3GPP TR 38.869 V18.0.0, Study on low-power wake-up signal and receiver for NR.