# 3GPP TSG RAN WG2 Meeting #129      	                        R2-2500261
Athens, Greece, 17th – 21th Feb., 2025

Agenda item:	8.1.3

Source:	Apple

Title:	Remaining issues on NW-sided data collection

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

  - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback

    - Identification related signalling is part of the above objective

  - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models

  - Signalling mechanism of applicable functionalities/models

The revised Rel-19 WID of AI/ML for air interface (WID RP-243244) was agreed in RAN#106 [1]. One of its sub-objectives is to specify data collection for NW-sided model, as highlighted copied below:

NW-sided data collection was discussed from RAN2#126 to RAN2#128. It was agreed to design a unified measurement framework, and immediate MDT was agreed to support L1 measurement logging and reporting with some leftover issues. Meanwhile, there is one post-meeting email discussion on event triggered logging [2]. In this contribution, we will also address both leftover issues in previous RAN2 agreements and remaining issues in [2].

Furthermore, RAN2 didn’t discuss NW-sided data collection of AI/ML based positioning and required data type for both NW-side model and UE-side model, which are included in the guidance of agenda item:

8.1.3 NW side data collection

Contributions should focus on the mechanisms and principles identified for data collection for network side model training during rel-18.   Contributions should discusss type of data required to be collected for NW sided model and UE sided model (common to NW sided and different).  Question to RAN1 should also be identified.

In this contribution, we further discuss NW-sided data collection from below aspects:

- NW-sided data collection for AI/ML based beam management

- NW-sided data collection for AI/ML based positioning

- Collected data type of NW-sided data collection

- Collected data type of UE-sided data collection

# 2 Discussion

## 2.1 NW-sided data collection for AI/ML based beam management

Based on latest agreements in RAN2#128 [3], our understanding on the whole procedure, including OAM centric and gNB centric data collection, is illustrated in Figure .1. We also label FFSs to be discussed in this contribution.

Figure. 1 Overall procedure of OAM centric and gNB centric NW-sided data collection

In the following, we discuss the below remaining issues based on Figure.1.

- Overall procedure

- AS procedure step 1: RRC framework of L1 measurement logging

- AS procedure step 2: How UE logs L1 measurement

- AS procedure step 3: Trigger reporting of logged L1 measurement

- AS procedure step 4: Reporting of logged L1 measurement

### Overall procedure

As illustrated in Figure.1, our understanding is:

- CN procedure (i.e. procedure between RAN and CN/OAM/UDM) is labelled in blue font.

  - We think RAN2 can assume this procedure reuses existing procedure of immediate MDT.

  - One possible exception is that maybe a new server instead of TCE can be introduced to store training data, but it is up to RAN3/SA5 to decide.

- AS procedure (i.e. procedure between RAN and UE) is labelled in orange font.

  - This procedure includes 4 steps as illustrated in Figure. 1 and is common to both OAM centric and gNB centric data collection.

  - Difference between OAM centric and gNB centric data collection is which NW entity (OAM vs gNB) to trigger data collection and generate data collection configuration.

Observation 1: Difference between OAM centric and gNB centric data collection is which NW entity (OAM vs gNB) to trigger data collection and generate data collection configuration. Their AS procedure is same.

We have UE privacy concern on gNB centric data collection because the collected data is generated by the UE which may include UE’s privacy sensitive information (e.g., the UE’s location information). Please note that the UE privacy requirement is already satisfied by OAM centric data collection because user consent for MDT is required before NW configures the UE to perform immediate MDT, according to TS 32.422 [4]:

4.9.1 Signalling based MDT

In case of signalling based MDT getting user consent before activating the MDT functionality is required because of privacy and legal obligations. It is the Operator responsibility to collect user consent before initiating an MDT for a specific IMSI, IMEI number or SUPI.

Collecting the user consent shall be done via customer care process. The user consent information availability should be considered as part of the subscription data and as such this shall be provisioned to the UDM database.

Observation 2: UE privacy requirement is preserved in OAM centric data collection due to UE consent mechanism in MDT, but it is not clear how to satisfy UE privacy requirement in gNB centric data collection.

Since RAN2 agreed to aim the same measurement framework for both gNB-centric data collection and OAM-centric data collection, we think similar UE consent mechanism in MDT should also be supported in gNB-centric data collection, i.e. user consent is required before NW configures the UE to gNB-centric data collection. The detailed signaling can be further discussed.

Proposal 1: To strive for a common measurement framework, similar UE consent mechanism in MDT is supported in gNB-centric data collection, i.e. UE consent is required before NW configures the UE gNB-centric data collection. FFS signaling details.

### Step 1: RRC framework of L1 measurement logging

#### 2.1.2.1 RRC framework (based on L1 CSI framework vs L3 RRM framework)

In legacy immediate MDT and L3 RRM, measurement configuration (i.e. MeasConfig) is included in RRCReconfiguration message. Whether it is immediate MDT or L3 RRM is transparent to the UE. The existing RRC structure is illustrated in Figure.2, where each MeasID links one instance of MeasObjectNR with one instance of ReportConfigNR.

Meanwhile, according to below RAN1#116b agreement [5], RAN1 has agreed to reuse existing CSI framework to configure AI/ML based beam management. The existing RRC structure of CSI framework is illustrated in Figure.3, where each csi-ResourceConfigId links one instance of csi-ResourceConfig with one instance of csi-ReportConfig.

Agreement

For network-sided AI/ML model for BM-Case1 and BM-Case2,

- support using existing CSI framework for configuration of Set A as the starting point

- support using existing CSI framework for configuration of Set B as the starting point

- Note: Purpose, such as above “For NW-sided model, for BM-Case1 and BM-Case2” and “Set A” and “Set B”, will not be specified in RAN 1 specifications

Figure. 2 Illustration of existing RRC structure of L3 RRM and immediate MDT (note: some unrelated IEs, e.g., offsetMO, are omitted)

Figure. 3 Illustration of existing RRC structure of L1 CSI framework (note: some unrelated IEs, e.g., csi-SSB-ResourceSet, are omitted)

Then, we have two alternatives of RRC structure to implement immediate MDT with L1 measurement logging:

- Alt-1: Integrate existing L1 CSI framework into L3 measurement framework

- Alt-2: Extend existing L1 CSI framework with logging and RRC reporting

Although both alternatives can work, we prefer Alt-1 because Alt-2 will bring the following issues:

- Alt-2 will require RAN1 to implement RAN2 agreements in RAN1 specification because the procedure text of L1 CSI framework is specified in Section 5.2.1 of TS 38.214.

- Alt-2 will restrict the use case of the RAN2 designed data collection framework only to AI/ML based beam management, which is not future proof for other AI/ML use cases (e.g. AI/ML based mobility).

- Alt-2 is conflicted with RAN2 agreement to use immediate MDT as baseline.

Observation 3: If we extend L1 CSI framework with logging and RRC reporting, it will bring below issues:

- It will require RAN1 to implement RAN2 agreements in RAN1 specification as procedure text of L1 CSI framework is now specified in Section 5.2.1 of TS 38.214.

- It will restrict the use case of the RAN2 designed data collection framework only to AI/ML based beam management, which is not future proof for other AI/ML use cases (e.g. AI/ML based mobility).

We provide one example of implementing Alt-1 in Figure. 4 with below explanation:

- New L1 CSI resource configuration (e.g. a list of NZP CSI-RS resource set IDs) and logging configuration (e.g. logging interval) are introduced within MeasObjectNR.

  - Note that the new L1 CSI resource configuration only needs to include resource set IDs which point to CSI resource pool configured in CSI-MeasConfig, instead of duplicating NZP CSI-RS resource configuration.

- New on-demand reporting configuration is introduced within ReportConfigNR to indicate at least reporting type (i.e. on-demand) and reporting quantity configuration (e.g. L1 RSPR and/or beam index).

Figure. 4 An example of how to integrate logged L1 measurement into L3 RRM framework (green modules are introduced for logged L1 measurement/reporting)

So, we propose:

Proposal 2: L3 measurement framework is extended to configure logged L1 measurements with below specification changes:

- Within MeasObjectNR: introduce L1 CSI resource configuration (e.g. a list of resource set IDs pointed to CSI-RS resource pool in CSI-MeasConfig) and logging configuration.

- Within ReportConfigNR: introduce on-demand reporting configuration to indicate at least reporting type (i.e. on-demand) and reporting quantity configuration (e.g. L1 RSPR and/or beam index).

Then, we address below two FFSs of RAN2#127 agreements [6]:

Agreements

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

#### 2.1.2.2 FFS#1: if multiple configurations can be provided to the UE

During offline discussion in RAN2#127b, we think that there are two different understandings on what “multiple configurations” means:

- Understanding 1: it means multiple configurations of logged L1 measurements

  - For example, allow multiple MeasConfig in example of Figure 4.

- Understanding 2: it means multiple CSI-RS resource sets in one logged L1 measurement configuration

  - For example, allow multiple L1-ResourceConfig in one MeasConfig in example of Figure 4.

We prefer understanding 2 because we understand the intention is to allow different L1 CSI-RS resource sets follow different measurement / logging configurations (e.g. different L1 CSI resource sets are measured with different periodicities). This is a valid use case because datasets collected from different L1 CSI resource sets may be used to train different AI/ML models by the NW. Furthermore, please note that existing L1 CSI framework already supports this feature. As illustrated in Figure. 3, multiple NZP-CSI-RS-ResourceSet and multiple CSI-ReportConfig can be configured where one NZP-CSI-RS-ResourceSet can be associated with one CSI-ReportConfig via CSI-ResourceConfigId.

Observation 4: Existing L1 CSI framework already supports multiple CSI-RS resource set configurations: multiple NZP-CSI-RS-ResourceSet and multiple CSI-ReportConfig can be configured where one NZP-CSI-RS-ResourceSet can be associated with one CSI-ReportConfig via CSI-ResourceConfigId.

We think it is straight forward to reuse this signaling framework. As illustrated in Figure. 4, under MeasObjectNR. multiple L1 CSI resource sets and multiple logging configuration can be configured, where one L1 CSI resource set is associated with one logging configuration via CSI-ResourceConfigId.

Proposal 3a: Do not support multiple configurations of logged L1 measurements (e.g. multiple MeasConfig).

Proposal 3b: Similar to existing L1 CSI framework, multiple L1 CSI resource sets can be configured to be linked with different logging configurations (e.g. with different logging periodicities) in MeasObjectNR.

#### 2.1.2.3 FFS#2: if dynamic activation/deactivation is support

In RAN2#128 [3], it was agreed not to support measurements on aperiodic CSI resources. However, it was not concluded whether to support measurements on Semi-Persistent (SP) CSI resource.

Agreements on NW side data collection - Measurements on aperiodic CSI resources are not reported for NW sided data collection.

We think it is not reasonable to support measurements on SP CSI resources. Enhanced MDT is intended to support data collection for model training which doesn’t have latency requirement. If NW needs the UE to perform data collection only for one particular time duration, it can just reconfigure / de-configurate the data collection via RRC rather than via SP CSI resources.

Observation 5: If NW needs the UE to perform data collection only for one particular time duration, it can just reconfigure / de-configurate the data collection via RRC rather than via SP CSI resources.

Thus, we propose not to support dynamic activation / deactivation of logged L1 measurements on SP CSI resources because it is sufficient to rely on RRC reconfiguration / de-configuration of periodic CSI resources.

Proposal 4: As target for offline training without latency requirement, no need to support dynamic activation/deactivation of logged L1 measurements on SP CSI resources (i.e. it is sufficient to rely on RRC reconfiguration / de-configuration of periodic CSI resources).

### Step 2: How UE logs L1 measurement

#### 2.1.3.1 FFS#1: Other measurement to log

In RAN2#127b [7], it was agreed to at least log L1-RSRP and/or beam ID in NW-sided model training, and we have an FFS on whether other data to be collected.

Agreements on NW side data collection

- For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress.

We think it is important to allow the NW to know when each sample of reported L1 measurement is measured. Thus, the UE can include the time stamp when the L1 measurement (e.g. L1-RSRP) is logged. Meanwhile, please note that logging time stamp was already supported in logged MDT:

LogMeasReport-r16 ::=                SEQUENCE {

absoluteTimeStamp-r16                AbsoluteTimeInfo-r16,

traceReference-r16                   TraceReference-r16,

traceRecordingSessionRef-r16         OCTET STRING (SIZE (2)),

tce-Id-r16                           OCTET STRING (SIZE (1)),

logMeasInfoList-r16                  LogMeasInfoList-r16,

logMeasAvailable-r16                 ENUMERATED {true}                   OPTIONAL,

logMeasAvailableBT-r16               ENUMERATED {true}                   OPTIONAL,

logMeasAvailableWLAN-r16             ENUMERATED {true}                   OPTIONAL,

...

}

Proposal 5: Besides L1 RSRP and/or beam-IDs, the UE also collects logging time stamp in its AS buffer.

#### 2.1.3.2 FFS#2: FFS on the memory size

We need to determine the minimum AS layer memory size. In legacy NR, logged MDT and QoE measurements (including paused measurements and IDLE/INACTIVE measurements) had specified the same minimum AS layer memory size, which is 64kB according to TS 38.306 [8]:

Logged MDT (From Section 4.2.18 of TS 38.306) loggedMeasurements-r16 Indicates whether the UE supports logged measurements in RRC_IDLE and RRC_INACTIVE state. A UE that supports logged measurements shall support both periodical logging and event-triggered logging. The minimum memory size of MDT logged measurements is 64KB. For eRedCap UE supporting this feature, the minimum memory size of MDT logged measurements is 16KB.; UE; No; No; No. QoE (from Section 6 of TS 38.306) AS layer memory size for QoE paused measurement reports; It is mandatory to support the minimum AS layer memory size of 64KB for QoE paused measurement reports for UEs which support qoe-Streaming-MeasReport-r17, qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17.. AS layer memory size for QoE measurement reports in RRC_IDLE and RRC_INACTIVE; It is mandatory to support the minimum AS layer memory size of 64KB for QoE measurement reports stored in RRC_IDLE/RRC_INACTIVE for UEs which support qoe-IdleInactiveMeasReport-r18 and any of qoe-Streaming-MeasReport-r17 or qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17. This memory size is additional to "AS layer memory size for QoE paused measurement reports".

Observation 6: In legacy NR, logged MDT, QoE measurements in IDLE/INACTIVE state and QoE paused measurements had specified the minimum AS layer memory size, which are all 64KB according to TS 38.306.

Meanwhile, RAN1 Reply LS (R1-2310681) [9] provided typical data size for model training in beam management. According to the highlighted parts, we can estimate that the typical training data size stored in AS layer memory is 100 bits8 instances (Set B) + 500 bits4 instances (Set A) = 2800 bits=0.35KB.

Observation 7: Based on the typical training data size of beam management provided in RAN1 Reply LS (R1-2310681), the typical training data size stored in AS layer memory is 100 bits8 instances (Set B) + 500 bits4 instances (Set A) =0.35KB.

From RAN1 reply LS (R1-2310681) LCM purpose; UE-side/NW-side models; Data content; Typical data size (per data sample); Typical latency requirement; Notes. Training; UE-side, NW-side; L1-RSRPs and/or beam-IDs; See Note 1 for L1-RSRPs; Relaxed. ==omit table rows on “Inference” and “Monitoring” === Note 1: There is no agreement on the data size of L1-RSRPs for Set A or Set B, but the following typical data size is provided as guidance for RAN2 discussion. Based on existing L1-RSRP reporting methodology, i.e., 7 bits for the strongest beam and 4 bits for the remaining beams, for Set B = 16 as an example, the typical data size would be 67 (hence up to ~100 bits), and for Set A = 128 as an example, the typical data size would be 515 (hence up to ~500 bits) if all beams in Set A were to be collected. For BM Case 2, the data size L1-RSRPs for Set A and Set B represents the data size per predicted future time instance and per history measurement time instance, respectively. Payload size may not be fixed. Note 2: There are no agreements on the reporting type. Note 4: Feasibility and necessity of the monitoring schemes listed in the table are under discussion. Note 5: For BM Case 2, the typical value of the number of history measurement time instance used in evaluations is up to 8 and typical value of the number of predicted future time instance is 1~4.

Based on above analysis, we think it is sufficient to reuse the same minimum AS layer memory size of logged MDT and QoE. Thus, we propose:

Proposal 6: The minimum AS layer memory size for L1 measurement logging is same as existing logged MDT and QoE (i.e. 64KB).

#### 2.1.3.3 FFS#3: Event triggered logging

In summary report of email discussion [2], we notice the following Proposal 1 and Rapporteur Summary 1:

Summary 1: 22 companies responded to this question. 17 companies are in support of the use of L3 event-based triggers, and of those companies, it is clear that at least an A1-like event should be supported, while the usefulness of an A2-like event has less support. Generally, companies expressed that L3 event triggers provide stability, which would avoid ping ponging in measurement logging. Some companies think that the existing L3 events could be used instead.

Proposal 1: Support the use of L3 event triggers to activate or deactivate logging. The mechanism for activating and deactivating logging in response to the trigger and the L3 event triggers can be used are FFS.

As highlighted above, 17 out of 22 companies support to introduce an A1-like L3 event trigger. It is already a majority view (>75% supporting ratio). To make progress, we have the further proposal on top of above P1:

Proposal 7: Support A1-like L3 event triggered logging (i.e. when L3 serving cell measurement becomes better than absolute threshold). FFS its details and whether support other L3 event trigger.

Meanwhile, we think it is necessary to clarify that event triggered logging should be one optional feature on top of periodical logging, i.e. NW should have flexibility to disable it.

Proposal 8: RAN2 confirm that L3 event triggered logging is optional on top of periodical logging.

### Step 3: Details of availability reporting

#### 2.1.4.1 Contents of availability reporting

We first discuss the contents of availability reporting. According to previous RAN2 agreements, the availability indication report can indicate the following 2 cases:

- Indicate that UE’s logged measurement is available for NW retrieval, as agreed in RAN2#127b [7]:

- RAN2#127b

- The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest. FFS trigger/definition of availability indication.   and FFS how data availability indication is sent to the network.

- Indicate UE’s internal status, including low power state, low memory state and AS buffer is full, as agreed in RAN2#128 [3]:

- RAN2#128

- The UE can report the reason for triggering of indication for the status (e.g. low power state, low memory).  FFS how this is signalled and if the reporting can be part of availability indication.

Thus, the availability reporting needs to include cause indication for the NW to differentiate these two cases. The detailed format can be further discussed (e.g. whether finer sub-indication on UE’s internal status).

Proposal 9: Availability reporting includes the cause indication for the NW to at least differentiate the following two cases. FFS whether introduce finer sub-indication on different UE internal status in Case 2.

- Case 1: indicate UE’s logged measurement is available for NW retrieval

- Case 2: indicate UE’s internal status, including low power state, low memory state and AS buffer is full.

#### 2.1.4.2 Signalling of availability reporting

In legacy logged MDT, the UE may include the availability indication in various RRC complete messages (i.e., RRCReconfigurationComplete / RRCSetupComplete / RRCResumeComplete / RRCReestablishmentComplete). Essentially, it means only NW triggered availability indication reporting is supported. It is sufficient for logged MDT because logged MDT measurement is performed when the UE is in RRC_IDLE / RRC_INACTIVE state and NW is aware when the UE enters RRC_CONNECTED state. However, logged L1 measurement is performed in RRC_CONNECTED state and NW may not be aware when the UE has available logged measurement.

Observation 8: In legacy logged MDT, only NW triggered availability indication reporting is supported via various RRC complete messages because it is performed when the UE is in RRC_IDLE / RRC_INACTIVE state and NW is aware when the UE enters RRC_CONNECTED state.

Thus, UE initiated availability reporting needs to be supported as well, and we think it is straight forward to reuse UAI for UE initiated reporting.

Proposal 10: Support both NW triggered availability reporting and UE initiated availability reporting:

- NW triggered: reporting as response to NW request via various RRC complete messages.

- UE initiated: it is up to UE implementation when to report via UAI message.

One related FFS is in agreement of RAN2#128 [3] on when the UE reports to the network after its buffer is full.

- The UE reports to the network when buffer is or may become full. FFS when it reports (before and/or after).

As RAN2 agreed the UE stops data collection when buffer is full, we think that the intention of the reporting is to notify the NW about the termination of data collection. Thus, the UE should report it after its buffer is full.

Proposal 11: The UE reports availability indication to the network after its buffer is full.

### 2.1.5 Step 4: Reporting of logged L1 measurement

RAN2#127b further discussed which SRB is used to report logged L1 measurement, and down scoped to between new SRB and SRB4 [7]:

Agreements on NW side data collection

- Low priority SRB will be used.    FFS new SRB or use of SRB4

We prefer to introduce a new SRB due to below consideration

- Introducing a new SRB is useful to have differentiated handling between transfer of AI/ML dataset and transfer of QoE measurements.

  - If reusing SRB4 for AI/ML dataset transfer, the UE/NW can’t differentiate between the AI/ML dataset transfer and QoE measurement. As QoE measurements typically have large payload amount, it may block the transfer of AI/ML dataset. Furthermore, dataset transfer may have latency requirement as identified in training collaboration option 4 in RAN1#116 [10]. In this case, transfer of QoE measurements may cause latency of AI/ML dataset transfer and finally lead to failure of training collaboration.

Agreement

To alleviate / resolve the issues related to inter-vendor training collaboration of AI/ML-based CSI compression using two-sided model, study the following options:

- Option 1: Fully standardized reference model (structure + parameters)

- Option 2: Standardized dataset

- Option 3: Standardized reference model structure + Parameter exchange between NW-side and UE-side

- Option 4: Standardized data / dataset format + Dataset exchange between NW-side and UE-side

- Option 5: Standardized model format + Reference model exchange between NW-side and UE-side

- Introducing a new SRB is a cleaner solution because some QoE specific features were specified in SRB4 (e.g. RAN visible application layer measurements and RRC segmentation).

- Introducing a new SRB is more forward compatible because enhanced MDT should be a general framework for NW-sided data collection.

- Specification work of introducing a new SRB is manageable.

  - The spec work will be quite similar to SRB4. We don’t foresee significant work.

Observation 9: Dataset transfer may have latency requirement as identified in training collaboration option 4 in RAN1#116. If reusing SRB4 for AI/ML dataset transfer, transfer of QoE measurements may cause latency of AI/ML dataset transfer and finally lead to failure of training collaboration.

Observation 10: Introducing a new SRB is a cleaner solution because some QoE specific features were specified in SRB4 (e.g. RAN visible application layer measurements and RRC segmentation).

The new SRB should have lower priority than SRB1 and SRB2. Its priority between SRB4 can be further discussed.

Proposal 12: Introduce a new SRB to report logged L1 measurements with priority lower than SRB1/SRB2. FFS whether its priority is higher than SRB4.

### 2.1.6 How to handle logged measurement in HO or IDLE/INACTIVE state

This issue was raised during online discussion of RAN2#127b [7], but no conclusion was made. In our understanding, there are two different views when the UE enters IDLE/INACTIVE state or during HO but has logged L1 measurements in its AS buffer:

- Alt-1: The UE keeps the logged L1 measurements in its AS buffer. When it returns to CONNECTED state or completes HO, it notifies NW on the availability of stored measurements for NW to retrieve.

- Alt-2: The UE discards all the logged L1 measurements in its AS buffer.

We don’t think Alt-2 makes sense:

- Alt-2 implies risk of UE wasting its power for NW-side data collection but result in nothing. With such risk, the UE may not be willing to perform data collection for NW.

- The stored L1 measurements may still be useful to NW because the UE may enter CONNECTED state shortly.

- It will introduce extra spec work. Please note that Alt-1 is similar to existing logged MDT and early measurement, and thereby the existing spec changes can be reused.

Thus, we propose to adopt Alt-1:

Proposal 13: When the UE enters IDLE/INACTIVE state or during HO but has logged L1 measurements in its AS buffer, it keeps the logged measurements in its AS buffer. When it returns to CONNECTED state or completes HO, it notifies NW on the availability of stored measurements for NW to retrieve.

## 2.2 NW-sided data collection for AI/ML based positioning

First, please note that the revised Rel-19 WID [1] has clearly indicated RAN1 will not discuss any proposals on 2nd priority issues. Thus, following the guidance from the updated WID, we think RAN2 can also postpone the NW-side data collection discussion on case 2a/2b which are 2nd priority issues.

Proposal 14: Following guidance of revised WID, RAN2 postpone the discussion on data collection of AI/ML based positioning Case 2a)/2b) which are 2nd priority issues.

Then, for case 3a)/3b), Section 7.2.1.3.1 of TR 38.843 captured below conclusion [11]:

From Section 7.2.1.3.1 of TR 38.843

For positioning use cases, when considering LMF-side inference, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF. While for LMF-side performance monitoring, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF.

Based on above conclusion, it seems to be straight forward for RAN2 to assume NPPRa will be used for exchange of dataset / measurement / label / inference outcome in case 3a/3b and RAN2 can just leave work to RAN1/RAN3. Thus, we propose:

Proposal 15: For case 3a)/3b) of AI/ML based positioning, RAN2 assume NPPRa is used to exchange dataset and inference outcome between gNB and LMF and RAN2 leave data collection design to RAN1/RAN3.

## 2.3 Collected data type of NW-sided data collection

NW-sided data collection includes NW-side model of AI/ML based management and case 2b)/3a)/3b) of AI/ML based positioning. Based on our discussion in Proposal 5, Proposal 14 and Proposal 15, we have the below observation:

- NW-side model of AI/ML based management: at least L1-RSRP, beam ID and logging time stamps are required data type. Whether other data to collect depends on RAN1.

- Positioning Case 2b): its discussion should be postponed.

- Positioning Case 3a)/3b): leave data collection design to RAN1/RAN3.

Thus, we don’t think RAN2 have immediate work on collected data type:

Proposal 16: On required data type for NW-sided data collection, RAN2 wait RAN1 conclusion on NW-side model of AI/ML based management and leave Positioning Case 3a)/3b) to RAN1/RAN3.

## 2.4 Collected data type of UE-sided data collection

UE-sided data collection includes UE-side model for AI/ML based beam management and positioning case 1/2a). As the UE-side model is deployed at UE side, the basic assumption is that UE-side model is trained by UE side, and thus the required data type corresponding to AI/ML model input is naturally known at UE side. Therefore, when UE uses UE-side AI/ML model for inference, the UE can generate the measurement for AI/ML model inference using the same type or format of the measurement as training phase by implementation. 3GPP should not restrict the data type used for input of AI/ML model training. Thus, we do not think it is necessary to specify the data type of AI/ML model input. 3GPP only need to specify model output for UE-side model, and reference signal (RS) used for model input measurement and its configuration.

Observation 11: As the UE-side model is deployed at UE side, UE-side model is trained by UE side, and thus the required data type corresponding to AI/ML model input is naturally known at UE side. 3GPP should not restrict the data type used for input of AI/ML model training.

Thus, we propose:

Proposal 17: On model training and inference for UE-sided model, RAN2 assume that 3GPP only specify model output, reference signal (RS) used for model input measurement and its configuration. The specific data used for model input and their intermediate processing is up to UE implementation.

Meanwhile, we agree that data type needs to be specified for performance monitoring, especially when the NW performs the monitoring. But it is RAN1 expertise and RAN1 is working on it for both AI/ML based management and AI/ML based positioning. Thus, to avoid potential duplicated work, we propose RAN2 to leave the work to RAN1.

Proposal 18: On performance monitoring for UE-sided model, RAN2 leaves to RAN1 on the required data type.

Finally, RAN2 should send LS to RAN1 on Proposal 17 and Proposal 18.

Proposal 19: RAN2 send LS to RAN1 to check any issue for assumption in Proposal 17 and Proposal 18.

# 3 Conclusion

In this contribution, we discuss remaining issues of NW-sided data collection. Our Observation are:

Observation 1: Difference between OAM centric and gNB centric data collection is which NW entity (OAM vs gNB) to trigger data collection and generate data collection configuration. Their AS procedure is same.

Observation 2: UE privacy requirement is preserved in OAM centric data collection due to UE consent mechanism in MDT, but it is not clear how to satisfy UE privacy requirement in gNB centric data collection.

Observation 3: If we extend L1 CSI framework with logging and RRC reporting, it will bring below issues:

- It will require RAN1 to implement RAN2 agreements in RAN1 specification as procedure text of L1 CSI framework is now specified in Section 5.2.1 of TS 38.214.

- It will restrict the use case of the RAN2 designed data collection framework only to AI/ML based beam management, which is not future proof for other AI/ML use cases (e.g. AI/ML based mobility).

Observation 4: Existing L1 CSI framework already supports multiple CSI-RS resource set configurations: multiple NZP-CSI-RS-ResourceSet and multiple CSI-ReportConfig can be configured where one NZP-CSI-RS-ResourceSet can be associated with one CSI-ReportConfig via CSI-ResourceConfigId.

Observation 5: If NW needs the UE to only perform data collection only for one time duration, it can just reconfigure / de-configurate the data collection via RRC rather than via SP CSI resources.

Observation 6: In legacy NR, logged MDT, QoE measurements in IDLE/INACTIVE state and QoE paused measurements had specified the minimum AS layer memory size, which are all 64KB according to TS 38.306.

Observation 7: Based on the typical training data size of beam management provided in RAN1 Reply LS (R1-2310681), the typical training data size stored in AS layer memory is 100 bits8 instances (Set B) + 500 bits4 instances (Set A) =0.35KB.

Observation 8: In legacy logged MDT, only NW triggered availability indication reporting is supported via various RRC complete messages because it is performed when the UE is in RRC_IDLE / RRC_INACTIVE state and NW is aware when the UE enters RRC_CONNECTED state.

Observation 9: Dataset transfer may have latency requirement as identified in training collaboration option 4 in RAN1#116. If reusing SRB4 for AI/ML dataset transfer, transfer of QoE measurements may cause latency of AI/ML dataset transfer and finally lead to failure of training collaboration.

Observation 10: Introducing a new SRB is a cleaner solution because some QoE specific features were specified in SRB4 (e.g. RAN visible application layer measurements and RRC segmentation).

Observation 11: As the UE-side model is deployed at UE side, UE-side model is trained by UE side, and thus the required data type corresponding to AI/ML model input is naturally known at UE side. 3GPP should not restrict the data type used for input of AI/ML model training.

Based on observations, our proposals can be found below:

Overall procedure of enhanced MDT

Proposal 1: To strive for a common measurement framework, similar UE consent mechanism in MDT is supported in gNB-centric data collection, i.e. UE consent is required before NW configures the UE gNB-centric data collection. FFS signaling details.

RRC framework of enhanced MDT

Proposal 2: L3 measurement framework is extended to configure logged L1 measurements with below specification changes:

- Within MeasObjectNR: introduce L1 CSI resource configuration (e.g. a list of resource set IDs pointed to CSI-RS resource pool in CSI-MeasConfig) and logging configuration.

- Within ReportConfigNR: introduce on-demand reporting configuration to indicate at least reporting type (i.e. on-demand) and reporting quantity configuration (e.g. L1 RSPR and/or beam index).

Proposal 3a: Do not support multiple configurations of logged L1 measurements (e.g. multiple MeasConfig).

Proposal 3b: Similar to existing L1 CSI framework, multiple L1 CSI resource sets can be configured to be linked with different logging configurations (e.g. with different logging periodicities) in MeasObjectNR.

Proposal 4: As target for offline training without latency requirement, no need to support dynamic activation/deactivation of logged L1 measurements on SP CSI resources (i.e. it is sufficient to rely on RRC reconfiguration / de-configuration of periodic CSI resources).

How the UE performs logging

Proposal 5: Besides L1 RSRP and/or beam-IDs, the UE also collects logging time stamp in its AS buffer.

Proposal 6: The minimum AS layer memory size for L1 measurement logging is same as existing logged MDT and QoE (i.e. 64KB).

Proposal 7: Support A1-like L3 event triggered logging (i.e. when L3 serving cell measurement becomes better than absolute threshold). FFS its details and whether support other L3 event trigger.

Proposal 8: RAN2 confirm that L3 event triggered logging is optional on top of periodical logging.

Details of availability reporting

Proposal 9: Availability reporting includes the cause indication for the NW to at least differentiate the following two cases. FFS whether introduce finer sub-indication on different UE internal status in Case 2.

- Case 1: indicate UE’s logged measurement is available for NW retrieval

- Case 2: indicate UE’s internal status, including low power state, low memory state and AS buffer is full.

Proposal 10: Support both NW triggered availability reporting and UE initiated availability reporting:

- NW triggered: reporting as response to NW request via various RRC complete messages.

- UE initiated: it is up to UE implementation when to report via UAI message.

Proposal 11: The UE reports availability indication to the network after its buffer is full.

Reporting of logged L1 measurement

Proposal 12: Introduce a new SRB to report logged L1 measurements with priority lower than SRB1/SRB2. FFS whether its priority is higher than SRB4.

Proposal 13: When the UE enters IDLE/INACTIVE state or during HO but has logged L1 measurements in its AS buffer, it keeps the logged measurements in its AS buffer. When it returns to CONNECTED state or completes HO, it notifies NW on the availability of stored measurements for NW to retrieve.

NW-sided data collection for AI/ML based positioning

Proposal 14: Following guidance of updated WID, RAN2 postpone the discussion on data collection of AI/ML based positioning Case 2a)/2b) which are 2nd priority issues.

Proposal 15: For case 3a)/3b) of AI/ML based positioning, RAN2 assume NPPRa is used to exchange dataset and inference outcome between gNB and LMF and RAN2 leave data collection design to RAN1/RAN3.

Collected data type of NW-sided data collection

Proposal 16: On required data type for NW-sided data collection, RAN2 wait RAN1 conclusion on NW-side model of AI/ML based management and leave Positioning Case 3a)/3b) to RAN1/RAN3.

Collected data type of UE-sided data collection

Proposal 17: On model training and inference for UE-sided model, RAN2 assume that 3GPP only specify model output, reference signal (RS) used for model input measurement and its configuration. The specific data used for model input and their intermediate processing is up to UE implementation.

Proposal 18: On performance monitoring for UE-sided model, RAN2 leaves to RAN1 on the required data type.

Proposal 19: RAN2 send LS to RAN1 to check any issue for assumption in Proposal 17 and Proposal 18.

# 4 References

[1] RP-243244, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm.

[2] R2-2500999, Report on [POST128][019][AI PHY] NW side data collection (Nokia), Nokia.

[3] RAN2#128, Chair Notes.

[4] TS 32.422-v17.8.0, Trace control and configuration management, 2022-9.

[5] RAN1#116b, Chair Notes.

[6] RAN2#127, Chair Notes.

[7] RAN2#127b, Chair Notes.

[8] TS 38.306-v18.0.0, User Equipment (UE) radio access capabilities, 2023-12.

[9] R1-2310681, Reply LS on Data Collection Requirements and Assumptions, RAN1.

[10] RAN1#116, Chair Notes.

[11] TR 38.843-v2.0.1, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), 2023-12.