# 3GPP TSG RAN WG2 Meeting #129      	                           R2-2500262
Athens, Greece, 17th – 21th Feb., 2025

Agenda item:	8.1.4

Source:	Apple

Title:	Further discussion on UE-side data collection

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]:

  - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection

  - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods

The new Rel-19 SID of AI/ML for air interface Phase 2 (SID RP-243245) was agreed in RAN#106 [1]. The SI objective on UE-side data collection can be found below:

According to RAN#106 discussion, the following potential aspects on UE-side data collection were identified for further study in RAN2 in Rel-19:

Discuss any of the aspects identified in RANP WF

o	Study RAN aspects related to data transfer over UP

o	Discuss level of NG-RAN involvement in the control and configuration of UE side data collection.

o	Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion).

o	Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain.

o	Discuss on the scalability aspects of CP

Based on our understanding, the above aspects can be classified into data collection configuration and dataset transfer. Thus, we discuss the above aspects according to below order:

- Data collection configuration

  - Level of NG-RAN involvement in the control and configuration of UE side data collection

- Dataset transfer

  - RAN perspective to enable the data transfer to CN domain or OAM domain.

  - NG-RAN involvement in the data transfer, including visibility discussion

  - RAN aspects related to data transfer over UP

  - Scalability aspects of CP

# 2 Discussion

## 2.1 Data collection configuration

In Reply LS to SA2 [2], RAN2 provide the following definition of data collection configuration:

- Data collection refers to the UE collecting data that involves the UE performing measurements (e.g., radio measurements).

Based on above definition, we discuss the level of NG-RAN involvement in the control and configuration.

### 2.1.1 Level of NG-RAN involvement in the control and configuration

RAN2#128 made below agreements on data collection configuration in AI/ML based beam management [3]:

- For BM use case for UE-side model, data collection related configuration(s) (e.g., measurement resources configuration) and associated ID(s) can be included in training data collection configuration.

- For data collection configuration UE-side model training, the UE can send a request for data collection.   FFS what the request contains.

- The network can provide the data collection configuration (at any point in time), with or without UE request.

- The following methods for network control of the initiation and configuration for data collection:

- The network can decide when to start/stop the data collection and send configuration.

- The network can configure whether UE is allowed to initiate request for data collection.

We think above agreements can be extended to general UE-side data collection.

Proposal 1: Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration:

- Data collection related configuration(s) (e.g., measurement resources configuration) and associated ID(s) can be included in training data collection configuration.

- For data collection configuration UE-side model training, the UE can send a request for data collection.

- The network can provide the data collection configuration (at any point in time), with or without UE request.

- The following methods for network control of the initiation and configuration for data collection:

  - The network can decide when to start/stop the data collection and send configuration.

  - The network can configure whether UE is allowed to initiate request for data collection.

Then on the UE request message, we think the request is usful at least from below three aspects:

- The UE request can indicate whether the data collection is needed based on its local / internal information, e.g. whether the device local model is ready for interence or needs further training.

- The UE request can indicate when the data collection can start or stop based on its local / internal informaion on, e.g. available remaining battery and memory information, whether any on-going operations which can’t be interrputed by data collection.

  - The UE can’t share such information with the NW because it may be UE’s internal implementation.

- The UE request can indicate what assistance information and/or reference signal (RS) from NW are needed bsaed on its local / internal informaion, e.g. required CSI-RS resource for measurements.

  - The UE may provide its suggestion to the NW and it is up to NW to decide whether to acommendate the UE requrst as legacy.

Observation 1: The following information is useful to be included in UE request signaling on data collection:

- Whether the data collection is needed based on UE’s local / internal information, e.g. whether the device local model is ready for interence or needs further training.

- When the data collection can start or stop based on UE’s local / internal information, e.g. remaining battery / memory status, whether any on-going operations which can’t be interrputed by data collection.

- What assistance information and/or reference signal (RS) from NW are needed for model training, e.g. required CSI-RS resource for measurements.

The above 1) and 2) can be combined into one bit indication on whether initializing / stopping data collection, and 3) can be mapped to the suggested measurement resource configuration. Meanwhile, as general principle in 3GPP, it is up to NW implementation whether to accommodate the UE request. Thus, we propose:

Proposal 2: The UE request may include the indication on whether initializing / stopping data collection and suggested measurement resource configuration. And it is up to NW implementation whether to accommodate the UE request.

In RAN2#128 [3], some companies proposed some solutions without UE request (e.g. requested by UE-side server). However, we think the discussion on detailed solution is out of RAN2 scope, and it can be left to other WGs.

Proposal 3: On data collection configuration without UE request, RAN2 leaves the detailed solution discussion to other WG.

## 2.2 Data Transfer

In Reply LS to SA2 [2], RAN2 provide the following definition of data transfer:

- Data transfer refers to the sending/transfer of the collected data from the UE to Server for data collection for UE-side model training/OTT server (as capture in Table 7.2.1.3.2-1) in TR38.843.

Based on above definition, we discuss the issues identified in RANP#106.

### 2.2.1 RAN aspects to enable the data transfer to CN domain or OAM domain

In RANP#106, option 1b was precluded. Thus, the data transfer to CN domain is mapped to option 2 and the data transfer to OAM domain is mapped to option 3.

Observation 2: As option 1b was precluded in RANP#106, the data transfer to CN domain is mapped to option 2 and the data transfer to OAM domain is mapped to option 3.

According to the LS from RAN to SA2/SA5 in RANP#106 [4], SA2 will evaluate UP solution of option 2 and SA5 will evaluate UP solution of option 3.

From RAN perspective, transfer of data over UP for Solution 2 in SA2 should be studied and provide feedback on fulfillment of visibility and controllability requirements.  Transfer of data over UP for Solution 3 can be studied by SA5

RAN has already concluded that transfer of data over CP for Solution 2 and 3 can meet the visibility and controllability RAN requirements, even though some scalability concerns have been raised for high amount of data transfer.  RAN is not expecting SA2 and SA5 to study the transfer of data over CP for now.

Thus, it is still not clear whether / how UP solution of option 2 and option 3 can work before SA2/SA5 provide feedback. Furthermore, even if UP solution is adopted, it is also not clear whether RAN is transparent to option 2/3 and whether RAN can differentiate option 2 and option 3. Thus, we think it is difficulty for RAN2 to discuss detailed RAN impacts between option 2 and 3 at this stage, i.e. RAN2 can only discuss potential RAN impacts common to option 2 and option 3. Thus, we propose:

Proposal 4: Before SA2/SA5 provide feedback, RAN2 only discuss potential RAN impacts common to option 2 and option 3.

Meanwhile, please note that it is indicated that RAN will make a final decision on the way forward taking into account SA inputs in the LS to SA2/SA5 [4]. In a simple, RAN do not task RAN2 to discuss solution down-selection.

TSG RAN will make a final decision on the way forward taking into account SA inputs when available.  TSG RAN will have a checkpoint in Q2, 2025

Thus, we don’t think RAN2 need to compare between option 2 and option 3, and their down-selection.

Proposal 5: Before RANP provide guidance, RAN2 don’t compare between option 2 (i.e. transfer to CN domain) and option 3 (i.e. transfer to OAM domain) and discuss their down-selection.

### 2.2.2 NG-RAN involvement in the data transfer

We think it needs to discuss separately for CP solution and UP solution:

CP solution

We discuss the controllability and visibility of CP solution. On controllability, it is common understanding that NG-RAN have the full control because SRB transmission is under NG-RAN control.

Proposal 6: RAN2 conclude that NG-RAN have the full control for dataset transfer over CP solution.

On visibility, it is also common understanding that NG-RAN have full visibility for standardized data because it is ASN.1 coding. However, one remaining issue is whether to support non-standardized data transfer over CP solution. We have the following concerns to support it:

- The potential leakage risk as it main contain proprietary implementation and UE privacy information.

- In 3GPP, all standardized data / measurement is specified with clear RAN4 requirements to ensure the data reported by the UE is valid for the NW. However, we doubt how to specify RAN4 requirements for non- standardized data.

Meanwhile, the UE can send the non-standardized data over the top.

Observation 3: Supporting non-standardized data transfer over CP solution has the following issues:

- The potential leakage risk as it may contain proprietary implementation and UE privacy information.

- It is not clear how to specify RAN4 requirements for non- standardized data to ensure validity for NW.

Meanwhile, the UE can send the non-standardized data over the top.

Thus, we propose not to support non-standardized data transfer over CP solution.

Proposal 7: RAN2 conclude that NG-RAN have full visibility for standardized dataset transfer over CP solution. Non-standardized data transfer over CP solution is not supported.

UP solution

The existing UP protocol stack is between the UE and UPF. 5GC manages the UP traffic with the granularity of PDU session, and NG-RAN is responsible to map QoS Flow(s) of the same PDU session to DRB(s), which is illustrated in Figure. 1.

Figure. 1 5G UP framework

Thus, we understand that NG-RAN only has the controllability and visibility on configuration of PDU session and QoS Flow. Whether NG-RAN can be aware of UE-side data transfer depends on SA2 design (e.g. whether to introduce AI/ML specific PDU session or AI/ML specific QoS flow).

Observation 4: On UP traffic, existing NG-RAN only has the controllability and visibility on configuration of PDU session and QoS Flow. Whether NG-RAN can be aware of UE-side data transfer depends on SA2 design (e.g. whether to introduce AI/ML specific PDU session or AI/ML specific QoS flow).

Before SA2 provide feedback, we don’t think RAN2 can conclude the level of NG-RAN involvement. Thus, we propose:

Proposal 8: RAN2 wait SA2 feedback before discussion on level of NG-RAN involvement on dataset transfer over UP solution, including its controllability and visibility.

### 2.2.2 RAN aspects related to data transfer over UP

As we discussed in Observation 4, we think that RAN2 need to wait SA2 conclusion on:

- Whether NG-RAN can be aware of AI/ML data transfer

- If NG-RAN is aware of AI/ML data transfer, what is the granularity of management? For example:

  - Alt-1: Introduce AI/ML specific QoS flow

  - Alt-2: Introduce AI/ML specific PDU session

  - Alt-3: Introduce AI/ML specific protocol stack (e.g. data plane)

Thus, we propose:

Proposal 9: On data transfer solution over UP, RAN2 wait SA2 conclusion on whether/how NG-RAN can be aware of AI/ML dataset transfer (e.g. via AI/ML specific QoS flow, or AI/ML specific PDU session, or AI/ML specific protocol stack, etc.) to configure DRB accordingly.

### 2.2.3 Scalability aspects of CP

The existing RRC message only supports up to 16 segments in UL (i.e. up to data size of 144KB). Supporting larger UL segment number will put extra memory requirement on both UE and gNB. Thus, whether it works needs to be decided based on specific value of dataset payload size.

Observation 5: Supporting UL RRC message with larger than 16 segments put extra memory requirement for both UE and gNB. Thus, whether it works needs to be decided based on dataset payload size.

For Option 4-1: sharing {target CSI, CSI feedback} dataset:

  - Data size for target CSI depends on the format. There is no agreement on the format or necessary precision of the target CSI. Some examples based on companies’ evaluations are: eType-II format (up to ~1000 bits), eType-II-like format (~ a few 1000 bits), and float32 format (up to ~ 150K bits). The data size may also vary depending on the configuration, and the captured value indicates the order of magnitude of the typical data size per sample as a guideline.

  - There is no agreement on the CSI feedback size. Values in the order of eType II payload size may be assumed (up to ~ 1000 bits) for RAN2 discussion.

  - There is no agreement on the total number of samples N1 in the dataset. The range of the number of samples N1 used in RAN1 evaluations is 40K-1200K. As a representative number, RAN2 may assume that the total number of samples is around N1=600K.

Summing up the above, and assuming eType-II-like format of a size 2000 bits, the total dataset size for Option 4-1 may be roughly in the order of 600K * (2000 bits + 1000 bits) / (8bits/Byte) = 225 MB. The size may be smaller for eType-II format. The size may be larger if we consider the dataset consisting of supporting multiple configurations.

In Rel-18 study phase, RAN1 sent Reply LS (R1-2310681) to RAN2 with analysis on data payload size for different use cases [5]. However, as it was early study phase, RAN1 didn’t provide specific values for most use cases. Recently, RAN1 sent another LS (R1-2410922) [6] to request RAN2 to evaluate feasibility of identified solutions on two-sided model, which provides a specific value for option 1 of CSI compression (225MB in Option 4-1):

Proposal 10: Before discussion whether dataset transfer over CP solution can work, RAN2 need to wait RAN1 conclusion on dataset payload size.

If 225MB is the typical data size, it means that maximum UL segment number needs to increase to 25600, which is impossible to support.

Thus, we think CP solution may not work if data size is too large, and RAN2 need to discuss based on specific value of dataset payload size. Given RAN1 only provide data size of CSI compression in LS (R1-2410922) [6], we think it is necessary to send LS to request RAN1 to provide data size of other AI/ML use cases, including beam management, positioning and CSI prediction.

Proposal 10: On scalability of CP solution, RAN2 conclude that it may not work if data size is too large (e.g., 225MB in CSI compression). RAN2 send LS to request RAN1 to provide data size of other AI/ML use cases, including beam management, positioning and CSI prediction.

# 3 Conclusion

In this contribution, we share our view on UE-side data collection. Our observations are:

Observation 1: The following information is useful to be included in UE request signaling on data collection:

- Whether the data collection is needed based on UE’s local / internal information, e.g. whether the device local model is ready for interence or needs further training.

- When the data collection can start or stop based on UE’s local / internal information, e.g. remaining battery / memory status, whether any on-going operations which can’t be interrputed by data collection.

- What assistance information and/or reference signal (RS) from NW are needed for model training, e.g. required CSI-RS resource for measurements.

Observation 2: As option 1b was precluded in RANP#106, the data transfer to CN domain is mapped to option 2 and the data transfer to OAM domain is mapped to option 3.

Observation 3: Supporting non-standardized data transfer over CP solution has the following issues:

- The potential leakage risk as it may contain proprietary implementation and UE privacy information.

- It is not clear how to specify RAN4 requirements for non- standardized data to ensure validity for NW.

Meanwhile, the UE can send the non-standardized data over the top.

Observation 4: On UP traffic, existing NG-RAN only has the controllability and visibility on configuration of PDU session and QoS Flow. Whether NG-RAN can be aware of UE-side data transfer depends on SA2 design (e.g. whether to introduce AI/ML specific PDU session or AI/ML specific QoS flow).

Observation 5: Supporting UL RRC message with larger than 16 segments put extra memory requirement for both UE and gNB. Thus, whether it works needs to be decided based on dataset payload size.

Based on observations, our proposals can be found below:

Level of NG-RAN involvement in the control and configuration of UE side data collection

Proposal 1: Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration:

- Data collection related configuration(s) (e.g., measurement resources configuration) and associated ID(s) can be included in training data collection configuration.

- For data collection configuration UE-side model training, the UE can send a request for data collection.

- The network can provide the data collection configuration (at any point in time), with or without UE request.

- The following methods for network control of the initiation and configuration for data collection:

  - The network can decide when to start/stop the data collection and send configuration.

  - The network can configure whether UE is allowed to initiate request for data collection.

Proposal 2: The UE request may include the indication on whether initializing / stopping data collection and suggested measurement resource configuration. And it is up to NW implementation whether to accommodate the UE request.

Proposal 3: On data collection configuration without UE request, RAN2 leaves the detailed solution discussion to other WG.

RAN aspects to enable the data transfer to CN domain or OAM domain

Proposal 4: Before SA2/SA5 provide feedback, RAN2 only discuss potential RAN impacts common to option 2 and option 3.

Proposal 5: Before RANP provide guidance, RAN2 don’t compare between option 2 (i.e. transfer to CN domain) and option 3 (i.e. transfer to OAM domain) and discuss their down-selection.

NG-RAN involvement in the data transfer

Proposal 6: RAN2 conclude that NG-RAN have the full control for dataset transfer over CP solution.

Proposal 7: RAN2 conclude that NG-RAN have full visibility for standardized dataset transfer over CP solution. Non-standardized data transfer over CP solution is not supported.

Proposal 8: RAN2 wait SA2 feedback before discussion on level of NG-RAN involvement on dataset transfer over UP solution, including its controllability and visibility.

RAN aspects related to data transfer over UP

Proposal 9: On data transfer solution over UP, RAN2 wait SA2 conclusion on whether/how NG-RAN can be aware of AI/ML dataset transfer (e.g. via AI/ML specific QoS flow, or AI/ML specific PDU session, or AI/ML specific protocol stack, etc.) to configure DRB accordingly.

Scalability aspects of CP

Proposal 10: On scalability of CP solution, RAN2 conclude that it may not work if data size is too large (e.g., 225MB in CSI compression). RAN2 send LS to request RAN1 to provide data size of other AI/ML use cases, including beam management, positioning and CSI prediction.

# 4 References

[1] RP-243245, New SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2, Qualcomm.

[2] R2-2411152, Reply LS to SA2 on AIML data collection.

[3] RAN2#128, Chair Notes.

[4] RP-243316, LS on AI/ML UE sided data collection.

[5] R1-2310681, Reply LS on Data Collection Requirements and Assumptions, RAN1.

[6] R1-2410922, LS on signalling feasibility of dataset and parameter sharing.