3GPP TSG-RAN WG2 Meeting #129	R2-2500270

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.2.2

Source: 	CATT

Title: 	Discussion on Paging for Ambient IoT

Document for: 	Discussion and Decision

1	Introduction

According to the WID [1] and agenda, this contribution discusses some essential paging aspects.

    - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs.

Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

2	Discussion

2.1	 Clarification on the identifier in paging message

According to the WID [1], only the paging message contains one identifier and that contains no identifier are in scope of Rel-19. With reference to the agreement achieved in RAN2#126, we understand that paging message with one identifier cover the cases of paging a single device and paging multiple devices with a group ID, while paging message with no identifier covers the case of paging all devices.

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion.

This is clarified as follows:

Proposal 1: The paging message containing no identifier means the reader addresses all the devices. The paging message containing one identifier means the reader addresses one device or a group of devices (respectively in the form of device ID or group ID).

2.2	 How to support subsequent paging

This section gives some detailed solutions on the support of paging procedure for the same service request from CN. Note that a paging procedure corresponding to a service request can consist of n (subsequent) AIoT paging messages, as shown in the figure.

1) ID used in A-IoT paging message

In RAN2#128, it was agreed that:

- An ID in paging message is beneficial to avoid duplicated response for same service request. FFS whether ID is generated by the reader or CN. FFS on the ID size.

For the FFS whether the ID is generated by the reader or CN, we have slight preference on the option that the ID is generated by reader. If the ID is generated by CN, the ID may be too long and will increase the cost of device. Considering the ID is used for the device to distinguish different service requests within reader scope, the ID can be defined with a short format, e.g., with a length of 2 bits or 4 bits, as long as the ID can avoid collision over the AIoT interface. The specific ID size can be determined in stage 3. For a given service request from CN, the ID should be contained with the same value in all the (subsequent) paging messages associated with this service request.

Proposal 2: For an AIoT paging procedure, the ID used to avoid duplicated response for the same service request is generated by the reader. All the (subsequent) paging messages associated with the same service request containing the same ID value.

2) Device behaviours

Referencing the following text extracted from TR [2], the device can determine whether to skip the subsequent paging messages according to the ID as mentioned in proposal 2.

It is supported that the reader can send multiple (subsequent) A-IoT paging messages that are associated with the same service request from the CN. The duplicated response from devices for the same service request should be avoided. The A-IoT paging message can include information to avoid this duplicated response from the device to a reader. It needs to be further discussed on how to design this information in A-IoT paging message (e.g., including stage-3 details and considering the related aspects from other WGs). Then, based on this information, the device determines whether to skip sending the response to A-IoT paging message or not (if the device had successfully responded the same service before). This information should be short and simple. This information is one ID, while it needs to be further discussed whether the ID is generated by the reader or by the core network. It needs to be further discussed on the size of this information.

That is, once the device has successfully responded to the service request with the ID included in an AIoT paging message, it should not respond to the subsequent paging messages containing the same ID value. It is worth noting that the reader may reuse the same ID for the following service requests at non-overlapping time (e.g., reuse the ID value after the current service is finished). For a certain service request, the “skip response” operation for this ID should be limited before the end of this service procedure to avoid missing the following service requests from CN.

Proposal 3: Once the device has successfully responded to the service which is indicated by the ID within the AIoT paging message, it shall not respond to the subsequent AIoT paging messages containing the same ID before the end of the whole paging procedure for this service. Otherwise, it shall respond to the subsequent AIoT paging messages (if received).

As for how the device determines it has successful responded the service, it is related to the discussion on random access mechanism and data transmission scheduling mechanism. In RAN2#128, we achieved the following agreement.

- The "R2D failure/success feedback indication" also applies to "following D2R data" after the random access procedure for re-access purpose

Based on this, the device can know whether the random access procedure and D2R data transmission succeeds. As an example, for the inventory service request, when the device sends a D2R message containing its device ID to the reader and receives an R2D success feedback indication, it considers itself responding to the service request successfully. The same logic applies to the command service request, for instance, the device receives the R2D success feedback indication after sending the last D2R message and thus consider it has successfully responded to the service request. The details on the design of R2D failure/success feedback indication can be left to the discussion on random access mechanism and data transmission mechanism.

Proposal 3a: The device determines whether it has successfully responded to the service request based on the R2D failure/success feedback indication.

During the paging procedure with subsequent paging messages, the device needs to store some status information for the ongoing service request, e.g., the ID as mentioned above and whether it has successfully responded to the service request or not. RAN1 has agreed to support the following two types of memory [3].

- Memory can include two types of memory: 1) Non-Volatile Memory (NVM) such as EEPROM for permanently storing device ID, etc, and 2) registers for temporarily keeping any information required for its operation only while energy is available in energy storage.

Considering the status information is used in short-term and changes frequently, stored in temporary memory is more suitable.

Proposal 4: The device needs to maintain some status information (e.g., the ID used to avoid duplicated response for same service request, whether it has successfully responded or not, etc.) for the service request in its temporary memory before the end of the corresponding paging procedure.

The maintained status information should be released when the paging procedure in AIoT interface is ended. So that the device is allowed to response to following service requests which reuses the ID. As for how the device determines the end of the paging procedure, i.e., before which point the paging messages with the same ID belong to the same service request, some candidate solutions are given as following. RAN2 could have a further discussion on this issue.

- Option 1: A timer is used by device to determine end of the paging procedure, the length of timer could be predefined in spec (e.g., 10s) or configured by reader in the R2D message.

- Option 2: The device is explicitly indicated by the reader with an “end” indication (e.g., to be a one bit indication) in the last paging message of a paging procedure.

Proposal 5: As for how the device determines the end of an AIoT paging procedure with subsequent paging messages, RAN2 further discusses which of the following options to adopt:

- Option 1: The device uses a timer to determine the end, where the length of timer can be predefined in spec or configured by reader in R2D message.

- Option 2: Explicitly indicated by the reader with an “end” indication in the last paging message of the paging procedure.

2.3	 Paging message content and format

1) Paging message content

As per analyzed in section 2.1 and 2.2, the paging message contains one identifier for the target device(s) and an ID used for differentiate paging procedures. As for other information or configuration which is possible to be contained in the paging message, we need wait for progress of other topic and work groups. In detail:

① Whether the command can be contained in the paging message

In RAN2#126, we reached the following agreement and sent LS to SA2 and SA3 [4], indicating that the feasibility of paging message contains command is finally depends on SA2 and SA3. Since no reply LS has been received from SA2 and SA3, RAN2 still needs to wait for their progress.

3 From RAN2 point of view we will study “Command only” use case. FFS the options on how to support it : Initial trigger message from the reader contains the command. Final feasibility depends on SA2 and SA3 work/conclusions. Use baseline procedure for “inventory and command”(i.e. first triggers inventory procedure and then sends command)

Proposal 6: Wait for response of SA2 and SA3 on whether the command can be contained in the paging message.

② Whether to contain the access configuration in the paging message.

According to the WID [1], CBRA and CFRA will be supported in Rel-19. The following figure is the general framework of slotted-ALOHA for AIoT random access procedure [2].

For these two types of random access, some aspects need to be determined before we discuss whether to contain the access configuration in the paging message and what information is contained, including:

- Whether an additional R2D message (shown as the Dotted Arrow in the figure) is introduced for the use of random access; if yes, should we use the paging message or the additional R2D message to contain the access configuration;

- What is the configuration format in time domain of CFRA and CBRA, i.e. how to indicate the access occasion(s); and whether an additional explicit indication is needed to differentiate access type of CBRA and CFRA;

- What is the configuration format in frequency domain (need to wait for progress of RAN1).

RAN2 should postpone the discussion on containing the access configuration in paging message until common understanding on the access occasion modelling and clear concept of the access configurations are achieved.

Proposal 7: Postpone the discussion on whether/how to contain the access configuration in paging message until RAN2 has common understanding on the access occasion modelling and clear concept of the access configurations (e.g., how to formulate the occasion number and frequency domain resources).

③ Whether the service type needs to be contained in the paging message

In RAN2#128, there is an FFS on whether to indicate the service type/command type to the device.

- Whether we include further information in paging message to indicate service type/command type or to indicate that further subsequent messages are coming can be discussed in WI phase once we understand more the paging design and AS ID.

The intention is to tell the device whether there are further subsequent messages so that the device can know when to stop monitoring R2D. However, the requirement can already be fulfilled based on the agreed R2D failure/success feedback indication.

- The "R2D failure/success feedback indication" also applies to "following D2R data" after the random access procedure for re-access purpose

More specifically, for a service request, there may be multiple rounds of D2R and R2D transmission between the device and reader. The reader only sends the R2D success feedback indication to device after successfully receiving the last D2R message while the previous D2R transmission can be implicitly confirmed by scheduling new UL grant (for next D2R transmission). By this way, the device can know the service is finished when receiving the R2D success feedback indication. Additional info for service type/command type in the paging message is not needed.

Proposal 8: Do not include information for service type/command type in the AIoT paging message.

2) Format of the paging message

The following is an initial discussion on the format of the paging message including the information confirmed till now, i.e. one identifier used for addressing the target device(s) and an ID used for differentiate subsequent paging messages for the same paging procedure. Considering the WID [1] indicates that only AIoT MAC layer is included, the paging message should be implemented as an AIoT MAC CE.

- The AIoT MAC CE has an identifier field. The identifier field is optional present. When present, the identifier is in the form of device ID or group ID and is used to addressing one device or a group of device; while absent means addressing all the devices. The size of this field is pending SA2’s progress.

- The AIoT MAC CE has an ID field used to avoid duplicated response for same service request. The service ID field is optional present. Present means the device needs to performs the corresponding operation for subsequent paging message; absent means there is no subsequent paging message for this service request.

Proposal 9: Define an AIoT MAC CE for the paging message. The AIoT paging MAC CE at least includes:

- One identifier field: absence of this field means addressing all the devices; if this field is in the form of device ID it means addressing a single device; if this field is in the form of group ID it means addressing a group of devices.

- An ID used to avoid duplicated response for same service request: absence of this field means there is no subsequent paging message for this service request; if present, the device performs the corresponding operation for subsequent paging message.

Considering the note in WID [1], the paging message format needs be able to contain multiple identifiers for forward compatibility purpose. This intention can be fulfilled by defining the paging message as variable length MAC CE. In the future version, the MAC CE can be extended as needed. Detailed design can be discussed in stage 3.

Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

Proposal 10: The AIoT MAC CE for the paging message is in a format with variable length for forward compatibility purposes.

4	Conclusion

In this contribution, paging functionality for A-IoT is discussed with giving corresponding observations and proposals:

Clarification on the identifier in paging message

Proposal 1: The paging message containing no identifier means the reader addresses all the devices. The paging message containing one identifier means the reader addresses one device or a group of devices (respectively in the form of device ID or group ID).

How to support subsequent paging

Proposal 2: For an AIoT paging procedure, the ID used to avoid duplicated response for the same service request is generated by the reader. All the (subsequent) paging messages associated with the same service request containing the same ID value.

Proposal 3: Once the device has successfully responded to the service which is indicated by the ID within the AIoT paging message, it shall not respond to the subsequent AIoT paging messages containing the same ID before the end of the whole paging procedure for this service. Otherwise, it shall respond to the subsequent AIoT paging messages (if received).

Proposal 3a: The device determines whether it has successfully responded to the service request based on the R2D failure/success feedback indication.

Proposal 4: The device needs to maintain some status information (e.g., the ID used to avoid duplicated response for same service request, whether it has successfully responded or not, etc.) for the service request in its temporary memory before the end of the corresponding paging procedure.

Proposal 5: As for how the device determines the end of an AIoT paging procedure with subsequent paging messages, RAN2 further discusses which of the following options to adopt:

- Option 1: The device uses a timer to determine the end, where the length of timer can be predefined in spec or configured by reader in R2D message.

- Option 2: Explicitly indicated by the reader with an “end” indication in the last paging message of the paging procedure.

Paging message content and format

Proposal 6: Wait for response of SA2 and SA3 on whether the command can be contained in the paging message.

Proposal 7: Postpone the discussion on whether/how to contain the access configuration in paging message until RAN2 has common understanding on the access occasion modelling and clear concept of the access configurations (e.g., how to formulate the occasion number and frequency domain resources).

Proposal 8: Do not include information for service type/command type in the AIoT paging message.

Proposal 9: Define an AIoT MAC CE for the paging message. The AIoT paging MAC CE at least includes:

- One identifier field: absence of this field means addressing all the devices; if this field is in the form of device ID it means addressing a single device; if this field is in the form of group ID it means addressing a group of devices.

- An ID used to avoid duplicated response for same service request: absence of this field means there is no subsequent paging message for this service request; if present, the device performs the corresponding operation for subsequent paging message.

Proposal 10: The AIoT MAC CE for the paging message is in a format with variable length for forward compatibility purposes.

5	Reference

- RP-243326  New Work Item: Solutions for Ambient IoT (Internet of Things) in NR.

- 3GPP TR 38.769  Study on solutions for ambient IoT (Internet of Things).

- RAN1#116 Chair’s notes.

- R2-2406150  LS on RAN2 agreements and assumptions for Ambient IoT

- 3GPP TR 23.700-13: Study on Architecture support of Ambient power-enabled Internet of Things.