3GPP TSG-RAN WG2 Meeting #129	                                                                    R2-2500271

Athens, Greece, Feb. 17th – 21st, 2025

Source:	CATT

Title:	Discussion on the Random Access for Ambient IoT

Agenda Item:	8.2.3

Document for:	Discussion and Decision

- Introduction

After the SI discussion, it was agreed in the last RAN plenary that we will firstly focus on the topology-1 for the A-IoT in the WI phase. In this paper, we give our views on the RACH-related issues for A-IoT according to the WID [1]. In detail, the following aspects are covered:

- On the use of unified design for 3-step and 2-step CBRA

- Further details on CBRA

- Further details on CFRA

- Discussion

- On the use of unified design for 3-step and 2-step CBRA

After RAN plenary discussion [1], it was determined that the 3-step CBRA (Solution 1 in the TR) is supported as baseline for A-IoT, and it is up to RAN2 to decide whether to support unified procedure for 3-step CBRA and 2-step CBRA (Solution 3 in the TR).

For the unified design for 3-step CBRA and 2-step CBRA, the general principles are summarized as follows,

1. Regardless of whether the device performs 2-step or 3-step CBRA, the device always sends 16-bit random number in A-IoT Msg1; 2. The device determines whether to send the data along with the random number based on the scheduled size of A-IoT Msg1. If larger than 16-bit, the device can include the data transmission in A-IoT Msg1;

Regarding the bullet 2 above, it is indeed an implicit indication based on the scheduled size of A-IoT Msg1 for the device to determine the CBRA type. That is to say, the unified solution (if supported) requires the device to support both 3-step and 2-step CBRA, which introduces additional complexity to the device for CBRA type distinction. From this perspective, we prefer to only support 3-step CBRA for A-IoT in this release.

Proposal 1:  Only 3-step CBRA is supported for A-IoT.

- Further details on CBRA

As per the current TR [2], the general framework of random access for slotted-ALOHA is as follows,

Figure 1: The general framework of random access for slotted-ALOHA

As can be seen from above Figure, the (subsequent) A-IoT paging is used to trigger a new access round. In the following, we further discuss some open issues related to slotted-ALOHA based on the modeling as shown Figure 1.

- A-IoT Msg1 transmission based on slotted-ALOHA

Whether to join the current access round

Upon receiving the A-IoT paging, it needs to be further confirmed that whether the device that needs to response this A-IoT paging should join the current access round. In RFID, the device firstly selects the round number, and then makes response upon the arrival of the corresponding round. For A-IoT, to reduce the device complexity, the most straightforward way is to just let all the devices that need to make response join the current access round, i.e., no need to further define the round selection mechanism. If the device suffers from access failure in the current round, it should perform re-access in the next round triggered by the following R2D message.

Proposal 2a: Upon receiving the A-IoT paging, all devices that need to make response join the current access round to perform CBRA.

Proposal 2b: If the device that joins the current round suffers from access failure, it should join the next round triggered by the subsequent A-IoT paging to perform re-access.

The configuration for slotted-ALOHA in A-IoT paging

During the SI discussion, it was agreed that upon receiving the (subsequent) A-IoT paging that needs a response, the device should perform random selection among the access occasions following the (subsequent) A-IoT paging message. Note that this also aligns with the RAN1 study captured in the TR 36.839 [2],

From the perspective of the physical layer, at least when a response is expected from multiple devices that are intended to be identified, an A-IoT contention-based access procedure initiated by the reader is used, for which at least slotted-ALOHA based access and FDMA, are studied. The study of FDMA includes how the frequency domain resources for Msg1 are allocated, and how a device determines that frequency-domain resource allocation. A R2D transmission triggering random access determines X time domain resource(s) for D2R transmission(s) for Msg1, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s).

Therefore, the (subsequent) A-IoT paging message should at least include the time domain resource configuration (i.e., the number of X access occasions following the (subsequent) A-IoT paging message) for A-IoT Msg1 transmission. Regarding how to support FDMA for A-IoT Msg1, it is still under RAN1 discussion, so we can further wait for RAN1 progress on the configuration related to FDMA for A-IoT Msg1.

Proposal 3: The (subsequent) A-IoT paging should indicate the number of X access occasions following the (subsequent) A-IoT paging. FFS on the configuration related to supporting FDMA for A-IoT Msg1.

- The design of A-IoT Msg2

On the content of A-IoT Msg2

During the SI discussion, we only confirmed that the random number for contention resolution is included in A-IoT Msg2. In the following, we further discuss other potential information that needs to be contained in A-IoT Msg2.

UL grant for A-IoT Msg3

To acquire the resource for A-IoT Msg3, similar as NR RACH, the A-IoT Msg2 should be able to include the UL grant assigned by the reader to the device identified by its random ID in A-IoT Msg1 for contention resolution. .

Proposal 4: The A-IoT Msg2 should include the UL grant assigned by the reader to the device identified by its random ID in A-IoT Msg1 for contention resolution.

Whether to have AS ID

According to the current TR 36.839 [2], it was captured in the data transmission section that,

The handling of data transmission failure has been studied. It is understood that the subsequent R2D data transmission after the D2R data transmission does not need to be always sent. In case of D2R data transmission failure, the A-IoT device follows the reader subsequent R2D instruction, if any. - For instance, the reader can repeat the R2D upper layer "command" to trigger the A-IoT device to re-send the same D2R upper layer "response" (i.e., the A-IoT device just follows the received R2D to transmit following D2R). - For instance, the reader can re-send A-IoT Msg2 to specific device(s) to echo the random ID(s), in case of failure reception of corresponding D2R data transmission (i.e. "Msg3") after the initial A-IoT Msg2, together with the corresponding D2R resource scheduling. This can trigger the A-IoT device to re-send the same D2R data transmision (i.e. "Msg3"). If it is supported to include the echoed random IDs for multiple devices, the re-sent A-IoT Msg2 only includes the random IDs of the devices, whose "Msg3" is not successfully received.

That means two candidate ways can be used for handling of D2R failure. One way is up to reader implementation to repeat the upper layer command to trigger the device to re-send the same response. The other is to re-send A-IoT Msg2 to the specific device to re-send the same response. No matter which way to use, the basic principle should be that only the “failed” device is triggered to re-send the same D2R message, instead of triggering all the devices to make response.

Observation 1: No matter which way to handle the D2R failure, only the device(s) that suffer from the D2R failure should be triggered to re-send the same D2R message.

To implement above mentioned purpose, the AS ID can be assigned by A-IoT Msg2 associated with the corresponding UL grant. By this way, if the reader detects the A-IoT Msg3 failure at a certain UL grant for a device, the reader can re-schedule this device using the AS ID associated with this UL grant, by re-sending the A-IoT Msg2. That means the AS ID is useful when the reader wants to re-send A-IoT Msg2 for the re-transmission of A-IoT Msg3 to the “specific” device. These are reflected by the following proposals.

Proposal 5a: For 3-step CBRA, support the reader to assign the AS ID in A-IoT Msg2 for the potential re-transmission of A-IoT Msg3 to the “specific” device(s).

Proposal 5b: If the reader detects the A-IoT Msg3 failure for a device at the corresponding UL grant assigned by A-IoT Msg2, the reader re-sends the A-IoT Msg2 to schedule the re-transmission of A-IoT Msg3 by including the AS ID assigned for this device.

Some further discussion related to the AS ID is covered in our contribution for the data transmission aspects in [3].

On the format of A-IoT Msg2

For the detailed format of A-IoT Msg2, RAN1 studied some candidate formats as follows [2]:

For Msg2 transmission in response to multiple Msg1 transmissions initiated by a R2D transmission triggering random access, the following options have been identified: Option 1: A PRDCH for Msg2 transmission corresponds to an A-IoT Msg1 received from one device. Option a: the starting time for Msg2 monitoring is separate for each Msg1 resource. Option b: the starting time for Msg2 monitoring is common for multiple Msg1 resources. Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices, the starting time for Msg2 monitoring is common for multiple Msg1 resources.

Therefore, we can further wait for RAN1 progress on whether an A-IoT Msg2 possibly corresponds to multiple A-IoT Msg1 from difference devices.

Proposal 6: For the detailed format of A-IoT Msg2, RAN2 to wait for RAN1 progress on whether it is possible that an A-IoT Msg2 corresponds to multiple A-IoT Msg1 from different devices.

- CBRA failure detection

According to the WID [1], the A-IoT paging should support addressing single device via one device ID, a group of devices via group ID or all devices via no identifier.

- A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs.

At least for the cases of paging a group of devices or paging all devices, since the subsequent A-IoT paging does not contain the specific device ID for the same service, the device should be aware of RACH failure, so that it can determine whether to respond to the subsequent A-IoT paging for re-access purpose.

Observation 2: For the cases of paging a group of devices or paging all devices, the device should be aware of RACH failure to determine whether to respond to the subsequent A-IoT paging for re-access.

There was below the RAN1 progress [2] related to the timing relationships:

TD2R_min: Minimum time between a D2R transmission and the corresponding R2D transmission following it. TD2R_max: Maximum time between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max], when a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device. See clause 6.3 for message descriptions.

Upon sending the A-IoT Msg1 for CBRA, the device is expected to receive the A-IoT Msg2 within [TD2R_min, TD2R_max]. Therefore, except for the case where the device suffers from the failure of contention resolution, the device should also perform re-access upon receiving the subsequent A-IoT paging for the same service, if the device does not receive A-IoT Msg2 within [TD2R_min, TD2R_max].

Proposal 7: The device considers the RACH failure, if it fails to receive the A-IoT Msg2 corresponding to its Msg1 within [TD2R_min, TD2R_max]. The device should perform re-access upon receiving the subsequent A-IoT paging for the same service.

Regarding when to start monitor A-IoT Msg2, it was discussed by RAN1 with the following assumptions [2]:

For Msg2 transmission in response to multiple Msg1 transmissions initiated by a R2D transmission triggering random access, the following options have been identified: Option 1: A PRDCH for Msg2 transmission corresponds to an A-IoT Msg1 received from one device. Option a: the starting time for Msg2 monitoring is separate for each Msg1 resource. Option b: the starting time for Msg2 monitoring is common for multiple Msg1 resources. Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices, the starting time for Msg2 monitoring is common for multiple Msg1 resources.

Therefore, we can also wait for further RAN1 progress in this regards.

Proposal 8: Wait for RAN1 further progress on the details related to the monitor of A-IoT Msg2.

- On the consideration of re-access for CBRA

Optional R2D message

Some further clarification needs to be further discussed related to the optional R2D message in Figure 1. According to SI discussion, some companies thought that the reader may observe a large number of devices joining the current round, which results in severe conflicts. Thus the reader may re-start a new round containing more access occasions by sending this optional R2D message. From our point of view, this optional R2D message can be the subsequent A-IoT paging, since each round triggered by the (subsequent) A-IoT paging can include different number of access occasions.

Proposal 9: It is up to reader implementation to stop the current round and trigger a new round with more access occasions by the subsequent A-IoT paging message. No need to introduce a separate R2D message for this purpose.

- Further details on CFRA

According to the WID for A-IoT paging [1],

- A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

In our understanding, in Rel-19, the A-IoT paging can include one explicit device ID. So the device can determine to perform CFRA if the device verifies only one explicit device ID is included in the A-IoT paging.

Proposal 10: In Rel-19, the device considers to perform CFRA, if the A-IoT paging only contains a single device ID corresponding to the device itself.

If the device determines to perform CFRA, the device will send the A-IoT data by the following D2R transmission. Therefore, the A-IoT paging should be able to provide the corresponding UL grant for the D2R data transmission, if the A-IoT paging only includes one explicit devcie ID.

Proposal 11: If the A-IoT paging only includes one explicit device ID, it should provide the UL grant for the corresponding D2R data transmission.

- Conclusion

In this paper, we discuss the RACH-related issues for A-IoT. The observations and proposals of this paper are summarized as follows,

Observation 1: No matter which way to handle the D2R failure, only the device(s) that suffer from the D2R failure should be triggered to re-send the same D2R message.

Observation 2: For the cases of paging a group of devices or paging all devices, the device should be aware of RACH failure to determine whether to respond to the subsequent A-IoT paging for re-access.

On the use of unified design for 3-step and 2-step CBRA

Proposal 1:  Only 3-step CBRA is supported for A-IoT.

Further details on CBRA

Proposal 2a: Upon receiving the A-IoT paging, all devices that need to make response join the current access round to perform CBRA.

Proposal 2b: If the device that joins the current round suffers from access failure, it should join the next round triggered by the subsequent A-IoT paging to perform re-access.

Proposal 3: The (subsequent) A-IoT paging should indicate the number of X access occasions following the (subsequent) A-IoT paging. FFS on the configuration related to supporting FDMA for A-IoT Msg1.

Proposal 4: The A-IoT Msg2 should include the UL grant assigned by the reader to the device identified by its random ID in A-IoT Msg1 for contention resolution.

Proposal 5a: For 3-step CBRA, support the reader to assign the AS ID in A-IoT Msg2 for the potential re-transmission of A-IoT Msg3 to the “specific” device(s).

Proposal 5b: If the reader detects the A-IoT Msg3 failure for a device at the corresponding UL grant assigned by A-IoT Msg2, the reader re-sends the A-IoT Msg2 to schedule the re-transmission of A-IoT Msg3 by including the AS ID assigned for this device.

Proposal 6: For the detailed format of A-IoT Msg2, RAN2 to wait for RAN1 progress on whether it is possible that an A-IoT Msg2 corresponds to multiple A-IoT Msg1 from different devices.

Proposal 7: The device considers the RACH failure, if it fails to receive the A-IoT Msg2 corresponding to its Msg1 within [TD2R_min, TD2R_max]. The device should perform re-access upon receiving the subsequent A-IoT paging for the same service.

Proposal 8: Wait for RAN1 further progress on the details related to the monitor of A-IoT Msg2.

Proposal 9: It is up to reader implementation to stop the current round and trigger a new round with more access occasions by the subsequent A-IoT paging message. No need to introduce a separate R2D message for this purpose.

Further details on CFRA

Proposal 10: In Rel-19, the device considers to perform CFRA, if the A-IoT paging only contains a single device ID corresponding to the device itself.

Proposal 11: If the A-IoT paging only includes one explicit device ID, it should provide the UL grant for the corresponding D2R data transmission.

- References

[1] RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

[2] 3GPP TR 38.769 “Study on solutions for ambient IoT (Internet of Things)” V2.0.1

[3] R2-2500272 “Discussion on the A-IoT Data Transmission and Other General Aspects” CATT