3GPP TSG-RAN WG2 Meeting #129	 R2-2500285

Athens, Greece, Feb 17-21, 2025

Agenda Item:	8.1.3

Source: 	NEC

Title:  	Discussion on NW-side data collection

Document for:	Discussion and Decision

- 1 Introduction

In last RAN2#128 meeting, the following agreements are made:

Agreements on NW side data collection - Focus on the following three radio condition event based logging - L3 serving cell measurement based (e.g. X1/X2 similar to A1/A2) - Beam based events (e.g. beam becomes top-1 beam and number of measurements is less than configured value) - L1 beam level measurement - Measurements on aperiodic CSI resources are not reported for NW sided data collection. - Data collection is controlled by the network. The UE will not autonomously stop when low power state is detected. - The UE reports to the network when the power state is low. We will not specify how the UE determines low power state. The network should de-configure the data collection (this can be captured in stage 2). - The UE reports to the network when buffer is or may become full. FFS when it reports (before and/or after). - The UE can report the reason for triggering of indication for the status (e.g. low power state, low memory). FFS how this is signalled and if the reporting can be part of availability indication.

In this contribution, we further discuss the remaining issue for NW-side data collection including availability indication, UE information procedure, etc.

- 2 Discussion

- 2.1 availability indication

In last meeting, RAN2 agreed that two causes are introduced to facilitate data collection logging and reporting, i.e., power state and buffer. In this section we further discuss more details.

There is FFS that how these causes are signalled, in our understanding, if the UE buffer is full, it is straightforward that the UE need to initiate availability indication to request NW to collect data, or if the UE power state is low, the UE need to request NW to stop data collection configuration, after that since the data collection will be de-configured by NW, the UE better to also initiate availability indication to request NW to collect data, otherwise it is not useful to keep storing training data without any data collection proceeding. In this sense, these two causes (i.e., full buffer and low power state) can be merged into availability indication instead of introducing two separate signalling which makes things a bit complicated.

Proposal-1: the causes, i.e., full buffer and low power state, are part of availability indication.

AS buffer event-based availability indication was discussed in RAN2 but no agreement is made yet, it means when the logged data size reach to UE buffer size limitation, the UE could send availability indication (instead of the logged data) to NW, then the follow-up on-demand reporting can be triggered by NW for collecting logged data. In our understanding, it is reasonable to consider such kind of combination between on-demand reporting and event-based reporting. First of all, compared with direct event-based data reporting, event-based availability indication can avoid NW resource overhead, meanwhile having buffer event make the UE trigger availability indication more precisely. Once the NW receive such event-based availability indication, it can facilitate NW decision on whether to initiate on-demand reporting. Therefore it is promising to support AS buffer event-based availability indication, i.e., when the logged data size reach to UE buffer size limitation, the UE send the availability indication to NW, then the NW can facilitate on-demand data collection. One more thing is how to signal availability indication, since it is a kind of UE assistance information, UAI is better to be considered.

Proposal-2: support event-based availability indication, once the logged data size reach to UE buffer limitation, the UE send availability indication via UAI.

- 2.2 UE information procedure

RAN2 has agreed that UE information procedure will be used for on-demand data reporting, in this section we further discuss more details.

In legacy, once receive UEInformationRequest from NW, the UE shall transmit UEInformationResponse including the recorded information (e.g., logged measurement) requested by NW. For data collection, the UE may have already logged a large amount of training data, one message may not be enough to carry all of training data at one time. But if NW send multiple UEInformationRequest messages to UE, signal overhead is increased, in this sense, the UE could send multiple UEInformationResponse to respond the corresponding UEInformationRequest, and each UEInformationResponse can indicate whether there is remaining training data available or indicate the remaining training data size. By this way, unnecessary signal overhead can be saved when a large amount of training data need to be reported.

Proposal-3: the UE can send multiple UEInformationResponse messages to respond the same one UEInformationRequest message when large amount of training data need to be reported and UEInformationResponse could include the remaining data availability indication or size.

- 2.3 others

Data collection and reporting may be a long term procedure which means the UE need to continuously log data and report data during a long time. For logging, the concern is from UE power state which can be solved by the above mentioned low power state indication. However for data reporting, it is related to NW uplink resource allocation, if large amount of UEs are operating training data reporting, it could cause NW uplink resource congestion, which impact on normal UL transmission for either SRB or DRB. Currently, RAN2 consider using a lower priority SRB to carry training data, but it still belong to signal radio bearer which is usually configured with a relatively higher priority than DRB, sometimes only modifying the priority is not enough. Therefore besides introducing lower priority SRB, lower layer priority handling can also be considered, e.g., MAC layer SR, BSR, and logical channel strategy can be considered.

Proposal-4: UL transmission priority handling (e.g., MAC layer) should be considered for training data reporting.

If the data collection is done, the NW could explicitly release the configuration. Besides explicit way, we think some mechanism from logged MDT can be referred, i.e., using maximum logging duration, or maximum data size required by NW could be considered to let UE release the configuration, in this way, NW-UE signalling interaction could be reduced. For example, the NW could configure the UE to perform data collection for specific duration, e.g., 3 hours at night, when duration is expired, the data collection configuration could be released by UE. Another example, the NW could configure the UE to perform data collection up to X MB data, and the configuration could be released once the UE has reported data up to X MB data.

Proposal-5: maximum logging duration or maximum data size can be considered to release data collection configuration, i.e., when the UE has already logged and/or reported data up to max duration/size, the UE release the configuration.

- 3 Conclusions

Proposal-1: the causes, i.e., full buffer and low power state, are part of availability indication.

Proposal-2: support event-based availability indication, once the logged data size reach to UE buffer limitation, the UE send availability indication via UAI.

Proposal-3: the UE can send multiple UEInformationResponse messages to respond the same one UEInformationRequest message when large amount of training data need to be reported and UEInformationResponse could include the remaining data availability indication or size.

Proposal-4: UL transmission priority handling (e.g., MAC layer) should be considered for training data reporting.

Proposal-5: maximum logging duration or maximum data size can be considered to release data collection configuration, i.e., when the UE has already logged and/or reported data up to max duration/size, the UE release the configuration.

- 4 References