3GPP TSG-RAN WG2 Meeting #129	R2-2500291

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.3.3

Source:	OPPO

Title:	Discussion on simulation results of measurement event prediction

Document for:	Discussion, Decision

Introduction

In RAN2#127bis, we agreed that

Measurement event prediction simulations will at least focus on intra-frequency FR2, case A, and second study goal (i.e. HO KPI improvement).   FFS what is KPI.

Companies can bring simulation results for intra-frequency measurement reduction for FR1 and report what they are doing.  Focus on temporal case B.

and

Companies will focus on sub-case 2 for measurement event prediction.   Companies can simulate other sub-cases if they wish and report what they are using.

In this contribution, we will present our simulation results for measurement event prediction, analysing both temporal domain case A and temporal domain case B.

This paper will provide both intermediate KPIs and system-level KPIs to compare the results of AI/ML methods against the legacy approach.

Given that RAN2 has agreed to focus on indirect prediction methodologies,

4	RAN2 focus on indirect prediction methodology.

no direct predictions will be included in this analysis.

Discussion

In RAN2#127bis, we have agreed that F1 score to be the metric for simulation evaluation. While there is no absolute standard for what constitutes a “good” F1 score, a widely accepted assumptions can be used as shown in Table 1.

Table 1: general assumption for evaluate performance with F1 score

| F1 score<0.7 | 0.7<=F1 score<0.9 | F1 score>=0.9 |
| --- | --- | --- |
| Poor performance | Acceptable performance | Good performance |

In this paper, we will reuse this assumption for performance evaluation. All the results are obtained through sliding filtering methods.

Measurement event prediction based on temporal domain case A

We have evaluated three different methods for calculating the intermediate KPIs of measurement event prediction in Case A. The primary distinction between the Legacy-driven prediction approach and SLS (system level simulation) approach lies in whether handover decisions are influenced by prediction results.

- For the legacy-driven prediction approach: Handovers are managed purely through legacy processes, while predictions are used solely to calculate the F1 score.

- For the SLS Approach: Prediction results directly influence handover decisions, integrating AI/ML predictions into the handover process. We simulated two handover options (i.e., Option 2 and Option 3) for the SLS approach.

The method for calculating intermediate KPIs differs accordingly. Below is an example illustrating this difference. Consider the current time instant is 0, and a real handover will occur at time instant 5.

- For the Legacy-driven prediction approach:

  - At time instant 0, if AI/ML predicts that a handover will happen at instant 5, the counter n3 (true prediction) increases by 1.

  - At time instant 1, if AI/ML again predicts that a handover will occur at instant 5, n3 increases by another 1.

  - This process repeats for each subsequent time instant where the same prediction is made, leading to multiple increments of n3.

- For the SLS Approach:

  - At time instant 0, when the handover at instant 5 is predicted, the UE initiates handover preparation and ceases further predictions until the handover is completed.

  - Consequently, n3 increases only once.

Therefore, the legacy-driven prediction approach records more true prediction events, leading to a higher F1 score compared to the SLS approach. This difference underscores the importance of the chosen method in evaluating the performance of AI/ML models for handover predictions.

For system-level simulations, two options for handover decisions are available:

Agreement: For measurement event prediction for temporal domain case A, company focus on option 2 or option 3.

Option 2: network transmit handover command purely based on actual measurement event regardless whether an actual measurement result(@t2) is earlier or later than predicted measurement event((@t1))

Option 3: For AI mobility, HO preparation starts when an event is predicted to happen (i.e., t0), and HO command is sent when A3 entering conditions are met based on actual/real measurement and an event is predicted to be met for the duration of TTT.

The table below provide a detailed comparison of precision, recall, and F1 scores for different methods used in prediction measurement event case A.

Table 2: Intermediate results of measurement event case A

| Method\metrics | Legacy-driven prediction approach | SLS option 2 | SLS option 3 |
| --- | --- | --- | --- |
| Precision | 0.97 | 0.91 | 0.93 |
| Recall | 0.97 | 0.88 | 0.90 |
| F1 | 0.97 | 0.90 | 0.91 |
| Average L3 cell RSRP difference (dB) | 0.68 |  |  |
| Last predicted point L3 cell RSRP difference (dB) | 1.41 |  |  |

All three methods demonstrate high accuracy in predicting measurement events, with F1 scores exceeding 0.9. In the context of Table 1, it means the prediction is good. Such high F1 scores indicate that these models are effective in identifying true positives while maintaining low rates of false positives and false negatives.

Observation 1: Measurement event case A: The prediction of measurement event is pretty good with F1 score consistently above 0.9.

One reason for the higher F1 score of legacy-driven prediction approach compared to the SLS options could be attributed to how true predictions are counted. In the context of legacy-driven prediction approach, it appears that certain true predictions may be counted multiple times, leading to an inflated performance metric. This practice can skew the results, making legacy-driven prediction approach appear more accurate than it might actually be under conditions where each prediction is counted only once.

Observation 2: Measurement event case A: Legacy-driven prediction approach (i.e. handovers are managed purely through legacy processes, while predictions are used solely to calculate the F1 score.) achieves a higher F1 score than SLS method (i.e., prediction results directly influence handover decisions), because the number of true predictions has been counted repeatedly.

While not as high as legacy-driven prediction approach, both SLS Option 2 and SLS Option 3 still achieve respectable F1 scores of 0.90 and 0.91, respectively. In the following table, we further check the system-level performance (in terms of HO failure rate) of these two SLS approaches and here are the results.

Table 3: System-level results of measurement event case A

| Method | HO failure rate (%) | HO attempts per UE per second |
| --- | --- | --- |
| baseline | 28.96 | 0.33 |
| SLS option 2 | 28.42 | 0.38 |
| SLS option 3 | 24.20 | 0.34 |

Both AI-driven options show a reduction in HO failure rates compared to the baseline method. Specifically, SLS Option 3 demonstrates the most significant improvement with a failure rate of 24.20%, representing a decrease of 4.76 percentage points from the baseline's 28.96%. This indicates that AI mechanisms can enhance the reliability of handovers, leading to more stable and efficient network performance.

Observation 3: Measurement event case A: AI/ML performs better than baseline (legacy) in terms of HO failure rate. Compared to the baseline, handover decision option 3 reduces HO failure rate from 28.96% to 24.20%.

The improved performance of AI-driven handover mechanisms highlights the potential benefits of integrating advanced AI algorithms into network management systems. Among the AI options, Option 3 outperforms Option 2 in terms of HO failure rate, achieving a lower rate of 24.20% compared to Option 2's 28.42%.

Observation 4: Measurement event case A: AI/ML prediction with handover decision option 3 outperforms option 2.

One possible explanation for this observation is that both Option 2 and Option 3 provide robust predictions of TTT satisfaction. However, Option 3 has a distinct advantage: it can issue the handover command as soon as the neighbouring cell's channel quality surpasses that of the serving cell. This immediate action reduces the likelihood of handover failure.

Measurement event prediction based on temporal domain case B

The previous agreement that measurement event prediction will focus on sub case 2 was initially proposed for Case A. However, when companies attempt to implement Case B, an issue arises: L3 measurements cannot be skipped at arbitrary intervals. To address this challenge, during RAN2#128, we reached a consensus that:

3 Company can report which filtering options is being used for the input L3 RSRP of sub-use case 2: option 1, option 2, option 3.

Here are the details of each filtering option:

- Filtering option 1: L3 filtering is based on its L1 filtered result and the immediate last skipped measurement result ;

- Filtering option 2: L3 filtering is based on its L1 filtered result i.e. no L3 filtering if the immediate last result is skipped;

- Filtering option 3: L3 filtering is based on the L1 filtered result and last actual measurement result i.e. the skipped result(s) in between is ignored.

Among the three filtering options for sub case 2, filtering option 1 may not be practical. Filtering option2 actually use no L3 filtering and the channel variation may deteriorate the handover performance. We believe filtering option 3 is the most practical one among the three.

In this section, we stimulated two methods for the measurement event prediction of temporal domain case B, i.e.,

- Sub case 1

- Sub case 2 with filtering option 3

The table below summarizes the performance of two sub-cases under different Max ETD:

Table 4: Intermediate results of measurement event case B

| Method | Sub case 1 |  | Sub case 2 with filtering option 3 |  |
| --- | --- | --- | --- | --- |
| Max ETD (ms) | 80 | 200 | 80 | 200 |
| Precision | 0.77 | 0.88 | 0.68 | 0.87 |
| Recall | 0.76 | 0.87 | 0.66 | 0.85 |
| F1 | 0.76 | 0.88 | 0.67 | 0.86 |
| Average L3 cell RSRP difference (dB) | 0.53 |  | 0.69 |  |

When the Max ETD is set at 80ms, sub case 1 achieves an F1 score of 0.76, which is considered acceptable performance (please refer to Table 1). In contrast, sub case 2 with filtering option 3 performs poorly with an F1 score of 0.67. This indicates that sub case 1 outperforms sub case 2 with filtering option 3 under this condition, suggesting that the filtering option 3 may not be as effective in this scenario.

Observation 5: Measurement event case B: When max ETD = 80ms, sub case 1 achieves an F1 score of 0.76 (acceptable performance), while sub case 2 with filtering option 3 reaches a lower F1 score of 0.67, indicating poorer performance. Sub case 1 demonstrates superior performance compared to sub case 2 with filtering option 3.

When the max ETD is increased to 200ms, both sub-cases achieve relatively high F1 scores exceeding 0.85. This suggests that AI/ML can effectively detect measurement events but may introduce some deviation between the actual and predicted event times.

Observation 6: Measurement event case B: When max ETD relaxed from 80ms to 200ms, both sub case 1 and sub case 2 with filtering option 3 achieve a relatively high F1 score (higher than 0.85). It means the sub case 2 has an higher ETD compared to sub case 1.

Table 4 shows a relatively lower F1 score compared to measurement evet case A as shown in Table 2. This discrepancy is intriguing, especially considering that the mean L3 cell RSRP difference between cases A and B is quite similar. An explanation can be derived from Figure 1, where solid dots represent the measured RSRP (ground-truth) values, and hollow dots denote the predicted RSRP values generated by AI/ML.

Let’s assume the average prediction error for cases A and B is comparable. For case A, the prediction errors across the three time instances are [0.1dB, 0.7dB, 1dB], whereas for case B, the prediction errors are [0.6dB, 0.5dB] at time instances 1 and 3, respectively. Despite the average RSRP being the same between the two cases, the judge of events diverges.

In case A, the impact of prediction errors appears minimal as instants 1, 2, and 3 all satisfy the A3 entering condition (A3-offset=2dB). Although the prediction error increases over time, the difference in RSRP between the serving cell and the neighboring cell grows at a faster rate. This rapid increase in RSRP discrepancy effectively accommodates the increment of prediction error, mitigating its impact on the overall performance. Conversely, in case B, the error at time instant 1 makes the prediction falsely indicating that the instant does not follow A3 entering condition. This delays the declaration of a TTT satisfactory compared to the actual event occurrence in case B, explaining why the F1 score increases much when Max ETD extends from 80ms to 200ms.

Figure 1: Illustration for F1 score difference between measurement event case A and case B

Observation 7: Measurement event case A: The increasing discrepancy in RSRP between the neighbouring cell and the serving cell can accommodate the increment of prediction error over time, thereby facilitating the precise prediction of measurement events.

Observation 8: Measurement event case B: A relatively even but high prediction error tends to delay the declaration of TTT satisfaction, indicating a potential mismatch between prediction and actual event timing.

By comparing cases A and B in Figure 1 and Tables 2 and 4, we can see that case A with 0.6dB average RSRP difference achieves a better F1 score than case B with 0.55 dB average RSRP difference. That implies that for case B to achieve an F1 score comparable to case A, it needs a higher prediction error (e.g., 0.1 dB) than case A.

Observation 9: For measurement event prediction, case B requires a higher prediction accuracy than case A in average to correctly predict the exact occurrence time of the event.

Besides intermediate KPI, we also evaluated system-level performance as shown in Table 5.

Table 5: System-level results of measurement event case B

| Method | HO failure rate (%) | HO attempts per UE per second |
| --- | --- | --- |
| baseline | 35.56 | 0.045 |
| Sub case 1 | 36.09 | 0.045 |
| Sub case 2 with filtering option 3 | 41.46 | 0.044 |

The HO failure rate for sub case 1 is very close to the baseline (36.09% vs. 35.56%), indicating minimal impact on system-level performance despite the introduction of measurement event predictions. Conversely, sub case 2 with filtering option 3 shows a significant degradation in system-level performance, with a 5.9% increase in HO failure rate compared to the baseline. This suggests that the filtering option 3 introduces additional errors or inefficiencies that negatively affect the overall system performance.

Observation 10: Sub case 1, with a 50% MRRT, achieves a HO failure rate very close to the baseline, indicating minimal degradation (0.53%) in system level performance. In contrast, Sub case 2 with filtering option 3 shows a 5.9% degradation in HO failure rate.

Taking observations 5 and 10 into account, it is clear that sub case 1 outperforms sub case 2 in multiple aspects. Our previous agreement was that

Companies will focus on sub-case 2 for measurement event prediction.   Companies can simulate other sub-cases if they wish and report what they are using.

This agreement is initially proposed for case A study and is not designed for case B. Recognizing that there are potentially more promising sub cases than sub case 2, it is advisable to explore other sub cases to fully understand their impact and performance. Given these considerations, the following proposal are recommended:

Proposal 1: For measurement event prediction in temporal domain case B, companies are encouraged to explore other sub cases, e.g., sub case 1.

Conclusion

In this contribution, we have shared our simulation results on RRM measurement prediction and have the following observations and proposals:

Measurement event prediction based on temporal domain case A:

Observation 1: Measurement event case A: The prediction of measurement event is pretty good with F1 score consistently above 0.9.

Observation 2: Measurement event case A: Legacy-driven prediction approach (i.e. handovers are managed purely through legacy processes, while predictions are used solely to calculate the F1 score.) achieves a higher F1 score than SLS method (i.e., prediction results directly influence handover decisions), because the number of true predictions has been counted repeatedly.

Observation 3: Measurement event case A: AI/ML performs better than baseline (legacy) in terms of HO failure rate. Compared to the baseline, handover decision option 3 reduces HO failure rate from 28.96% to 24.20%.

Observation 4: Measurement event case A: AI/ML prediction with handover decision option 3 outperforms option 2.

Observation 7: Measurement event case A: The increasing discrepancy in RSRP between the neighboring cell and the serving cell can accommodate the increment of prediction error over time, thereby facilitating the precise prediction of measurement events.

Measurement event prediction based on temporal domain case B:

Observation 5: Measurement event case B: When max ETD = 80ms, sub case 1 achieves an F1 score of 0.76 (acceptable performance), while sub case 2 with filtering option 3 reaches a lower F1 score of 0.67, indicating poorer performance. Sub case 1 demonstrates superior performance compared to sub case 2 with filtering option 3.

Observation 6: Measurement event case B: When max ETD relaxed from 80ms to 200ms, both sub case 1 and sub case 2 with filtering option 3 achieve a relatively high F1 score (higher than 0.85). It means the sub case 2 has an higher ETD compared to sub case 1.

Observation 8: Measurement event case B: A relatively even but high prediction error tends to delay the declaration of TTT satisfaction, indicating a potential mismatch between prediction and actual event timing.

Observation 9: For measurement event prediction, case B requires a higher prediction accuracy than case A in average to correctly predict the exact occurrence time of the event.

Observation 10: Sub case 1, with a 50% MRRT, achieves a HO failure rate very close to the baseline, indicating minimal degradation (0.53%) in system level performance. In contrast, Sub case 2 with filtering option 3 shows a 5.9% degradation in HO failure rate.

Proposal 1: For measurement event prediction in temporal domain case B, companies are encouraged to explore other sub cases, e.g., sub case 1.