3GPP TSG-RAN WG2 Meeting #129	R2-2500293

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.7.4.2

Source: 	Canon Research Centre France

Title:	Discussion on Delay status report

Document for:	Discussion and decision

# Introduction

During RAN2#128 in Orlando the following agreements related to DSR were recorded in [1]:

Thresholds configuration Let the network configure the triggering and reporting thresholds without constraints. RAN2 understanding is that the data that has been already reported in the DSR should not trigger another DSR The existing cancelling and triggering of Rel-18 DSR is reused for the enhanced DSR. Non-delay critical data ahead in the queue The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

# Discussion

In this paper we discuss the MAC CE design:

We propose the following format for the MAC CE:

| LCG7 | LCG6 | LCG5 | LCG4 | LCG3 | LCG2 | LCG1 | LCG0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S7 | S6 | S5 | S4 | S3 | S2 | S1 | S0 |
| New 1 | Delay 1/buffer size 1 |  |  |  |  |  |  |  |
| Buffer Size 1 |  |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |
| New m | Delay m/buffer size m |  |  |  |  |  |  |  |
| Buffer Size m |  |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |
| S7 | S6 | S5 | S4 | S3 | S2 | S1 | S0 |
| New 1 | Delay 1/buffer size 1 |  |  |  |  |  |  |  |
| Buffer Size 1 |  |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |
| New m | Delay m/buffer size m |  |  |  |  |  |  |  |
| Buffer size m |  |  |  |  |  |  |  |  |

Figure 2-1 Enhanced DSR MAC CE

In detail, the message is composed of:

- LCGi: LCGs flags similar to legacy DSR

- Si: For each LCG, section bitmap indicates the presence of the delay status for each section

- New: For at least one, or each section, indicates whether the section is new or existing. If the section is new, then the associated delay information is reported. If the section is not new (i.e., existing) then the delay information is not included

- Delay: Similar to legacy Remaining time

- Buffer size: Similar to legacy buffer size

Proposal 1: Reuse legacy DSR LCG field definition

Proposal 2: For each LCG a bit map Si identifies the reported pairs

Proposal 3: For each reported pair, a bit (new) indicate if it is the first time that the pair is reported

Proposal 4: do not report remaining time if the new bit is set to 0

Proposal 5: Reuse legacy remaining time field definition

Proposal 6: Reuse legacy buffer size field definition

# Conclusion

According to the discussion in section 2, it is proposed:

Proposal 1: Reuse legacy DSR LCG field definition

Proposal 2: For each LCG a bit map Si identifies the reported pairs

Proposal 3: For each reported pair, a bit (new) indicate if it is the first time that the pair is reported

Proposal 4: do not report remaining time if the new bit is set to 0

Proposal 5: Reuse legacy remaining time field definition

Proposal 6: Reuse legacy buffer size field definition

# References

- R2-2410915	Report from session on R18 MBS, R18 QoE and R19 XR	Session chair (Huawei)