3GPP TSG-RAN WG2 Meeting #129	R2-2500296

Athens, Greece, 17-21 Feb 2025

Agenda Item:	8.1.5

Source:	NEC

Title:	Signalling feasibility of dataset and parameter sharing

Document for:	Discussion, Decision

# Introduction

According to RAN1 LS (R1-2410922), RAN2 is expected to evaluate feasibility of the RAN1 identified solutions on two-sided model for CSI compression. This paper serves for this purpose.

# Discussion

RAN1 has been discussing various ways for alleviating/resolving the issues related to inter-vendor training collaboration to support the use case of CSI compression(two-sided model). In particular, RAN1’s discussion requires the following procedure:

- NW-side shares model parameters and/or dataset to the UE or UE-side via standardized signaling, where the model parameters and/or dataset received at the UE or UE-side go through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., model (re-)training.

RAN1 is considering the following Sub-Options for further study with potential further down-selection.

- Option 4-1: sharing dataset containing N1 samples of {target CSI, CSI feedback} corresponding to the input and output of a nominal encoder, respectively

- Option 3a-1 without target CSI: sharing parameters corresponding to a nominal encoder

- Option 3a-1 with target CSI: sharing parameters corresponding to a nominal encoder, along with dataset containing N2 samples of {target CSI}

According to RAN1 LS in R1-2410922, the size of the dataset and model parameters are as below:

- Option 4-1: sharing {target CSI, CSI feedback} dataset:  around 225MB

- Option 3a-1 without target CSI: sharing encoder parameter: around 11.6 MB

- Option 3a-1 with target CSI: sharing encoder parameters +{target CSI} dataset: 11.6MB+data set

RAN1 concluded that a standardized signaling, if feasible and specified, can be used for parameter and/or dataset exchange to alleviate/resolve the inter-vendor training collaboration complexity.

RAN1 concluded that the standardized signaling may be Over-the-air or other approaches.

At first, our understanding is that the required standardized signaling is between the gNB and the UE over the air interface. It should be noted that currently at most of the cases, the RRC messages sent from the gNB to UE is less than 9000 bytes. However, some extremely large RRCReconfiguration or RRCResume message may outsize (e.g. for the RRC message carrying measurement configuration), in which case, downlink RRC message segmentation may be required to fit into a single PDCP SDU. Taking account of the possible RRC message segmentation, the maximum signalling transferred from gNB to the UE should be less than 144K in general, since the current maximum segments allowed is 16 over DL DCCH. With this said, the support of option 3a-1 sharing parameters corresponding to a nominal encoder (up to 11.6MB) from the gNB to the UE is not realistic by pure RRC signalling if the current NR L2 design is maintained. The essential reason here is a RRC signalling with a size up to 11.6MB may require unreasonable amount of message segmentations in order for one segment to fit into a single PDCP SDU. Needless to say on the support of Option 4-1 sharing dataset (with a size up to 225MB).

Proposal-1: It is not realistic to support the transmission of sharing dataset (with a size up to 225MB) or sharing parameters corresponding to a nominal encoder (up to 11.6MB) from the gNB to the UE by RRC signalling if the current NR L2 design is maintained.

Meanwhile, we think that sharing dataset (with a size up to 225MB)or sharing parameters corresponding to a nominal encoder (up to 11.6MB) from the gNB to the UE via data transmission tunnel (UP) may be possible. However traditionally the UP data tunnel is established between the UE and UPF in the 5GC. RAN2 need to either work on a solution to allow the gNB to establish a UP tunnel to be supported by a DRB between the UE and gNB, or otherwise, the gNB may need to send the dataset or parameters to the core network, that is able to distribute the content to the UE via traditional UP tunnel. In this manner, the sharing dataset or the sharing parameters will be transmitted as pure data, not signalling.

Proposal-2: User plane based solution can be used to support the transmission of sharing dataset or sharing parameters corresponding to a nominal encoder from the gNB to the UE.

Proposal-3: the gNB sends the dataset or parameters to the core network, that is able to distribute the content to the UE via traditional UP tunnel.

Considering the fact that sharing the data set and the parameter for AIML training from the network may apply to multiple UEs, there may be a possibility to use broadcast or multicast based transmission over the air. In addition, as the transmission of the data set and the parameter for AIML training takes a long time and the delay requirement is very low for such transmission. Hence there may be possible for the UE to receive such data set and the parameter when transiting to RRC Idle or RRC Inactive state.

Proposal-4: the UE may receive the data set and the parameter for AIML training via broadcast or multicast based reception.

Proposal-5: the UE can receive the data set and the parameter for AIML training during RRC Idle or RRC Inactive state.

# Conclusion and Proposal

We have the following proposals:

Proposal-1: It is not realistic to support the transmission of sharing dataset (with a size up to 225MB) or sharing parameters corresponding to a nominal encoder (up to 11.6MB) from the gNB to the UE by RRC signalling if the current NR L2 design is maintained.

Proposal-2: User plane based solution can be used to support the transmission of sharing dataset or sharing parameters corresponding to a nominal encoder from the gNB to the UE.

Proposal-3: the gNB sends the dataset or parameters to the core network, that is able to distribute the content to the UE via traditional UP tunnel.

Proposal-4: the UE may receive the data set and the parameter for AIML training via broadcast or multicast based reception.

Proposal-5: the UE can receive the data set and the parameter for AIML training during RRC Idle or RRC Inactive state.