3GPP TSG-RAN WG2 Meeting #129	R2-2500297

Athens, Greece, 17-21 Feb 2025

Agenda Item:	8.2.3

Source:	NEC

Title:	Random Access for Ambient IoT device

Document for:	Discussion, Decision

# Introduction

A new WID was approved in RP-243326 [1] for Solutions for Ambient IoT (Internet of Things) in NR in RP#106. The following aspects of A-IoT Random access is included in the corresponding RAN2 scope within the WID:

A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129).

This paper intends to discuss detailed design of A-IoT Random access following the progress made during the study phase of A-IoT.

# Discussion

## Unified solution for contention-based random access

Following the objective of A-IoT Random access, during RAN2#129, RAN2 need to decide if a unified solution should be used for contention-based random access.

The A-IoT device determines the random access type from the A-IoT paging message. If the random access is contention-based random access, the A-IoT device performs access occasion/resource selection.

There are three candidate solutions as below studied for the contention resolution during AIoT study item:

-	Solution 1: A-IoT Msg1 without data

-	A-IoT Msg1: When the A-IoT device identifies the start of its own access occasion, it sends one 16-bit random ID generated by the A-IoT device to the reader.

-	A-IoT Msg2: The reader responds with the successfully received random ID.

If the A-IoT device receives the A-IoT Msg2 including a random ID, which is the same as the previously transmitted one in A-IoT Msg1, it considers the contention resolution as successful.

-	Solution 2: A-IoT Msg1 with data

-	A-IoT Msg1: When the A-IoT device identifies the start of its own access occasion, it sends the A-IoT Msg1 including the upper layer data, which can be the device ID and/or any other upper layer data, in addition to one 16-bit random ID generated by the A-IoT device to the reader.

-	A-IoT Msg2: The reader may respond with the successfully received random ID.

If the A-IoT device receives the A-IoT Msg2 including a random ID, which is the same as the previously transmitted one in A-IoT Msg1, it considers the contention resolution as successful. If the A-IoT Msg2 is not received by the device, the re-access is not autonomously performed i.e., the re-access is always controlled by the reader.

-	Solution 3: A-IoT Msg1 optionally includes data (unified solution supporting both Solution 1 and 2)

-	A-IoT Msg1: When the A-IoT device identifies the start of its own access occasion, it sends one 16-bit random ID generated by the A-IoT device to the reader. In addition, it is controlled by the reader on whether to also include the upper layer data, which can be the device ID and/or any other upper layer data.

-	A-IoT Msg2: The reader responds with the successfully received random ID.

If the A-IoT device receives the A-IoT Msg2 including a random ID, which is the same as the previously transmitted one in A-IoT Msg1, it considers the contention resolution as successful.

Firstly, as summarized in the TR[2], Solution 1 can support relatively large coverage for A-IoT Msg1 than Solution 2, whereas Solution 2 has the message number efficiency in case of low probability of A-IoT Msg1 collision (e.g. it can reduce one step for data transmission interaction in case of success).

Secondly, as discussed and replied in the LS S2-2501241/R2-250062, the latency related requirements associated with the completion time of a given A-IoT service is still under discussion. If the latency related requirement is agreed, only solution 2 can meet such latency requirement. Hence, we think that the unified solution (covering solution 1 and solution 2) is more forward compatible.

Thirdly, in our understanding, if the A-IoT device can properly indicate its support of solution 1 (3-step random access) or solution 2 (2-step random access) via an indication in MSG1 header, for example, indicating whether the upper layer data is included or not in MSG1, then the reader will know the format of MSG1 accordingly. In this manner, the A-IoT device can support one solution, i.e., either solution 1 (3-step random access) or solution 2 (2-step random access), or both solutions. Hence, there is no increase on the complexity of A-IoT implementation by specifying the unified solution.

Considering above, we think the unified solution covering both Solution 1 and Solution 2 for the contention resolution is the best way forward for A-IoT random access for achieving a simple and forward compatible design without increasing the complexity of A-IoT implementation.

Proposal-1: The unified solution (i.e., Solution-3) covering both Solution 1 and Solution 2 is used for the contention resolution of A-IoT contention-based random access.

In addition, the A-IoT reader may be not in a best position to decide whether any upper layer data should be carried by A-IoT devices in MSG1 since it is lack of the knowledge of the coverage of A-IoT device, and it may not know the latency requirements of the A-IoT services. Different from the reader, A-IoT devices have the full knowledge on its own coverage situation and may also know the latency requirements of the A-IoT services, therefore we propose to support the A-IoT devices in determining whether to include upper-layer data in Msg1 and indicate it via MSG-1 header.

Proposal-2: Support the A-IoT devices in determining whether to include upper-layer data in Msg1 and indicate it via MSG-1 header.

## Contention-free random access

As discussed during study phase of Ambient IoT, for contention-free based access, the A-IoT device will just select the indicated D2R occasion/resource and performs data transmission directly by skipping the contention resolution step which needed for contention-based random access.

During the study phase, it was also studied for the possible unification between the above contention-free access and contention-based random access, from the device perspective. In the unified solution, when the A-IoT device identifies the start of its own access occasion, it sends at least one 16-bit random ID generated by the A-IoT device to the reader (regardless of the contention-free or contention-based access occasion).

In our understanding, the motivation to include the random ID in contention-free access is for the purpose of subsequent D2R scheduling and R2D reception, i.e., support subsequent D2R scheduling and R2D reception via the random ID as the “AS ID”. However, as captured in the TR, the disadvantage of including the random ID only in contention-free access is that the reader may not know whether the intended device is responding on the given resources or not. If the unified solution between contention-free access and contention-based random access is supported, to avoid the above disadvantage, we think in the unified solution when the A-IoT device identifies the start of its own access occasion for contention-free access, it shall send both 16-bit random ID generated by the A-IoT device and the device ID together to the reader.

Observation-1: If the unified solution between contention-free access and contention-based random access is supported, when the A-IoT device identifies the start of its own access occasion for contention-free access, it sends both 16-bit random ID generated by the A-IoT device and the device ID together to the reader.

Since the motivation of including the random ID in contention-free access is for the purpose of subsequent D2R scheduling and R2D reception, whether to support the unified solution also depends on the discussion of the “AS ID”. So far, regarding the “AS ID”, there are still 3 options on the table [2].

-	Option 1: a random ID (if used in first D2R message) can be reused;

-	Option 2: the reader assigns this "AS ID" to the device. It needs to be further discussed via which R2D message.

-	Option 3: It is up to the reader whether to reuse the random ID (if used in first D2R message) as the "AS ID" or to assign a new "AS ID". It needs to be further discussed via which R2D message.

When Option 1 is supported, then the unified solution (with sending both 16-bit random ID and the device ID together) may be preferred. On the other hand, if Option 2 or Option 3 is supported, when the A-IoT device identifies the start of its own access occasion for contention-free access, it just sends device ID to the reader.

Proposal-3: If a random ID in the first D2R message is agreed to be used as “AS ID” for subsequent D2R scheduling and R2D reception, support the unified solution between the contention-free access and contention-based random access (i.e., including both 16-bit random ID and the device ID in MSG-1).

## RACH resource selection for Ambient IoT device

In order to select a time slot to respond to the reader via random access procedure, the AIoT device needs to know the exact time slots that can be used to initiate its random access. In addition, according to the discussion at RAN1, the FDM approach was also discussed on top of the TDM based access. FDM based approach may be used to afford large number of A-IoT device access, which allows different devices to access the reader via a different frequency bandwidth at the same time slot. Based on this background, we discuss below what type of parameters the reader should announce before the A-IoT device can initiate its random access.

The initial triggering message (i.e., A-IoT paging message) sent by the reader should include a set of access occasions allocated for RA (e.g. M time domain resources and N frequency domain resource) within one access round.

Firstly, we focus on the discussion of time domain aspects. Assuming intended devices information (i.e., filter criteria from CN) is also provided in the initial trigger message, each device matching the filter criteria may generate a random number according to the number of the available time resources (e.g. M) provided in the initial triggering message for random access. The random number should be in the range [0, (M-1)].

Proposal-4: Each A-IoT device matching the filter criteria generates a random number for time resource determination according to the number of the available time resources provided in the initial triggering message for random access.

Based on the random number generated, the A-IoT device may start “RACH-time resource” counting down since receiving “initial triggering message” command (i.e., A-IoT paging message).

Proposal-5: A-IoT device starts “RACH-time resource” counting down after randomly generating a random number for time resource determination.

However, how devices to determine its transmission/access occasion is still unclear.

In RFID, within an access round triggered by “Query” command, determination of transmission/access occasion is based on reception of “QueryRep” command which instructs devices to decrement their slot counters and, if slot count=0 after decrementing, to backscatter an RN16 to the reader immediately.

During the study phase, RAN1 studied TDMA for MSG-1transmissions from multiple devices corresponding to a PRDCH providing a triggering message. Followings are captured in Clause 6.1.4 [2].

A R2D transmission triggering random access determines X time domain resource(s) for D2R transmission(s) for Msg1, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s).

The study includes:

-	X=1 and X>1 and X>=1. The maximum value of X>1 should be set considering the device implementation complexity, device power consumption, the resource usage efficiency affected at least by SFO, and inventory latency.

-	Size(s) for resource allocation in the time domain.

-	Determination of the X time domain resource(s) by the device.

-	Addressing timing errors for adjacent time domain resources due to residual SFO of the device.

In our understanding, A-IoT paging message acts as “Query” command in A-IoT, a “QueryRep”-like R2D message (we call it access occasion start indication message since it is used to indicate the start of a new time resource for random access) is needed for determination of transmission/access occasion. With this access occasion start indication message, RFID-like mechanism for determining device’s own access occasion can be reused.

Proposal-6: Support access occasion start indication message (like “QueryRep” command in RFID) for determination of device’s own access occasion in time domain.

Proposal-7: The access occasion start indication message instructs devices to decrement their slot counters and, if slot count=0 after decrementing, to backscatter an RN16 to the reader immediately.

Proposal-5 and Proposal-6 can be considered as the baseline and RAN2 can further discuss necessary mechanism for X>1 and X>=1 cases after RAN1 making further progresses.

In addition to the RA time resource selection, the device also needs to randomly selects one frequency resource among the available frequency resources (e.g. N) in the initial triggering message for sending MSG-1 to the reader.

Proposal-8: In addition to the RA time resource selection, the device also needs to randomly select one frequency resource among the available frequency resources (e.g. N) in the initial triggering message for sending MSG-1 to the reader.

Meanwhile, it is very important to note that when there are multiple frequency domain resources for performing RA, RAN2 may need to study additional mechanisms or enhancements for AIoT device access, since anyway the slotted ALOHA mechanism cannot be simply reused any more.

As discussed above, an RO in A-IoT can be defined as a combination of a time resource and a frequency resource. In some scenarios, the neighbor readers may allocate different combinations of time and frequency resource as available resources to devices for interference controlling and/or other purposes. As captured in TR [2], “as to the A-IoT paging message, it can additionally indicate the information from which the device(s) can determine the resource(s) to be used for D2R response message(s)”, it is nature to assume that the total available ROs (for example, a sub set from M time resources and N frequency resources) is indicated by A-IoT paging message.

Proposal-9: The total available ROs (including a sub set in both time and frequency resources) is indicated by A-IoT paging message.

# Conclusion and Proposal

We have the following observations and proposals:

Observation-1: If the unified solution between contention-free access and contention-based random access is supported, when the A-IoT device identifies the start of its own access occasion for contention-free access, it sends both 16-bit random ID generated by the A-IoT device and the device ID together to the reader.

Proposal-1: The unified solution (i.e., Solution-3) as captured in the TR38.769 is used for the contention resolution of AoT contention-based random access.

Proposal-2: Support the A-IoT devices in determining whether to include upper-layer data in Msg1 and indicate it via MSG-1 header.

Proposal-3: If a random ID in the first D2R message is agreed to be used as “AS ID” for subsequent D2R scheduling and R2D reception, support the unified solution between the contention-free access and contention-based random access (i.e., including both 16-bit random ID and the device ID in MSG-1).

Proposal-4: Each A-IoT device matching the filter criteria generates a random number for time resource determination according to the number of the available time resources provided in the initial triggering message for random access.

Proposal-5: A-IoT device starts “RACH-time resource” counting down after randomly generating a random number for time resource determination.

Proposal-6: Support access occasion start indication message (like “QueryRep” command in RFID) for determination of device’s access occasion in time domain.

Proposal-7: The access occasion start indication message instructs devices to decrement their slot counters and, if slot count=0 after decrementing, to backscatter an RN16 to the reader immediately.

Proposal-8: In addition to the RA time resource selection, the device also needs to randomly select one frequency resource among the available frequency resources (e.g. N) in the initial triggering message for sending MSG-1 to the reader.

Proposal-9: The total available ROs (including a sub set in both time and frequency resources) is indicated by A-IoT paging message.

# Reference

- RP-243326 New WID for Ambient IoT

- TR38.769