3GPP TSG-RAN WG2 Meeting #129	R2-2500300

Athens, Greece, 17 –21 Feb 2025

Agenda Item:	8.13.3

Source:	NEC

Title:	Control Plane aspects for Multi-hop Relay

Document for:	Discussion, Decision

# Introduction

In RAN2#128 meeting, the following agreements were reached for the basic aspects of Control Plane for Multi-hop Relay:

In one approach (“approach 1”) of U2N relays, each of the Intermediate Relay UEs must be in RRC_CONNECTED when the U2N remote UE is in RRC_CONNECTED.  Connection establishment in the U2N remote UE first requires that each Intermediate Relay UE which is in RRC_IDLE/RRC_INACTIVE first enters RRC_CONNECTED.  FFS whether connection establishment of an Intermediate Relay UE (other than the Last Relay UE) is captured in specification as connection establishment of a remote UE or a relay UE.  FFS if SRB0 would be forwarded before connection establishment of each intermediate relay UE.

The figure and description under P1 of R2-2410006 serves as a baseline connection establishment procedure for discussion for multi-hop U2N Relays if Approach 1 (all relay UEs must be in RRC_CONNECTED when the remote UE is in RRC_CONNECTED) is adopted.

In one approach (“approach 2”) of U2N relays, Intermediate Relay UEs (other than the Last Relay UE) can be in any RRC state when the U2N remote UE is in RRC_CONNECTED.

In approach 2, any intermediate relay UE which happens to be in RRC_CONNECTED towards the last relay UE’s serving gNB and is operating as a remote UE is assumed to obtain its relaying configuration (RLC channel configuration, SRAP configuration, etc.) in dedicated signalling from the last relay UE’s serving gNB via the U2N connection.

The figure and description under P4 of R2-2410006 serves as a baseline connection establishment procedure for discussion for multi-hop U2N Relays if Approach 2 (relays other than the Last Relay may/may not remain in RRC_IDLE/RRC_INACTIVE when the remote UE is in RRC_CONNECTED) is adopted.

For the baseline procedure, the PDB for each hop is provided by the network according to gNB implementation.   This does not preclude the possibility of assistance information from the UE(s).

If enhancements allow some intermediate relay UEs to remain in idle/inactive, the PDB split between the Uu hop and all remaining hops is performed by the network.  FFS how to split the PDB over each of the individual remaining hops.

In multi-hop, the U2N Remote UE acquires the SI of the cell of the Last Relay UE, which is forwarded via the Intermediate Relay UE(s).  FFS how to perform the forwarding and whether an intermediate relay UE can forward available SI directly (rather than retrieving it from the last relay UE).

Furthermore, during RAN2#128 meeting, the following agreements were reached for the way forward of the RRC connection establishment baseline procedure for SL Multi-hop Relay:

Agreement:

Support a baseline procedure for the case in which the intermediate relay UEs for a remote UE all transition to RRC_CONNECTED (if not already there) when the remote UE goes to RRC_CONNECTED.

1. Support this case based on existing U2N framework, with all the UEs RRC_CONNECTED to the last relay UE’s serving cell.

2. Continue to discuss whether/how to support the case that intermediate relay UEs do not all move to RRC_CONNECTED when the remote UE triggers connection establishment.

All agreements in this WI apply to both these cases unless otherwise specified.

There was a post meeting email discussion on Control plane baseline solution for Multi-hop Relay: [POST128][401][Relay].

This paper discusses the open issues that were not resolved so far on CP aspects for Multi-hop Relay.

# Discussion

## First UL RRC message forwarding

As concluded in the last RAN2 meeting, RAN2 will support a baseline procedure for the case in which the intermediate relay UEs for a remote UE all transition to RRC_CONNECTED (if not already there) when the remote UE goes to RRC_CONNECTED.

Following our understanding that the following Approach-1 should be used for the RRC connection establishment baseline procedure for SL Multi-hop Relay [1].

Approach-1: Each of the Intermediate Relay UEs must be in RRC_CONNECTED when the U2N remote UE is in RRC_CONNECTED. Connection establishment in the U2N remote UE first requires that each Intermediate Relay UE which is in RRC_IDLE/RRC_INACTIVE first enters RRC_CONNECTED.

The signalling procedure for approach-1 is depicted in figure-1 as below.

Figure-1: RRC connection establishment procedure for SL Multi-hop Relay of approach-1

The current Rel-17 UL first RRC message may apply to the last Relay UE for Multiple Hop sidelink relaying but cannot apply to the first Relay UE and intermediate Relay UE, especially when the first Relay UE and/or intermediate Relay UE are not in RRC Connected state. More specifically, if first Relay UE and/or intermediate Relay UE are not in RRC Connected state, the first Relay UE and intermediate Relay UE needs to send their own first RRC messages and these RRC messages will be subject to forwarding. The question is in the case how the first Relay UE and intermediate Relay UE handle the forwarding for the Remote UE’s first UL RRC message and for its own first UL RRC message.

In Rel-17, if the U2N Relay UE is not in RRC_CONNECTED, it will establish an RRC connection with the gNB before forwarding the first UL RRC message from Remote UE. Rel-17 mechanism basically requests the Relay UE to buffer the first UL RRC messages for Remote UE until it is RRC connected, which is suitable for single hop relay scenario. This Rel-17 mechanism may be reused for Rel-19 Multiple Hop sidelink relaying for simplicity. However, with the increase of hop(s) for sidelink relay, that mechanism will increase significantly the delay for the Remote UE to establish a RRC connection, which then downgrade the sidelink relay performance.

For Rel-19 Multiple Hop sidelink relaying case, one possibility can be the Relay UE always immediately forwards the first UL RRC message received from its downstream node (i.e., the SRAP Data PDU received from SL-RLC0) regardless of its RRC state, to its parent Relay UE via determined egress RLC channel (e.g. specified PC5 RLC channel). With this option, if the Relay UE is not in RRC Connected state, the Relay UE will firstly forward the first UL RRC message received from Remote UE to its parent Relay UE. And then the Relay UE may send its own first UL RRC message (e.g. RRCSetupRequest) its parent Relay UE. Then the parent Relay UE can forward these RRC messages along with the relay path towards the gNB serving the last Relay UE. With this option, the delay for the RRC connection procedure for Remote UE will be significantly reduced.

Proposal-1: Immediate first UL RRC messages forwarding at the first Relay UE and intermediate Relay UE is supported, when they are not in RRC_CONNECTED state

The abovementioned immediate forwarding procedure may be applicable to last Relay UE also. But as for Rel-19 multi-hop relay, the assumption is that the last Relay UE may simply inherit the Relay UE behaviour specified for Rel-17 U2N Relay, we are not sure if the last relay UE follows immediate forwarding procedure when it receives the forwarded first UL RRC messages from the intermediate Relay UE, when the last relay UE is not in RRC_CONNECTED state.

## System Information acquisition for Remote UE

During RAN2#128 meeting, it was agreed that for multi-hop, the U2N Remote UE acquires the SI of the cell of the Last Relay UE, which is forwarded via the Intermediate Relay UE(s). But it is FFS how to perform the forwarding and whether an intermediate relay UE can forward available SI directly (rather than retrieving it from the last relay UE).

It should be noted that the first Relay UE receiving the on demand SIB request cannot request the SIB transmission from the network by itself. The first Relay UE needs to forward the on demand SIB request to its upstream relay node, and the upstream relay node needs to keep forwarding the on demand SIB request until reaching the last Relay UE for the Remote UE. The forwarding is not based on SRAP protocol, but on the PC5 RRC message. In case of one hop based forwarding for SI request (i.e., Rel-17), it does not need to indicate the destination of the SIB request message, since the Relay UE is assumed to terminate this message. In case of multiple hop based forwarding(i.e., Rel-19), the intermediate Relay UE may need to know the destination of such message to make correct message forwarding over PC5 RRC. The destination can be expressed by the identity of the Remote UE.

In addition, the intermediate Relay UE may need to record the routing information for the Remote UE, since the intermediate Relay UE may need to rely on this routing information to forward back the request SIBs when it receives the request SIBs from its parent Relay UE.

Proposal 2:	The destination of on demand SIB request message (Remote UE ID) can be included in the on demand SIB request message over PC5 RRC for Remote UE.

There may be Relay UE(s) in the relaying path that may temporally store a valid version of the SIB(s) requested by the Remote UE. We think that allowing the intermediate Relay UE to provide the available SIBs to Remote UE per its request can reduce signaling overhead over the air and also reduce the relay for the Remote UE to acquire the SIBs. But if the intermediate Relay UE only holds a subset of the SIBs requested by the Remote UE, it may not terminate the SIB Request from Remote UE and continue to further forward the SIB request message to its parent Relay UE.

Proposal 3:	The intermediate Relay UE is allowed to provide the request SIBs to the Remote UE if there is a valid version of the request SIBs.

Proposal 4:	The intermediate Relay UE continues to further forward the SIB request message to its parent Relay UE if the intermediate Relay UE only holds a subset of the SIBs requested by the Remote UE.

## SRAP Data forwarding

Based on the existing methods of U2N relay and U2U relay, the connection between the remote UE and the gNB can be divided into two parts:

-	The U2U-like connection between the remote UE and the last relay UE, and

-	The connection between the last relay UE and the gNB serving it.

Therefore, the data forwarding can also be divided into two parts accordingly.

Based on the U2U-like connection, the U2U-like SRAP method, such as the local ID pair of the remote UE and the first relay UE, can be used for the data forwarding between the remote UE and the first relay UE. Additionally, the U2N-like SRAP method, wherein only the local ID of the remote UE is used, can be used for the data forwarding between the last relay UE and the gNB. For example, the last relay UE will remove the local ID of the remote UE form the SRAP PDU when it forwards the data to the gNB. And in the opposite direction, the last relay UE will add the local ID of the remote UE into the SRAP PDU.

Proposal-5:	The U2U-like SRAP method, such as the local ID pair of the remote UE and the first relay UE, can be used for the data forwarding between the remote UE and the first relay UE.

Proposal-6:	The U2N-like SRAP method, wherein only the local ID of the remote UE is used, can be used for the data forwarding between the last relay UE and the gNB.

## Link failure handling

In Rel17 U2N relay, the remote UE triggers the RRC re-establishment procedure when detecting PC5 RLF and it can suspend RLM if it is connected to gNB via relay UE. This basic principle of Rel-17 U2N relay PC5 failure handling may be applicable to multi-hop case.

Proposal 7:	For PC5 Link failure handling of Remote UE, this basic principle of Rel-17 U2N relay PC5 failure handling applies to multi-hop case.

In addition, in Rel17 U2N relay, the radio link failure of relay UE will trigger the notification message transfer or PC5 link release. For the case of multi-hop, we believe, any PC5 radio link failure of intermediate relay UE(s) can lead to the suspension of the multi-hop path.

Proposal 8:	Any PC5 radio link failure of intermediate relay UE(s) can lead to the suspension of the multi-hop path.

# Conclusion and Proposal

We have the following proposals:

Proposal-1: Immediate first UL RRC messages forwarding at the first Relay UE and intermediate Relay UE is supported, when they are not in RRC_CONNECTED state

Proposal 2:	The destination of on demand SIB request message (Remote UE ID) can be included in the on demand SIB request message over PC5 RRC for Remote UE.

Proposal 3:	The intermediate Relay UE is allowed to provide the request SIBs to the Remote UE if there is a valid version of the request SIBs.

Proposal 4:	The intermediate Relay UE continues to further forward the SIB request message to its parent Relay UE if the intermediate Relay UE only holds a subset of the SIBs requested by the Remote UE.

Proposal-5:	The U2U-like SRAP method, such as the local ID pair of the remote UE and the first relay UE, can be used for the data forwarding between the remote UE and the first relay UE.

Proposal-6:	The U2N-like SRAP method, wherein only the local ID of the remote UE is used, can be used for the data forwarding between the last relay UE and the gNB.

Proposal 7:	For PC5 Link failure handling of Remote UE, this basic principle of Rel-17 U2N relay PC5 failure handling applies to multi-hop case.

Proposal 8:	Any PC5 radio link failure of intermediate relay UE(s) can lead to the suspension of the multi-hop path.

# Reference

- RAN2#128   meeting minutes

- R2-2410006  Report of [POST127][402][Relay] Multi-hop relay control plane

- R2-25NNNNN Report of [Post128][401][Relay] Control plane baseline solution

- R2-2409796  CP and SRAP for Multi-hop Relay