3GPP TSG-RAN WG2 Meeting #129	R2-2500304

Athens, Greece, Feb. 17th- 21st, 2025

Agenda item:	8.6.2

Source:	Samsung

Title:	Further Considerations to Support Inter-CU LTM

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

Introduction

In RAN2#128 meeting, RAN2 reached the following agreements for inter-CU LTM procedure and some details:

Agreements on inter-CU LTM:

- Rel-19 Set ID is configured for a candidate configuration. For DRB:

- Inter-CU MCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> UE performs PDCP re-establishment to all radio bearers.

- Inter-CU SCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> For MN terminated bearer without change of termination point, UE does not perform PDCP re-establishment. PDCP recovery is needed if it is split bearer.

> For SN terminated bearer without change of termination point, UE performs PDCP re-establishment.

> If there is change of termination point for a radio bearer (the keyToUse in the RadioBearerConfig is different from the keyToUse in the current UE configuration), UE performs PDCP re-establishment.

- For SRBs in inter-CU SCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3.

- For SRB1/2 in inter-CU SCG LTM, PDCP re-established is not performed based on NW configuration (PDCP re-establishment flag and SDU discard).

- RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it.

- RAN2 confirms that the inter-CU MCG LTM with intra-SN PSCell change is supported in Rel19.

- From RAN2 perspective, the following coexistence cases in NR-DC can be supported:

- Case 1: Intra-CU MCG LTM + Inter-CU MCG LTM

- Case 2: Intra-CU SCG LTM + Inter-CU SCG LTM

7.	In coexistence cases of inter-CU MCG/SCG LTM and intra-CU MCG/SCG LTM, when inter-CU MCG or SCG LTM is executed, it’s up to the NW to ensure that maintained LTM candidate configurations are valid, e.g. reconfigure or release invalid intra-CU MCG/SCG LTM candidate configurations. UE does not autonomously release invalid intra-CU candidate configurations.

8.	RAN2 to support intra-CU SCG LTM in MN RRC message (i.e. MN RRCReconfiguration message), in addition to SN RRC message.

9.	RAN2 to support intra-CU MCG LTM with SCG configuration.

10.	It’s up to NW to ensure that the complete configuration includes the MCG part and SCG part configuration when UE combines the reference and candidate configuration for inter-CU SCG LTM.

11.	RAN2 assumes that how to indicate the list of candidate PSCells from source SN to MN is up to RAN3. From RAN2 perspective, in INM, source SN may send measurement results of candidate PSCells to the MN. The MN then forwards the measurement results to the candidate SN(s), and then the candidate SN(s) determines the LTM candidate cells based on the measurement results and the upper limit for the number of PSCells that can be prepared by each candidate SN. The existing IEs defined in INM can be reused as a baseline.

In this contribution, we will further address inter-CU LTM in details.

Discussion (non-DC part)

Issue 1: Security change ID

According to the running CR, the security change ID (i.e., ltm-NoSecurityChangeID-r19, ltm-ServingCellNoSecurityChangeID-r19) was introduced to determine the security key update during LTM. Originally, the security key change is only applicable for inter-CU LTM, which means that the security change ID can implicitly indicate whether the LTM is inter-CU or intra-CU. However, the IE design in RRC CR does not link such security change ID with inter-CU LTM only. In other words, depending on NW implementation, the security key change for intra-CU LTM is not forbidden, as long as the ltm-ServingCellNoSecurityChangeID-r19 is different from ltm-NoSecurityChangeID-r19 of the target cell. Thus, RAN2 needs to discuss whether to support the security key change for intra-CU LTM in Rel-19. If not, how to develop the specification to indicate such not-allowed case.

Proposal 1-1: RAN2 is kindly asked to discuss whether the new Rel-19 security change ID can be used to indicate the security key update during intra-CU LTM in Rel-19.

Issue 2: Number of candidate configurations

For inter-CU LTM, the LTM configuration can include the LTM candidates for both normal LTM and condition LTM. Meanwhile, according to the following agreements:

L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC),

The candidate configurations for CHO can be also configured to the UE. Considering the storage limitation, the number of candidate configurations at the UE side cannot be too much. In Rel-19, the maximum number of candidate configurations at UE side should take CHO, LTM and CLTM into account. The following options can be considered:

- Option 1: one number is shared by LTM, CLTM, and CHO

- Option 2: separate number is defined for each mobility scheme

- Option 3: normal LTM and CLTM share the same number, while CHO uses a separate number

In our understanding, the LTM and CLTM can share the same LTM-config IE so that the maximum number of candidate configurations can be shared between two schemes. Thus, option 3 is preferred.

Proposal 1-2: RAN2 is kindly asked to discuss the maximum number of canidate configurations stored at the UE side considering the support of LTM, CLTM and CHO.

Issue 3: Early Sync RACH resource allocation

During Rel-18 discussion, RAN2 made the following agreement

- RAN2#121bis-e From RAN2 perspective, to enable shared preamble resource among multiple UEs, it is beneficial that the information that identifies the allocated CFRA resource (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) can be indicated in the PDCCH order (as legacy intra-cell PDCCH order). - RAN2#123-bis RAN2 understands that the source DU needs to know the early RACH configuration for each candidate cell, so that the source cell can know how to set the PDCCH order information for early RACH; The candidate DU provides the TA value and its associated information to the source DU via the CU, e.g. preamble index, RO information (i.e. RA-RNTI) and candidate cell identity, so that the source DU can identify the UE. RAN3 can design the necessary network signalling.

The above agreements indicated that the early sync RACH resource allocated by one candidate cell is shared by multiple UEs. This sharing method can save the RACH resource at each LTM candidate cell. Following this RAN2 agreement, RAN3 designed that the early sync resource is shared among UEs in the same gNB-DU. Thus, the early sync. RACH resource at each candidate cell is allocated per gNB-DU, as shown by an example in Fig. 2. Specifically, the Cell1 allocates three early sync. RACH resource sets, i.e.,

- Set0: used for the LTM cell switch from Cell0 to Cell1

- Set1: used for the LTM cell switch from Cell2 to Cell1

- Set2: used for the LTM cell switch from Cell3 to Cell1

Fig. 1 Early sync. RACH resource sharing per gNB-DU

In Rel-19, the inter-CU LTM has the similar issue, i.e., whether the early sync. RACH resource is shared among multiple UEs or not. The motivation in Rel18 on shared resource is also applicable for inter-CU LTM case.

Proposal 1-3: RAN2 is kindly asked to agree that the early sync. RACH resource of each LTM candidate cell is shared among multiple UEs for inter-CU LTM.

To realize the resource sharing, two options can be considered, i.e., per gNB-DU sharing or per gNB sharing. Specifically, as shown in the following figure, the source gNB-CU has three connected gNB-DUs. For per gNB-DU sharing, the candidate cell1 prepares three isolated sets of early TA RACH resource, each of which is shared by the UEs under one gNB-DU. While for per gNB sharing, candidate cell 1 prepares one set of early TA RACH resource which can be shared by all three gNB-DUs. For example, whenever one gNB-DU wants to trigger an UE to perform early TA acquisition, such gNB-DU can request one RACH resource from the source gNB-CU. Obviously, compared with per gNB-DU sharing, per gNB sharing has higher resource utilization efficiency. Nevertheless, the specification impacts for both options are mainly in RAN3 scope, RAN2 can indicate the benefit of the per-gNB sharing and ask RAN3 to make their decision.

Proposal 1-4: RAN2 is kindly asked to agree that the per-gNB early sync. RACH resource sharing is beneficial for the RACH resource utilization, and send an LS to RAN3.

Fig. 2. Per gNB-DU/gNB early sync. RACH resource sharing

Discussion (DC part)

Inter-CU MCG LTM

According to agreements in the past, the following inter-CU MCG LTM cases are supported:

- Inter-CU MCG LTM with intra-SN PSCell change

- Inter-CU MCG LTM with SCG addition

- Inter-CU MCG LTM with SCG reconfiguration

- Inter-CU MCG LTM + intra-CU MCG LTM

- Inter-CU MCG LTM +intra-CU SCG LTM

Also, the intra-CU MCG LTM with SCG configuration is supported.

Issue 1: reference configuration for inter/intra-CU MCG LTM

Similar to no-DC case, the reference configuration including MCG part can be configured to the UE. The question is whether the reference configuration can include SCG part or not. For MCG LTM, the SCG configuration can be added/modified. Thus, the SCG reference configuration is beneficial to reduce the configuration overhead at the UE side. Moreover, since the SN is unchanged, when preparing the SCG part, the SN can evaluate whether the reference configuration for SCG part can save signalling or not. If it is needed, SN can provide the reference configuration to MN directly (without request from MN), and the MN can configure the reference configuration including both MCG and SCG part.

Proposal 2-1: For inter-CU MCG LTM with unchanged SN, RAN2 is kindly asked to agree the followings:

- The reference configuration can include the SCG part

- During inter-CU MCG LTM preparation, the SN can determine whether to provide the reference configuration of SCG part to the MN or not.

Issue 2: Applying SCG (re)configuration during MCG LTM

In release 18, when the UE executes LTM cell switch, it first releases the current configuration related to the cell group for which LTM cell switch is triggered and applies the complete LTM configuration, either generated using the reference configuration and non-complete LTM candidate configuration or received as a complete LTM candidate configuration. Since R19 supports Intra-CU and Inter-CU MCG LTM with SCG configuration, while executing MCG LTM cell, UE needs to release the current SCG configuration also in such cases. An alternative suggested by some companies during the CR discussion is to always use mrdc-ReleaseAndAdd. However, for this to work, MN need to always use mrdc-ReleaseAndAdd.  It will be much simpler to follow the existing principle by releasing the SCG configuration before applying the LTM candidate configuration during the MCG LTM cell switch including SCG configuration. If the MN wants to release and add SCG, it is always possible to use mrdc-ReleaseAndAdd. But making such behaviour mandatory reduces flexibility and doesn’t bring any value.

Proposal 2-2: During MCG LTM switch which changes the SCG configuration, UE releases the current SCG configuration before applying the LTM candidate configuration.

Issue 3: Default configuration during LTM cell switch

During LTM cell switch execution, UE applies the default MAC configuration and the default values for RLF timers and counters (T310, T311,N310, N311) for the cell group for which the LTM cell switch is triggered before applying the LTM candidate configuration.

Release-18 LTM cell switch procedure specifies this behaviour as below:

1>	use the default values specified in 9.2.3 for timers T310, T311 and constants N310, N311 associated with the cell group for which the LTM cell switch procedure is triggered, where T310, N310, and N311 are for both MCG and SCG, and T311 is only for the MCG;

- apply the default MAC Cell Group configuration as specified in 9.2.2 for the cell group for which the LTM cell switch procedure is triggered;

We need to consider the case where MCG LTM cell switch includes SCG configuration and SCG LTM cell switch has ltm-Config associated to MCG, while setting the default values for the RLF timers/counters and applying the default MAC Cell Group Configuration. If the LTM cell switch is triggered on MCG and the applied RRC Reconfiguration changes the SCG configuration, default MAC configuration and RLF timers need to be applied for SCG as well. Similarly, if the LTM cell switch is triggered on SCG, but the ltm-Config is associated to MCG, the application of default MAC configuration and RLF timers need to be performed for MCG also. In short, the default configurations need to be applied based on the RRCReconfigurations present rather than the cell group which triggered LTM cell switch. A good baseline can be the SCPAC procedure.

Proposal 2-3: Before the LTM candidate configuration is applied, UE applies default MAC configuration and RLF timers/counters for all the cell group(s) for which the RRCReconfiguration is applied (rather than for the cell groups for which LTM cell switch is triggered as in R18).

Issue 4: SN RRCReconfigurationComplete

In Rel-18 and in the running CR for LTM, UE sends SN RRCReconfigurationComplete using ULInformationTransferMRDC message, as given below.

2>	if the RRCReconfiguration is applied due to an LTM cell switch execution:

3>submit the RRCReconfigurationComplete message via SRB1 embedded in NR RRC message ULInformationTransferMRDC as specified in clause 5.7.2a.3.

In RAN2#127-bis, RAN2 already made an agreement that for Inter-SN SCG LTM, UE sends SN RRCReconfigurationComplete in MN RRCReconfigurationComplete message as below.

=>Upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message.

RAN2 also agreed to support Intra-CU SCG LTM + Inter-CU SCG LTM and to support intra-CU SCG LTM in MN RRC message (i.e. MN RRCReconfiguration message), in addition to SN RRC message.

There is no agreement on how the SN RRCReconfigurationComplete is send for Intra-CU SCG LTM in MN RRCReconfiguration message, and the running CR considers that it is included in ULInformationTransferMRDC message. We think  the  SN RRCReconfigurationComplete needs to be included in MN RRCReconfigurationComplete message, in this case rather than in ULInformationTransferMRDC message.

Proposal 2-4: Upon execution of intra-SN SCG LTM where the LTM configuration is associated with MCG, UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message.

Discussion on running RRC CR

In the following, our views on each editor’s note are given.

| Editor’s Note | Comments |
| --- | --- |
| FFS whether additional text for key security change in LTM (at the MCG and SCG) is needed. | We assume some changes are needed since new scheme for inter-CU LTM is needed. However, this depends on the progress on SA3. We can leave FFS as it is for now. |
| FFS whether we use “LTM” for both LTM and CLTM (CLTM will only be used when something applies only to conditional LTM). | We prefer to use LTM for both LTM and CLTM, unless some texts are applicable to CLTM only. |
| Whether to have a new field or re-use the existing selectedSK-Counter is FFS. | Prefer to using a new field so that this can help NW differentiate whether such SK-counter update is due to LTM or not. |
| FFS whether for the SRBs we use the field reestablishPDCP within the LTM candidate configuration (same as for the RLC re-establishment in Rel-18) | DRB part: for inter-CU SCG LTM, this can rely on the keyToUse IE in source cell and the target cell. If it is different, PDCP re-establishment is needed, e.g., 2> for each drb-Identity value that is part of the current UE configuration: … 3> if the LTM cell switch is triggered on the MCG, or, 3> if the LTM cell switch is triggered on the SCG, and the keyToUse in the RadioBearerConfig is changed 4> re-establish the PDCP entity of this DRB as specified in TS 38.323 [5], clause 5.1.2; SRB part: for inter-CU SCG LTM, it can rely on whether the secondary key is used or not, e.g., 2> for each srb-Identity value that is part of the current UE configuration … 4> if the LTM cell switch is triggered on the MCG, or, 4> if the LTM cell switch is triggered on the SCG, and SRB identified by the srb-Identity is associated with the secondary key: 5> re-establish the PDCP entity of this SRB as specified in TS 38.323 [5]; |
| FFS about the relationship between the Rel-18 IDs used for the L2 reset and the Rel-19 IDs used for the security key change/PDCP re-establishment. | The Rel-18 ID is checked if the Rel19 ID is the same or the Rel19 ID is not configured. Thus, it is better to add “else”, e.g., - else: 2 if the LTM-Candidate IE in ltm-Config or ltm-ConfigSCG indicated by lower layers or for the selected cell in accordance with 5.3.7.3 does not contain the field ltm-NoResetID and if the UE does not have any value stored of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID; or 2> if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config or ltm-ConfigSCG indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID: |
| The interaction about when the UE receives an ltm-ReferenceConfiguration in ltm-Config and ltm-ConfigSCG is FFS | The current text is clear, and further clarification is not needed. |
| How to release ltm-ConfigSCG is FFS | Prefer to use a separate text |
| FFS is the field ltm-ReferenceConfiguration can be part of ltm-ConfigSCG in case it containes both MCG and SCG configurations. | Ltm-referenceConfiguration is associated with the LTM configuration. Thus, ltm-ConfigSCG should contain such IE. |
| FFS whether the list of sk-counters is part of the LTM-Config or is provided outside (same level as ltm-Config in the RRCReconfiguration message) | As agreed, we can use SCPAC design, i.e., sk-counter list is per security key change set ID |

Proposal 3-1: RAN2 is kindly asked to develop the RRC CR based on the comments in the above table.

Conclusion

- Non-DC

Security change ID:

Proposal 1-1: RAN2 is kindly asked to discuss whether the new Rel-19 security change ID can be used to indicate the security key update during intra-CU LTM in Rel-19.

Number of candidate configurations:

Proposal 1-2: RAN2 is kindly asked to discuss the maximum number of canidate configurations stored at the UE side considering the support of LTM, CLTM and CHO.

Early sync RACH resource allocation

Proposal 1-3: RAN2 is kindly asked to agree that the early sync. RACH resource of each LTM candidate cell is shared among multiple UEs for inter-CU LTM.

Proposal 1-4: RAN2 is kindly asked to agree that the per-gNB early sync. RACH resource sharing is beneficial for the RACH resource utilization, and send an LS to RAN3.

- DC

Inter-CU MCG LTM

Proposal 2-1: For inter-CU MCG LTM with unchanged SN, RAN2 is kindly asked to agree the followings:

- The reference configuration can include the SCG part

- During inter-CU MCG LTM preparation, the SN can determine whether to provide the reference configuration of SCG part to the MN or not.

Proposal 2-2: During MCG LTM switch which changes the SCG configuration, UE releases the current SCG configuration before applying the LTM candidate configuration.

Proposal 2-3: Before the LTM candidate configuration is applied, UE applies default MAC configuration and RLF timers/counters for all the cell group(s) for which the RRCReconfiguration is applied (rather than for the cell groups for which LTM cell switch is triggered as in R18).

Proposal 2-4: Upon execution of intra-SN SCG LTM where the LTM configuration is associated with MCG, UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message.

- RRC CR related

Proposal 3-1: RAN2 is kindly asked to develop the RRC CR based on the comments in the above table.

References