3GPP TSG-RAN WG2 Meeting #129	R2-2500308

Athens, Greece, Feb. 17th- 21st, 2025

Agenda item:	8.13.3

Source:	Samsung

Title:	Consideration on CP issues for multi-hop SL relay

WID/SID:	NR_SL_relay_enh2

Document for:	Discussion and Decision

Introduction

In Nov meeting, the control plane procedure for the multi-hop SL relay was discussed, and the following agreement were reached:

Agreements:

For the baseline procedure, the PDB for each hop is provided by the network according to gNB implementation.   This does not preclude the possibility of assistance information from the UE(s).

If enhancements allow some intermediate relay UEs to remain in idle/inactive, the PDB split between the Uu hop and all remaining hops is performed by the network.  FFS how to split the PDB over each of the individual remaining hops.

Agreement:

Support a baseline procedure for the case in which the intermediate relay UEs for a remote UE all transition to RRC_CONNECTED (if not already there) when the remote UE goes to RRC_CONNECTED.

1. Support this case based on existing U2N framework, with all the UEs RRC_CONNECTED to the last relay UE’s serving cell.

2. Continue to discuss whether/how to support the case that intermediate relay UEs do not all move to RRC_CONNECTED when the remote UE triggers connection establishment.

All agreements in this WI apply to both these cases unless otherwise specified.

In this contribution, we will share our further view on CP related procedures.

Discussion

Baseline procedure

As the baseline procedure, the intermediate relay UE has to establish RRC connection first before relaying the packets from its child node. The benefit of such procedure is that the design of Rel-17 single hop SL relay can be almost reused. However, such baseline procedure results in large accessing latency of the remote UE. Thus, the baseline procedure may result in the connection establishment/re-establishment/resume failure at the remote UE. For example, during RRC connection setup procedure, T300 is started to monitor whether the RRC connection setup is failed or not. The largest value of T300 is 2000ms. In multi-hop SL relay, the remote UE may not be able to finish RRC connection setup procedure within 2000ms due to the RRC connection establishment of first/intermediate/last relay UEs.  Thus, RAN2 needs discuss how to avoid the connection establishment failure caused by the connection setup of first/intermediate/last relay UE. One possible way is to set the T300 according to different number of hops, i.e., the T300 can be increased with the number of hops.

Proposal 1-1: RAN2 is kindly asked to discuss how to avoid the connection establishment failure caused by the connection setup of first/intermediate/last relay UE (e.g., hop number based T300/T301/T319 setting).

On the other hand, RAN2 can discuss potential enhancement to reduce the latency. One possible direction is the Approach 2 proposed in last meeting (i.e., allow the intermediate relay UE in RRC_IDLE/INACTIVE state). However, such method has some critical issues, e.g., authorization, QoS split, etc. Thus, before discussing the potential enhancement, it is better to have some fundamental principles:

- No authorization issue appears

- The E2E QoS can be well controlled by the network

Proposal 1-2: to reduce the connection establishment latency, RAN2 aims at the enhancement without authorization issue and E2E QoS issue.

The latency of baseline procedure is caused by the hop-by-hop connection establishment, i.e., each intermediate relay UE can perform the connection establishment only after its parent relay UE completes the connection. To reduce the latency, one possible way is to allow the parallel connection establishment of intermediate relay UEs. Specifically, whenever an intermediate relay UE receives the first RRC connection setup request message from its child node, it can trigger its own RRC connection establishment procedure; meanwhile, it can forward the received the RRC connection setup request message to its parent node. To realize this parallel connection setup, the following issue needs to be resolved:

- Issue 1: PC5 RLC channel configuration for SRB0

Each intermediate relay UE has to have the PC5 RLC channel for SRB0. This can be relies on the pre-configuration from the gNB side, e.g., via system information, via the configuration when releasing the UE

- Issue 2: the authorization of intermediate UE

Compared to RRC_IDLE, the UE in RRC_INACTIVE is authorized in the core network. Thus, the security may not be issue to the RRC_INACTIVE UE. Thus, such parallel procedures can be implemented as long as the intermediate relay UE is in RRC_INACTIVE, i.e., when the intermediate relay UE in RRC_INACTIVE receives RRC connection setup request message from its child node, it can forward such message to its parent node and start its own RRC resume procedure.

Proposal 1-3: to reduce the connection latency, RAN2 is kindly asked to discuss the parallel connection establishment procedure with the following features:

- the intermediate relay UE is in RRC INACTIVE state

- the PC5 RLC channel for SRB0 is preconfigured to the intermediate relay UE in RRC INACTIVE

- the intermediate relay UE can perform forwarding of the RRC connection setup request message from its child node and its own connection resume procedure at the same time.

After the connection establishment of each relay UEs along the routing path, gNB can configure the bearer mapping of remote UE hop-by-hop, and the E2E QoS of remote UE can be well controlled by gNB. Meanwhile, the signalling from each intermediate relay UE to the gNB, e.g., measurement reporting, will increase the burden of the whole routing path. In order to reduce such burden, the gNB can release the intermediate relay UE to RRC Inactive state by providing some configurations, e.g., mapping configuration, routing configuration, etc., to maintain the data forwarding for the remote UEs. Those configurations can be continuously applied for the data forwarding.

Proposal 1-4: As an optimization, RAN2 is kindly asked to discuss whether to allow releasing the intermediate relay UE to INACTIVE state after establishing the data forwarding path for the remote UE.

After connection establishment, the gNB may release the remote UE to the IDLE/INACTIVE state, or gNB may perform the path switch to another direct or indirect path. This requires to release configurations (e.g., mapping configuration, routing configuration) along the routing path for the remote UE. In order to reduce the packet loss, the gNB should know whether the packets from the remote UE have been completely received or not before releasing the configuration.

Proposal 1-5: RAN2 is kindly asked to discuss how to handle the remote UE’s packets undertaking forwarding in the routing path before releasing the configuration for the remote UE at the intermediate relay UE.

Failure case handling

- Remote UE

The legacy failure case handling can be reused. For example, the remote UE can suspend RLM if it is connected to gNB via relay UE; it triggers the RRC re-establishment procedure when detecting PC5 RLF.

Proposal 2-1: RAN2 is kindly asked to agree that the legacy failure handling can be reused by remote UE.

- Last relay UE

In R17, several cases will trigger the notification message transfer. In Rel-19, the last relay UE has the same functionalities as the relay UE in the single hop SL relay. Thus, the handling on the failure case at the last relay UE can reuse the legacy design (i.e., the initiation of notification message).

Proposal 2-2: RAN2 is kindly asked to agree that the legacy procedure for Notification message initiation can be reused for the last relay UE.

The resultant question is whether and how to deliver such message to the following-up intermediate relay UEs towards to the remote UE. This depends on the discussion on how to handle the failure cases. The following options can be considered:

- Option 1: transparently delivery

In this option, whenever such notification message is received from the last relay UE, each node just simply forward this message to the next-hop, so that such message is transmitted hop-by-hop to the remote UE.

- Option 2: conditionally transparent delivery

In this option, the receiving node may perform some recovery operations. If the recovery is successful, the delivery is not needed; otherwise, the delivery is performed. Thus, this option may result in that the notification message may not finally reach the remote UE.

- Option 3: regenerated delivery

In this option, during the delivery, if intermediate relay UE also triggers its own notification message, it may generate a new notification message considering its own state or the received notification from the last relay UE.

Proposal 2-3: RAN2 is kindly asked to discuss the delivery of the last relay UE’s notification message based on the following options:

- Option 1: transparently delivery

- Option 2: conditionally transparent delivery

- Option 3: regenerated delivery

- Intermediate relay UE

In R17, when receiving a notification message from the relay UE, the remote UE will trigger the RRC re-establishment procedure. However, in Rel-19, multiple intermediate relay UEs are along the transmission path. Whenever there is a PC5 link problem, the whole transmission path will be interrupted. Thus, to reduce the interruption time, one of important topics for Rel-19 is how to recover the transmission path quickly when there is PC5 link problem. In general, the notification message is useful to link problem notification among intermediate relay UEs. With such message, two options can be considered:

- Option 1: E2E recovery

Whenever an intermediate relay UE has a PC5 link problem, it will send the Notification message to its child node, and such message will be forwarded to the remote UE hop-by-hop. Then, after receiving it, the remote UE can follow the legacy design, i.e., initiate RRC re-establishment procedure. This option may not be a problem for single hop relay. For multi-hop relay, it also cause large recovering time since the remote UE needs carry out re-establishment procedure.

- Option 2: Hop-by-hop recovery

To resolve the issue of Option 1, another method is that the recovering is performed hop-by-hop. Specifically, when an intermediate relay UE has PC5 link problem or receives the notification message from the parent node, it can try to recovery by itself. If a new relay UE is reselected, the transmission path from the remote UE to the gNB can be established quickly.

Proposal 2-4: RAN2 is kindly asked to discuss the following two options for failure recovery at the intermediate relay UE:

  - Option 1: E2E recovery

  - Option 2: Hop-by-hop recovery

Conclusion

Baseline procedure

Proposal 1-1: RAN2 is kindly asked to discuss how to avoid the connection establishment failure caused by the connection setup of first/intermediate/last relay UE (e.g., hop number based T300/T301/T319 setting).

Proposal 1-2: to reduce the connection establishment latency, RAN2 aims at the enhancement without authorization issue and E2E QoS issue.

Proposal 1-3: to reduce the connection latency, RAN2 is kindly asked to discuss the parallel connection establishment procedure with the following features:

- the intermediate relay UE is in RRC INACTIVE state

- the PC5 RLC channel for SRB0 is preconfigured to the intermediate relay UE in RRC INACTIVE

- the intermediate relay UE can perform forwarding of the RRC connection setup request message from its child node and its own connection resume procedure at the same time.

Proposal 1-4: As an optimization, RAN2 is kindly asked to discuss whether to allow releasing the intermediate relay UE to INACTIVE state after establishing the data forwarding path for the remote UE.

Proposal 1-5: RAN2 is kindly asked to discuss how to handle the remote UE’s packets undertaking forwarding in the routing path before releasing the configuration for the remote UE at the intermediate relay UE.

Failure case handling

Proposal 2-1: RAN2 is kindly asked to agree that the legacy failure handling can be reused by remote UE.

Proposal 2-2: RAN2 is kindly asked to agree that the legacy procedure for Notification message initiation can be reused for the last relay UE.

Proposal 2-3: RAN2 is kindly asked to discuss the delivery of the last relay UE’s notification message based on the following options:

- Option 1: transparently delivery

- Option 2: conditionally transparent delivery

- Option 3: regenerated delivery

Proposal 2-4: RAN2 is kindly asked to discuss the following two options for failure recovery at the intermediate relay UE:

  - Option 1: E2E recovery

  - Option 2: Hop-by-hop recovery

References