3GPP TSG-RAN WG2 Meeting #129                                          R2-2500318

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.3.2.2

Source:	Mediatek Inc.

Title:	Model Generalization for AIML RRM Prediction

Document for:	Discussion, Decision

Introduction

Agreements - Reuse the evaluation methodology in TR38.843 for generalization study, i.e., the generalization performance is evaluated with the following cases, - Baseline: The AI/ML model is trained using the dataset with Configuration #B and tested using the dataset with Configuration #B. - Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset with Configuration #A but tested using the dataset with Configuration #B. - Generalization Case #2 (GC#2): The AI/ML model is trained using mixed datasets with both configurations and tested using the dataset with Configuration #B. - Companies can choose which case they compare with and should report it with simulation results. - Generalization issues on RRM measurement prediction are prioritized. - Start the study with generalization issue with RRM measurement prediction in temporal domain. Companies can choose to study frequency domain prediction cases and report what they have simulated. Study generalization over UE speeds The simulation assumption of FR1 temporal domain case B is reused for generalization study with 3 UE speeds i.e. 30Km/h, 60Km/h, and 90Km/h. FFS on combinations The simulation assumption of FR2 temporal domain case A is reused for generalization study with 3 UE speeds i.e. 60Km/h, 90Km/h, and 120Km/h. FFS on combinations

In RAN2 #128 meeting and email discussion [1], the agreements on the model generalization are given as follows.

Further agreements on generalization

- Companies that would like to study inter-frequency generalization can start with input 2GHz and output 4GHz, and 4GHz to 2GHz.   FFS if we introduce a third frequency.

- Study model generalization across different cell configurations (e.g. ISD, gNB height, power, beam pattern, etc).   FFS which parameters we prioritize.

According to the agreement, in this contribution, we present the simulation results of model generalization in the following use cases.

- RRM_Scen2_Gen: RRM temporal domain case B with different UE speeds (e.g., 30, 60, 90km/hr)

- RRM_Scen3_Gen: RRM inter-frequency prediction with different frequency prediction (e.g., 2Ghz=>4Ghz, 4Ghz=>2Ghz )

- RRM_Scen4_Gen: RRM temporal domain case A with different UE speeds (e.g., 60, 90, 120km/hr)

Observations from each experiment are provided. Since there are still many other generalization parameters under consideration [1]. We also provide our view on the selection of generalization parameters for the next steps.

Discussion

Simulation results and observations

RRM temporal domain case B with different UE speeds

In this case, we examine the impact of model generalization for the RRM temporal domain case B. The AI model is trained and used to perform inference at different UE speeds, e.g., 30, 60, and 90km/hr. We consider two different measurement reduction ratios, i.e., MRRT = 0.5 and 0.8. The observation and prediction patterns are given below.

Figure 1: observation and prediction pattern for (a) MRRT = 0.5 (b) MRRT = 0.8

In this simulation, we consider the sliding case, sub-use case 2, and cluster-based approach where both AI model input and output are L3 cell level RSRP of 21 cells. The approach is selected because, in the sliding case, UE can update the measurement filter more frequently, the sub-use case 2 can achieve the highest accuracy among all sub-use cases, and the cluster-based approach generally outperforms the cell-based approaches in our previous simulation result. Detailed AI model parameters and settings are given in the below table. The general simulation setting can be found in the Appendix.

| Setting/Parameters | Value |
| --- | --- |
| AI model | CNN |
| Flop | 3.02M |
| Model parameters | 204KB |
| Cluster approaches setting | Input: 21 cells, output: 21 cells |
| AI input/output | Input: L3 cell level RSRP, output: L3 cell level RSRP |

Table 1:AI model parameters and settings for intra-frequency temporal domain case B

In the following simulation, we examine the impact of different UE speeds. The baseline refers to the case where the training dataset and inference dataset are generated with the same UE speed. The GC#1 refers to the case where the training dataset and inference dataset are mismatched, e.g., 90km/hr for training and 30km/hr for inference. The GC#2 refers to the case where the training dataset is a hybrid dataset that contains different UE speeds, e.g., 30, 60, 90km/hr data. We consider two different experiments A and B, where Experiment A considers the inference dataset with 30km/hr UE speed and Experiment B considers the inference dataset with 90km/hr. A non-AI result, e.g., sample and hold, is also provided as a reference.

| FR1, Temporal domain case B, sliding, sub-use case 2, MRRT = 0.5 | Training dataset | Inference dataset | L3 RSRP diff | L3 RSRP diff (sample and hold) |
| --- | --- | --- | --- | --- |
| Baseline A | 30km/hr | 30km/hr | 0.059 | 0.1244 |
| GC#1 A(60km/hr) | 60km/hr |  | 0.058 |  |
| GC#1 A(90km/hr) | 90km/hr |  | 0.058 |  |
| GC#2 A | 30,60,90km/hr |  | 0.057 |  |
| GC#1 B(30km/hr) | 30km/hr | 90km/hr | 0.064 | 0.1492 |  |
| GC#1 B(60km/hr) | 60km/hr |  | 0.06 |  |  |
| Baseline B | 90km/hr |  | 0.06 |  |  |
| GC#2 B | 30,60,90km/hr |  | 0.059 |  |  |

Table 2:  Model generalization - RRM temporal domain case B with different UE speed (MRRT=0.5)

| FR1, Temporal domain case B, sliding, sub-use case 2, MRRT = 0.8 | Training dataset | Inference dataset | L3 RSRP diff | L3 RSRP diff (sample and hold) |  |
| --- | --- | --- | --- | --- | --- |
| Baseline A | 30km/hr | 30km/hr | 0.241 | 0.292 |  |
| GC#1 A(60km/hr) | 60km/hr |  | 0.240 |  |  |
| GC#1 A(90km/hr) | 90km/hr |  | 0.247 |  |  |
| GC#2 A | 30,60,90km/hr |  | 0.236 |  |  |
| GC#1 B(30km/hr) | 30km/hr | 90km/hr | 0.287 | 0.355 |
| GC#1 B(60km/hr) | 60km/hr |  | 0.277 |  |
| Baseline B | 90km/hr |  | 0.276 |  |
| GC#2 B | 30,60,90km/hr |  | 0.273 |  |

Table 3: Model generalization - RRM temporal domain case B with different UE speed (MRRT=0.8)

Our result shows that the generalization of different UE speeds does not significantly impact AI performance. One possible reason is that the AI model may learn to predict measurements based on historical measurements with similar logic, regardless of the UE speed. Another possible reason is the short prediction distance in this case, e.g., 40ms or 160ms, where the channel does not vary much. It should be noticed that all generalization cases still outperform the non-AI approach, e.g., sample and hold, but the improvement is limited, which is not surprising since the improvement is upper bounded by the baseline and we already know that the margin for AI enhancement in this case is limited.

Observation 1: AI approaches demonstrate robust generalization performance across different UE speeds in the RRM temporal domain case B.

Observation 2: Both GC#1 and GC#2 have similar performance compared to the baseline and slightly outperform the non-AI approach, e.g., sample and hold.

RRM Inter-frequency prediction with different frequency prediction

In this section, we examine the model generalization in co-located inter-frequency prediction, where 4GHz and 2GHz cells are deployed at the same place. We investigate the impact when the AI model is trained and used to predict measurements at different frequencies, for example, the AI model is trained to predict 4Ghz measurement based on 2Ghz measurement as model input. After training, the AI model is then used to predict 2Ghz measurement based on 4Ghz measurement as model input. Although in typical situations, UE will know its serving frequency, i.e., observation frequency, this experiment can examine the model’s capability and determine if the model can be reused even if the serving frequency changes. Detailed AI model parameters and settings are given in the below table. The general simulation setting can be found in the Appendix.

| Setting/Parameters | Value |
| --- | --- |
| AI model | CNN |
| AI input/output | Input: L3 cell level RSRP, output: L3 cell level RSRP |
| Cluster approaches setting | Input: 21 cells, output: 21 cells |
| Flop | 3M |
| Model parameters | 201KB |

Table 4: AI model parameters and settings for inter-frequency prediction

The simulation results are provided in the following table. Experiment A considers the case where AI is used to predict 4GHz from 2GHz measurement. In baseline A, the model is trained on a matched dataset, i.e., predicting 4GHz from 2GHz. In GC#1 A, the model is trained on the opposite type of dataset, i.e., predicting 2GHz from 4GHz. In GC#2 A, the AI model is trained on a hybrid dataset that contains both predicted directions. For experiment B, we consider reverse inference directions, where AI is used to predict 2GHz from 4GHz measurement.

We observe that when the training and inference datasets are mismatched, e.g., in the GC#1 A case, the prediction accuracy significantly decreases. This is because the AI model only learns how to predict 2GHz from 4GHz measurement. The RPSR of a 2GHz signal is usually ~6dB higher compared to the RSRP of a 4GHz signal due to the pathloss. If we use the model to predict 4GHz based on 2GHz measurement as input, it will output a higher value compared to the input, resulting in the roughly 12dB L3 RSRP difference.

However, it should be noted that in GC#1, we do not perform any preprocessing based on the information of predicted frequency. The prediction accuracy could be further enhanced if we utilize the knowledge of predicted frequency. For the hybrid dataset case, e.g., in the GC#2 A case, such significant performance loss is not observed. Once the AI model recognizes that there may be different prediction directions from the training dataset, even though we do not specify the prediction direction during AI inference, the prediction accuracy is acceptable and better than the non-AI approach that assumes the predicted target is equal to the observed target minus the path loss difference.

| FR1, inter-freq, sub-use case 2 | Training dataset | Inference dataset | L3 RSRP diff | L3 RSRP diff (non-AI) |
| --- | --- | --- | --- | --- |
| Baseline A | 2GHz => 4GHz | 2GHz => 4GHz | 1.589 | 2.309 |
| GC#1 A | 4GHz => 2GHz |  | 11.92 | 2.309 |
| GC#2 A | 2GHz => 4GHz and 4GHz => 2GHz |  | 2.035 | 2.309 |

| Baseline B | 4GHz => 2GHz | 4GHz => 2GHz | 1.52 | 2.309 |
| --- | --- | --- | --- | --- |
| GC#1 B | 2GHz => 4GHz |  | 11.432 | 2.309 |
| GC#2 B | 2GHz => 4GHz and 4GHz => 2GHz |  | 2.08 | 2.309 |

Table 5:  Model generalization - RRM inter-freq prediction with different frequency prediction

Observation 3: Considering the generalization parameter of different predicted frequencies in inter-freq prediction, the GC#1 case without any preprocessing based on the information of predicted frequency suffers from significant performance loss.

Observation 4: For GC#2 case, the prediction accuracy is acceptable and outperforms the non-AI approach.

RRM Intra-frequency temporal domain case A with different UE speeds

For intra-frequency temporal domain case A, we use continuous historical measurement results to predict continuous future measurement results without considering any measurement reduction. The following is the default setting in our simulation if no further specification.

| Setting/Parameters | Value |
| --- | --- |
| AI model | CNN |
| Cluster approaches | Input: 21 cells, output: 21 cells |
| Sliding/non-sliding | Sliding |
| OW/PW | OW=400ms, PW=400ms |
| AI input/output (sub-use cases) | Sub-use case 2, input: L3 cell RSRP; output: L3 cell RSRP |

In this case, we examine the impact of different UE speeds between training and inference datasets. In Experiment A, we fixed the inference dataset with UE speed = 60km/hr. The results show that the AI model trained on different UE speed datasets achieves similar RSRP prediction accuracy. A similar result can be found in experiment B, which fixed the inference dataset with UE speed = 90km/hr. The possible reason is similar to what we mentioned in the above temporal domain case B section. The AI model actually learns a similar relation between observation measurement and prediction from different UE speed datasets.

| FR2, Temporal domain case A | Training dataset | Inference dataset | L3 RSRP diff | L3 RSRP diff (sample and hold) |
| --- | --- | --- | --- | --- |
| Baseline A | 60km/hr | 60km/hr | 0.61 | 0.84 |
| GC#1 A(90km/hr) | 90km/hr |  | 0.64 |  |
| GC#1 A(120km/hr) | 120km/hr |  | 0.63 |  |
| GC#2 A | 60,90,120km/hr |  | 0.64 |  |
| GC#1 B(60km/hr) | 60km/hr | 120km/hr | 0.66 | 1.13 |
| GC#1 B(90km/hr) | 90km/hr |  | 0.65 |  |
| Baseline B | 120km/hr |  | 0.64 |  |
| GC#2 B | 60,90,120km/hr |  | 0.66 |  |

Table 3: Model generalization - RRM temporal domain case A with different UE speeds

Observation 5: AI approaches demonstrate robust generalization performance across different UE speeds in the RRM temporal domain case A.

Observation 6: Both GC#1 and GC#2 have similar performance compared to the baseline and outperform the non-AI approach.

Proposal 1: RAN2 capture the above observations in the TR.

Selection of Generalization Parameters

Since testing model generalization is crucial to verify whether the AI approach can be applied in practice, we may need to consider more generalization parameters beyond UE speeds and different predicted frequencies. However, due to the heavy workload and time constraints, RAN2 should carefully select which parameters to consider next. We believe the selection of generalization parameters should include one of the following properties:

- Hard to obtain: If the parameter is hard to obtain, then the device has no chance to change its AI model, and thus, the AI model should have enough generalization capability to cover different scenarios.

- Frequent change: If the parameter can be detected by the device, but updating the AI model based on it too frequently would result in a huge overhead, we would expect the AI model to have enough generalization capability to cover the different scenarios.

For the first category, we propose considering the different channel scenarios such as UMa and UMi. Since it is difficult for the device to verify which kind of channel it is facing, and it is also hard to obtain detailed parameters, e.g., ISD, the height of BS, BS Tx power… etc, in practice. Although we can consider each parameter and examine the model generalization for all combinations, this would result in a huge workload. Hence, we propose jointly considering multiple parameters, e.g., ISD, height of BS, BS Tx power, …etc., in different configurations instead of considering each of them and all their combinations. For example, for FR1, we can consider two different channel configurations: UMa and UMi. For FR2, we can consider two different channel configurations: UMi_1 and UMi_2. The detailed parameters, e.g., ISD, the height of BS, BS Tx power, ...etc., for the UMa, UMi, UMi_1, and UMi_2 can be FFS.

For the second category, we propose verifying if the AI model can be applied across different cells. More specifically, compare the results of the following cases:

  - Baseline: AI Model is trained on the data from Cell B and is used to infer the result of Cell B.

  - GC#1: AI Model is trained on the data from Cell A and is used to infer the result of Cell B.

  - GC#2: AI Model is trained on the data from a set of cells and is used to infer the result of Cell B and/or a set of cells

Even though the device will know its serving cell and predict targets, it may not try to update AI model every time when the serving cell or predict target changes due to the model update overhead. This experiment can help to verify the need for frequent model updates once the serving cell and predicted targets are changed.

Proposal 2: RAN2 consider model generalization for different channel configurations. Multiple parameters, e.g., ISD, the height of BS, BS Tx power,…etc, should be jointly considered in these configurations instead of evaluating each parameter and all their combination individually.

Proposal 3: RAN2 consider two different configurations, i.e., UMa and UMi for the FR1 case, and consider UMi_1 and UMi_2 for the FR2 case. Detail parameters for UMa, UMi, UMi_1, and UMi_2 can be FFS.

Proposal 4: RAN2 consider to evaluate generalization performance cross different numbers of cells. Specifically, compare the results of the cases:

- Baseline: AI Model is trained on the data from Cell B and is used to infer the result of Cell B

- GC#1: AI Model is trained on the data from Cell A and is used to infer the result of Cell B

- GC#2: AI Model is trained on the data from a set of cells and is used to infer the result of Cell B and/or a set of cells

Conclusion

Observations:

Simulation results: Model generalization of intra-frequency temporal domain case B

Observation 1: AI approaches demonstrate robust generalization performance across different UE speeds in the RRM temporal domain case B.

Observation 2: Both GC#1 and GC#2 have similar performance compared to the baseline and slightly outperform the non-AI approach, e.g., sample and hold.

Simulation results: Model generalization of Inter-frequency prediction

Observation 3: Considering the generalization parameter of different predicted frequencies in inter-freq prediction, the GC#1 case without any preprocessing based on the information of predicted frequency suffers from significant performance loss.

Observation 4: For GC#2 case, the prediction accuracy is acceptable and outperforms the non-AI approach.

Simulation result: Model generalization of Intra-frequency temporal domain case A

Observation 5: AI approaches demonstrate robust generalization performance across different UE speeds in the RRM temporal domain case A.

Observation 6: Both GC#1 and GC#2 have similar performance compared to the baseline and outperform the non-AI approach.

Proposals:

Observation of simulation results

Proposal 1: RAN2 capture the above observations in the TR.

Selection of generalization parameters

Proposal 2: RAN2 consider model generalization for different channel configurations. Multiple parameters, e.g., ISD, the height of BS, BS Tx power,…etc, should be jointly considered in these configurations instead of evaluating each parameter and all their combination individually.

Proposal 3: RAN2 consider two different configurations, i.e., UMa and UMi for the FR1 case, and consider UMi_1 and UMi_2 for the FR2 case. Detail parameters for UMa, UMi, UMi_1, and UMi_2 can be FFS.

Proposal 4: RAN2 consider to evaluate generalization performance cross different numbers of cells. Specifically, compare the results of the cases:

- Baseline: AI Model is trained on the data from Cell B and is used to infer the result of Cell B

- GC#1: AI Model is trained on the data from Cell A and is used to infer the result of Cell B

- GC#2: AI Model is trained on the data from a set of cells and is used to infer the result of Cell B and/or a set of cells

Reference

[1] 	[Post128][018][AI Mob] generalization (Apple)

Appendix: Simulation Setting

| parameters | Value for FR1 | Value for FR2 |
| --- | --- | --- |
| Frequency Range | FR1@4GHz, SCS: 30KHz | FR2 @ 30 GHz; SCS: 120 kHz |
| Deployment | 2-tier model (7 sites, 3 sectors/cells per site) | 2-tier model (7 sites, 3 sectors/cells per site) |
| Channel model | UMa With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading and LOSsoft; Inter-frequency correlation model is optional. | Umi With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading and LOSsoft; |
| System BW | 20MHz | 80MHz |
| UE speed | 30,60, 90 km/h for temporal domain case B | 60, 90,120 km/h for temporal domain case A |
| UE distribution | 100% outdoor | 100% outdoor |
| BS Antenna Configuration | (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ | (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ |
| BS Antenna pattern | 3-sector antenna radiation pattern, 8 dBi | TR 38.802 Table A.2.1-6, |
| UE Antenna Configuration | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) | Antenna setup and port layouts at UE: (1, 4, 2, 1, 1, 1, 1) |
| UE Antenna pattern | Omni-direction | TR 38.802 Table A.2.1-8, |
| Tx/Rx beam | Tx: 4, Rx:1 | Tx:32, Rx:4 |
| BS Tx Power | 44dBm | 40 dBm |
| Maximum UE Tx Power | 23dbm | 23 dBm |
| BS receiver Noise Figure | 5db | 7 dB |
| UE receiver Noise Figure | 9dB | 10 dB |
| Inter site distance | 500m | 200 m |
| BS Antenna height | 25m | 10m |
| UE Antenna height | 1.5m | 1.5 m |
| Spatial consistency | Procedure A in TR38.901 with coarse update | Procedure A in TR38.901 with coarse update |
| UE trajectory model | Option 1 | Option 1 |
| UE trajectory boundary processing model | Option 2: circle-bouncing model, | Option 2: circle-bouncing model, |
| Sampling period | 40ms | 20ms *4 = 80ms |