3GPP TSG-RAN WG2 Meeting #129	R2-2500319

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.3.3

Source:	Mediatek Inc.

Title:	Preliminary results of indirect event prediction and simulation assumption for direct event prediction

Document for:	Discussion, Decision

Introduction

In the previous #128 meeting, RAN2 made the following agreements on indirect measurement event prediction for temporal case A and B, and SLS evaluation for AI/ML mobility of event prediction.

Agreements on measurements events 1 Agreed values for Case A. NOTE1 indirect prediction only Parameters; baseline value; Note. A3 event offset (db); 2; Open for 3db. TTT (ms); 320; Open for one shorter value. UE speed (km/h); 90; Open for 60 and 120km/h. OW length (ms, note1); N/A; Up to implementation. PW length (ms, note1); 400; Open for more values. Max ETD (ms, note1); 80; Open for more values. 2 Agreed values for Case B. NOTE1 indirect prediction only Parameters; baseline value; Note. A3 event offset (db); 2; Open for 3db. TTT (ms); 320; Open for one shorter value. UE speed (km/h); 30; Open for 60 and 90km/h. OW length (ms,note1); N/A; Up to implementation. PW length (ms,note1); 200 (non-sliding) 40ms (sliding); Open for more values. Max ETD (ms, note1); 80; Open for more values. MRRT; 50%; Open for more values. 3 Company can report which filtering options is being used for the input L3 RSRP of sub-use case 2: option 1, option 2, option 3. to capture the 3 options into TR.

Agreements for system level simulation

1 	For measurement event prediction for temporal domain case B:

UE reports when A3 event is satisfied with actual measurements and predicted results.   And handover command will be received after handover preparation.

Remove T0 from the picture

2	For measurement event prediction for temporal domain case A, company focus on option 2 or option 3.

Option 2: network transmit handover command purely based on actual measurement event regardless whether an actual measurement result(@t2) is earlier or later than predicted measurement event((@t1))

Option 3: For AI mobility, HO preparation starts when an event is predicted to happen (i.e., t0), and HO command is sent when A3 entering conditions are met based on actual/real measurement and an event is predicted to be met for the duration of TTT.

3	Handover Preparation time is 40ms and handover execution time: 40ms

4  RAN2 focus on indirect prediction methodology

In this paper, we give our preliminary results of indirect measurement event prediction and further evaluate direct measurement event prediction.

Discussion

Motivation

The goal of measurement event prediction is to improve mobility system performance, the question is what the potential scenarios are where AI-driven event prediction can be beneficial, what measurement event prediction entails and how to utilize the outcome of measurement event prediction to improve mobility performance in these scenarios.

With the evolution of cellular technology and the adoption of higher frequency bands like FR2 and beamforming, the handover can increase even further due to dynamic channel conditions, dense deployments, and smaller cell sizes. Legacy L3 HO mechanism introduces TTT to reduce unnecessary HO and ping-pong, which also leads to additional latency. This reactive approach, involving measurements, reporting, and handover execution, may still result in suboptimal mobility performance, as it may not always facilitate the seamless transition of UE to the best cell in a timely manner.

Observation 1: As cellular technology advances with higher frequency bands like FR2 and beamforming are utilized, prompting the usage of TTT within legacy L3 handover mechanisms to mitigate unnecessary transitions and reduce "ping-pong" effects, though at the cost of introducing latency and achieving only suboptimal mobility performance.

In mobility procedure, the UE experiences continuous handover, some of them are unnecessary handover, for example, from cell A to cell B, and then back to cell A; or from cell A to B and then change to cell C at a short time; or UE received handover Command in cell A to switch to Cell B. However, a Fail occurred on the way handover to cell B. Such continuous and unnecessary handovers may significantly affect the mobility performance and user experience, leading to issues like game freezing, delays, and unsmooth video services.

Observation 2: UE may experiences continuous and unnecessary handover and significantly affect the mobility performance and user experience, leading to issues like game freezing, delays, and unsmooth video services.

Following two applications of measurement event prediction can be considered:

- To enable a proactive measurement report(MR) and handover: the UE predict measurement during the TTT period and proactively trigger the measurement report without the necessity of actually waiting for the entire TTT duration, thus reducing the HO latency.

- To avoid unnecessary handover: the UE avoids sending unnecessary measurement reports based on the prediction and consequently avoids the execution of unnecessary handover.

Proposal 1: In improve the mobility performance, measurement event prediction can be utilized to: 1. enable proactive measurement reporting, thereby reducing handover latency; 2. prevent the transmission of unnecessary measurement reports that could lead to unnecessary handovers.

Considering above applications, AI approaches could be applied to predict the measurement event in the following aspects:

- Option 1: Utilize AI to predict if certain event(s) is about to happen, i.e., the outcome of the predictions is whether the measurement event(s) condition is fulfilled in the future/the prediction window.

- Option 2: Utilize AI to predict if certain event(s) are valid, i.e., the outcome of the prediction is whether the occurring event is valid/invalid in the future/the prediction window. An event that leads to unnecessary HO or HOF is regarded as an invalid event, otherwise, it is considered a valid event.

In option 2, the occurring event means that the entering condition of the event is fulfilled. We define the corresponding event which leads to unnecessary handover or handover failure as invalid. If the entering condition of the occurring event is remain satisfied in the future prediction window, the occurring is considered as valid.

Proposal 2: RAN2 study the following two options of measurement event prediction, i.e. what to predict:

- Optoin 1: UE predicts if certain event is about to happen in the future/the prediction window.

- Option 2: UE predicts if the occurring event is valid in the future/ the prediction window. The occurring event is the event that the entering condition is fulfilled.

With the measurement event prediction, if the UE predicts the occurring event is valid or the entering condition of the event is fulfilled during the whole prediction window (which can be longer than TTT), the UE determine to trigger an early MR to reduce the handover latency and enhance overall mobility performance. If the UE predicts the occurring event is invalid or predicts the entering condition of the event is fulfilled during TTT, but the not satisfied after TTT, the UE determines to refrain from sending the measurement report (MR) or still send the measurement report with information to hint NW the invalid event to reduce the unnecessary handover.

Proposal 3: Following the measurement event prediction, the UE further determines when and whether to send the MR or prevent transmission of the MR.

Preliminary Results for Temporal Domain Case A of System-Level Simulation

During the simulation of AI-based mobility procedures, we utilize option 2 from proposal 2 to predict whether events that meet the entering condition at t0 still remain valid in the future prediction window. Upon the entering condition is fulfilled at t0, the UE initiates AI indirect event prediction inference. Based on the AI prediction results, determining whether an event continues to fulfil the condition during TTT or longer time at t1, which means the occurring event is predicted to be valid. In alignment with option 3 of the Handover (HO) modelling agreed by RAN2#128[1], the UE sends predicted MR and starts HO preparation at t0.

Figure 1: HO modelling option3 for system level simulation

Concurrently, the ground-truth label for real event at t2 are obtained based on actual measurement results. The results of n1, n2, and n3 are counted based on whether the absolute value of the time difference between the prediction results time t1 and the ground-truth labels time t2 is less than or equal to the max ETD and further leads to the calculation of intermediate KPIs (e.g., F1 score). It is important to note that the ground-truth labels obtained from actual measurements are not used for the actual execution of MR and HO but are only used for the calculation of intermediate KPIs. The actual execution of MR and HO relies entirely on the AI prediction results.

In our AI-based approach for indirect event prediction, we utilize 5 previously available time samples measurement results (totalling 400ms, with each sample interval 80ms), i.e., L3 cell RSRP with sliding window, to predict the subsequent 5 future time samples (totalling 400ms, with each sample interval 80ms) cell level measurement result.

Table 1 gives system level results for temporal case A of indirect event prediction. In the HO modelling of option 3, the HO command is sent in advance based on the prediction results, and the most intuitive system gain brought by AI is the increase in the time or probability of staying in the best cell, without increasing the occurrence of HOF and ping-pong or short ToS.

|  | TTT (ms) | HOF rate | Total number of HO attempts per UE per second | Total number of HOF per UE per second | Short ToS | Average ToS (ms) | Ping-pong | Camp on best cell |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AI | 160 | 23.7 | 0.221 | 0.0525 | 45 | 4535 | 27 | 0.71 |
|  | 320 | 26.6 | 0.225 | 0.06 | 46 | 4451 | 32 | 0.66 |
| Legacy | 160 | 18.8 | 0.378 | 0.06 | 136 | 2611 | 107 | 0.64 |
|  | 320 | 23.4 | 0.266 | 0.0625 | 51 | 3726 | 30 | 0.62 |

Table 1 System level results for temporal case A of indirect event prediction

The results indicates that AI improves the efficiency of handover attempts, increases average ToS, reduces ping-pong/short ToS, and increases the probability of camping on the best cell meanwhile without increase the number of HOF compared with legacy HO procedure, enhancing overall network performance and user experience. Therefore, we believe that AI is beneficial in this case for indirect event prediction temporal domain case A.

Observation 3: For indirect event prediction temporal domain case A, AI-based HO can provide ~11% gain (in terms of camp on best cell rate) and ~42% gain (in terms of average ToS) compared to the legacy HO approach and without increase of the total number of HOF per UE per second. The improvement is more significant in the short TTT case, e.g., 160ms, than in the longer TTT cases, e.g., 320ms.

We delve into the effects of missed and false event predictions on system performance. Missed event predictions can lead to situations such as:

- Although the cell handover is not performed in time, the quality of the original serving cell is still good enough, resulting in no RLF/HOF occurring.

- Avoidance of unnecessary handovers that could have resulted in ping-pong effects or short Time of Stay (ToS), due to the missed prediction.

- Deterioration in the current serving cell's quality due to the absence of a timely handover, leading to RLF/HOF.

False event predictions can lead to the following situations:

- Handovers to a suboptimal cell, which, despite not being the best choice, but the quality of that cell is still good enough, resulting in no RLF/HOF occurring.

- Handover to an incorrect cell, leading to a ping-pong/short ToS.

- A decline in the predicted serving cell's quality, leading to RLF/HOF.

The predicted missed and false events did not result in a significant increase in Handover Failures (HOFs). Moreover, the AI predictions contributed to a reduction in the occurrence of ping-pong and short ToS, we believe that AI for indirect event prediction is beneficial.

Proposal 4: RAN2 capture the observation in the TR that the system performance of temporal domain case A can be improved by indirect measurement event prediction with the method that the UE starts inference when the entering condition of the event is fulfilled, and AI predicts if the occurring event is valid in the prediction window. Evaluation on other methods is not precluded.

However, we observed legacy HO procedure has a lower HOF rate compared to AI-based HO for both TTT values. We further analysis of the detailed system performance results, and we found that the number of HOF per UE per second did not increase. According to the definition of HOF rate in TR36.839[2], which is Handover failure rate = (number of handover failures) / (Total number of handover attempts), we discover that the increase in HOF rate is due to a significant reduction in the number of HO attempts, rather than an increase in the number of HOF.

Observation 4: AI approach has a marginally elevated HOF rate compared to legacy HO procedures. The increase in HOF rate is due to the reduction in the number of HO attempts, rather than an increase in the number of HOF.

Therefore, we think the HOF rate cannot fully reflect the impact of AI on system-level performance, as the HOF rate is also influenced by the number of HO attempts. We propose to include the number of HOFs as a system-level performance metric would allow for a more intuitive analysis of system performance directly from our results table.

Proposal 5: RAN2 considers including the “total number of HOF per UE per second” as one of the system-level performance indicators.

Table 2 present the summarized intermediate evaluation results for TTT setting of 160ms and 320ms at a speed of 120km/h. The results indicate that the average L3 cell RSRP difference (between prediction and genie-aided label) is smaller than 1 dB. Furthermore, the RSRP difference at the last predicted point is larger than the average of the predicted RSRP differences.

| TTT(ms) | F1 score | Precision | Recall | Average L3 cell RSRP difference (dB) | Last predicted point L3 cell RSRP difference (dB) |
| --- | --- | --- | --- | --- | --- |
| 160 | 0.915 | 0.964 | 0.87 | 0.64 | 1.22 |
| 320 | 0.874 | 0.941 | 0.816 | 0.64 | 1.22 |

Table 1 Intermediate results for temporal case A of indirect event prediction

As the TTT increases from 160 ms to 320 ms, there is a noticeable decrease in the F1 score, Precision, and Recall. This indicates that a longer TTT negatively impacts the performance of the model performance. Specifically, the F1 score decreases from 0.915 to 0.874 as TTT increases. This suggests that the overall balance between precision and recall is better at a lower TTT. Precision decreases from 0.964 to 0.941 with an increase in TTT. This means that the model's ability to correctly identify positive instances (true positives) out of all predicted positives is slightly better at a lower TTT. Recall decreases more significantly from 0.87 to 0.816 as TTT increases. This indicates that the model's ability to identify all actual positive instances is more adversely affected by a longer TTT.

We analyse that this is because the model exhibits a larger RSRP difference when predicting over a longer prediction window, and therefore, within the same model prediction window, the performance of a longer TTT also deteriorates.

Observation 5: A lower TTT (160 ms) has better intermediate performance, including F1 score, Precision, and Recall), compared to a higher TTT (320 ms). The model demonstrates enhanced accuracy and reliability with a reduced TTT duration.

However, the intermediate results are significantly affected by the implementation method for ground-truth labelling, which may not accurately represent the actual effect on system performance. For a more detailed discussion, please consult another paper [3] from our work.

Direct measurement event prediction

For direct event prediction, RAN2 has agreed that the model output is the probability of event occurrence within a time window. And the time window is equivalent to the prediction window. However, in the process of implementing direct prediction, the definition of the prediction window alone is insufficient. We still want to clarify the definition of prediction window and “tolerance window” similar to the max ETD in indirect event prediction. The two concepts are for different purposes, the prediction window is the time window of making prediction, while the prediction tolerance window refers to the permissible error tolerance window for deriving performance metrics. To differentiate them, we use "prediction window" and "tolerance window" to refer to the two “time windows”. Since the tolerance window is for purpose of deriving the intermediate performance metrics, it may not have specification impact in the future.

If we confuse the two, the length of the prediction window will affect the statistics of intermediate KPIs, which is highly unreasonable.

Observation 6: The prediction window and tolerance window are two different concepts and with different usage, i.e. one is for inference operation and the other is for deriving performance metrics for intermediate KPI.

We depict the prediction window and tolerance window separately in the figure 1. The AI will predict the event that occurs in the prediction window (PW), i.e., t0 to tp. The AI model output the probability of prediction window, where the probability is higher than certain threshold and we think there is an event occurrence at t1. If a real event occurs within the [t1 - tolerance window/2, t1 + tolerance window/2] of the predicted event time t1, we consider it a True event prediction. If an event occurs but is predicted as not happening, it is a Missed event prediction. If no event occurs within this tolerance window but is predicted to happen, it is a False event prediction. With this interpretation, we can clearly distinguish between prediction window and tolerance window.

Figure 1: Interpretation of prediction window and tolerance window

Proposal 6: RAN2 clarify the interpretation of time window, including prediction window and tolerance window. The prediction window is consecutive to the observation window starts from t0. The AI will predict the event that occurs in the prediction window (PW), i.e., t0 to tp. With the output in prediction window, the tolerance window is used to derive the intermediate performance metrics.

Proposal 7: RAN2 clarify the interpretation of the prediction window is the time window of model inference output.

Proposal 8: RAN2 clarify the interpretation of the tolerance window is the time window used to derive the intermediate KPI (i.e., F1 score) based on the output of prediction window.

Proposal 9: For direct measurement event prediction, take the interpretation of prediction window and tolerance window as the baseline.

As shown in Figure2, if an event occurs at a specific time instance within the window, and if we only output one probability for the prediction window, it's hard to determine the exact predicted occurrence time within the window. Therefore, the AI model's output can be the probability of the prediction window and the event occurrence probabilities for all time points within the PW. We propose to take the largest probability higher than certain threshold within the prediction window as the timing of the predicted event occurrence.

Figure 2: The output/label of the direct prediction

Proposal 10: For direct measurement event prediction, the model output should be the probability of each time instance of within the prediction window and the highest probability time instance higher than certain threshold is the time instance of event occurs in the prediction window.

Proposal 11: For direct measurement event prediction, with the highest probability time instance higher than certain threshold in prediction window, the tolerance window can be used to determine the True event prediction”, “Missed event prediction”, and “False event prediction” to derive the F1 score.

Conclusion

Motivation

Observation 1: As cellular technology advances with higher frequency bands like FR2 and beamforming are utilized, prompting the usage of TTT within legacy L3 handover mechanisms to mitigate unnecessary transitions and reduce "ping-pong" effects, though at the cost of introducing latency and achieving only suboptimal mobility performance.

Observation 2: UE may experiences continuous and unnecessary handover and significantly affect the mobility performance and user experience, leading to issues like game freezing, delays, and unsmooth video services.

Proposal 1: In improve the mobility performance, measurement event prediction can be utilized to: 1. enable proactive measurement reporting, thereby reducing handover latency; 2. prevent the transmission of unnecessary measurement reports that could lead to unnecessary handovers.

Proposal 2: RAN2 study the following two options of measurement event prediction, i.e. what to predict:

- Optoin 1: UE predicts if certain event is about to happen in the future/the prediction window.

- Option 2: UE predicts if the occurring event is valid in the future/ the prediction window. The occurring event is the event that the entering condition is fulfilled.

Proposal 3: Following the measurement event prediction, the UE further determines when and whether to send the MR or prevent transmission of the MR.

System-Level Simulation results for indirect prediction

Observation 3: For indirect event prediction temporal domain case A, AI-based HO can provide ~11% gain (in terms of camp on best cell rate) and ~42% gain (in terms of average ToS) compared to the legacy HO approach and without increase of the total number of HOF per UE per second. The improvement is more significant in the short TTT case, e.g., 160ms, than in the longer TTT cases, e.g., 320ms.

Observation 4: AI approach has a marginally elevated HOF rate compared to legacy HO procedures. The increase in HOF rate is due to the reduction in the number of HO attempts, rather than an increase in the number of HOF.

Observation 5: A lower TTT (160 ms) has better intermediate performance, including F1 score, Precision, and Recall), compared to a higher TTT (320 ms). The model demonstrates enhanced accuracy and reliability with a reduced TTT duration.

Proposal 4: RAN2 capture the observation in the TR that the system performance of temporal domain case A can be improved by indirect measurement event prediction with the method that the UE starts inference when the entering condition of the event is fulfilled, and AI predicts if the occurring event is valid in the prediction window. Evaluation on other methods is not precluded.

Proposal 5: RAN2 considers including the “total number of HOF per UE per second” as one of the system-level performance indicators.

Solution for direct event prediction

Observation 6: The prediction window and tolerance window are two different concepts and with different usage, i.e. one is for inference operation and the other is for deriving performance metrics for intermediate KPI.

Proposal 6: RAN2 clarify the interpretation of time window, including prediction window and tolerance window. The prediction window is consecutive to the observation window starts from t0. The AI will predict the event that occurs in the prediction window (PW), i.e., t0 to tp. With the output in prediction window, the tolerance window is used to derive the intermediate performance metrics.

Proposal 7: RAN2 clarify the interpretation of the prediction window is the time window of model inference output.

Proposal 8: RAN2 clarify the interpretation of the tolerance window is the time window used to derive the intermediate KPI (i.e., F1 score) based on the output of prediction window.

Proposal 9: For direct measurement event prediction, take the interpretation of prediction window and tolerance window as the baseline.

Proposal 10: For direct measurement event prediction, the model output should be the probability of each time instance of within the prediction window and the highest probability time instance higher than certain threshold is the time instance of event occurs in the prediction window.

Proposal 11: For direct measurement event prediction, with the highest probability time instance higher than certain threshold in prediction window, the tolerance window can be used to determine the True event prediction”, “Missed event prediction”, and “False event prediction” to derive the F1 score.

Reference

[1] R2_128_ChairNotes_24-11-22_17-00_eom

[2] 3GPP TR 36.839.

[3] R2-2500324