3GPP TSG-RAN WG2 Meeting #129		R2- 2500336

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.5.4

Source:	vivo

Title:	Discussion on adaptation of common signal transmissions

Document for:	Discussion and Decision

Introduction

In RAN2#128 meeting, adaptation of paging occasion for network energy saving was further discussed and the agreements below were achieved [1]:

Agreements on paging adaptation

- Introduce value for Ns=8. FFS on 16.

- Introduce value for N= T/32. FFS on T/64, T/128, T/256.

Moreover, RAN1 and RAN4 had made some progress on the PRACH adaptation and SSB adaptation in time domain. In this contribution, we will further provide our consideration on the adaptation of paging occasions in time domain, solutions for PRACH adaptation and answer to the LS of RAN4 for SSB adaptation.

Discussion

## 2.1 Adaptation of paging occasions in the time domain

- New value N in option b

Agreement in RAN2#128 2. Introduce value for N= T/32. FFS on T/64, T/128, T/256.

We take a UE operating in non-eDRX mode for example. According to the current specification, a UE determines its DRX cycle to be the shortest of the UE specific DRX value configured by RRC (if any), the UE specific DRX value configured by upper layers (if any), and a default DRX value broadcast in system information [2]. The UE specific DRX value configured by upper layers refers to the CN configured DRX cycle [3], which cannot be controlled by the RAN node. As a consequence, if the RAN node broadcasts a value nAndPagingFrameOffset with one of {T/64, T/128, T/256}, a UE with a CN DRX cycle T = 32rf will calculate N to be smaller than 1, which means there will be only 1 Paging Frame existing on every 2 DRX cycle of this UE, which is against the WI requirement as following [4]:

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

Considering this situation, companies have provided the following solutions from RAN side in the last meeting:

- Solution 1: Set N = 1;

- Solution 2: Switch to legacy paging mechanism;

- Solution 3: Apply default paging cycle;

Solution 1 does not work as expected to achieve NES gain. For instance, for the UEs with DRX cycle T = 32rf, if nAndPagingFrameOffset is configured as T/64, then N = 1/2. If we set N from 1/2 to 1 for this situation, then PF will occur once every 32rf for the UEs with DRX cycle T = 32rf, according to the PF calculation formula [TS 38.304]:

SFN for the PF is determined by: (SFN + PF_offset) mod T = (T div N)*(UE_ID mod N)

Therefore, although paging latency is not impacted by solution 1, the expected NES gain by setting nAndPagingFrameOffset to 1/64T (i.e. 1 PF every 64rf) can not be achieved and hence this configuration is meaningless.

Solution 2 relies on NW implementation. However, the new nAndPagingFrameOffset values {T/64, T/128, T/256} are meaningless since a NES cell has no way so far to guarantee that it does not have any UEs configured with CN DRX cycle = 32rf camping on it. Thus, the best strategy an NES cell can apply is not to configure a value nAndPagingFrameOffset with one of {T/64, T/128, T/256}, even if RAN2 agrees to support. Otherwise, it will cause paging latency to the UEs configured with CN DRX cycle = 32rf and the RAN node is not aware of it.

Solution 3 will cause paging latency to the UEs configured with CN DRX cycle = 32rf when default paging cycle equals to one of {64rf, 128rf, 256rf}, which is against the WID requirement.

To sum up, none of the solutions on the table solves the problem when N is determined to be smaller than 1, and thus we propose RAN2 not to support new nAndPagingFrameOffset (i.e. N) value {T/64, T/128, T/256} for option b.

Observation 1: If new value nAndPagingFrameOffset = {T/64, T/128, T/256} is introduced and configured, for the UEs with the DRX cycle smaller than 64rf, UE will calculate and determine an N value smaller than 1, which will increase the paging latency of such UEs. The solutions on the table for this situation can not properly address this issue.

Proposal 1: Not to support new nAndPagingFrameOffset (i.e. N) values {T/64, T/128, T/256} for option b.

If the majority companies still prefer and agree to have new nAndPagingFrameOffset values {T/64, T/128, T/256}, one solution to make the introduced values meaningful is that the CN ensures all the UEs in the NES cell have at least one PF per DRX cycle when N value {T/64, T/128, T/256} is configured. For example, when the CN determines all the UEs in the Tracking Area where the NES cell belongs can have DRX cycle larger than 32rf, the CN negotiates with the NES cell so that the NES cell can apply a value nAndPagingFrameOffset with one of {T/64, T/128, T/256} to achieve better NES gain without worrying to impact any UE’s paging latency. Therefore, if RAN2 agrees to support new nAndPagingFrameOffset values {T/64, T/128, T/256}, we propose to send an LS to SA to ask about the feasibility of whether the CN can assist the RAN node to configure N = {T/64, T/128, T/256} for better NES gain.

Proposal 2: If RAN2 agrees to support new nAndPagingFrameOffset (i.e. N) value {T/64, T/128, T/256}, send an LS to SA to ask about the feasibility of whether the CN can assist the RAN node to configure nAndPagingFrameOffset (i.e. N) = {T/64, T/128, T/256} for better NES gain.

- Ns value and signaling overhead of option b

Agreement in RAN2#128 - Introduce value for Ns=8. FFS on 16. Agreement in RAN2#127bis 2. R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values)

In RAN2#128 meeting, RAN2 agreed that the number of POs per PF includes at least 8, and FFS 16. For legacy case, the maximum total number of POs (N*Ns in one T) that can be supported is 128 when T = 32 rf, N = T and Ns = 4. For NES scenario, when N = T/32, if we only support the Ns = 8, the paging capacity reduces 16 times comparing with the legacy case. Thus, taking paging capacity into account, we think the Ns values can be extended to 16.

Proposal 3: Support new Ns value = 16 for option b.

In the table below, we analyze the signaling overhead of option b when SCS is 30kHz, Ns = {8, 16}, {T=32 rf, N=1/32T} and {T=64 rf, N=1/64T} for instance.

| SCS=30kHz T=320ms N=1/32T | firstPDCCH-MonitoringOccasionOfPO=32 radio frames =320 sub frames =640 slots =8960 symbols The number of bits used for a PO offset: 14 bits |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The number of POs of one PF | Ns =1 | Ns =2 | Ns = 4 | Ns = 8 | Ns =16 |
| The signaling of firstPDCCH-MonitoringOccasionOfPO | 14 bits | 28 bits | 56 bits | 112 bits | 224 bits |
| SCS=30kHz T=640ms N=1/64T | firstPDCCH-MonitoringOccasionOfPO=64 radio frames =640 sub frames =1280 slots =17920 symbols The number of bits used for a PO offset: 15 bits |  |  |  |  |
| The number of POs of one PF | Ns=1 | Ns =2 | Ns =4 | Ns =8 | Ns =16 |
| The signaling of firstPDCCH-MonitoringOccasionOfPO | 15 bits | 30 bits | 60 bits | 120 bits | 240 bits |

Table 1: signaling overhead of option b.

As the number of POs increases, the bits of firstPDCCH-MonitoringOccasionOfPO will be increased to 14*x or 15*x from the examples we listed, where x is the number of POs in one PF. When the number of POs in one PF is greater than or equal to 8, the signaling overhead of firstPDCCH-MonitoringOccasionOfPO has exceeded hundreds of bits. However, the maximum SIB1 size is only 2976 bits. We think that the hundreds of signaling overhead caused by firstPDCCH-MonitoringOccasionOfPO is unacceptable and should be optimized.

TS 38.331 NOTE 2: The physical layer imposes a limit to the maximum size a SIB can take. The maximum SIB1 or SI message size is 2976 bits.

Observation 2: When the number of POs in one PF equals 8 or 16, the signaling overhead of firstPDCCH-MonitoringOccasionOfPO has exceeded hundreds of bits

When one PF per DRX cycle is configured, the maximum value range of firstPDCCH-MonitoringOccasionOfPO extends to the length of one DRX cycle because the value range of firstPDCCH-MonitoringOccasionOfPO depends on the length between two adjacent PFs. To achieve better NES gain, it is unnecessary to distribute the POs throughout the DRX cycle. Otherwise, the NW can simply configure more than one PF. Therefore, the easiest way is to restrict the maximum value range of firstPDCCH-MonitoringOccasionOfPO. For different SCS and N, the new value range can be set to (0…x) for which x can be predefined, i.e.,

- firstPDCCH-MonitoringOccasionOfPO-v19xy   CHOICE {

- SCS15KHZonethirtysecondT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..X1),

- SCS30KHZonethirtysecondT                   SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..X2),

- SCS60KHZonethirtysecondT                                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0.. X3)

- SCS120KHZonethirtysecondT        SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0.. X4),

- SCS480KHZonethirtysecondT     SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0.. X5)

- }      OPTIONAL,           -- Need R

Observation 3: The value range of firstPDCCH-MonitoringOccasionOfPO corresponding to N = 1/32T does not need to span the whole DRX cycle, considering the target scenario is for NES. Otherwise, the network can simply configure N = 1/16T or 1/8T etc.

Proposal 4: For signaling overhead optimization, restrict the maximum value range of firstPDCCH-MonitoringOccasionOfPO for different SCS corresponding to the new value nAndPagingFrameOffset (i.e. N).

- Paging Early Indication (PEI)

RAN2#128 Proposal-3: R2 further discuss to whether 1) based on a single PEI-O configuration, and rely on network implementation to have separate PEI:s for the two types of PO:s, or 2) based on separate PEI-O configurations, and thus automatically separate PEIs for the two types of PO:s. Where the two types of POs are 1) PO:s monitored by legacy UE (may or may not shared with R19 UE, up to network configuration), and 2) PO:s monitored only by R19 UE Proposal-4: R2 assume to leave it to network implementation to set po-NumPerPEI and maxPEI-perPF properly, in order to be compatible with Ns=8 without further spec effort RAN2 will revisit PEI configuration and PEI transmission aspects with the consideration of offline discussion in the next meeting.

RAN2 discussed the impact to the PEI configuration in the last meeting. The issue needs to be discussed is whether to allow a single PEI configuration for both legacy PO and R19 NES PO. There are two options have been discussed as follows:

- Option 1: based on a single PEI-O configuration, and rely on network implementation to have separate PEIs for the two types of POs;

  - Understanding 1: single PEI configuration with separate PEI transmission;

  - Understanding 2: single PEI configuration with single PEI transmission;

- Option 2: based on separate PEI-O configurations, and thus automatically separate PEIs for the two types of POs;

There are two understandings of option 1 from the last meeting. For understanding 1, it can be achieved by the figure 1. For example, relying on network implementation to send two PEIs associated legacy PF and R19 NES PF respectively based on the single PEI configuration with the same frame offset (pei-FrameOffset) and number of POs associated with one PEI monitoring occasion (po-NumPerPEI=8 ).

Figure 1: single PEI configuration with separate PEI transmission.

PEI-Config-r17 ::=                        SEQUENCE {

po-NumPerPEI-r17                          ENUMERATED {po1, po2, po4, po8},

payloadSizeDCI-2-7-r17                    INTEGER (1..maxDCI-2-7-Size-r17),

pei-FrameOffset-r17                       INTEGER (0..16),

subgroupConfig-r17                        SubgroupConfig-r17,

lastUsedCellOnly-r17                      ENUMERATED {true}                                                OPTIONAL,  -- Need R

}

Figure 2: PEI related configuration.

Agreement in RAN2#127bis 1. Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration.

For understanding 1 of option 1, the network can not flexibly handle the case where PEI can associate with different numbers of NES POs and legacy POs. Take figure 4 as an example. When the number of legacy PO in one PF is 2 and the PEI is associated with two legacy PFs (which means the number of total legacy POs is 4), the same single PEI configuration can not adapt to configure 8 NES POs in one NES PF.

Figure 3: PEI associated with different number of NES POs and legacy PO

Observation 4：For understanding 1 of option 1, the network can not associate the PEI with different numbers of NES POs and legacy PO with one PEI configuration.

For understanding 2 of option 1, there are two cases needs to be discussed.

- Case 1: one PEI associated with NES PF and legacy PF, which are overlapping.

- Case 2: one PEI associated with NES PF and legacy PF, which are not overlapping.

For case 1, legacy UEs and NES-capable UEs can use the single PEI configuration with the same frame offset to determine the location of PEI. The UEs that supporting CN assigned subgrouping can respectively monitor their own subgroups in PEI relying on network implementation to divide legacy UEs and NES-capable UEs into different subgroups based on the UE capability. If the UEs only support UE_ID based subgrouping, the network may wake up legacy UEs and NES-capable UEs simultaneously even if the network only wants to wake up legacy UEs or NES-capable UEs. For case 2, NES-capable UEs can not use the same frame offset as legacy UEs because they are not overlapping. Reading legacy PEI for NES-capable UE will be complicated because the location of legacy PEI is calculated based on the legacy PF. The NES-capable UEs do not know which legacy PF is associated with the PEI unless indicated by the network.

Figure 4: single PEI configuration with single PEI transmission in case 2.

Observation 5：For case 1 of understanding 2, if the UEs only supports UE_ID based subgrouping, the network may wake up legacy UEs and NES-capable UEs simultaneously even if the network only wants to wake up legacy UEs or NES-capable UEs.

Observation 6：For case 2 of understanding 2, the NES-capable UEs do not know which legacy PF is associated with the PEI unless indicated by the network.

We propose to adopt separate PEI-O configuration as it is cleaner and more flexible.

Proposal 5: RAN2 to support separate PEI-O configurations for the two types of POs.

- UE capability reporting

For paging adaptation, the UE can report the capability of supporting paging adaptation to the gNB after registration with AMF, and the gNB can report the capability to the AMF. After receiving this capability from the UE, AMF can forward this capability to the anchor gNB for RAN paging. The AMF forwards this capability information to all gNBs in the latest TA of the UE for CN paging. Similarly, the anchor gNB can also forward this capability information to other gNBs for RAN paging. Then, the serving gNB can identify whether the paging is for a UE supporting paging adaptation or not based on the exchanged UE capability, which assists the network to determine the paging location. For example, when the network wants to page a UE that supports paging adaptation, if the network is not aware of whether paging adaptation is supported by this UE, the NW will not know whether to apply legacy configuration or the NES configuration. Thus, we think the capability of supporting paging adaptation should be introduced.

Proposal 6: A new UE capability is defined for the NES-capable UEs to indicate the support of paging adaptation.

## 2.2 Adaptation of SSB in time domain

Agreements: - RAN4 to discuss whether there are any impacts on at least the following RRM requirements due to SSB adaption: - L1-RSRP for serving cell - L1-SINR - BFD - CBD - SCell activation - TCI state switching - NR intra-frequency L3 measurement - NR inter-frequency L3 measurement However, L3 measurement requirement in RAN4 is defined based on SMTC configuration rather than SSB configuration in legacy. RAN4 would like to ask RAN2 that whether there is any SMTC impact due to SSB adaptation.

From the above question in RAN4’s LS [5], RAN4 asked RAN2 whether there is any SMTC impact due to SSB adaptation. Currently, UE performs L3 measurement based on the SMTC configured to the UE. Considering the adaptation of SSB, there are two options to be considered:

- Option 1: The network ensures that the SMTC periodicity is larger than the periodicity of adapted SSB. There is no SMTC impact due to SSB adaptation.

- Option 2: SMTC periodicity can be adapted with the SSB periodicity.

For option 1, the network can ensure that the UEs always have SSB during measurement. For example, if there is a cell, its SSB periodicity adapts from 20ms to 80ms, the network can configure SMTC to 80ms directly. There is no SMTC impact due to SSB adaptation. If the periodicity of adapted SSB is smaller than SMTC periodicity, the network can reconfigure the SMTC periodicity.

For option 2, multiple SMTC configurations need to be introduced based on the SSB periodicity, which will introduce extra signaling overhead. In addition, if the SMTC is for neighbour cell measurements and the neighbour cell supports SSB adaptation, it needs target gNB dynamically informs serving gNB the SSB periodicity, which will also have extra delay and signaling overhead. Thus, option 1 is the easiest way that we prefer.

Proposal 7: Send LS reply to RAN4 to inform there is no SMTC impact due to SSB adaptation, e.g. the network ensures that the SMTC periodicity is larger than the periodicity of adapted SSB.

## 2.3 Adaptation of PRACH resources in time domain

- Legacy UE handling

To achieve the NES purpose in PRACH adaptation, the network could configure legacy PRACH resources sparsely to reduce the monitoring time between legacy ROs when the system load is low. When the number of camping UEs increases a lot, additional NES-related PRACH resources with shorter period can be activated for RACH offloading. Meanwhile, the legacy UEs can still perform random access procedure with the legacy PRACH resources. The following note in the WID [4] for adaptation of common signal/channel transmissions also points out that there shall be no negative impact on legacy UE. Since we do not see significant benefits to bar the legacy UEs, we propose RAN2 to not introduce cell barring mechanism for PRACH adaptation.

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown.

Proposal 8: The NW does not bar legacy UEs for the purpose of PRACH adaptation.

- RACH triggering events

Currently, a number of triggering events for random access are specified in TS38.300 as follows:

The random access procedure is triggered by a number of events: - Initial access from RRC_IDLE; - RRC Connection Re-establishment procedure; - DL or UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure (see clause 18.0) is ongoing, when UL synchronisation status is "non-synchronised"; - UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when there are no PUCCH resources for SR available; - Handover, except for when RACH-less HO is configured; - SR failure; - Explicit request by RRC upon synchronous reconfiguration; - RRC Connection Resume procedure from RRC_INACTIVE; - To establish time alignment for a primary or a secondary TAG; - Request for Other SI (see clause 7.3); - Beam failure recovery; - Consistent UL LBT failure on SpCell; - SDT in RRC_INACTIVE (see clause 18); - Positioning purpose during RRC_CONNECTED requiring random access procedure, e.g., when timing advance is needed for UE positioning; - Early UL synchronization with an LTM candidate cell; - RACH-based LTM cell switch.

RAN2 needs to discuss whether all the above triggering events can be supported for additional PRACH resources. In RAN1#116bis [6], RAN1 had agreed to support additional PRACH resources for RRC_IDLE/INACTIVE and RRC_CONNECTED UEs, so the corresponding triggering events of RRC_IDLE/INACTIVE and RRC_CONNECTED should be considered. For the events like “handover”, “Early UL synchronization with an LTM candidate cell” and “RACH-based LTM cell switch”, even if the PRACH resources are configured for target cells, the function of the additional PRACH resources is no different from that of the legacy PRACH resources, since the random access in additional PRACH resources is only to reduce the PRACH congestion when the legacy PRACH resources may be configured sparsely for NES. Therefore, the PRACH procedure for all triggering events of legacy PRACH resources should also be supported for additional PRACH resources.

RAN1#116bis Agreement Support adaptation mechanisms of PRACH in time-domain for following: - UE in idle/inactive mode - UE in connected mode

Proposal 9: The PRACH procedure for all triggering events of legacy PRACH resources should also be supported for additional PRACH resources.

- Selection of PRACH resources

In RAN1#116bis meeting, the below agreements on adaptation of PRACH were made [6]:

Agreement For adaptation of PRACH in time-domain, support at least the following: - Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any) - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs - Configuration of additional PRACH resources is provided by semi-static signalling - FFS: details including whether there is overlap of additional PRACH resources and PRACH resources for legacy UEs - FFS: adaptation mechanism for additional PRACH resources - Note: No change to the existing PRACH configuration tables in 38.211

Based on the above agreements, we can see that RAN1 has agreed to support additional PRACH resources for NES-capable UEs. NES-capable UEs can use both additional PRACH resources and legacy PRACH resources. One potential RAN2 impact is how NES-capable UE selects PRACH resources if additional PRACH resources are enabled. If NES-capable UEs still select legacy PRACH resources when there are additional PRACH resources, the congestion of RACH in legacy PRACH resources would be increased heavily since the legacy UE can only use legacy PRACH resources. Therefore, to reduce the impact on legacy UEs, NES-capable UEs should prioritize selecting the additional PRACH resources. If the number of RACH failures exceeds a threshold in additional PRACH resources, NES-capable UEs can select the legacy PRACH resources.

Proposal 10: The NES UE prioritizes selecting the additional NES RO if both additional NES RO and legacy RO are configured.

Proposal 11: The NES UE can switch to legacy ROs after a configured number of times of RACH attempt in additional NES ROs.

Conclusion

Based on the discussion, we have the following proposals:

Adaptation of paging occasions in the time domain

Observation 1: If new value nAndPagingFrameOffset = {T/64, T/128, T/256} is introduced and configured, for the UEs with the DRX cycle smaller than 64rf, UE will calculate and determine an N value smaller than 1, which will increase the paging latency of such UEs. The solutions on the table for this situation can not properly address this issue.

Proposal 1: Not to support new nAndPagingFrameOffset (i.e. N) values {T/64, T/128, T/256} for option b.

Proposal 2: If RAN2 agrees to support new nAndPagingFrameOffset (i.e. N) value {T/64, T/128, T/256}, send an LS to SA to ask about the feasibility of whether the CN can assist the RAN node to configure nAndPagingFrameOffset (i.e. N) = {T/64, T/128, T/256} for better NES gain.

Proposal 3: Support new Ns value = 16 for option b.

Observation 2: When the number of POs in one PF equals 8 or 16, the signaling overhead of firstPDCCH-MonitoringOccasionOfPO has exceeded hundreds of bits

Observation 3: The value range of firstPDCCH-MonitoringOccasionOfPO corresponding to N = 1/32T does not need to span the whole DRX cycle, considering the target scenario is for NES. Otherwise, the network can simply configure N = 1/16T or 1/8T etc.

Proposal 4: For signaling overhead optimization, restrict the maximum value range of firstPDCCH-MonitoringOccasionOfPO for different SCS corresponding to the new value nAndPagingFrameOffset (i.e. N).

Observation 4：For understanding 1 of option 1, the network can not associate the PEI with different numbers of NES POs and legacy PO with one PEI configuration.

Observation 5：For case 1 of understanding 2, if the UEs only supports UE_ID based subgrouping, the network may wake up legacy UEs and NES-capable UEs simultaneously even if the network only wants to wake up legacy UEs or NES-capable UEs.

Observation 6：For case2 of understanding 2, the NES-capable UEs do not know which legacy PF is associated with the PEI unless indicated by the network.

Proposal 5: RAN2 to support separate PEI-O configurations for the two types of POs.

Proposal 6: A new UE capability is defined for the NES-capable UEs to indicate the support of paging adaptation.

Adaptation of SSB adaptation in the time domain

Proposal 7: Send LS reply to RAN4 to inform there is no SMTC impact due to SSB adaptation, e.g. the network ensures that the SMTC periodicity is larger than the periodicity of adapted SSB.

Adaptation of PRACH resources in time domain

Proposal 8: The NW does not bar legacy UEs for the purpose of PRACH adaptation.

Proposal 9: The PRACH procedure for all triggering events of legacy PRACH resources should also be supported for additional PRACH resources.

Proposal 10: The NES UE prioritizes selecting the additional NES RO if both additional NES RO and legacy RO are configured.

Proposal 11: The NES UE can switch to legacy ROs after a configured number of times of RACH attempt in additional NES ROs.

# 4. Reference

- RAN2#128 meeting minutes

- TS 38.304, V18.4.0

- TS 23. 501, V19.2.1

- RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson.

- R4-2416911, LS on SSB adaptation, Apple.

- RAN1#116bis meeting minutes