3GPP TSG-RAN WG2 Meeting #129		R2-2500346

Athens, Greece, 17th – 21st Feb. 2025

Source:	vivo

Title:	Discussion on inter-CU LTM

Agenda Item:	8.6.2

Document for:	Discussion and Decision

- Introduction

In the last meeting, the following agreements were made for inter-CU LTM [1]:

Agreements on inter-CU LTM

- Attempt to have RAN2 consensus on the definition of subsequent inter-CU LTM in offline and include it to reply LS. If failed, we will copy the WID aim that is written in the WID.

- Reply to SA3 that RAN2 think avoiding RRC configuration between cell switches for subsequent inter-CU LTM is important and it needs to be respected in solution selection.

- Include the time plan information to indicate until when their feedback is required or until when RAN2 needs to down-select the options. We assume in RAN2#130, to complete WI.

- RAN2 to postpone the discussion on fast recovery upon inter-CU LTM failure until inter-CU LTM security solution is finalized, if fast recovery is feasible.

- Rel-19 Set ID is configured for a candidate configuration. For DRB:

- Inter-CU MCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> UE performs PDCP re-establishment to all radio bearers.

- Inter-CU SCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> For MN terminated bearer without change of termination point, UE does not perform PDCP re-establishment. PDCP recovery is needed if it is split bearer.

> For SN terminated bearer without change of termination point, UE performs PDCP re-establishment.

> If there is change of termination point for a radio bearer (the keyToUse in the RadioBearerConfig is different from the keyToUse in the current UE configuration), UE performs PDCP re-establishment.

- For SRBs in inter-CU SCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3.

- For SRB1/2 in inter-CU SCG LTM, PDCP re-established is not performed based on NW configuration (PDCP re-establishment flag and SDU discard).

- RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it.

- RAN2 confirms that the inter-CU MCG LTM with intra-SN PSCell change is supported in Rel19.

- From RAN2 perspective, the following coexistence cases in NR-DC can be supported:

- Case 1: Intra-CU MCG LTM + Inter-CU MCG LTM

- Case 2: Intra-CU SCG LTM + Inter-CU SCG LTM

- In coexistence cases of inter-CU MCG/SCG LTM and intra-CU MCG/SCG LTM, when inter-CU MCG or SCG LTM is executed, it’s up to the NW to ensure that maintained LTM candidate configurations are valid, e.g. reconfigure or release invalid intra-CU MCG/SCG LTM candidate configurations. UE does not autonomously release invalid intra-CU candidate configurations.

- RAN2 to support intra-CU SCG LTM in MN RRC message (i.e. MN RRCReconfiguration message), in addition to SN RRC message.

- RAN2 to support intra-CU MCG LTM with SCG configuration.

- It’s up to NW to ensure that the complete configuration includes the MCG part and SCG part configuration when UE combines the reference and candidate configuration for inter-CU SCG LTM.

- RAN2 assumes that how to indicate the list of candidate PSCells from source SN to MN is up to RAN3. From RAN2 perspective, in INM, source SN may send measurement results of candidate PSCells to the MN. The MN then forwards the measurement results to the candidate SN(s), and then the candidate SN(s) determines the LTM candidate cells based on the measurement results and the upper limit for the number of PSCells that can be prepared by each candidate SN. The existing IEs defined in INM can be reused as a baseline.

In this paper, we first discuss the remaining open issues for inter-CU LTM in the SA scenario. Then we discuss the scenarios and issues of inter-CU LTM with DC.

- Discussion

  - Remaining issues for Inter-CU LTM in SA

UE-based TA measurement

In RAN2#126 meeting, whether UE-based TA measurement can be supported for inter-CU LTM was discussed. Some companies believe that RAN4 has already defined the UE requirement for UE-based TA measurement and it applies to both intra-CU and inter-CU scenarios. From our understanding, UE-based TA measurement is supported in synchronized cases in Rel-18 according to the reply LS by RAN4 [2]. Therefore, when the serving cell and candidate cells in Rel-19 from different CUs are synchronized, UE-based TA measurement is supported obviously and the Rel-18 mechanism can be reused.

Observation 1: If the serving cell and candidate cells are from different CUs and are synchronized, UE-based TA measurement can be supported by reusing the Rel-18 mechanism.

Proposal 1: RAN2 confirms that UE-based TA measurement is supported in the synchronized inter-CU LTM scenario, and no specification impact is expected.

However, for Rel-19 inter-CU LTM, as the serving cell and the candidate cells may belong to different CUs and are more likely to be unsynchronized, it seems highly beneficial to support UE-based TA measurement for unsynchronized scenarios. The decision on whether UE-based TA measurement for unsynchronized cases can be supported and how to support it should be made by RAN1 and RAN4. Therefore, an LS should be sent to RAN1 and RAN4 to inquire about the feasibility and approach for supporting UE-based TA when the serving cell and the candidate cell are unsynchronized.

Proposal 2: RAN2 confirms that supporting UE-based TA measurement in the unsynchronized case is beneficial for inter-CU LTM from the RAN2 perspective.

Proposal 3: Send an LS to RAN1/4 to ask whether it is feasible to support UE-based TA measurement in unsynchronized cases.

CFRA resource in LTM Cell Switch Command MAC CE

For RACH-based LTM, reserving specific CFRA resources for each UE via pre-configured RRC configuration will result in a huge waste of CFRA resources for the candidate cells. To resolve the issue in Rel-18, CFRA resources are supported to be indicated in the LTM cell switch command, which means the CFRA resource can be allocated when the S-DU determines to trigger LTM and prevent the waste of RACH resources.

To address the potential waste of CFRA resources caused by inter-CU LTM, we propose supporting the inclusion of CFRA resources in the LTM Cell Switch Command MAC CE for inter-CU LTM.

Proposal 4: CFRA resources can be included in the LTM Cell Switch Command MAC CE for inter-CU LTM. Send an LS to inform RAN3.

Coexistence of inter-CU LTM with CHO

The coexistence of inter-CU LTM with CHO was supported based on the agreement reached in RAN2#127 meeting.

RAN2#127 Agreement

- L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM):

- When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network.

- The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations.

- For the execution order between CHO and LTM, Rel-18 principle is applied.

However, since CHO is triggered by the UE and RRCReconfiguraiton is initiated from the network, there may be cases where the RRCReconfiguration from the source gNB conflicts with an ongoing conditional handover. This can result in the UE failing to receive the RRCReconfiguration message. If the RRCReconfiguraiton message includes information related to LTM configuration update, the target gNB of CHO must be aware of the updated LTM configuration in order to reconfigure the UE accordingly. Otherwise, a misalignment may occur between the UE and the network, potentially leading to the failure of subsequent LTM cell switch. To illustrate this issue, we refer to the example presented in the following figure.

Figure 1. Conflict of CHO and RRCReconfiguration for inter-CU LTM

To address the issue, interaction between the source gNB and the target gNB of CHO is essential. RAN3 is expected to design the corresponding signaling enhancement.

Proposal 5: Send an LS to ask RAN3 to consider the LTM configuration mismatch issue between UE and the network caused by CHO execution.

Coexistence of inter-CU LTM with other features

In Rel-18 intra-CU LTM, we agreed that it can coexist with some other features, such as coverage enhancement, RedCap, and MIMO 2TA. As for the Rel-19 inter-CU LTM, we believe that there is no difference regarding the coexistence issue. Therefore, we propose that RAN2 confirms that inter-CU LTM can coexist with coverage enhancement, RedCap, and MIMO 2TA.

Proposal 6: RAN2 confirms that inter-CU LTM can coexist with coverage enhancement, RedCap, and MIMO 2TA (i.e., following R18).

  - Discussion on inter-CU LTM with DC

2.2.1 Inter-CU SCG LTM

L2 handling

In the last meeting, the following inter-CU SCG LTM L2 reset operations were agreed:

- Rel-19 Set ID is configured for a candidate configuration. For DRB:

- Inter-CU MCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> UE performs PDCP re-establishment to all radio bearers.

- Inter-CU SCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> For MN terminated bearer without change of termination point, UE does not perform PDCP re-establishment. PDCP recovery is needed if it is split bearer.

> For SN terminated bearer without change of termination point, UE performs PDCP re-establishment.

> If there is change of termination point for a radio bearer (the keyToUse in the RadioBearerConfig is different from the keyToUse in the current UE configuration), UE performs PDCP re-establishment.

- For SRBs in inter-CU SCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3.

- For SRB1/2 in inter-CU SCG LTM, PDCP re-established is not performed based on NW configuration (PDCP re-establishment flag and SDU discard).

However, the L2 reset operation for the RLC entity for inter-CU SCG LTM has not been discussed yet.

For DRBs in inter-CU SCG LTM, the L2 handling for RLC is the same as in the non-DC case. Therefore, the agreement reached in RAN2#127bis regarding L2 handling for the non-DC case can be reused.

RAN2#127bis Agreement

- For non-DC case, if the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18.

RAN2#127bis Agreement

- For non-DC case, if the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18.

Proposal 7: For DRBs in DC case, if the new Rel-19 IDs of the serving cell and the target cell have different values, the UE shall perform RLC re-establishment. Otherwise, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding RLC behaviour as defined in Rel-18.

For SRB3, since the PSCell has changed after performing SCG LTM, the signaling from the source PSCell may be invalid. Therefore, RLC re-establishment should be performed in any case when SCG LTM is performed. Therefore, the Rel-18 mechanism for SRB3 RLC handling (i.e., perform RLC re-establishment based on NW indication) can be reused.

Proposal 8: For SRB3, RLC re-establishment is performed based on NW indication (i.e., following R18 LTM).

For split SRB1/2, if inter-CU or intra-CU inter-DU SCG LTM is performed, RLC re-establishment should be performed since the location of RLC entity has changed. However, for intra-DU SCG LTM, there is no need to perform RLC re-establishment. In Rel-18 LTM, the L2 handling for split SRB1/2 has not been discussed, and types of SRBs will follow the NW indication(i.e., the field reestablishRLC) to perform RLC re-establishment.

Therefore, there are two options for the L2 handling of RLC for split SRB1/2:

- Option 1: Follow the Rel-18 LTM SRBs L2 handling mechanism and always perform RLC re-establishment based on the NW indication when executing SCG LTM.

- Option 2: Follow the same L2 handling mechanism as DRBs and not perform RLC re-establishment when executing intra-DU SCG LTM.

Proposal 9: RAN2 to discuss the L2 handling of SCG RLC for split SRB1/2 based on the following two options:

- Option 1: RLC re-establishment is performed based on NW indication (i.e., following R18 LTM).

- Option 2: if the new Rel-19 IDs of the serving cell and the target cell have different values, the UE shall perform RLC re-establishment. Otherwise, if the ltm-ServingCellNoResetID and ltm-NoResetID of the target cell have different values, the UE shall perform RLC re-establishment(i.e., following the RLC handling behavior in R18 LTM for DRBs). In other cases, the UE shall not perform RLC re-establishment.

2.2.2 Inter-CU MCG LTM with SN released or unchanged

In RAN2#127 meeting, the following agreements were made for the case of inter-CU MCG LTM with SN released or unchanged:

RAN2#127 Agreements

- SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration).

- Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied).

- For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

Security key handling

During inter-CU MCG LTM with DC, the SN key is updated concurrently with any MN key change. As per TS 33.501[3], the MN shall set the SN Counter to ‘0’ when a new AS root key, KNG-RAN, in the associated 5G AS security context is established. Therefore, when the KNG-RAN is updated due to inter-CU MCG LTM, the sk-Counter utilized by UE for generating the SN key should be set to zero.

TS 33.501

6.10.3.1	SN Counter maintenance

The MN shall set the SN Counter to ‘0’ when a new AS root key, KNG-RAN, in the associated 5G AS security context is established. The MN shall set the SN Counter to ‘1’ after the first calculated KSN, and monotonically increment it for each additional calculated KSN. The SN Counter value '0' is used to calculate the first KSN.

In our view, there are two options for ensuring that sk-Counter = 0 is used when inter-CU MCG LTM with SN unchanged or SN addition is performed:

- Option 1: Default sk-Counter = 0 is used by UE for generating the SN key when security key update is performed in MCG.

- Option 2: Network configures sk-Counter = 0 in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG.

Both the options work well and RAN2 can select one for handling the security key in this scenario.

Proposal 10: RAN2 to discuss and choose one of the two options for the security key handling of inter-CU MCG LTM with DC:

- Option 1: Default sk-Counter = 0 is used by UE for generating the SN key when security key update is performed in MCG.

- Option 2: Network configures sk-Counter = 0 in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG.

In addition, similar to Rel-17 CHO with SCG, inter-CU MCG LTM with SN unchanged also requires the signaling enhancement between the candidate MN and SN to make the SN recognize that the Addition/Modification is related to an LTM. For instance, an indicator should be added in the SN Addition Request and SN Modification Request message. Furthermore, the source MN ID and UE ID should also be included in the SN Addition Request to help the SN to optimize resource allocation.

Proposal 11: Network signaling enhancements between MN and SN are needed for inter-CU MCG LTM with SN unchanged (e.g., the candidate MN informs SN the Addition/Modification is related to an LTM, includes source MN ID and UE ID in the SN Addition Request to help the SN to optimize resource allocation). Send an LS to inform RAN3.

- Conclusion

In this paper, we discuss the inter-CU LTM remaining open issues and also the specification impact of inter-CU LTM in DC scenarios. We have the following observations and proposals:

Remaining issues for Inter-CU LTM in SA

Observation 1: If the serving cell and candidate cells are from different CUs and are synchronized, UE-based TA measurement can be supported by reusing the Rel-18 mechanism.

Proposal 1: RAN2 confirms that UE-based TA measurement is supported in the synchronized inter-CU LTM scenario, and no specification impact is expected.

Proposal 2: RAN2 confirms that supporting UE-based TA measurement in the unsynchronized case is beneficial for inter-CU LTM from the RAN2 perspective.

Proposal 3: Send an LS to RAN1/4 to ask whether it is feasible to support UE-based TA measurement in unsynchronized cases.

Proposal 4: CFRA resources can be included in the LTM Cell Switch Command MAC CE for inter-CU LTM. Send an LS to inform RAN3.

Proposal 5: Send an LS to ask RAN3 to consider the LTM configuration mismatch issue between UE and the network caused by CHO execution.

Proposal 6: RAN2 confirms that inter-CU LTM can coexist with coverage enhancement, RedCap, and MIMO 2TA (i.e., following R18).

Inter-CU SCG LTM

Proposal 7: For DRBs in DC case, if the new Rel-19 IDs of the serving cell and the target cell have different values, the UE shall perform RLC re-establishment. Otherwise, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding RLC behaviour as defined in Rel-18.

Proposal 8: For SRB3, RLC re-establishment is performed based on NW indication (i.e., following R18 LTM).

Proposal 9: RAN2 to discuss the L2 handling of SCG RLC for split SRB1/2 based on the following two options:

- Option 1: RLC re-establishment is performed based on NW indication (i.e., following R18 LTM).

- Option 2: if the new Rel-19 IDs of the serving cell and the target cell have different values, the UE shall perform RLC re-establishment. Otherwise, if the ltm-ServingCellNoResetID and ltm-NoResetID of the target cell have different values, the UE shall perform RLC re-establishment(i.e., following the RLC handling behavior in R18 LTM for DRBs). In other cases, the UE shall not perform RLC re-establishment.

Inter-CU MCG LTM with SN released or unchanged

Proposal 10: RAN2 to discuss and choose one of the two options for the security key handling of inter-CU MCG LTM with DC:

- Option 1: Default sk-Counter = 0 is used by UE for generating the SN key when security key update is performed in MCG.

- Option 2: Network configures sk-Counter = 0 in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG.

Proposal 11: Network signaling enhancements between MN and SN are needed for inter-CU MCG LTM with SN unchanged (e.g., the candidate MN informs SN the Addition/Modification is related to an LTM, includes source MN ID and UE ID in the SN Addition Request to help the SN to optimize resource allocation). Send an LS to inform RAN3.

- References

- RAN2#128_ChairNotes.

- R4-2314455, Reply LS beam application time and UE based TA measurement for LTM. Ericsson.

- TS 33.501 v19.0.0, Security architecture and procedures for 5G system (Release 19).