3GPP TSG-RAN WG2 Meeting #129		R2-2500347

Athens, Greece, 17th – 21st Feb. 2025

Source:	vivo

Title:		Discussion on LTM measurement event evaluation and reporting

Agenda Item:		8.6.3

Document for:	Discussion and Decision

- Introduction

In RAN2 #128 meeting, some agreements related to MAC CE in L1 event-triggered reporting have been reached, as follows[1]:

- SSBRI and CRI is used to represent the candidate beam ID in LTM MR MAC CE.

- For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE.

- For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE.

- A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals.

- Support the truncated measurement report MAC CE.

In this contribution, we would like to further discuss the content and format of MAC CE in L1 event-triggered reporting.

- Discussion

  - L1 Event Triggered Measurement Report MAC CE format

Till now, the information we agreed to be included in L1 Event Triggered Measurement Report MAC CE is as follows in RAN2 #127bis[2]:

Agreements on L1 event triggered MR

- Basic information included in MR MAC CE:

- Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id)

- Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams

- Triggered event information (e.g., ReportConfigID)

MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not).

N is configurable by NW.

- Additional information included in MR MAC CE:

- The information and quantity of current beam, based on NW configuration.

Besides all the information we agreed so far, we understand another information should also be considered, which is the number of beams satisfying the measurement event condition in the measurement report. The reason is that when beams satisfying and unsatisfying the measurement event condition are both included in a single MAC CE, the network cannot know which beam result(s) does not satisfy the measurement event condition, because the network can only know the RSRP value without knowing whether the value fulfils the TTT requirement.

Observation 1: The network cannot know which beam result(s) in L1 reporting MAC CE refers to the beam not satisfying the event condition, because there is no information about whether the corresponding RSRP value fulfils the TTT.

In our understanding, the simplest way is to add another information e.g. the satisfied number, which represents the number of beams which fulfil the triggering event in the L1 event triggered measurement report MAC CE, e.g. if the number of beams which fulfil the triggering event is M, then the ‘satisfied number’ should be M in the MAC CE, and the network can know the first M beams within the MAC CE fulfil the triggering event while the remaining N-M beams (if the UE reports N in total) do not fulfil the triggering event.

Proposal 1: The number of beams which fulfil the triggering event is included in the L1 event triggered measurement report MAC CE.

Then as illustrated in the section of ‘6.1.3.x L1 Event Triggered Measurement Report MAC CE’ in the running MAC CR[3], we can include the information of number of satisfied beams in the MAC CE, followed by the unsatisfied beam, and the example of the format of this MAC CE is as follows. Here we assume that maximum 4 candidate cells with maximum 4 beams each can be reported as R18, so the satisfied number is 16, the final decision of number of reporting beams could be determined by RAN1 as suggested by [POST128][108][MOB] RRC running CR[4]:

- Proposal4: Ask RAN1 what should be the maximum number of beam measurement results that can be reported in event-triggered measurement report.

Figure 1. Example of L1 event triggered measurement report and truncated L1 event triggered measurement report MAC CE [for absolute RSRP]

Meanwhile, in RAN2 #128 meeting, it was also discussed that differential RSRP values may be utilized in designing MAC CE format[1].

R2-2410441 On L1 Measurement Reporting Enhancements for Rel-19 LTM Nokia discussion Rel-19 NR_Mob_Ph4-Core Proposal 18: RAN2 to study the RSRP value reporting using differential RSRP mechanism in MAC CE. Discussion: [Samsung, Xiaomi, Lenovo]: Since we have MR MAC CE, it would be easier and more straightforward to include all explicit values (rather than using differential values). [Apple]: It is already supported in Rel-18. [IDC]: Agree with the proposal, it would be good to optimize the MR MAC CE size. [CATT]: Agree with proposal. [Huawei]: In general, ok with the proposal. Wonder if there needs any RAN4 impact for mapping. [Lenovo]: It is an optimization and not sure how many bits can be saved compared to the case using all explicit values. - Revisit it once we can have better understanding on MR MAC CE format and how much bits actually can be saved from differential RSRP values.

According to TS 38.212[5], using differential RSRP would take 4 bits, rather than 7 bits for normal RSRP.

Table 6.3.1.1.2-6: CRI, SSBRI, RSRP, and CapabilityIndex

| Field | Bitwidth |
| --- | --- |
| CRI |  |
| SSBRI |  |
| RSRP | 7 |
| Differential RSRP | 4 |
| CapabilityIndex | 2 |

With this assumption in MAC CE format design, the potential format of L1 Event Triggered Measurement Report MAC CE as in [3] is as follows:

Figure2. Example of L1 event triggered measurement report and truncated L1 event triggered measurement report MAC CE [for differential RSRP]

By making the RSRP 4 bits, additional 3 bits can be saved for other information e.g. per-beam indication of whether the beam is the triggered one or fulfilling the event, etc. However, as RSRI is 9 bits and anyway two octets are needed for RSRI and RSRP, it seems that there is no real difference on whether using differential RSRP or not in the MAC CE design. Therefore, we propose to adopt the MAC CE format in Figure.1 as the baseline.

Moreover, in the MAC CE format, we assume that the max N is total number of all CANDIDATE beams included into MR MAC CE without counting the serving beam. This is not explicitly mentioned in the previous agreement so we think it is also better to clarify it. Therefore, we propose:

Proposal 2: RAN2 to clarify that the max N (total number of all beams included into MR MAC CE) is the number of all candidate beams without counting current serving beam.

Proposal 3: RAN2 to agree the MAC CE format in Figure.1 as the baseline.

  - Truncated L1 Event Triggered Measurement Report MAC CE

In RAN2 #128 meeting, it was agreed:

- Support the truncated measurement report MAC CE.

In this section, we would like to discuss the issues related to supporting truncated measurement report MAC CE.

Generally, we think it should be clarified about the scenarios that the truncated MAC CE should be used. Considering the following cases, where N is the total number of all candidate beams included into MR MAC CE, and K is the number of satisfied beams:

| Different cases | Beams included in the MAC CE | Truncated MAC CE or not |
| --- | --- | --- |
| Case 1: N>K, UL grant can accommodate N beam results | K satisfied beams, Up to N-K unsatisfied beams | No |
| Case 2: N>K, UL grant can accommodate X beam results (X<K) | X satisfied beams | Yes |
| Case 3: N>K, UL grant can accommodate K beam results but not N beam results | K satisfied beams, Less than N-K unsatisfied beams | ? (prefer No) |
| Case 4: N<K, UL grant can accommodate N beam results | N satisfied beams | ? (prefer Yes) |

In our understanding, case 1 and case 2 are less controversial where in case 2 truncated MAC CE should be used and in case 1 it should not. But for case 3 and 4, more discussion is needed.

In case 3, although the UL grant cannot accommodate N beam results, K satisfied beams are all included in the MAC CE to be reported. We think truncated MAC CE may not be used in this case as anyway the unsatisfied beam results should be in a best-effort way to be reported.

In case 4, although UL grant can accommodate N beam results, not all the satisfied beam results can be reported. If truncated MAC CE is used, then the network can know that there are satisfied beams not been reported so new UL grant may be provided to the UE, thus we prefer to also use truncated MAC CE in this case.

In summary, we think a general principle can be that truncated MAC CE is used when the reported number of beams is less than the satisfied number of beams, e.g. when the UL grant cannot accommodate an event-triggered L1 measurement report MAC CE containing all satisfied measurement result, or when the UE can only include N satisfied beams but the actual number of satisfied beams is larger than N.

Observation 2: The number of reported candidate beams may be less than the number of candidate beams fulfilling triggering event in the following 2 cases:

- when the UL grant cannot accommodate an event-triggered L1 measurement report MAC CE containing all satisfied measurement result;

- when the UE can only include N satisfied beams but the actual number of satisfied beams is larger than N.

Proposal 4: If the number of reported candidate beams is less than the number of candidate beams fulfilling triggering event, truncated L1 Event Triggered Measurement Report MAC CE is used.

Another issue we would like to discuss here is that how the UE should select beams to be included in the MAC CE. For satisfied beams, the UE is supposed to report all of them unless the above 2 cases. For unsatisfied beams, how to select beams can be tricky. Selecting unsatisfied beams by RSRP value may not be optimal because for L1 measurement the fluctuation can be severe, so high RSRP value may mean nothing. In our understanding, the better method for UE to select the unsatisfied beams should be selecting the ones that are better than the threshold associated to the entry condition for some time but does not reach the TTT length. The longer its RSRP is above the threshold, the better the beam quality is. On the other hand, if all the RSRP of unsatisfied beams is below the threshold, then it may be left to UE implementation. Therefore, we propose:

Proposal 5: When UE reports unsatisfied beams in MR MAC CE, they are selected based on the time length that the beam fulfils the event threshold.

  - Padding L1 measurement report MAC CE

Considering truncated measurement report MAC CE is already agreed to be supported, we could take one step further to consider whether padding measurement report MAC CE could also be supported.

In legacy BSR and SL-BSR discussion, the introduction of Padding BSR is driven by the following key considerations:

1/ Resource Utilization Optimization

When the uplink resources (UL Grant) allocated by the gNB are not fully utilized by the UE’s pending data, the remaining space would otherwise be filled with padding bits. Padding BSR enables the UE to utilize these padding bits to report buffer status, thereby minimizing resource waste and improving uplink efficiency.

2/ Reduced Signalling Overhead and Latency

Unlike Regular BSR, which requires explicit scheduling requests (SR) or dedicated grants, Padding BSR can be "piggybacked" on existing resource allocations. This eliminates additional signalling interactions, reducing latency and overhead, especially for scenarios with frequent buffer updates.

3/ Dynamic Buffer State Updates

While Regular and Periodic BSRs rely on specific triggers or timers, Padding BSR leverages padding space to dynamically report buffer status during any resource allocation. This allows the gNB to capture real-time data volume changes and optimize subsequent scheduling decisions.

4/ Support for Multi-Priority Logical Channels

When padding space is limited, Padding BSR employs Truncated BSR to prioritize reporting for the highest-priority LCG.

When it comes to L1 measurement reporting, similar reason could be considered, e.g., the L1 measurement results can update frequently, or the UL grant may not be able to accommodate all beam measurement results. Therefore, padding bits can be used to report beams such as:

- The updated beam measurement results that are already reported and do not satisfy the leaving condition

- The unsatisfied beam measurement results that has not been reported yet because of no triggering event is satisfied.

Therefore, we suggest that RAN2 to discuss whether padding L1 measurement report MAC CE should be supported. We prefer to support it considering the benefits it brings and the spec impact seems not much.

Proposal 6: Introduce padding L1 measurement report MAC CE.

- Conclusion

In this contribution, we discussed the content and format of MAC CE in L1 event-triggered reporting, as well as Truncated/padding L1 measurement report MAC CE details. We have the following Observations and Proposals:

Observation 1: The network cannot know which beam result(s) in L1 reporting MAC CE refers to the beam not satisfying the event condition, because there is no information about whether the corresponding RSRP value fulfils the TTT.

Observation 2: The number of reported candidate beams may be less than the number of candidate beams fulfilling triggering event in the following 2 cases:

- when the UL grant cannot accommodate an event-triggered L1 measurement report MAC CE containing all satisfied measurement result;

- when the UE can only include N satisfied beams but the actual number of satisfied beams is larger than N.

Proposal 1: The number of beams which fulfil the triggering event is included in the L1 event triggered measurement report MAC CE.

Proposal 2: RAN2 to clarify that the max N (total number of all beams included into MR MAC CE) is the number of all candidate beams without counting current serving beam.

Proposal 3: RAN2 to agree the MAC CE format in Figure.1 as the baseline.

Proposal 4: If the number of reported candidate beams is less than the number of candidate beams fulfilling triggering event, truncated L1 Event Triggered Measurement Report MAC CE is used.

Proposal 5: When UE reports unsatisfied beams in MR MAC CE, they are selected based on the time length that the beam fulfils the event threshold.

Proposal 6: Introduce padding L1 measurement report MAC CE.

- References

- RAN2 #128 meeting chair notes;

- RAN2 #127bis meeting chair notes;

- R2-2500xxx, Running MAC CR for Mob Ph4 (vivo);

- R2-2500xxx, Report of [POST128][108][MOB] RRC running CR (Huawei);

- 3GPP TS 38.212 V18.5.0 (2024-12);