3GPP TSG-RAN WG2 Meeting #129	R2-2500349

Athens, Greece, 17th – 21st Feb. 2025

Source:	vivo

Title:	Discussion on RRM measurement gaps enhancements

Agenda Item:	8.7.3

Document for:	Discussion and Decision

- Introduction

In RAN2#128 [1], the following agreements were made regarding the MG skipping for XR transmission enhancements:

Agreements on RRM measurement gaps impacts - No MG-specific enhancements is needed on DSR operation. - RAN2 assumes that UE follows DRX pattern as currently, even when MG is indicated as skipped - No MG-specific enhancements is needed for DRX operation.

In this contribution, we first discuss the RAN2 impacts of measurement gap skipping for the DCI-based solution agreed in RAN1. Then we discuss the RRC-based MG skipping solution and the UE assistant information for the MG skipping enhancement.

- Discussion

RAN2 impacts of measurement gap skipping

Impacts on MAC specification

In TS 38.321, certain data transmission and reception cannot be performed during the activated measurement gaps.

TS 38.321

5.14	Handling of measurement gaps

During an activated measurement gap, the MAC entity shall, on the Serving Cell(s) in the corresponding frequency range of the measurement gap configured by measGapConfig as specified in TS 38.331 [5]:

1>	not perform the transmission of HARQ feedback, SR, and CSI;

1>	not report SRS;

1>	not transmit on UL-SCH except for Msg3 or the MSGA payload as specified in clause 5.4.2.2;

1>	if the ra-ResponseWindow or the ra-ContentionResolutionTimer or the msgB-ResponseWindow is running, or if there is an ongoing RACH-less LTM cell switch, or if there is an ongoing RACH-less handover:

2>	monitor the PDCCH as specified in clauses 5.1.4, 5.1.5, and 5.7.

1>	else:

2>	not monitor the PDCCH;

2>	not receive on DL-SCH.

TS 38.321

5.14	Handling of measurement gaps

During an activated measurement gap, the MAC entity shall, on the Serving Cell(s) in the corresponding frequency range of the measurement gap configured by measGapConfig as specified in TS 38.331 [5]:

1>	not perform the transmission of HARQ feedback, SR, and CSI;

1>	not report SRS;

1>	not transmit on UL-SCH except for Msg3 or the MSGA payload as specified in clause 5.4.2.2;

1>	if the ra-ResponseWindow or the ra-ContentionResolutionTimer or the msgB-ResponseWindow is running, or if there is an ongoing RACH-less LTM cell switch, or if there is an ongoing RACH-less handover:

2>	monitor the PDCCH as specified in clauses 5.1.4, 5.1.5, and 5.7.

1>	else:

2>	not monitor the PDCCH;

2>	not receive on DL-SCH.

With the DCI-based measurement gap skipping solution, when a measurement gap occasion is indicated as skipped by the lower layer, the UE should behave as if there is no activated measurement gap during the skipped measurement gap occasion. This means that all the data transmission and reception activities that are typically restricted during an activated measurement gap, as defined in the MAC specification, can be performed during the skipped measurement gap occasion.

- The UE behaves as if there is no activated measurement gap during a skipped measurement gap occasion.

Impacts on RRC specification

In RAN1#119 meeting, the following agreements were made for the DCI-based solution:

Agreements in RAN1#119

For explicit indication by DCI to skip a particular gap/restriction, in addition to DCI formats 0_1/1_1 and 0_2/1_2, one bit indication can be included as a part of DCI formats 0_3/1_3.

- Explicit indication can be configured for each DCI format individually by higher layer.

- The skipping indication corresponds to the first gap/restriction occasion among co-scheduled cells by the DCI (Type-1C field)

Above applies only for intra-band CA cases (same SCS for cells in the set of co-scheduled cells).

Agreements in RAN1#119

For explicit indication by DCI to skip a particular gap/restriction, in addition to DCI formats 0_1/1_1 and 0_2/1_2, one bit indication can be included as a part of DCI formats 0_3/1_3.

- Explicit indication can be configured for each DCI format individually by higher layer.

- The skipping indication corresponds to the first gap/restriction occasion among co-scheduled cells by the DCI (Type-1C field)

Above applies only for intra-band CA cases (same SCS for cells in the set of co-scheduled cells).

According to the RAN1 agreement, an explicit RRC indication is needed for each DCI format to inform the UE whether the one-bit indication will be included in the corresponding DCI format.

- An explicit indication per DCI format is configured by RRC. Details are based on RAN1 parameters.

RRC-based MG skipping solution

In RAN2#127bis meeting, the RRC-based MG (Measurement Gap) skipping solution was discussed, and many companies expressed interest in supporting this RRC-based approach. From RAN2 perspective, we believe that the RRC-based solution offers advantages over the DCI-based solution in at least the following two scenarios:

- Scenario 1: When MG occasions overlap with data whose arrival can be predicted.

- Scenario 2: When CG scheduling is configured.

For scenario 1, given that XR traffic is quasi-periodic (e.g., with periodicity of 16.67ms with jitter), it is possible to predict the overlap of MG occasions with a significant portion of packets based on the traffic period, jitter range, and MG configuration. Therefore, it is more appropriate to employ an RRC-based MG skipping solution (i.e., RRC configures the MG skipping patterns) rather than a DCI-based solution for these packets. The RRC-based solution addresses situations where the overlap of MG occasions and data can be anticipated, whereas the DCI-based solution addresses situations where such overlapping cannot be predicted due to packet arrival jitter and changes in data burst size. The RRC and DCI-based solutions are complementary and could be jointly utilized for measurement gap skipping.

As for scenario 2, the CG scheduling method is a kind of main scheduling method for UL XR traffic, if CG is configured and DCI-based MG skipping solution is used, the network may need to indicate UE before the CG occasions to skip the MG every time with DCI which have no data to schedule, which is inefficient.

Considering the above analysis, we think RAN2 should support the RRC-based MG skipping solution. Naturally, detailed discussions about this approach can start once we have more information about the DCI-based solution from RAN1/4 and determine what is required for the RRC-based solution.

- RAN2 confirms that RRC-based solution indicating the occasions where measurement gaps/restrictions should be skipped is supported.

UE assistant information

Discussion on whether and what kind of UE assistance information should be introduced for measurement gap/restrictions related enhancements have taken place in RAN1. Since measurement occasions affect RAN4, it should be RAN4 to make the decision on this issue. RAN1 has sent an LS to RAN4 [2] in RAN1#117 meeting. In our opinion, RAN2 should wait for RAN4’s conclusions regarding UE assistance information related to measurement occasions. If RAN4 agrees to introduce information related to measurement occasions in UAI, then RAN2 can proceed with discussing the corresponding signaling and procedural design.

- RAN2 waits for RAN4’s conclusions on UE assistance information related to measurement occasions before further discussion.

- Conclusion

In this paper, we have discussed the RAN2 impacts of measurement gap skipping for the DCI-based solution agreed in RAN1. Then we discuss the RRC-based MG skipping solution and the UE assistant information for the MG skipping enhancement. The following proposals were made:

RAN2 impacts of measurement gap skipping

- The UE behaves as if there is no activated measurement gap during a skipped measurement gap occasion.

- An explicit indication per DCI format is configured by RRC. Details are based on RAN1 parameters.

RRC-based MG skipping solution

- RAN2 confirms that RRC-based solution indicating the occasions where measurement gaps/restrictions should be skipped is supported.

UE assistant information

- RAN2 waits for RAN4’s conclusions on UE assistance information related to measurement occasions before further discussion.

- References

- RAN2-127bis, Chair notes

- R1-2405736, LS on UE assistance information, RAN1#117