3GPP TSG-RAN2 Meeting #129	R2-2500421

Athens, Greece, 17-21 February, 2025

Agenda Item:	8.13.4

Source:	ASUSTeK

Title:	RRC states of candidate Relay UEs for path switching

Document for:	Discussion and Decision

1	Introduction

The following intra-gNB path switching scenarios are specified in the new WID on NR sidelink multi-hop relay [1]:

First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

RAN2#128 made the following agreement on control plane procedures:

In one approach (“approach 1”) of U2N relays, each of the Intermediate Relay UEs must be in RRC_CONNECTED when the U2N remote UE is in RRC_CONNECTED.  Connection establishment in the U2N remote UE first requires that each Intermediate Relay UE which is in RRC_IDLE/RRC_INACTIVE first enters RRC_CONNECTED.

This document discusses RRC states of candidate Relay UEs for path switching.

2	Discussion

2.1	Limitation in WID for scenarios C and D

We have the following statement in the WID on NR sidelink multi-hop relay [1]:

The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell.

This statement seems to limit that all the relay UEs in a candidate relay path should be in RRC_CONNECTED and connect to the same cell.  In our opinion, requiring all relay UEs in RRC_CONNECTED is quite restrictive and would result in path switching failure if there is no eligible candidate relay path with all the relay UEs in RRC_CONNECTED nearby.

Observation 1: Requiring all relay UEs in RRC_CONNECTED would result in path switching failure if there is no eligible candidate relay path with all relay UEs in RRC_CONNECTED nearby.

According to clause 16.12.6.2 in TS 38.300 [2], in the legacy single-hop U2N Relay, the gNB can select a L2 U2N Relay UE in any RRC state (i.e. RRC_IDLE, RRC_INACTIVE, or RRC_CONNECTED) as a target L2 U2N Relay UE for direct to indirect path switch. Besides, RAN2#128 also agreed that connection establishment in the U2N remote UE requires that each Intermediate Relay UE which is in RRC_IDLE/RRC_INACTIVE first enters RRC_CONNECTED, which implies the relay UEs could be initially in RRC_IDLE/RRC_INACTIVE when multi-hop U2N remote UE connection establishment is initiated.

Observation 2: A candidate U2N Relay UE in RRC_IDLE/RRC_INACTIVE is allowed in the legacy single-hop U2N Relay

Observation 3:  The relay UEs could be initially in RRC_IDLE/RRC_INACTIVE when multi-hop U2N remote UE connection establishment is initiated.

A clear drawback of the relay UEs in RRC_IDLE/RRC_INACTIVE is that more latency would be induced during path switching. However, since the gNB knows RRC states of the relays in each candidate relay path in case of intra-gNB path switching, the gNB can prioritize the candidate relay paths with relay UEs in RRC_CONNECTED over the candidate relay paths with relay UEs in RRC_IDLE/RRC_INACTIVE when selecting a target relay path.

Observation 4: Since the gNB knows RRC states of the relays in each candidate relay path in case of intra-gNB multi-hop path switching, the gNB can prioritize the candidate relay paths with relay UEs in RRC_CONNECTED (if available) when selecting a target relay path.

In view of the above, we think it is beneficial to consider the candidate relay paths with relay UEs in RRC_IDLE/RRC_INACTIVE for multi-hop path switching, which also aligns the RRC states of the candidate U2N Relay UEs in both scenarios of multi-hop U2N remote UE connection establishment and multi-hop path switching.

Proposal 1:  RAN2 to confirm that relay UEs in a candidate relay path for multi-hop path switching could be in any RRC state (i.e. RRC_IDLE, RRC_INACTIVE, or RRC_CONNECTED) as in the legacy single-hop U2N Relay.

3 	Conclusion

In this contribution, we raise the following observations and proposals:

Observation 1: Requiring all relay UEs in RRC_CONNECTED would result in path switching failure if there is no eligible candidate relay path with all relay UEs in RRC_CONNECTED nearby.

Observation 2: A candidate U2N Relay UE in RRC_IDLE/RRC_INACTIVE is allowed in the legacy single-hop U2N Relay

Observation 3: The relay UEs could be initially in RRC_IDLE/RRC_INACTIVE when multi-hop U2N remote UE connection establishment is initiated.

Observation 4: Since the gNB knows RRC states of the relays in each candidate relay path in case of intra-gNB multi-hop path switching, the gNB can prioritize the candidate relay paths with relay UEs in RRC_CONNECTED (if available) when selecting a target relay path.

Proposal 1:  RAN2 to confirm that relay UEs in a candidate relay path for multi-hop path switching could be in any RRC state (i.e. RRC_IDLE, RRC_INACTIVE, or RRC_CONNECTED) as in the legacy single-hop U2N Relay.

- References

[1] RP-241609, New WID on NR sidelink multi-hop relay

[2] TS 38.300-i40, NR; NR and NG-RAN Overall Description; Stage 2 (Release 18)