3GPP TSG-RAN WG2 Meeting #129                                     R2-2500452

Athens, Greece, Feb. 17th – 21st, 2025

Source: 			OPPO

Title: 	Discussion on paging procedure for Ambient IoT

Agenda item:		8.2.2

Document for: 	Discussion and Decision

- Introduction

In the last RAN plenary meeting, new Work Item on solutions for Ambient IOT has been approved, and the RAN2 scope is captured as follows:

- RAN2 scope:

  - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signaling procedure to enable DO-DTT and DT data transmission:

    - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs.

Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

    - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129).

    - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R.

    - Only MAC layer is included

In the last RAN2#128 meeting, regarding paging procedure for Ambient IOT, following agreements have been achieved:

- An ID in paging message is beneficial to avoid duplicated response for same service request. FFS whether ID is generated by the reader or CN. FFS on the ID size.

- Capture the scenario that different readers may page the same device for same service request.  We will discuss this in WI phase (if in scope of WI) and consider progress from all the WGs

- Whether we include further information in paging message to indicate service type/command type or to indicate that further subsequent messages are coming can be discussed in WI phase once we understand more the paging design and AS ID.

In this contribution, we aim to provide our views on the paging procedure for the A-IOT system.

- Discussion

- 2.1 Resource allocated for msg1 transmission

Overall, the paging message should indicate the time-frequency resource for A-IOT device to perform msg1 transmission, especially considering FDM and/or TDM may be applied to accommodate more than one A-IOT device for D2R transmission.

For contention-based RACH procedure, RAN2 #126 has already agreed that A-IoT paging message indicates information for the device to determine resources to be used for response (D2R message). But details of information of the message are still FFS. Also, for the cases of paging message containing a single A-IOT device ID or a group ID, A-IOT device(s) needs to know its resource(s) to be used for transmission of msg1.

In our point of view, we can easily mark each such resource with an index, if the mapping relationship between the resource and index is known. On example is provided in Figure 1. Subsequently, if a A-IOT device knows its resource index from the A-IOT paging message, it can perform msg1 transmission in the correct resource.

Figure 1: an example of mapping relationship between the resource location and resource index

Such index could come from two sources. Firstly, if the contention-based RACH procedure is applied, the UE can determine the resource index randomly in a range autonomously, similar with the RFID system wherein the RFID tag determines its access slot according to the received Q value. Secondly, for the contention-free RACH procedure, the paging message shall provide the resource indices for the target A-IOT devices. One simple way is to allocate the resource indices for A-IOT devices covered by a group ID in an ascending manner. For example, the resource index #1 is allocated to the A-IOT device with a ID: group ID+1, the resource index#2 is allocated to the A-IOT device with a ID: group ID+2, and so on so forth.

In all, there are two pieces of configuration information that are important for the A-IOT device to derive its D2R resource. The first one is the mapping relationship between resource indices and the resource locations, and the second one is the mapping relationship between the resource index and the A-IOT device ID. In our opinion, the first one configuration could be defined in the specification, e.g., the resource index increases along the frequency domain firstly to the maximum bound, and then increases along the time domain and so on so forth, instead of being indicated explicitly in the A-IOT paging message. The second configuration may need to be indicated explicitly in the A-IOT paging message. Anyway, we suggest RAN2 to discuss how to capture these two pieces of configuration information to let the A-IOT device derive the location of its D2R transmission resource after receiving the paging message.

Proposal 1: RAN2 to discuss how to let the A-IOT device know its D2R resource location, considering two sets of key information may be included in the A-IOT paging message or defined in the specification:

- the mapping relationship between the resource location and resource index

- the mapping relationship between the resource index and A-IOT device ID

- 2.2 Avoiding duplicate response from devices

In the last RAN2#128 meeting, RAN2 agreed that an ID in paging message is beneficial to avoid duplicated response for same service request, but FFS whether this ID is generated by the reader or the CN and also FFS on the ID size.

Since the service request comes from the CN, if the ID is to be generated by the reader, the reader will need to translate the ID from CN to a AS-layer ID, and then includes the AS-layer ID in the paging message. In addition, to avoid duplicated response from an A-IOT device, the readers with overlapped coverage area may coordinate with each other on the AS-layer ID before sending the A-IOT paging message, which brings additional complexity. So we suggest RAN2 to agree on applying the CN side ID for avoiding duplicated response from A-IOT device.

Since the service request comes from the CN, if the ID is to be generated by the reader, the reader will need to translate the ID from CN to a AS-layer ID, and then includes the AS-layer ID in the paging message. Then, an A-IOT device in an overlapped coverage area is highly probably to respond more than once to a service request coming from the CN, provided that the AS-layer IDs generated by readers with overlapped coverage area are different, which is indicated as follows:

Figure 1: an A-IOT device in an overlapped coverage area by Reader 1 and Reader 2 will respond to the same service request more than once, if the AS-layer ID solution without inter-reader coordination is adopted

One of the solutions to avoid occurrence of this is to require readers to coordinate with each other on the AS-layer ID before sending the A-IOT paging message. But obviously this will bring addition complexity. So, we suggest RAN2 to agree on applying the CN side ID for avoiding duplicated response from A-IOT device.

Observation 1: an A-IOT device in a overlapped coverage area is highly probably to respond more than once to a service request coming from the CN, provided that the AS-layer IDs included in the A-IOT paging messages are different from each other.

Observation 2: for the AS-layer ID solution, to avoid responding to the same service request from the CN,  the readers needs to coordinate with each other on the AS-layer ID before sending the A-IOT paging message, which brings additional complexity.

Proposal 2: applying the CN side ID for avoiding duplicated response from A-IOT device.

Since for this solution, ID comes from the CN side, and it falls in the SA2’s scope to determine the ID length. Hence, RAN2 does not need to discuss the ID size.

- 2.3 Whether or not to support the case that different readers may page the same device for same service request？

It could be easily known that the overlapped coverage area of two or more Readers is smaller than the coverage area of any Reader. So we understand that the purpose of letting an A-IOT device to respond more than one A-IOT paging message coming from different readers for the same service request is to improve the accuracy of localization of A-IOT devices. Currently, according to the TR 38.769, whether or not multiple Reader IDs can be included by the A-IOT RAN node in the Inventory Report/Command Response message needs further discussion. It seems on this point other work groups, e.g., RAN3, has not yet made conclusion. Hence, we suggest RAN2 to postpone the discussion on this topic until other working groups make progress and send LS to us.

Observation 3: For now, according to the TR 38.769, whether or not multiple Reader IDs can be included by the A-IOT RAN node in the Inventory Report/Command Response message needs further discussion by other working group(s), e.g., RAN3.

Proposal 3: postpone discussion on whether or not different Readers may page the same device for the same service request until other working group, e.g., RAN3 makes further progress on the discussion regarding whether or not multiple Reader IDs can be included in the report sent by the Reader to the CN node.

- 2.4 Whether or not to indicate service type/command type or to indicate that further subsequent messages are coming in the paging message?

In the last RAN2#128 meeting, we discussed whether or not to include the service type/command type or indication regarding if there will be subsequent command data coming in the A-IOT paging message, but no conclusion has been made. Supporting companies claimed that with the indication received in the A-IOT paging message, A-IOT devices can know if they need to wait for the subsequent R2D message after transmission of msg1.

According to RAN1 study, TD2R_min and TD2R_max define the minimum and maximum time between the D2R transmission and the corresponding R2D transmission following it, respectively, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max]. Obviously, if the indication is not included in the A-IOT paging message, the A-IOT device only needs to monitor for the potential R2D message for a limited time at most after transmission of every D2R message, e.g., msg1, which seems not a big problem.

Observation 4: without the indication regarding the service type/command type or indication regarding if there will be subsequent command data coming included in the A-IOT paging message, the A-IOT device only needs to monitor for the potential R2D transmission in a limited time duration bounded by TD2R_max.

Proposal 4: does not pursue inclusion of the service type/command type or indication regarding if there will be subsequent command data coming in the A-IOT paging message.

- 2.5 How the network configures the RA type?

- 2.5.1 Contention-based vs. contention-free

It should be noted that one-to-one mapping relationship between each A-IOT device and access occasion should be exhibited, when a group/mask ID is included in the paging message, if contention-free RACH procedure is applied. The access occasion order could be just determined by the ascending/descending order of devices IDs belonging to the group/mask ID, when the group/mask ID is applied. On the other hand, for the contention-based RACH procedure, the access occasions allocated to the targeted devices may not be sufficient, there are collisions between different devices for performing accessing towards the network. In such cases, a such parameter, functioning as the Q to randomize the slot counter for the RFID system, should be included in paging message to let the targeted A-IOT devices to randomize the access occasion indices. It can be concluded that absence or presence of such parameter implies that it is contention-free or contention-based RACH procedure being configured by the network, which is aligned with the agreement we made in RAN2#126 meeting that AIOT paging message indicates information from which the device can determine resources to be used for response (D2R message).

Proposal 5: presence (absence) of the parameter for randomizing the devices’ access indices implies the contention-based (-free) RACH procedure.

- 2.5.2 3-step vs. 2-step

As mentioned in our accompany paper [1], we suggest RAN2 to adopt the unified solution of random access for A-IOT system. In the unified solution, for the 3-step RACH procedure, only the device generated RN16 needs to be transmitted towards the network. But for the 2-step RACH procedure, additionally, the A-IOT device needs to transmit data/upper layer ID towards the network. Therefore, the TB size allocated to devices for msg1 transmission in these two scenarios are totally different, which could be leveraged by A-IOT devices to differentiate whether to apply the 2-step or 3-step RACH procedure.

Observation 5: the TB size allocated to devices for msg1 transmission in these two scenarios are totally different, which could be leveraged by A-IOT devices to differentiate whether to apply the 2-step or 3-step RACH procedure.

Proposal 6: the device could determine to apply 2-step or 3-step RACH procedure by investigating the TBS allocated to the msg1 transmission.

- Conclusion and proposals

In the contribution, following proposals and observations are made:

Resource allocated for msg1 transmission:

Proposal 1: RAN2 to discuss how to let the A-IOT device know its D2R resource location, considering two sets of key information may be included in the A-IOT paging message or defined in the specification:

- the mapping relationship between the resource location and resource index

- the mapping relationship between the resource index and A-IOT device ID

Avoiding duplicate response from devices:

Observation 1: an A-IOT device in a overlapped coverage area is highly probably to respond more than once to a service request coming from the CN, provided that the AS-layer IDs included in the A-IOT paging messages are different from each other.

Observation 2: for the AS-layer ID solution, to avoid responding to the same service request from the CN,  the readers needs to coordinate with each other on the AS-layer ID before sending the A-IOT paging message, which brings additional complexity.

Proposal 2: applying the CN side ID for avoiding duplicated response from A-IOT device.

Whether or not to support the case that different readers may page the same device for same service request？

Observation 3: For now, according to the TR 38.769, whether or not multiple Reader IDs can be included by the A-IOT RAN node in the Inventory Report/Command Response message needs further discussion by other working group(s), e.g., RAN3.

Proposal 3: postpone discussion on whether or not different Readers may page the same device for the same service request until other working group, e.g., RAN3 makes further progress on the discussion regarding whether or not multiple Reader IDs can be included in the report sent by the Reader to the CN node.

Whether or not to indicate service type/command type or to indicate that further subsequent messages are coming in the paging message?

Observation 4: without the indication regarding the service type/command type or indication regarding if there will be subsequent command data coming included in the A-IOT paging message, the A-IOT device only needs to monitor for the potential R2D transmission in a limited time duration bounded by TD2R_max.

Proposal 4: not pursue inclusion of the service type/command type or indication regarding if there will be subsequent command data coming in the A-IOT paging message.

How the network configures the RA type?--- Contention-based vs. contention-free

Proposal 5: presence (absence) of the parameter for randomizing the devices’ access indices implies the contention-based (-free) RACH procedure

How the network configures the RA type?--- 3-step vs. 2-step

Observation 5: the TB size allocated to devices for msg1 transmission in these two scenarios are totally different, which could be leveraged by A-IOT devices to differentiate whether to apply the 2-step or 3-step RACH procedure.

Proposal 6: the device could determine to apply 2-step or 3-step RACH procedure by investigating the TBS allocated to the msg1 transmission.

Reference:

[1] R2-2500455, Discussion on random access for A-IOT, OPPO