3GPP TSG-RAN WG2 #129	R2-2500504

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:		8.6.2	Inter-CU LTM

Title:		Discussion on inter-CU LTM

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

During the last meeting, RAN2 further discussed several aspects to support inter-CU LTM in non-DC and DC scenario, including configuration, behaviour and signalling procedures at different LTM stages, some agreements for inter-CU LTM have been reached [1].

Agreements on inter-CU LTM:

- Rel-19 Set ID is configured for a candidate configuration. For DRB:

- Inter-CU MCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> UE performs PDCP re-establishment to all radio bearers.

- Inter-CU SCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> For MN terminated bearer without change of termination point, UE does not perform PDCP re-establishment. PDCP recovery is needed if it is split bearer.

> For SN terminated bearer without change of termination point, UE performs PDCP re-establishment.

> If there is change of termination point for a radio bearer (the keyToUse in the RadioBearerConfig is different from the keyToUse in the current UE configuration), UE performs PDCP re-establishment.

- For SRBs in inter-CU SCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3.

- For SRB1/2 in inter-CU SCG LTM, PDCP re-established is not performed based on NW configuration (PDCP re-establishment flag and SDU discard).

- RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it.

- RAN2 confirms that the inter-CU MCG LTM with intra-SN PSCell change is supported in Rel19.

- From RAN2 perspective, the following coexistence cases in NR-DC can be supported:

- Case 1: Intra-CU MCG LTM + Inter-CU MCG LTM

- Case 2: Intra-CU SCG LTM + Inter-CU SCG LTM

7.	In coexistence cases of inter-CU MCG/SCG LTM and intra-CU MCG/SCG LTM, when inter-CU MCG or SCG LTM is executed, it’s up to the NW to ensure that maintained LTM candidate configurations are valid, e.g. reconfigure or release invalid intra-CU MCG/SCG LTM candidate configurations. UE does not autonomously release invalid intra-CU candidate configurations.

8.	RAN2 to support intra-CU SCG LTM in MN RRC message (i.e. MN RRCReconfiguration message), in addition to SN RRC message.

9.	RAN2 to support intra-CU MCG LTM with SCG configuration.

10.	It’s up to NW to ensure that the complete configuration includes the MCG part and SCG part configuration when UE combines the reference and candidate configuration for inter-CU SCG LTM.

11.	RAN2 assumes that how to indicate the list of candidate PSCells from source SN to MN is up to RAN3. From RAN2 perspective, in INM, source SN may send measurement results of candidate PSCells to the MN. The MN then forwards the measurement results to the candidate SN(s), and then the candidate SN(s) determines the LTM candidate cells based on the measurement results and the upper limit for the number of PSCells that can be prepared by each candidate SN. The existing IEs defined in INM can be reused as a baseline.

In this contribution, we further discuss the remaining issues or details for inter-CU LTM and provide our consideration.

# 2. Discussion

2.1 Inter-CU SCG LTM

In CPAC, upon execution of CPAC procedure, the UE transmits a MN RRCReconfigurationComplete including SN RRCReconfigurationComplete message and the identity of the applied conditional reconfiguration. The motivation behind is to assist the MN to identify the target SN of the CPAC procedure. During previous meeting, it was agreed that upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message. Similar as CPAC, to assisted the MN to identify the target SN, the UE should include the identity of the applied LTM candidate configuration in the MN RRCReconfigurationComplete message.

Proposal 1. For inter-CU SCG LTM, to assist the MN to identify the target SN, UE includes the identity of applied LTM candidate configuration in the MN RRCReconfigurationComplete message.

Additionally, in Rel-18 intra-CU LTM, it was agreed to include the identity of the applied LTM candidate configuration in the RRCReconfigurationComplete message, with the motivation behind is to indicate to the gNB-CU that the RRCReconfigurationComplete is for the LTM cell switch procedure, in case that LTM and CHO are configured for the same candidate cell, or multiple LTM configuration are associated with the same candidate cell. The corresponding chairman’s note can be seen in the below table. For SN/SCG, it is also possible that LTM and CPAC are configured for the same candidate PSCell, or multiple LTM configuration are associated with the same candidate PSCell. Therefore, for inter-CU SCG LTM, to assist the target SN to identify the applied LTM candidate configuration, the UE should also include the identity of the applied LTM candidate configuration in the SN RRCReconfigurationComplete message.

R2-2403177 [E068][E231][E074][E240][S792]Resolution of remaining RILs for LTM Ericsson discussion Rel-18 NR_Mob_enh2-Core Late E74: “Transaction ID” - Apple agrees with Ericsson proposal, and think this is helpful for R19 as well. - CATT think it is not a valid case that multiple configurations are applicable for one target cell. - MTK think now we have some cases where this may apply and think this is helpful for R19 - HW think the only scenario is LTM and CHO config for the same cell, think only 1 bit is needed. - vivo prefer to not do any optimization, can configure different C-RNTI for different config. - LGE think that with extended transaction ID space this is resolved. - Xiaomi think that complete message already contains the conditional configuration ID. - Ericsson think there can be multiple LTM configs per target cell, and think that CRNTI check does not work. - Intel support this proposal, it seems to be a clean solution. Lenovo as well. - Nokia wonders if this is really a real problem .. Upon an LTM cell switch, the UE includes the applied LTM candidate configuration identifier within the RRCReconfigurationComplete message.

Proposal 2. For inter-CU SCG LTM, to assist the SN to identify the applied LTM candidate configuration, UE includes the identity of applied LTM candidate configuration in the embedded SN RRCReconfigurationComplete message.

2.2 L2 handling not supported in Rel-18 due to too late

During discussion of Rel-18 mobility correction last meeting, triggering of PDCP recovery for AM DRB with RLC bearer released in case of LTM cell switch without L2 reset was discussed, and companies think that it is technical correct, but it is too late to support. The Rel-18 network implementation should ensure that the concerned case never happens.

R2-2410063 Remaining issues on L2 reset for LTM NEC discussion Rel-18 NR_Mob_enh2-Core Proposal: In case of LTM cell switch without L2 reset, UE triggers PDCP recovery for an AM DRB, if one of its associated RLC bearer is released. The draft CR in Annex is adopted. Not pursued. [Apple]: Wonders if the scenario is really valid one. [Ericsson]: Considering it is NBC change, prefer not to have it. We still can survive w/o this change. [Xiaomi]: If companies are not favourable with this change, it’s good to capture that NW ensures this situation doesn’t happen in the meeting minutes. [Samsung, Qualcomm]: Although we agree in technical point of view, also share the concern with Ericsson. It may be too late to specify it now. [Vivo]: It’s ok not to agree with this correction, but considering it is valid scenario since it was already considered in SCPAC and specified it, it would be better to capture that NW ensures this situation doesn’t happen. [Ericsson]: Prefer to do nothing. Reasonable NW implementation takes care of it. [MediaTek]: Agree with Ericsson. [Google]: Agree with Ericsson. We may consider for Rel-19.

For legacy handover procedure, RLC bearer release can happen in case of intra-CU handover case, which means that there is no such restriction compared with LTM. To the remove this restriction for LTM, the related UE behavior can be supported in Rel-19.

Proposal 3. Support in Rel-19 that the UE triggers PDCP recovery for an AM DRB, if one of its associated RLC bearer is released in case of LTM cell switch without L2 reset.

# 3. Conclusion

In this contribution we discussed some issues related to inter-CU LTM and made the following proposals.

Proposal 1. For inter-CU SCG LTM, to assist the MN to identify the target SN, UE includes the identity of applied LTM candidate configuration in the MN RRCReconfigurationComplete message.

Proposal 2. For inter-CU SCG LTM, to assist the SN to identify the applied LTM candidate configuration, UE includes the identity of applied LTM candidate configuration in the embedded SN RRCReconfigurationComplete message.

Proposal 3. Support in Rel-19 that the UE triggers PDCP recovery for an AM DRB, if one of its associated RLC bearer is released in case of LTM cell switch without L2 reset.

# 4. References

[1] Draft_RAN2_128_Meeting_Report_v1