3GPP TSG-RAN WG2 Meeting #129	R2-2500506

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.1.3

Source:	Samsung

Title:	Discussion on NW side data collection

Document for:	Discussion & Decision

Introduction

This document discusses on NW side data collection for AI/ML air interface WI.

Discussion

Data content

In the last meeting, RAN1 has discussed the data content for NW-sided data collection, and the moderator made the following proposal for BM-Case1 (although not agreed) [1]:

##### Proposal C Send LS to RAN 2, to inform RAN 2 that from RAN 1 perspective, for content for data collection for training for NW-sided model via higher layer signaling, for BM-Case1, the following three types of contents for each instance can be considered: - Type 1: All L1-RSRPs of measured based the resources (for Set A/B) configured to UE - Type 2: All L1-RSRPs measured based on resources (for Set B) configured to UE, and beam information of K beams based on some resources (for Set A) configured to UE - Type 3: All L1-RSRPs measured based on resources (for Set B) configured to UE, and beam information and L1-RSRPs of K beams based on some resources (for Set A) configured to UE - Wherein the K beams are the beams with the largest K L1-RSRPs based on the resources (for Set A) in one instance - K is configured by NW.

With Type 1, full L1-RSRPs for Set A and B are collected. Type 1 can be used to train a regression model (i.e., using L1-RSRPs of Set A as label). Also, a classification model (i.e., using top-1/K beams of Set A as label) can be trained with Type 1, but it may have redundant information and quantized L1-RSRP may fail to identity top-1/K beams for label. Type 2 can be used for classification model by addressing the aforementioned issues in Type 1 (i.e., by removing L1-RSRP for Set A and instead including top-K beam information). However, Type 2 is not relevant for regression model training. Type 3 includes L1-RSRP of top-K beams on top of Type 2, but it also may not provide enough labelled data for typical regression model training. RAN1 discussed on these three types and understood the motivation of each type, but did not reach consensus on whether to down-select types.

The reason why RAN1 tried to send an LS to RAN2 is it is RRC signalling. So we think it could be discussed/agreed in RAN2. We suggest RAN2 to support all three options to assist RAN1’s further progress.  This is because each type has pros and cons regarding overhead aspect and applicable model category (i.e., whether classification model and/or regression model). Besides, in the perspective of RAN2, supporting all three types would not bring out much standard impact (e.g., just defining one more parameter) compared to down-selection.

Observation 1. RAN1 almost concluded that three types of data content for NW-side data collection. Each type has pros and cons regarding overhead aspect and applicable model category (i.e., whether classification model and/or regression model is used).

Proposal 1. RAN2 supports the following 3 types of data contents for BM-Case 1 for NW-side data collection:

- Type 1: All L1-RSRPs of measured based the resources (for Set A/B) configured to UE

- Type 2: All L1-RSRPs measured based on resources (for Set B) configured to UE, and beam information of K beams based on some resources (for Set A) configured to UE

- Type 3: All L1-RSRPs measured based on resources (for Set B) configured to UE, and beam information and L1-RSRPs of K beams based on some resources (for Set A) configured to UE

- Wherein the K beams are the beams with the largest K L1-RSRPs based on the resources (for Set A) in one instance

- K is configured by NW.

Memory capability

The following agreement was made in RAN2#127, in order for UE to log training data for NW side data collection:

<Agreements (RAN2#127)> - UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size. This is across all use cases.

In legacy, there are existing memory requirements for UE to log measurement results. Firstly, for logged MDT, the fixed size (i.e., 64KB) of minimum memory requirement is defined. This is commonly mandatory for UEs who support logged MDT, and no separate capability is used for this memory requirement.

<TS 38.306> loggedMeasurements-r16 Indicates whether the UE supports logged measurements in RRC_IDLE and RRC_INACTIVE state. A UE that supports logged measurements shall support both periodical logging and event-triggered logging. The minimum memory size of MDT logged measurements is 64KB. For eRedCap UE supporting this feature, the minimum memory size of MDT logged measurements is 16KB.; UE; No; No; No.

Meanwhile, there are two separate memories defined for QoE measurement.

- Memory 1. This is the memory for UEs to log QoE reports in RRC_CONNECTED while QoE reporting is temporarily paused by NW. The mandatory minimum requirement for this memory is 64KB as highlighted below.

- Memory 2. This is the memory for UE to log QoE reports in RRC_IDLE and RRC_INACTIVE for MBS broadcast service. The mandatory minimum requirement for this memory is 64KB as highlighted below. Besides, UE optionally may support more than 64KB for this memory. In this case, UE indicates the size (i.e., larger than 64KB) it supports to NW via an explicit capability indication.

<TS 38.306> # 6 Conditionally mandatory features without UE radio access capability parameters Features; Condition. Acquisition of positioning SI messages with 80 milliseconds offset position compared to SI messages in schedulingInfoList; It is mandatory to support acquisition of positioning SI messages with 80 milliseconds offset position compared to SI messages in schedulingInfoList for UEs which support the acquisition of the posSIB types in posSchedulingInfoList as specified in TS 38.331 [9].. Acquisition of SI messages with explicit SI window positions; It is mandatory to support acquisition of SI messages with explicit SI window positions for UEs which support the SIB types in schedulingInfoList2 as specified in TS 38.331 [9].. AS layer memory size for QoE paused measurement reports; It is mandatory to support the minimum AS layer memory size of 64KB for QoE paused measurement reports for UEs which support qoe-Streaming-MeasReport-r17, qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17.. AS layer memory size for QoE measurement reports in RRC_IDLE and RRC_INACTIVE; It is mandatory to support the minimum AS layer memory size of 64KB for QoE measurement reports stored in RRC_IDLE/RRC_INACTIVE for UEs which support qoe-IdleInactiveMeasReport-r18 and any of qoe-Streaming-MeasReport-r17 or qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17. This memory size is additional to "AS layer memory size for QoE paused measurement reports". <SKIP> ## 4.2 UE Capability Parameters <SKIP> qoe-AdditionalMemoryMeasReport-r18 Indicates the minimum AS layer memory size the UE supports for QoE measurement in RRC_IDLE and RRC_INACTIVE in addition to the "AS layer memory size for QoE paused measurement reports". Value kB128 means the UE supports at least 128 kilobytes for this purpose, and so on. A UE supporting this feature shall also support qoe-IdleInactiveMeasReport-r18.; UE; No; No; No. <TS 38.331> #### – AppLayerMeasParameters The IE AppLayerMeasParameters is used to convey the capabilities supported by the UE for application layer measurements. AppLayerMeasParameters information element -- ASN1START -- TAG-APPLAYERMEASPARAMETERS-START AppLayerMeasParameters-r17 ::= SEQUENCE { qoe-Streaming-MeasReport-r17 ENUMERATED {supported} OPTIONAL, qoe-MTSI-MeasReport-r17 ENUMERATED {supported} OPTIONAL, qoe-VR-MeasReport-r17 ENUMERATED {supported} OPTIONAL, ran-VisibleQoE-Streaming-MeasReport-r17 ENUMERATED {supported} OPTIONAL, ran-VisibleQoE-VR-MeasReport-r17 ENUMERATED {supported} OPTIONAL, ul-MeasurementReportAppLayer-Seg-r17 ENUMERATED {supported} OPTIONAL, ..., [[ qoe-IdleInactiveMeasReport-r18 ENUMERATED {supported} OPTIONAL, qoe-NRDC-MeasReport-r18 ENUMERATED {supported} OPTIONAL, qoe-AdditionalMemoryMeasReport-r18 ENUMERATED {kB128, kB256, kB512, kB1024} OPTIONAL, qoe-PriorityBasedDiscarding-r18 ENUMERATED {supported} OPTIONAL, srb5-r18 ENUMERATED {supported} OPTIONAL ]] } -- TAG-APPLAYERMEASPARAMETERS-STOP -- ASN1STOP

If UE does not indicate this optional capability in UECapabilityInformation message, it means the UE only supports the mandatory 64 KB memory for QoE measurement in RRC_IDLE and RRC_INACTIVE. The motivation for this optional capability is UEs may have different memory capabilities and NW may want to select the UE who has big memory capability for more and consistent data collection.

We assume the same benefit can be expected, if we have similar capability for NW side data collection. i.e., UE mandatorily supports the fixed size of minimum AS layer memory without UE capability signalling, and optionally reports via UE capability a larger minimum AS layer memory size. i.e., Taking different sizes of buffer capability from UEs into account, NW can select/configure UEs to perform NW-side data collection. The exact numbers (e.g., 64KB) could be discussed after RAN1/2’s progress on data content for NW-side data collection (as discussed in Proposal 1).

Proposal 2. UE mandatorily supports the fixed size of minimum AS layer memory without UE capability signalling, and optionally reports via UE capability a larger minimum AS layer memory size. FFS for the exact numbers.

Availability for logging

From last meetings, RAN2 agreed periodic logging and event-triggered logging for NW-side data collection:

<Agreements (RAN2#127bis)> - Periodic logging is supported for training data collection procedure in R19 - Event-triggered data logging will be supported. At least radio condition based event triggered logging will be supported. FFS the details of radio condition based event. FFS if other events are supported.

Meanwhile, regarding data reporting, RAN2 agreed to support only on-demand request based reporting:

<Agreements (RAN2#127bis)> - Periodic reporting of logged data is not supported. - On-demand reporting of the logged measurements will be specified - UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection. FFS of details of the message

In addition, the availability by UE was agreed as trigger for UEInformationRequest from NW.

<Agreements (RAN2#127bis)> - The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest. FFS trigger/definition of availability indication. and FFS how data availability indication is sent to the network.

Taking all aforementioned agreements into account, we assume the following procedure for NW side data collection:

- 1) NW provides measurement configuration for NW side data collection to UE (e.g., via RRCReconfiguration message).

- 2) UE logs the measurement data based on periodic logging and/or event-triggered logging.

- 3) UE indicates the availability for logged data to NW.

- 4) NW sends UEInformationRequest message to UE to request transmission of logged data.

- 5) UE reports the logged data via UEInformationResponse message.

Based on this procedure, we think there is no need for UE’s autonomous reporting (e.g., AS buffer event based reporting), which was left as FFS in the RAN2#127:

<Agreement (RAN2#127)> - FFS whether AS buffer event based reporting is supported. FFS if we send availability indication or full report if it is supported

Given multiple UEs may be configured in a cell to collect sufficient data for completion of NW side model training, UE’s autonomous reporting may cause severe network congestion. It is safer for UE to send reports based on NW’s on-demand request.

Proposal 3. UE’s autonomous reporting (including AS buffer event based reporting) is NOT supported.

For the availability indication UE sends in Step 3, we assume the availability indication can be transferred via UAI. We do not see the need to design a new separate message for this indication. Another issue for availability indication is when UE triggers transmission of the availability indication. Assume NW configures CSI-RS resource to measure for NW-side data collection. Then, (assuming periodic logging) UE immediately starts to measure and log the measurement results. If UE transmits the availability indication as soon as any measurement result is obtained, it may not be the useful information to NW, because UE would report the availability indication immediately after NW configuration. Therefore, we think there should be a threshold for logged data size to trigger the transmission of the availability indication, and it should be configurable by NW depending on its needs. For example, NW can decide a threshold considering UE’s memory capability (as discussed in Section 2.2). Also, NW can set a certain threshold to retrieve data, before UE stops measurement and data collection (i.e., UE’s buffer becomes full).

Proposal 4. UE sends the availability indication via UAI, when the size of logged data exceeds a threshold which is configured by NW.

Meanwhile, RAN2 agreed not to support RRC segmentation for NW side data collection.

<Agreement (RAN2#127)> - The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is needed (as for the logged MDT measurements)

So, when UE stores a big size of logged data, all the logged data may not be accommodated in a single UEInformationResponse message. In this case, it would be beneficial if UE can indicate the availability indication for the residual logged data when sending the first UEInformationResponse message. Based on this information, NW can request additional UE’s reports by sending another UEInformationRequest message. This is similar to logMeasAvailable indication in UEInformationResponse message used for logged MDT.

Proposal 5. UE can send the availability indication in UEInformationResponse message to indicate the further residual logged data.

Full buffer

In last meetings, RAN2 agreed UE autonomously stops measurement for NW-side data collection in case of full buffer and indicates it to NW. It is still FFS when exactly UE indicates it (i.e., “upon” full buffer and/or “before” full buffer).

<Agreement (RAN2#127)> - When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging. <Agreement (RAN2#128)> - The UE reports to the network when buffer is or may become full. FFS when it reports (before and/or after).

The reason why this full buffer indication is needed is to enable NW to stop sending measurement resources (e.g., CSI resources) while UE stops measurement due to full buffer. Therefore, it is obvious this indication should be sent “upon” full buffer (i.e., when UE stops measurement). It could be done via UAI. Someone may think it is also beneficial if UE indicates to NW “before” the buffer is full. However, we think it is more like an availability indication. For example, as we discussed/proposed in Proposal 4, NW can configure a data size threshold to trigger availability indication to retrieve data before UE’s buffer becomes full.

Proposal 6. Before full buffer, no separate indication is needed other than availability indication. Upon full buffer, UE indicates it (e.g., via UAI) (and stops measurement as agreed in RAN2#128).

Besides, RAN2 also need to discuss on when UE and NW resumes suspended measurement due to full buffer. Simply, UE can autonomously resume measurement when any memory becomes free (i.e., when any data report is sent to NW after full buffer). NW can also know UE’s resumption based on UE’s report after the full buffer indication, so it can resume sending measurement resources.

Proposal 7. After full buffer, UE autonomously resumes when any memory becomes free (i.e., when any data report is sent to NW).

To summarize what we have discussed/proposed in this section, an example of procedure regarding full buffer is like:

- 1) NW provides measurement configuration for NW side data collection to UE (e.g., via RRCReconfiguration message).

- 2) UE logs the measurement data based on periodic logging and/or event-triggered logging.

- 3) UE sends the availability indication for logged data to NW (e.g., via UAI)

- 4) When the buffer becomes full, UE indicates it to NW (e.g., via UAI) and stops measurement.

- 5) NW may stop sending measurement resources based on UE’s indication (but no standard impact is expected).

- 6) NW sends UEInformationRequest message to UE to request transmission of logged data.

- 7) UE reports some logged data via UEInformationResponse message.

  - Now UE has some free space in the memory, UE resumes the measurement for NW-side data collection.

  - Meanwhile, NW may resume sending measurement resources (if stopped in Step 5) (but no standard impact is expected) based on UE’s report.

Event-based logging

In the post email discussion i.e., [POST128][019][AI PHY] NW side data collection (Nokia), RAN2 discussed on event based logging. There may be some benefit for reporting overhead reduction and UE buffer/power consumption saving with event-based logging. However, RAN1/RAN2 have not fully studied on data processing/categorization at UE side (i.e., event-based logging) i.e., it may be beneficial or detrimental for model training. To ensure maximum performance, it is definitely better to collect all data by NW and then (if needed) do pre-processing before training with full flexibility, rather than defining UE-side events without sufficient study/verifying the performance. Therefore, any event that may impact on the performance shall be well studied and justified. Besides, we would like to note RAN1 has ongoing proposals and discussion on similar topics (e.g., data omission for NW-side data collection). So, we suggest to leave RAN1 to discuss on the related topics including event-based logging.

Proposal 8. RAN2 leaves RAN1 to discuss on event-based logging.

Others

We assume a training entity (i.e., gNB or OAM) may want to keep or release the configuration for NW-side data collection during handover. For example, if NW-side additional conditions remain same during handover, a training entity (i.e., OAM) may want to keep the OAM centric configuration after handover to collect training data continuously. On the other hand, for example, if gNB changes and NW-side additional conditions change during handover, target gNB may not want to continue gNB centric NW-side data collection configured by source gNB. Therefore, similar to most RRC configurations, we propose to leave as NW implementation whether to keep or release the configuration for NW-side data collection.

Proposal 9. During handover, it is up to NW implementation whether to keep or release the configuration for NW-side data collection.

Meanwhile, when UE transits to RRC_IDLE or RRC_INACTIVE, it is reasonable for UE to release the configuration for NW-side data collection, as it is not used any longer in RRC_IDLE and RRC_INACTIVE.

Proposal 10. Upon going to RRC_IDLE or RRC_INACTIVE, UE releases the configuration for NW-side data collection.

When UE releases the configuration for NW-side data collection (for any reason), UE may still log some data which has not been sent to NW. Then, UE should discard it as there is no configuration for it any more.

Proposal 11. When UE releases the configuration for NW-side data collection (for any reason), UE discards any unsent data.

Conclusion

RAN2 is requested to discuss and agree the following proposals:

Proposal 1. RAN2 supports the following 3 types of data contents for BM-Case 1 for NW-side data collection:

- Type 1: All L1-RSRPs of measured based the resources (for Set A/B) configured to UE

- Type 2: All L1-RSRPs measured based on resources (for Set B) configured to UE, and beam information of K beams based on some resources (for Set A) configured to UE

- Type 3: All L1-RSRPs measured based on resources (for Set B) configured to UE, and beam information and L1-RSRPs of K beams based on some resources (for Set A) configured to UE

- Wherein the K beams are the beams with the largest K L1-RSRPs based on the resources (for Set A) in one instance

- K is configured by NW.

Proposal 2. UE mandatorily supports the fixed size of minimum AS layer memory without UE capability signalling, and optionally reports via UE capability a larger minimum AS layer memory size. FFS for the exact numbers.

Proposal 3. UE’s autonomous reporting (including AS buffer event based reporting) is NOT supported.

Proposal 4. UE sends the availability indication via UAI, when the size of logged data exceeds a threshold which is configured by NW.

Proposal 5. UE can send the availability indication in UEInformationResponse message to indicate the further residual logged data.

Proposal 6. Before full buffer, no separate indication is needed other than availability indication. Upon full buffer, UE indicates it (e.g., via UAI) (and stops measurement as agreed in RAN2#128).

Proposal 7. After full buffer, UE autonomously resumes when any memory becomes free (i.e., when any data report is sent to NW).

Proposal 8. RAN2 leaves RAN1 to discuss on event-based logging.

Proposal 9. During handover, it is up to NW implementation whether to keep or release the configuration for NW-side data collection.

Proposal 10. Upon going to RRC_IDLE or RRC_INACTIVE, UE releases the configuration for NW-side data collection.

Proposal 11. When UE releases the configuration for NW-side data collection (for any reason), UE discards any unsent data.

References

[1] R1-2410736, FL summary #3 for AI/ML in beam management, Samsung (Moderator)