3GPP TSG-RAN WG2 Meeting #129	R2-2500509

Athens, Greece, Feb. Feb. 17th – 21st, 2025

Agenda Item	: 8.13.3

Source	: LG Electronics Inc.

Title	: Discussion on Control Plane Procedure for multi-hop U2N relay

Document for	: Discussion and Decision

1.	Introduction

The work item on NR sidelink multi-hop relay was approved for Rel-19 and the WID is shown in RP-241609 including the following objectives.

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] - Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In this document, we mainly investigate the condition to send and the cause value of the notification message from the intermediate Relay UE, and the FFS parts of the “[Post128][401][Relay] Control plane baseline solution”.

2.	Discussion

2.1 Resource allocation mode for intermediate Relay UE

In Rel-17 U2N Relay, the U2N Relay UE was allowed to be configured resource allocation mode 1 and mode 2. However, the U2N Remote UE was only allowed to be configured resource allocation mode 2. This is because it’s too complicated if the mode 1 resource allocation is configured to the U2N Remote UE. The same principle can be applied to muti-hop relay operation. Therefore, both mode 1 and mode 2 resource allocation are supported for the last Relay UE. However, only mode 2 resource allocation is supported for the intermediate Relay UE and Remote UE.

Proposal 1: Both mode 1 and mode 2 resource allocation are supported for the last Relay UE.

Proposal 2: Only mode 2 resource allocation is supported for the intermediate Relay UE and Remote UE.

2.2 Notification message transmission depending on discovery model

According to the approved 23.304 CR in SA2(S2-2501295), the intermediate Relay UE shall establish a Layer-2 link with its parent intermediate Relay UE before sending the Discovery Announcement message. In Discovery model A, the intermediate Relay UE can select its parent Relay UE by its own decision. However, in the Discovery model B, the Remote UE decides the entire path toward the last Relay UE. This means that the intermediate Relay UE can select its parent intermediate Relay UE or last Relay UE when relay reselection is triggered, if the connection is made by Discovery model A.  However, the intermediate Relay UE that triggered relay reselection cannot select its parent intermediate Relay UE or last Relay UE, if the connection is made by Discovery model B.

<23.304 CR: “S2-2501295”> ##### 6.4.3.8.1 Layer-2 Link establishment and management Each of the 5G ProSe Intermediate UE-to-Network Relay shall establish a Layer-2 Link with its parent Intermediate UE-to-Network Relay or the 5G ProSe UE-to-Network Relay, before it can serve the 5G ProSe Remote UE, or a child Intermediate UE-to-Network Relay. The 5G ProSe Intermediate UE-to-Network Relay shall establish the Layer-2 Link with a parent relay before sending the 5G ProSe UE-to-Network Relay Discovery Announcement message. <omit> …

We believe that the whole intermediate Relay UEs and Remote UEs connected to the same last Relay UE establish the connection by using the same discovery model. This is because the RSC (Relay Service Code) value in the discovery message is only one value. Each intermediate Relay UE decides whether to forward the received discovery message or not depending on the RSC value. We understand that if the intermediate Relay UE uses an RSC for the Discovery model A, the intermediate Relay UE can only forward the discovery message with the same RSC value, i.e. the Discovery model A. Based on the SA2 approved CR, the intermediate Relay UE that has connected using Discovery model A can perform relay reselection by itself if the triggering condition is met.

Observation 1: According to the agreed topologies in RAN2, only one PC5 link can exit between last Relay UE and an intermediate Relay.

Observation 2: The RSC (Relay Service Conde) in the intermediate Relay UE is only one value.

Observation 3: We believe that the last Relay UE, intermediate Relay UE and Remote UE may have the same RSC value to be connected for the multi-hop relay.

Observation 4: According to the 23.304 CR (S2-2501295), the intermediate Relay UE has PC5 connection with its parent Relay UE before forwarding announcement message.

So, we can deduce the following proposals.

Proposal 3: We assume that if the intermediate Relay UE establishes the SL connection using Discovery model A, all the connected last/intermediate Relay UE(s) and Remote UE(s) of the multi-path are connected using Discovery model A.

Proposal 4: We assume that if the intermediate Relay UE establishes the SL connection using Discovery model B, all the connected last/ intermediate Relay UE(s) and Remote UE(s) of the multi-path are connected using Discovery model B.

Proposal 5: If relay reselection is triggered at the intermediate Relay UE, the intermediate Relay UE connected using Discovery model A can trigger relay reselection by itself.

Proposal 6: If relay reselection is triggered at the intermediate Relay UE, the intermediate Relay UE connected using Discovery model B cannot trigger relay reselection by itself.

If the intermediate Relay UE is connected using discovery model A and relay reselection is triggered, the intermediate Relay UE can select to establish a PC5 connection with a new parent Relay UE on its own decision. In this case, the child intermediate Relay UE and the Remote UE need to perform RRC reestablishment procedure in L2 relay operation. However, the child intermediate Relay UE(s) and the Remote UE(s) doesn’t need to find a new parent Relay UE for themselves. In other words, the remaining behind path of the intermediate Relay UE can be reused. Thus, the intermediate Relay UE connected by using Discovery model A can send a notification message toward the Remote UE with a cause value (e.g., relay reselection success). Upon receiving this notification message, the Remote UE(s) and the child intermediate Relay UE(s) perform a connection re-establishment procedure via the current parent Relay UE. In this case, the Remote UE(s) and the child intermediate Relay UE(s) doesn’t need to trigger relay reselection.

However, if the intermediate Relay UE is connected using Discovery Model A and Relay Reselection is triggered, the intermediate Relay UE may fail to connect to the PC5 with the new parent Relay UE. In this case, the child intermediate Relay UE(s) or Remote UE(s) behind the intermediate Relay UE need to trigger relay reselection procedure. The intermediate Relay UE sends notification message with a cause value (e.g., relay reselection fail) to the child intermediate Relay UE or Remote UE. Upon receiving this notification message, each child intermediate Relay UE(s) or Remote UE(s) triggers relay reselection and attempts to establish a PC5 connection with a new parent Relay UE. This process can be performed serially.

If the intermediate Relay UE is connected using Discovery model B and relay reselection is triggered, the intermediate Relay UE cannot select a new parent Relay UE for the child intermediate Relay UE(s) or Remote UE(s). The intermediate Relay UE acting as a Remote UE can only trigger relay reselection for itself. The other child intermediate Relay UE(s) or Remote UE(s) have to trigger relay reselection for themselves. The intermediate Relay UE connected using Discovery model B sends a notification message with a cause value when PC5 RLF is detected, HO occur, cell reselection is performed, Uu RRC connection establishment/Resume fails, or the upper layer requests the release of the PC5-RRC connection. Upon receiving this notification message, the child intermediate Relay UE(s) or Remote UE(s) will trigger relay reselection.

Proposal 7: When the relay reselection is triggered at the intermediate Relay UE, the sending condition of notification message can be different depending on the discovery model as followings;

- In the Discovery model A, the intermediate Relay UE sends a notification message after PC5 connection establishment with a new selected parent relay UE. Upon receiving this notification message, the Remote UE performs an RRC connection re-establishment procedure via the current intermediate Relay UE (no triggering relay reselection).

- In the Discovery model A, the intermediate Relay UE sends a notification message to the child intermediate or Remote UE if relay reselection has failed. Upon receiving this notification message, the child intermediate Relay UE or Remote UE triggers relay reselection to find a new parent Relay UE.

- In the Discovery model B, the intermediate Relay UE sends a notification message when PC5 RLF is detected, HO occurs, cell reselection is performed, Uu RRC connection establishment/Resume is failed, or the upper later requests the release of the PC5-RRC connection. Upon receiving this notification message, the Remote UE triggers relay reselection to find new (multi-hop) relay path.

2.3 The Remaining discussion issues from “[Post128][401][Relay] Control plane baseline solution”

(Remaining issue 1) Proposal 9: When an intermediate relay UE is in RRC_IDLE/RRC_INACTIVE it can obtain the SI required by it or requested by the remote UE by requesting SI from the parent relay UE in PC5-RRC (e.g., using RemoteUEInformationSidelink). FFS if it can also receive it directly from SIB broadcast by the cell on Uu (when the the intermediate relay UE is in coverage).

Basically, we prefer to inherit the legacy Rel-17 U2N Relay operation. The last Relay UE is responsible for SIB monitoring for the other intermediate Relay UE(s) and Remote UE(s). It is guaranteed that the last Relay UE is always in coverage due to the configured Uu-RSRP threshold. Even if the intermediate Relay UE in RRC_IDLE/INACTIVE can monitor the SIB requested by the Remote UE(s), it is not guaranteed that the intermediate Relay UE is always in coverage. Depending on the movement of the intermediate Relay UEs, either the last Relay UE or the intermediate Relay UEs may monitor the requested SIB. Taking into account the movement of the intermediate Relay UE may cause control complexity of the intermediate Relay UE and the last Relay UE.

Proposal 8: Even if the intermediate Relay UE in RRC_IDLE/INACTIVE can receive SIB directly from the cell, the intermediate Relay UE receives SIB from the last Relay UE.  Only the last Relay UE is responsible for SIB monitoring for the intermediate Relay UE(s) and Remote UE(s).

(Remaining issue 2) Proposal 21: An intermediate relay UE in RRC_CONNECTED releases its paging-related information in the parent UE at least for itself. FFS whether it also releases it for the child UEs and how the child UEs receive the paging when the intermediate relay UE is in RRC_CONNECTED.

Similarly, we believe that the last Relay UE is responsible for the paging monitoring. Therefore, when the intermediate Relay UE become in RRC_CONNECTED, it only needs to release its own paging-related information. If the intermediate Relay UE in RRC_CONNECTED has to receive paging information for the Remote UE, the intermediate Relay UE has to control the paging monitoring related information of the Remote UE depending on its own RRC state transition. We believe that it causes control complexity of the intermediate Relay UE.

Proposal 9: When the intermediate Relay UE in RRC_CONNECTED sends a request message to release its paging-related information to its parent Relay UE, the intermediate Relay UE does not send a message to release the information related to the child intermediate Relay UE(s) or Remote UE(s). Only the last Relay UE is responsible for paging monitoring for the intermediate Relay UE(s) and Remote UE(s).

(Remaining issue 3) Proposal 22: When an intermediate relay UE is in RRC_IDLE/RRC_INACTIVE it can obtain paging from the parent relay UE in PC5-RRC (e.g., using RemoteUEInformationSidelink). FFS if it can also receive it directly on Uu (when the the intermediate relay UE is in coverage). Proposal 23: What triggers the intermediate relay UE to request paging monitoring by the parent relay (intermediate relay or last relay) in PC5-RRC (e.g., in RemoteUEInformationSidelink)? - when there is a change in the paging information of the intermediate UE or child UE - when the intermediate UE enters RRC_IDLE/RRC_INACTIVE - when the intermediate UE enters RRC_CONNECTED (to cancel paging monitoring request) - upon reception of paging monitoring request from a remote UE/child relay UE? FFS upon change in the ability of the intermediate UE to monitor paging on Uu (e.g., moving in/out of coverage) to initiate/cancel paging monitoring by the parent relay).

In the legacy Rel-17 U2N relay operation, the U2N Remote UE sends a message to the U2N Relay UE to release paging monitoring or set the information for paging monitoring when the state transition occurs at the U2N Remote UE. It does not depend on whether the U2N Remote UE is in coverage or out of coverage. We can apply the same principle to the Rel-19 multi-hop relay operation. It will not generate a new message transmission triggering condition comparing to the Rel-17 U2N relay. We think it’s better that the intermediate Relay UE receives the paging message via the last Relay UE regardless of whether the intermediate Relay UE is in coverage or not.

Proposal 10: Even though the intermediate Relay UE in RRC_IDLE/INACTIVE can receive its paging directly from the Uu, the default behaviour of an intermediate Relay UE is to receive its paging from the last Relay UE.

.

3.	Conclusion

Proposal 1: Both mode 1 and mode 2 resource allocation are supported for the last Relay UE.

Proposal 2: Only mode 2 resource allocation is supported for the intermediate Relay UE and Remote UE.

Proposal 3: We assume that if the intermediate Relay UE establishes the SL connection using Discovery model A, all the connected last/intermediate Relay UE(s) and Remote UE(s) of the multi-path are connected using Discovery model A.

Proposal 4: We assume that if the intermediate Relay UE establishes the SL connection using Discovery model B, all the connected last/ intermediate Relay UE(s) and Remote UE(s) of the multi-path are connected using Discovery model B.

Proposal 5: If relay reselection is triggered at the intermediate Relay UE, the intermediate Relay UE connected using Discovery model A can trigger relay reselection by itself.

Proposal 6: If relay reselection is triggered at the intermediate Relay UE, the intermediate Relay UE connected using Discovery model B cannot trigger relay reselection by itself.

Proposal 7: When the relay reselection is triggered at the intermediate Relay UE, the sending condition of notification message can be different depending on the discovery model as followings;

- In the Discovery model A, the intermediate Relay UE sends a notification message after PC5 connection establishment with a new selected parent relay UE. Upon receiving this notification message, the Remote UE performs an RRC connection re-establishment procedure via the current intermediate Relay UE (no triggering relay reselection).

- In the Discovery model A, the intermediate Relay UE sends a notification message to the child intermediate or Remote UE if relay reselection has failed. Upon receiving this notification message, the child intermediate Relay UE or Remote UE triggers relay reselection to find a new parent Relay UE.

- In the Discovery model B, the intermediate Relay UE sends a notification message when PC5 RLF is detected, HO occurs, cell reselection is performed, Uu RRC connection establishment/Resume is failed, notification message is received, or the upper later requests the release of the PC5-RRC connection. Upon receiving this notification message, the Remote UE triggers relay reselection to find new (multi-hop) relay path.

Proposal 8: Even though the intermediate Relay UE in RRC_IDLE/INACTIVE can receive SIB directly from the cell, the intermediate Relay UE receives SIB from the last Relay UE.  Only the last Relay UE is responsible for SIB monitoring for the intermediate Relay UE(s) and Remote UE(s).

Proposal 9: When the intermediate Relay UE in RRC_CONNECTED sends a request message to release its paging-related information to its parent Relay UE, the intermediate Relay UE does not send a message to release the information related to the child intermediate Relay UE(s) or Remote UE(s). Only the last Relay UE is responsible for paging monitoring for the intermediate Relay UE(s) and Remote UE(s).

Proposal 10: Even though the intermediate Relay UE in RRC_IDLE/INACTIVE can receive its paging directly from the Uu, the default behaviour of an intermediate Relay UE is to receive its paging from the last Relay UE.

4.	Reference

- RP-241609, Approved WID: NR sidelink multi-hop relay, LG Electronics Inc., InterDigital, FirstNet.