3GPP TSG-RAN WG2 Meeting #128	R2-2500510

Orlando, US, 18-22 August, 2024

Agenda Item	: 8.13.4

Source	: LG Electronics Inc.

Title	: Discussion on service continuity for multi-hop U2N relay

Document for	: Discussion and Decision

1.	Introduction

The work item on NR sidelink multi-hop relay was approved for Rel-19 and the WID is shown in RP-241609 including the following objectives.

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] - Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In this document, we propose the service continuity for muti-hop U2N relay operation, especially, for the second priority case in the WID[1], case C and case D, since the first priority case was handled at the last meeting.

2.	Discussion

  - Service continuity for intra-gNB multi-hop indirect to direct path

According to the WID multi-hop Relay, RAN2 handles the following cases with second priority:

- Case C) Intra-gNB direct to multi-hop indirect path switching

- Case D) Intra-gNB single-hop indirect to multi-hop indirect path switching

We can use the same procedure as the legacy Rel-17 U2N direct-to-indirect path switching. In the legacy Rel-17 U2N path switching, the service continuity of the Remote UE was supported when the target path was a direct path to a single-hop indirect path. Therefore, we believe that the same procedure can be applied to the above case C. The difference with the legacy Rel-17 U2N service continuity is that the target path has a multi-hop U2N connection. However, according to the WID, the last Relay UE is directly RRC connected mode and all the other intermediate Relay UE is indirectly RRC connected mode for the target path. So, we believe that the same path switching procedure Rel-17 U2N direct-to-indirect path switching can be applied even simply because the target path are all in RRC connected state.

Proposal 1: For scenario C, the legacy path switching command and procedure are reused for the Remote UE and the target first Relay UE. Upon receiving the path switching command, the Remote UE performs the legacy UE behaviour toward the target first Relay UE. The (first)intermediate/last Relay UE belonging to the target path should be in the RRC connected state.

Based on the last meeting agreement for the service continuity, the way like group path switching cannot be allowed. The same principle can be applied to the case D. Thus, the first Relay UE in the target path is a new relay UE which is not on the source relay path. The legacy indirect-to-indirect path switching procedure can be reused for case D.

Proposal 2: For scenario D, the first Relay UE in the target path is a new relay UE which is not on the source relay path. The legacy path switching command and procedure are reused for the Remote UE and the fist Relay UE in the target path. Upon receiving the path switching command, the Remote UE performs legacy UE behaviour toward the first Relay UE on the target path. The (first)intermediate/last Relay UE belonging to the target path should be in the RRC connected state.

3.	Conclusion

Proposal 1: For scenario C, the legacy path switching command and procedure are reused for the Remote UE and the target first Relay UE. Upon receiving the path switching command, the Remote UE performs the legacy UE behaviour toward the target first Relay UE. The (first)intermediate/last Relay UE belonging to the target path should be in the RRC connected state.

Proposal 2: For scenario D, the first Relay UE in the target path is a new relay UE which is not on the source relay path. The legacy path switching command and procedure are reused for the Remote UE and the fist Relay UE in the target path. Upon receiving the path switching command, the Remote UE performs legacy UE behaviour toward the first Relay UE on the target path. The (first)intermediate/last Relay UE belonging to the target path should be in the RRC connected state.

4.	Reference

- RP-241609, Approved WID: NR sidelink multi-hop relay, LG Electronics Inc., InterDigital, FirstNet.