3GPP TSG-RAN WG2 #129 meeting	R2-2500512

# Athens, Greece, 17th Feb – 21st Feb, 2025

Source:		NEC

Agenda item:		8.7.4.2	DSR enhancements

Title:		Considerations on DSR enhancements for XR

Document for:		Discussion and decision

# 1. Introduction

In the RAN2#128 meeting, agreements [1] on DSR enhancements for XR were made as below:

Let the network configure the triggering and reporting thresholds without constraints. RAN2 understanding is that the data that has been already reported in the DSR should not trigger another DSR The existing cancelling and triggering of Rel-18 DSR is reused for the enhanced DSR. The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

In this contribution, we will further discuss the remaining issues related to the enhanced DSR.

# 2. Discussion

Enhanced DSR and legacy DSR

RAN2 has agreed that triggering and reporting thresholds for enhanced DSR can be configured without constraints. In other words, the largest configured reporting threshold for enhanced DSR may be greater than, equal to, or lower than the remainingTimeThreshold for the legacy DSR.

Figure 1: issues when legacy and enhanced DSR coexist

- If the largest configured reporting threshold is greater than or equal to remainingTimeThreshold, the reported enhanced DSR shall cover all the delay information of legacy DSR. In this case, the legacy DSR can be cancelled if enhanced DSR is triggered.

- If the largest configured reporting threshold (e.g., T2) is lower than the remainingTimeThreshold (e.g., T3), as illustrated in Figure 1,

  - If legacy DSR is cancelled or not triggered upon triggering the enhanced DSR, then the delay information of unreported residual delay-critical data with remaining time range [T2, T3) will be missed.

  - If legacy DSR is not cancelled upon triggering the enhanced DSR, and there is unreported residual delay-critical data with remaining time range [T2, T3) after reporting of enhanced DSR, then legacy DSR shall also be reported. In this case, legacy DSR should avoid reporting redundent delay information with the remaining time range [0, T2) that has been reported by enhanced DSR.

Observation 1: The coexistence of legacy and enhanced DSR may result in unreported residual delay-critical data and the reporting of redundant delay information.

To ensure that all delay-critical data's delay information is accurately reported without redundancy, and to distinguish the operations (e.g., cancellation of a pending DSR) of legacy DSR and enhanced DSR, we must prevent PDCP SDUs from being associated with both DSR types. That is, it is essential to differentiate PDCP SDUs associated with legacy DSR from those with enhanced DSR. For example, we can consider the following definitions as a starting point.

A PDCP SDU is considered to be associated with a DSR (e.g., legacy DSR) if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU (as defined in TS 38.323 [4]) associated with the logical channel which triggered the DSR and is not associated with an enhanced DSR. A PDCP SDU is considered to be associated with an enhanced DSR if it has not been transmitted in any MAC PDU and is a PDCP SDU with remaining value of the running PDCP discardTimer below the largest configured reporting threshold and associated with the logical channel which triggered the DSR.

Proposal 1: RAN2 agrees to differentiate PDCP SDUs associated with legacy DSR from those associated with enhanced DSR.

If the reported enhanced DSR fails to encompass all delay information, such as that of residual delay-critical data with remaining time range [T2, T3), UE may still need to report legacy DSR to ensure all delay information of delay-critical data is reported. However, it may lead to increased overhead and extra delay. Consequently, RAN2 can consider how to report all delay information in a single DSR instance whenever feasible.

Proposal 2: To ensure all the delay information is reported without increased overhead, RAN2 can consider how to report all delay information in a single DSR instance whenever feasible.

Furthermore, since legacy DSR and enhanced DSR differ primarily in granularity, with the enhanced DSR offering finer information, if an enhanced DSR encompasses all the delay information of a legacy DSR, the legacy DSR can be cancelled.

Proposal 3: If an enhanced DSR encompasses all the delay information of legacy DSR, the legacy DSR can be cancelled.

non-delay critical data volume

To calculate the buffer size of non-delay critical data ahead of delay critical data, we need to explicitly identify which types of data can be categorized as non-delay critical data. For instance, the non-delay critical data should encompass not only PDCP SDUs or PDU Sets whose remaining time exceeds a configured threshold, but also low importance PDU Sets.

Proposal 4: Regarding the non-delay critical data volume calculation, it should encompass not only PDCP SDUs or PDU Sets whose remaining time exceeds a configured threshold, but also low importance PDU Sets.

Furthermore, as all PDUs in a PDU Set share the same PSDB, if all PDUs belonging to a PDU Set are needed (e.g., if pdu-SetDiscard is configured), and at least one SDU belonging to a PDU Set becomes delay-critical, then all SDUs belonging to the PDU Set should be counted in the delay-critical data volume.

Proposal 5: If all PDUs belonging to a PDU Set are needed and at least one SDU belonging to a PDU Set becomes delay-critical, then all SDUs belonging to the PDU Set should be counted in the delay-critical data volume.

# Summary

This contribution provides our analysis on the enhancement of DSR, and has the following proposals:

Observation 1: The coexistence of legacy and enhanced DSR may result in unreported residual delay-critical data and the reporting of redundant delay information.

Proposal 1: RAN2 agrees to differentiate PDCP SDUs associated with legacy DSR from those associated with enhanced DSR.

Proposal 2: To ensure all the delay information is reported without increased overhead, RAN2 can consider how to report all delay information in a single DSR instance whenever feasible.

Proposal 3: If an enhanced DSR encompasses all the delay information of legacy DSR, the legacy DSR can be cancelled.

Proposal 4: Regarding the non-delay critical data volume calculation, it should encompass not only PDCP SDUs or PDU Sets whose remaining time exceeds a configured threshold, but also low importance PDU Sets.

Proposal 5: If all PDUs belonging to a PDU Set are needed and at least one SDU belonging to a PDU Set becomes delay-critical, then all SDUs belonging to the PDU Set should be counted in the delay-critical data volume.

# References

[1] Report of 3GPP TSG RAN WG2 meeting #128, Orlando, the USA

[2] TS 38.321, Medium Access Control (MAC) protocol specification, v18.4.0