3GPP TSG-RAN WG2 Meeting #129	 R2-2500522

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.9.4

Source: 	CATT

Title: 	Remaining issues on support of PWS for NB-IoT NTN UE

Document for: 	Discussion and Decision

Introduction

Below conclusions were reached in RAN2 #128 on the support of PWS [1]:

1.	We will clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements.

2.	We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed

3.	Add the following PWS indication in Paging-NB:

- etws-Indication;

- cmas-Indication.

This contribution addresses the remaining open issues on the support of PWS for NB-IoT NTN UE.

Discussion

If SIB1-NB acquisition is needed

About acquisition on PWS immediately, we made the agreement “We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed” last meeting. It is FFS on whether SIB1-NB acquisition is needed; the intention of avoiding acquiring the SIB1-NB was to reduce the latency for acquisition of PWS.

In LTE, UE is not required to periodically check schedulingInfoList contained in SIB1, and the eNB may not send paging message to indicate to the UE the update of the schedulingInfoList due to start or stop scheduling the PWS. Hence the UE should first read the SIB1 to acquire the scheduling information for the corresponding PWS message, and then to acquire the PWS message, as follows [2]:

NOTE: The UE is not required to periodically check schedulingInfoList contained in SystemInformationBlockType1, but Paging message including the etws-Indication triggers the UE to re-acquire schedulingInfoList contained in SystemInformationBlockType1 for scheduling changes for SystemInformationBlockType10 and SystemInformationBlockType11. The UE may or may not receive a Paging message including the etws-Indication and/or systemInfoModification when ETWS is no longer scheduled.

If the similar mechanism is reused by the NB-IoT NTN UE, it also requires the UE to acquire the SIB1-NB to get the scheduling information for the corresponding PWS message upon reception of the indication of PWS.

However, the periodicity of SIB1-NB (2560 ms) is much longer than that for SIB1 of LTE (80 ms); acquiring SIB1-NB will thus introduce more latency which may increase the possibility that the requirement for acquisition of PWS is not met. If this were regarded as an issue to be solved, RAN2 could consider a solution to avoid UE acquiring SIB1-NB right before acquisition of PWS message. Specifically, the NW which supports PWS can reserve the position of SI window for PWS in the schedulingInfoList, and the NW really broadcasts the corresponding PWS in the reserved position only when the NW starts sending the indication of PWS. Otherwise, the resources located in the SI window could be scheduled for other data transmission.

Proposal 1: RAN2 down-selects the following solutions to resolve the issue whether SIB1(-NB) needs to be first acquired upon PWS indication:

- Solution 1: Reuse the same mechanism as in LTE, i.e. the indication of PWS triggers the UE to re-acquire schedulingInfoList contained in SIB1-NB to get scheduling info for corresponding PWS message.

- Solution 2: The schedulingInfoList contained in SIB1-NB includes the scheduling information for PWS message even when eNB does not broadcast PWS message, and the indication of PWS triggers UE to acquire the corresponding PWS message immediately based on the stored schedulingInfoList.

Content of the SIB10-NB/SIB11-NB/SIB12-NB

Regarding what is included in SIB10-NB/SIB11-NB/SIB12-NB to be introduced, it was agreed in RAN2#127bis to take the content within the corresponding LTE SIBs as the baseline, but to also check whether we can skip some unnecessary fields as signaling optimization.

Based on the content of existing SIB10/11/12, most of the fields refer to other specs which are not within the RRC scope. Take SIB10 as an example: we can find that the messageIdentifier/serialNumber/warningType refer to TS 36.413; hence, it seems not possible to determine at the RRC level which of these fields are necessary or unnecessary. The possibility of skipping some of these fields (if really possible) should be determined and specified in TS36.413.

SystemInformationBlockType10 ::= SEQUENCE { messageIdentifier BIT STRING (SIZE (16)), serialNumber BIT STRING (SIZE (16)), warningType OCTET STRING (SIZE (2)), dummy OCTET STRING (SIZE (50)) OPTIONAL, -- Need OP ..., lateNonCriticalExtension OCTET STRING OPTIONAL }

As for those RRC-defined fields, e.g. warningMessageSegmentNumber/warningMessageSegmentNumber in SIB11, they are mainly used for segmentation. Considering the limited size for SI transmission in NB-IoT which is even smaller than in LTE, the segmentation function should still be supported for NB-IoT, so that these fields cannot be skipped either.

SystemInformationBlockType12-r9 ::=	SEQUENCE {

messageIdentifier-r9				BIT STRING (SIZE (16)),

serialNumber-r9						BIT STRING (SIZE (16)),

warningMessageSegmentType-r9		ENUMERATED {notLastSegment, lastSegment},

warningMessageSegmentNumber-r9		INTEGER (0..63),

warningMessageSegment-r9			OCTET STRING,

dataCodingScheme-r9					OCTET STRING (SIZE (1))		OPTIONAL,	-- Cond Segment1

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

...,

[[	warningAreaCoordinatesSegment-r15		OCTET STRING	OPTIONAL	-- Need OR

]]

}

As for warningAreaCoordinatesSegment defined in SIB12, it is used to indicate the geographical area where the CMAS warning message is valid, and remains needed especially for NTN cell. Also, its definition refers to TS 23.041 [4].

Observation 1: RAN2 cannot determine skipping the parameters in SIB10/11/12 whose contents are defined by other WGs' Specs. As for the parameters defined by RRC in SIB10/11/12, they are used for segmentation and thus cannot be skipped.

Considering the above situation, we think there is no need to further pursue the optimization on potential field skipping, in order to simplify the discussion and avoid further inter-WG coordination. So we have Proposal 2 as follows:

Proposal 2: From RAN2 perspective, the content within the corresponding LTE SIBs can be reused for SIB10-NB/SIB11-NB/SIB12-NB (except for the dummy field in SIB10). Optimization on field skipping is not pursued.

Intended service area for ETWS

In the WID, one note is captured "Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome".

About the intended service area for ETWS for NR NTN, SA1 reply LS [3] informs RAN2 that the warning notification with geofencing requirements for ETWS is added and applicable to Satellite access. The related impact on RAN2 spec will be discussed under the AI of NR NTN Phase 3. In order to avoid duplicated discussion, NB-IoT NTN can wait for the progress of NR NTN and consider reusing the same mechanism directly, if possible.

Proposal 3: Wait for the progress on the intended service area design for ETWS in NR NTN, and consider reusing it directly to NB-IoT NTN (if possible).

Conclusion

In this contribution, the remaining issues on support of PWS for NB-IoT UE are discussed with the following observations and proposals:

Observation 1: RAN2 cannot determine skipping the parameters in SIB10/11/12 whose contents are defined by other WGs' Specs. As for the parameters defined by RRC in SIB10/11/12, they are used for segmentation and thus cannot be skipped.

Proposal 1: RAN2 down-selects the following solutions to resolve the issue whether SIB1(-NB) needs to be first acquired upon PWS indication:

- Solution 1: Reuse the same mechanism as in LTE, i.e. the indication of PWS triggers the UE to re-acquire schedulingInfoList contained in SIB1-NB to get scheduling info for corresponding PWS message.

- Solution 2: The schedulingInfoList contained in SIB1-NB includes the scheduling information for PWS message even when eNB does not broadcast PWS message, and the indication of PWS triggers UE to acquire the corresponding PWS message immediately based on the stored schedulingInfoList.

Proposal 2: From RAN2 perspective, the content within the corresponding LTE SIBs can be reused for SIB10-NB/SIB11-NB/SIB12-NB (except for the dummy field in SIB10). Optimization on field skipping is not pursued.

Proposal 3: Wait for the progress on the intended service area design for ETWS in NR NTN, and consider reusing it directly to NB-IoT NTN (if possible).

Reference

- Draft_RAN2_128_Meeting_Report_v1

- 3GPP TS 36.331: "Universal Terrestrial Radio Access (UTRAN); Radio Resource Control (RRC)"

- S1-244857, Reply LS on requirements for ETWS primary notification

- TS 23.041