3GPP TSG-RAN WG2 Meeting #129	R2-2500523

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.8.4

Source: 	CATT

Title: 	Further discussion on support of broadcast service in NR NTN

Document for: 	Discussion and Decision

Introduction

Based on current progress and agreements, this contribution further discusses the signalling design of the intended service area, service continuity aspects and ETWS geo-fencing for NR NTN.

Discussion

Signalling design of the intended service area

It was agreed to introduce a new SIB to include a list of intended service areas [1]. But there is still FFS on how to associate the intended service areas in the new SIB and the MBS session configured in MCCH.

Introduce a new SIB to include a list of intended service areas and related pointer (FFS if we point to the intended services areas via the index in the list or with an ID or another way)

Considering that SIB is broadcast signalling always in a way of full configuration, introducing an explicit ID for each entry of the service area list does not make much sense, because it is unable to be used to perform delta configurations as in the dedicated signalling, but introduce unnecessary extra signalling overhead in the other way around.

In the legacy, it is possible to use the position of each entry in the full list to reference the entry itself. Hence, it is proposed to follow this manner instead of introducing an explicit ID for each service area configured.

Proposal 1: Use the ordinal position among the entries of service areas list configured in the new SIB to reference the associated intended service area(s) for each MBS session configured in MCCH.

In RAN2#128, the service announcement based solution was discussed, i.e. providing the service area ID to UE via the MBS Service Announcement procedure, without reaching consensus. The intention was to relax the MCCH acquisition [2]. For this solution, we hold the following views. On the one hand, to provide service area ID via the MBS Service Announcement procedure, coordination between gNB and CN is needed to align the mapping between service area and its corresponding ID. This will bring non-trivial standard efforts (at least time for discussion) on RAN3 and SA2. Considering that SA2 is going to complete the exception of Rel-19 normative work in 2025 Q1, and there has been no relevant discussion on this new feature till now, it is thus impossible to introduce this solution in Rel-19 time frame at least from CN perspective.

Observation 1: Providing service area ID via the MBS Service Announcement procedure will lead to non-trivial Spec impacts which result in extra workload to RAN3 and SA2. Considering that there has been no relevant discussion on this aspect in SA2 that is to complete Rel-19 exception in 2025 Q1, it is too late to introduce this new feature in Rel-19 time frame as a new feature in SA2 Spec.

On the other hand, from a technical perspective, this service announcement based solution may have benefits only when the intended service area related information in the new SIB and MBS Service Announcement is changed but the MCCH configuration is not changed. Otherwise, as long as MCCH is changed (e.g. MBS session addition/removal/update), there is no way to relax MCCH re-acquisition.

However, RAN2 reached common understanding that the geographic area information for the intended service areas can be semi-static and does not cause frequent updates. This means that the intended service area change w/o MBS session related configuration in MCCH is a corner case, making the benefit of this service announcement based solution marginal.

5.	RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates.

Observation 2: The solution of providing the intended service area ID via MBS Service Announcement procedure has benefit to avoid reacquiring MCCH, only when there is change of intended service area info in new SIB and service announcement but no change of MBS session related information in MCCH. However, this is a corner case based on previous RAN2 common understanding that the intended service area is semi-static and not causes frequent updates, making this solution with marginal benefit.

With above observations, the enhancement of providing intended service area ID via MBS service announcement is not supported.

Proposal 2: Do not enhance the MBS Service Announcement procedure to provide intended service area ID to UE.

Also, as per our analyses in section 2.2 in [3], the optimization of MCCH acquisition in NTN scenario, w.r.t. how the UE avoids acquiring MCCH outside the intended service area, can be left to UE implementation. More specifically, when UE enters an NTN cell, it reads the MCCH according to current spec and acquires the service areas of MBS sessions also based on the new SIB. Once the UE acquires the service area info, it could store this information to determine whether to re-acquire the up-to-date MCCH. That is, the UE is not required to acquire the MCCH when it is outside any service area of its interested MBS sessions as determined based on its stored information. How to use the stored service area to avoid re-acquiring MCCH, as illustrated above, can be up to UE implementation, and this does not need to be specified in normative text.

Proposal 3: It is up to UE implementation to use the stored service area information acquired via the new SIB to avoid acquiring the MCCH when the UE is outside the service area of the interested MBS session. No specified UE behaviour on this aspect is needed.

Service continuity enhancements

In past meetings, MBS service continuity enhancements in the context of NTN were also discussed. Based on our analyses as follows, we found these enhancements are not essential and only have limited benefits.

Potential enhancements on service area for FSAI

Companies proposed to introduce service area for FSAI, so that the UE could handle the frequency priority with the consideration of whether it is within the service area of neighbour frequencies/cells. However, it is hard to introduce the service area for FSAI into SIB21 based on the current signalling structure.

According to TS 38.331 [4], the FSAI list in SIB21 is configured in a per frequency manner.

SIB21-r17 ::= SEQUENCE { mbs-FSAI-IntraFreq-r17 MBS-FSAI-List-r17 OPTIONAL, -- Need R mbs-FSAI-InterFreqList-r17 MBS-FSAI-InterFreqList-r17 OPTIONAL, -- Need R lateNonCriticalExtension OCTET STRING OPTIONAL, ... } MBS-FSAI-List-r17 ::= SEQUENCE (SIZE (1..maxFSAI-MBS-r17)) OF MBS-FSAI-r17 MBS-FSAI-InterFreqList-r17 ::= SEQUENCE (SIZE (1..maxFreq)) OF MBS-FSAI-InterFreq-r17 MBS-FSAI-InterFreq-r17 ::= SEQUENCE { dl-CarrierFreq-r17 ARFCN-ValueNR, mbs-FSAI-List-r17 MBS-FSAI-List-r17 }

In addition, according to TS 29.571 [5], one MBS session is associated with a list of FSAI, and one FSAI is allowed to be associated with multiple MBS sessions.

- 5.9.4.6 Type: MbsSession Table 5.9.4.6-1: Definition of type MbsSession Attribute name; Data type; P; Cardinality; Description. mbsSessionId; MbsSessionId; C; 0..1; MBS session identifier (TMGI and/or SSM, and NID for an SNPN) (NOTE 1). [unrelated part is omitted]. mbsFsaIdList; array(MbsFsaId); O; 1..N; List of MBS Frequency Selection Area Identifiers, for a broadcast MBS session. This attribute may be present if the "serviceType" attribute indicates a broadcast MBS session..

Above quotation implies that, if service areas were to be introduced specifically for FSAI in SIB21, the service area should be provided per FSAI and per MBS session. This brings excessively huge singling overhead for the system information considering the limited SI size, making such signalling design unacceptable.

Observation 3: If service areas were to be introduced specifically for FSAI in SIB21, they should be provided per FSAI and per MBS session. This brings excessively huge signalling overhead for the system information, and the signalling design is unacceptable still due to the SI size limitation concern.

Having checked the existing FSAI mechanisms, we found that the intention raised by company can already be realized by UE and NW implementation. In RAN2#128, it was agreed that the intended service area may include geographic areas across the current serving cell and overlapping neighbour cell(s).

4.	A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbor cell(s).

That is, the UE could obtain the service area info of an MBS session which covers not only the service areas of the serving cell but also those of the neighbour cells. Combining the info in USD and SIB21, the UE can determine whether the UE can obtain the service from neighbour cells. Specifically:

- For a certain MBS session, the UE acquires the FSAI list of an MBS session ID via upper layer procedure [5][6], and acquires related frequencies of FSAI via SIB21. This means, the UE, can know the frequencies applicable for each MBS session based on current USD and SIB21.

- Based on the intended service areas associated with the MBS session, the UE can know the intended service area within the scope of both serving cell and neighbour cells for each MBS session.

Then, the UE could determine whether it may prioritize the frequency(ies) associated with its intended service area (as per bullet "i"), based on whether it is still located within its intended service as determined by its current GNSS location and bullet "ii". That is, the UE may further prioritize a frequency(ies) in SIB21 for its interested service, if it finds itself located in the intended service area. The information in the exiting USD and the introduced service areas info in new SIB are already sufficient, and additional ASN.1 impact is not needed.

Proposal 4: Do not introduce service area information specific for FSAI in SIB21.

Also, since anyway this MBS specific frequency prioritization behaviour is up to UE implementation (with "... the UE may consider that frequency to be the highest priority during the MBS broadcast session…" in TS 38.304), we think RAN2 can also confirm that the UE can still considers whether to prioritize a frequency providing its interested MBS service (based on existing SIB21 and USD) as per the current procedure in TS 38.304, and discuss whether any informative text is needed to clarify the use of the intended service area related information (e.g. clarify that the UE may also take into account whether it is within the intended service area based on the introduced intended service area for the MBS session in the MCCH).

Proposal 4a: The UE can determine whether to prioritize the frequency in SIB21 for its interested MBS service(s) (based on the existing SIB21 and USD) as per the current procedure in TS 38.304. RAN2 discusses whether any informative text is needed, e.g. to further clarify that UE may take into account whether it is located in the intended service area based on the introduced intended service area for the MBS session in MCCH.

Introduction of service area for neighbour cell list in MCCH

Companies proposed to introduce service area specific for neighbour cells in MCCH, so that the UE could determine whether to request unicast before reselecting to a neighbour cell.

On the one hand, introducing service area for the neighbour cell will increase the signalling overhead, especially if the service areas are contained in SIB. On the other hand, the intention raised by company can also be realized by implementation without ASN.1 impact. In detail, for a certain MBS session, its intended service area may cover geographic areas across the current serving cell and overlapping neighbour cell(s) as per last meeting agreement. Based on this, the UE could determine whether a neighbour cell is providing this MBS session based on its GNSS location. If the UE is still within the service area and the reselected cell is within the neighbour cell list provided in MCCH, the UE considers the neighbour cell providing this MBS session. No need to further introduce per neighbour cell service area distinction.

Observation 4: It was agreed that the intended service area for an MBS session may include geographic areas within neighbour cell(s). If the UE is still within the service area and the reselected cell is within the neighbour cell list provided in MCCH, the UE can consider the neighbour cell providing this MBS session.

Also, we think whether/how the UE considers the service area related configuration in MCCH in addition to the neighbour cell list in MCCH can still be left to UE implementation, when it determines whether the reselected cell providing its interested MBS session at its location.

Proposal 5: Do not introduce service area per neighbour cell in MCCH.

Proposal 5a: In addition to the neighbour cell list provided in MCCH, it can be left to UE implementation whether/how the UE considers the intended service area configured in MCCH for the MBS session, when the UE determines whether the reselected cell is providing its interested MBS session at its location. RAN2 discusses whether any informative text is needed in the Spec.

ETWS geo-fencing for NTN

In [7], SA1 replies with the requirement of ETWS geo-fencing for NTN. ETWS geo-fencing requirement for NTN was added as follows. Accordingly, RAN2 needs to discuss how to broadcast the Warning Notification Area(s) over the Uu.

## 5.6 Additional PWS Requirements Specific to ETWS Subject to local/regional regulatory requirements, in addition to ETWS Primary and Secondary Notifications, ETWS shall be able to support broadcasting Warning Notification Area(s). This requirement applies to Satellite E-UTRAN and Satellite NG-RAN. Subject to local/regional regulatory requirements, an ETWS-capable UE shall present the received ETWS Notification(s) (Primary or Secondary) if there is a corresponding Warning Notification Area(s) and if the ETWS-capable UE is able to determine that its location is within the corresponding Warning Notification Area(s). Otherwise, if an ETWS-capable UE is unable to or fails to determine its location, the ETWS-capable UE shall present the ETWS Notification(s) (Primary or Secondary) to the user.

Considering that the Warning Notification Area(s) is introduce for the UE to determine whether to present the ETWS notification to the user, it should be contained in the Primary Notification, i.e., SIB6 (as it is too late to contain in the Secondary Notification). As for the format of warning notification area, we think the format used for CMAS (SIB8) can be reused. Considering that the Primary Notification shall be small enough to achieve timely information delivery as stated in [8], it is proposed to discuss whether it is feasible to reuse the format of warningAreaCoordinatesSegment for CMAS with a restriction of no segmentation to support ETWS Geo-fencing.

SIB8 ::= SEQUENCE { messageIdentifier BIT STRING (SIZE (16)), serialNumber BIT STRING (SIZE (16)), warningMessageSegmentType ENUMERATED {notLastSegment, lastSegment}, warningMessageSegmentNumber INTEGER (0..63), warningMessageSegment OCTET STRING, dataCodingScheme OCTET STRING (SIZE (1)) OPTIONAL, -- Cond Segment1 warningAreaCoordinatesSegment OCTET STRING OPTIONAL, -- Need R lateNonCriticalExtension OCTET STRING OPTIONAL, ... }

warningAreaCoordinatesSegment If present, carries a segment, with one or more octets, of the geographical area where the CMAS warning message is valid as defined in [28]. The first octet of the first warningAreaCoordinatesSegment is equivalent to the first octet of Warning Area Coordinates IE defined in and encoded according to TS 23.041 [29] and so on.

Proposal 6: Introduce a Warning Notification Area in SIB6. RAN2 discusses whether it is feasible to reuse the format of Warning Notification Area for CMAS in SIB8 with a restriction of no segmentation to support ETWS Geo-fencing.

Conclusion

In this contribution, supportive of broadcast service via NR NTN is discussed with giving corresponding proposals:

Signalling design of the intended service areas

Observation 1: Providing service area ID via the MBS Service Announcement procedure will lead to non-trivial Spec impacts which result in extra workload to RAN3 and SA2. Considering that there has been no relevant discussion on this aspect in SA2 that is to complete Rel-19 exception in 2025 Q1, it is too late to introduce this new feature in Rel-19 time frame as a new feature in SA2 Spec.

Observation 2: The solution of providing the intended service area ID via MBS Service Announcement procedure has benefit to avoid reacquiring MCCH, only when there is change of intended service area info in new SIB and service announcement but no change of MBS session related information in MCCH. However, this is a corner case based on previous RAN2 common understanding that the intended service area is semi-static and not causes frequent updates, making this solution with marginal benefit.

Proposal 1: Use the ordinal position among the entries of service areas list configured in the new SIB to reference the associated intended service area(s) for each MBS session configured in MCCH.

Proposal 2: Do not enhance the MBS Service Announcement procedure to provide intended service area ID to UE.

Proposal 3: It is up to UE implementation to use the stored service area information acquired via the new SIB to avoid acquiring the MCCH when the UE is outside the service area of the interested MBS session. No specified UE behaviour on this aspect is needed.

Service continuity enhancements

Observation 3: If service areas were to be introduced specifically for FSAI in SIB21, they should be provided per FSAI and per MBS session. This brings excessively huge signalling overhead for the system information, and the signalling design is unacceptable still due to the SI size limitation concern.

Observation 4: It was agreed that the intended service area for an MBS session may include geographic areas within neighbour cell(s). If the UE is still within the service area and the reselected cell is within the neighbour cell list provided in MCCH, the UE can consider the neighbour cell providing this MBS session.

Proposal 4: Do not introduce service area information specific for FSAI in SIB21.

Proposal 4a: The UE can determine whether to prioritize the frequency in SIB21 for its interested MBS service(s) (based on the existing SIB21 and USD) as per the current procedure in TS 38.304. RAN2 discusses whether any informative text is needed, e.g. to further clarify that UE may take into account whether it is located in the intended service area based on the introduced intended service area for the MBS session in MCCH.

Proposal 5: Do not introduce service area per neighbour cell in MCCH.

Proposal 5a: In addition to the neighbour cell list provided in MCCH, it can be left to UE implementation whether/how the UE considers the intended service area configured in MCCH for the MBS session, when the UE determines whether the reselected cell is providing its interested MBS session at its location. RAN2 discusses whether any informative text is needed in the Spec.

ETWS geo-fencing for NTN

Proposal 6: Introduce a Warning Notification Area in SIB6. RAN2 discusses whether it is feasible to reuse the format of Warning Notification Area for CMAS in SIB8 with a restriction of no segmentation to support ETWS Geo-fencing.

Reference

- RAN2#128 ChairNotes.

- R2-2410862	Support for broadcast services in NR NTN	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

- R2-2409670	Further discussion on support of broadcast service in NR NTN	CATT, CBN	discussion

- 3GPP TS 38.331: "NR: Radio Resource Control (RRC) protocol specification".

- 3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces".

- 3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services".

- S1-244857 	Reply LS on requirements for ETWS primary notification

- 3GPP TS 22.268: "Public Warning System (PWS) requirements".