3GPP TSG-RAN WG2 Meeting #129	R2-2500550

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.0

Source:	Nokia

Title:	UE usage of the RAT restrictions

WID/SID:	ECRATU - Release 19

Document for:	Discussion and Decision

# 1	Introduction

CT1 is approaching RAN2 on UE usage of RAT restrictions with LS C1-247193:

1. Overall Description:

With the work item ECRATU (Enhancement of controlling RAT utilization) CT1 is introducing an optional feature for the network to indicate RAT utilization restriction information to the UE with EMM and 5GMM procedures. The network could further indicate whether a configured RAT restriction is applicable:

- only for the PLMN from which it is received; or

- for the PLMN from which it is received and the equivalent PLMNs.

The following access technologies can be restricted: GERAN, UTRAN, E-UTRAN, satellite E-UTRAN, NG-RAN, satellite NG-RAN. E-UTRAN access technology encompasses all terrestrial E-UTRAN cells. NG-RAN access technology encompasses all terrestrial NG-RAN cells.

The UE will store the received RAT restriction in the list of "PLMNs with associated RAT restrictions", which is stored in non-volatile memory.

The UE shall not consider any PLMN/access technology combination for which a RAT restriction is configured in the list of "PLMNs with associated RAT restrictions" as a candidate PLMN/access technology for PLMN selection purpose.

The above aspects are taken care of in the current or future versions of TS 23.122, TS 24.301 and TS 24.501.

The following aspect is not yet covered in any stage-3 specification and CT1 would like to involve RAN2:

If an access technology is restricted in a PLMN, the UE shall not select any cell of the PLMN deployed in the restricted access technology during cell selection and re-selection for normal service. The restrictions are not applicable if the UE is selecting a cell for limited service.

2. Actions:

To RAN WG2 group.

ACTION:

CT1 kindly asks RAN2 to discuss the aspects not yet covered in any 3GPP specification and check if any changes in their specifications are required and provide feedback to CT1

In this paper we analyze this LS and possible way forward in RAN2..

# 2	Discussion

In XX.304 specifications we have following interactions between AS-NAS defined for PLMN selection:

For PLMN selection there is already a functionality between AS/NAS to limit PLMN to specific RATs and we believe this same functionality can be used for ECRATU without changes in RAN2. For PLMN restrictions it is explicitly written also that restrictions are PLMN specific i.e. it seems to be possible to be based on AS/NAS interactions to realize this requirement from CT1:

- only for the PLMN from which it is received; or

- for the PLMN from which it is received and the equivalent PLMNs.

Observation 1: For PLMN selection there is already a functionality between AS/NAS to limit PLMN to specific RATs

For cell selection we have following interactions in xx.304:

For cell selection there is already a functionality between AS/NAS to limit cell selection to specific RATs and we believe this same functionality can be used for ECRATU without changes in RAN2.

Observation 2: For cell selection there is already a functionality between AS/NAS to limit cell selection to specific RATs within PLMN

It seems that cell selection RAT restrictions actually are limited to selected PLMN only and it won’t be possible to realize these restrictions only to be applicable for (as requested by CT1);

- only for the PLMN from which it is received; or

- for the PLMN from which it is received and the equivalent PLMNs.

Observation 3: For cell selection it does not seem to be possible to have these RAT restrictions based in AS/NAS interactions associated to also EPLMNs.

Then for reselection there is following interactions between AS-NAS:

As can be seen there is no explicitly defined RAT restrictions defined for limiting which RATs are applicable for reselection:

Observation 4: For reselection there is no explicit statement in RAN2 specifications in AS-NAS interactions to limit reselections of cells of specific RAT(s)

As CT1 indicated in the LS they have included PLMN specific aspects already in their specs but:

The following aspect is not yet covered in any stage-3 specification and CT1 would like to involve RAN2:

If an access technology is restricted in a PLMN, the UE shall not select any cell of the PLMN deployed in the restricted access technology during cell selection and re-selection for normal service. The restrictions are not applicable if the UE is selecting a cell for limited service.

With following actions to RAN2:

CT1 kindly asks RAN2 to discuss the aspects not yet covered in any 3GPP specification and check if any changes in their specifications are required and provide feedback to CT1

As shown above we noticed that with current RAN2 specifications it is not possible to cover RAT restrictions for PLMNs (and/or EPLMN(s)) – it should be noted that as shown above the restriction signaling in CT1 so far only covers PLMN specific restrictions:

There seems to be at least following options how this could potentially be realized:

- To put all the TAs of the RAT in forbidden list,

- To send RAT restriction information to AS layer and AS layer can use this info to avoid selection/re-selection

- Make it up to UE implementation

- To disable N1 mode/S1 mode capability.

We assume that there needs to be firstly some signaling at least in NAS to signal restrictions of RATs and their applicability to cell selection/reselection:

Observation 5: in order to have RAT restrictions having any network control one needs to have signaling form NW to UE (we assume NAS signaling) to indicate which RATs are allowed for cell selection/reselection (similarly as already agreed for PLMN case)

C option seem to be basically leaving this functionality for UE implementation which does not seem to be really achieving what CT1 requests to have NW control which RATs are used for cell selection/reselection:

Observation 6: leaving the feature to UE implementation will not achieve CT1 requirements to have network control

D option seems very difficult to realize as CT1 seems to require either limitation to be applicable only to “selected PLMN” and/or EPLMNs – with disabling feature then of course it will always be applicable to all PLMNs

Observation 7:  D option (disable N1/S1 mode) does not work

Option A may be working if it is feasible to have list of all Location/Tracking/Routing areass of each RAT listed  - and LA/TA/RA as not shared between RATs. Additionally, this approach has drawback that UE has to always first reselection/select to the RAT and then finding out that corresponding LA/TA/RA is in the forbidden list.

Observation 8: Option A does not seem very practical as it requires huge lists of LA/TA/RAs and it assumes that LA/TA/RA are not shared between RATs. It also requires UE to first make “wrong” cell selection/reselection to the RAT even though it is restricted before it find it is forbidden.

Thus it seems that easiest and most efficient approach is to add interactions for RAT restrictions for cell selection/reselection to xx.304 specifications (including GERAN):

Proposal 1: Add restrictions to xx.304 specifications e.g. along lines described in Annex A

# 3	Conclusion

Observation 1: For PLMN selection there is already a functionality between AS/NAS to limit PLMN to specific RATs

Observation 2: For cell selection there is already a functionality between AS/NAS to limit cell selection to specific RATs within PLMN

Observation 3: For cell selection it does not seem to be possible to have these RAT restrictions based in AS/NAS interactions associated to also EPLMNs.

Observation 4: For reselection there is no explicit statement in RAN2 specifications in AS-NAS interactions to limit reselections of cells of specific RAT(s)

Observation 5: in order to have RAT restrictions having any network control one needs to have signaling form NW to UE (we assume NAS signaling) to indicate which RATs are allowed for cell selection/reselection (similarly as already agreed for PLMN case)

Observation 6: leaving the feature to UE implementation will not achieve CT1 requirements to have network control

Observation 7:  D option (disable N1/S1 mode) does not work

Observation 8: Option A does not seem very practical as it requires huge lists of LA/TA/RAs and it assumes that LA/TA/RA are not shared between RATs. It also requires UE to first make “wrong” cell selection/reselection to the RAT even though it is restricted before it find it is forbidden.

Proposal 1: Add restrictions to xx.304 specifications e.g. along lines described in Annex A

# Annex A: TP for 38.304 (similar should be added to 36.304 and 25.304 and

## 4.2	Functional division between AS and NAS in RRC_IDLE state and RRC_INACTIVE state

Table 4.2-1 presents the functional division between UE non-access stratum (NAS) and UE access stratum (AS) in RRC_IDLE state and RRC_INACTIVE states. The NAS part is specified in TS 23.122 [9] and the AS part in the present document.

Table 4.2-1: Functional division between AS and NAS in RRC_IDLE state and RRC_INACTIVE state

| RRC_IDLE and RRC_INACTIVE state Process | UE Non-Access Stratum | UE Access Stratum |
| --- | --- | --- |
| PLMN Selection and SNPN Selection | For a UE not operating in SNPN access mode, perform the following: Maintain a list of PLMNs in priority order according to TS 23.122 [9]. Select a PLMN using automatic or manual mode as specified in TS 23.122 [9] and request AS to select a cell belonging to this PLMN. For each PLMN, associated RAT(s) may be set. Evaluate reports of available PLMNs and any associated CAG-IDs from AS for PLMN selection. Maintain a list of equivalent PLMN identities. Maintain a list of "PLMNs not allowed to operate at the present UE location". Maintain applicable disaster roaming information for available PLMNs including potential disaster PLMNs for available PLMNs. To support manual CAG selection, provide request to search for available CAGs and evaluate reports of available CAGs from AS for CAG selection. For a UE operating in SNPN access mode, perform the following: Maintain a list of SNPNs according to TS 23.122 [9]. Select a SNPN using automatic or manual mode as specified in TS 23.122 [9] and request AS to select a cell belonging to this SNPN. Evaluate reports of available SNPNs from AS for SNPN selection. Maintain a list of equivalent SNPN identities. | For a UE not operating in SNPN access mode, search for available PLMNs. If associated RAT(s) is (are) set for the PLMN, search in this (these) RAT(s) and other RAT(s) for that PLMN as specified in TS 23.122 [9]. For a UE operating in SNPN access mode, search for available SNPNs only consider NR cells. Perform measurements to support PLMN/SNPN selection. Synchronise to a broadcast channel to identify found PLMNs/SNPNs. Report available PLMNs and any associated CAG-IDs with associated RAT(s) to NAS on request from NAS or autonomously. Report applicable disaster roaming information for available PLMNs autonomously including potential disaster PLMNs. For a UE operating in SNPN access mode, report available SNPNs to NAS autonomously; report information related to SNPN access with subscription of a different Credentials Holder, indicator whether onboarding is enabled, and the list of supported GINs to NAS autonomously, as specified in TS 38.331 [3]. To support manual CAG selection, perform the following: Search for cells broadcasting a CAG-ID. Read the HRNN (if broadcast) for each CAG-ID if a cell broadcasting a CAG-ID is found. Report CAG-ID(s) of found cell(s) broadcasting a CAG-ID together with the associated manual CAG selection allowed indicator, HRNN and PLMNto NAS. On selection of a CAG by NAS, select any acceptable or suitable cell belonging to the selected CAG and give an indication to NAS that access is possible (for the registration procedure) To support manual SNPN selection, report available SNPNs together with associated HRNNs (if available) to NAS on request from NAS. |
| Cell Selection | Control cell selection for example by indicating RAT(s) associated with the selected PLMN to be used initially in the search of a cell in the cell selection. For each PLMN, associated RAT(s) may be set. Maintain a list of "Forbidden Tracking Areas" and provide the list to AS. Maintain a list of "PLMNs not allowed to operate at the present UE location" and provide the list to AS. For a UE not operating in SNPN access mode: Maintain Allowed CAG list and optional CAG-only indication along with associated PLMN ID(s) on which the UE is allowed access and provide these lists to AS. To support manual CAG selection, select a CAG and request AS to select a cell belonging to this CAG. | Perform measurements needed to support cell selection. Detect and synchronise to a broadcast channel. Receive and handle broadcast information. Forward NAS system information to NAS. Search for a suitable cell. The cells broadcast one or more 'PLMN identity' or 'SNPN identity' (for a UE operating in SNPN access mode) in the system information. Respond to NAS whether such cell is found or not. If associated RATs is (are) set for the PLMN, perform the search in this (these) RAT(s) and other RATs for that PLMN as specified in TS 23.122 [9]. If a cell is found which satisfies cell selection criteria, camp on that cell. |
| Cell Reselection | Control reselection by indicating RAT(s) associated with a PLMN and/or EPLMN(s) to be used for reselection.. For a UE not operating in SNPN access mode, maintain a list of equivalent PLMN identities and provide the list to AS. For a UE operating in SNPN access mode, maintain a list of equivalent SNPN identities and provide the list to AS. Maintain a list of "Forbidden Tracking Areas" and provide the list to AS. Maintain a list of "PLMNs not allowed to operate at the present UE location" and provide the list to AS. For a UE not operating in SNPN access mode, maintain Allowed CAG list and optional CAG-only indication along with associated PLMN ID(s) on which the UE is allowed access and provide these lists to AS. Maintain the network slice(s) and NSAG information and provide network slice(s) and NSAG information to AS. | Perform measurements needed to support cell reselection. Detect and synchronise to a broadcast channel. Receive and handle broadcast information. Forward NAS system information to NAS. Change cell if a more suitable cell is found. Derive cell reselection priorities for slice-based cell reselection. If associated RATs is (are) set for the PLMN for reselection, perform reselection to these RAT(s) for that PLMN as specified in TS 23.122 [9]. |
| Location registration | Register the UE as active after power on. Register the UE's presence in a registration area, for instance regularly or when entering a new tracking area. Deregister UE when shutting down. Maintain a list of "Forbidden Tracking Areas". Maintain a list of "PLMNs not allowed to operate at the present UE location". Control and restrict location registration for a UE in eCall Only Mode. | Report registration area information to NAS. |
| RAN Notification Area Update | Not applicable. | Register the UE's presence in a RAN-based notification area (RNA), periodically or when entering a new RNA. |