3GPP TSG-RAN WG2 Meeting #129													     R2-2500560

Athens, Greece, 17-21 Feb, 2025

Agenda item:	8.3.3

Source:	Apple

Title:	On measurement event prediction

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we discuss the measurement prediction evaluation results, with focus on direct prediction.

Furthermore, considering that the results are rather promising, we provide (high level at this stage) outline of potential standards impact to support measurement event prediction

# 2   	Discussion

## 2.1   	Direct Event Prediction Results

In this contribution we provide our evaluation results for the following scenario:

- Direct measurement event prediction

- Temporal domain prediction in FR1

- Case A, second study goal (i.e. HO KPI improvement)

We realize this is a slight departure from the RAN2 agreed prioritization, however we believe this scenario is the most relevant for practical deployments.

The motivation to focus on FR1 probably needs no explanation – this is the frequency range which is being deployed and therefore this is where AI/ML optimization can bring real benefits. The motivation to focus on case A (and the second study goal) stems from the fact that we’ve already established benefits of case B (and the first study goal) in the context of RRM prediction. For example, we’ve shown in [2] that MRR of 50% (and even lower) are possible with sufficiently high (pending confirmation by SLS) measurement prediction accuracy.

Case A, on the other hand, is not necessarily helpful for RRM prediction, as it is not immediately obvious how case A prediction can be useful for this. In contrast, for measurement event prediction, case A makes the perfect sense as predicting a measurement event ahead of time can be used for HO optimization (as we show below).

Therefore, our motivation here is to “make the case” for case A has, i.e. to show its benefits.

Note: in this contribution we focus on FR1 and case A, as the benefit of case B has already been established (in the context of RRM prediction study item objective), whereas the benefits of case A are still a subject of open study.

The rest of the assumptions we’ve made is in accordance with the RAN2 agreements, specifically:

- A3 event offset: 2dB

- TTT: 320ms

- PW length: 200ms

With regards to the UE behavior modeling after A3 is triggered (which was left for individual company implementation decision), we implemented a simple HO where the UE is handed over to the best cell (i.e. the cell with the strongest RSRP measurement).

For the AI/ML model evaluation, we considered the following models: Logistic Regression, XGBoost, RandomForest, MLP, CNN, Transformer. We’ve used similar models in [2] and [3] (and other contributions), and therefore model description is omitted here for brevity.

In the results provided we used Observation Window (OW) of 800ms. Larger OW sizes we’ve experimented with didn’t provide any visible gains, 800ms appears to be the sweet spot. We followed the cluster-approach (in terms of input measurements), as it has been already established that it provides performance gains. Specifically, we uses measurements from 3 cells as inputs. The model, being a direct prediction model, is illustrated by the Figure 1 below.

Figure 1: Direct measurement event prediction model

We applied 50% probability threshold. As the data obviously is unbalanced (i.e. there are much more “no event” samples that “events”), we were initially concerned that either

We evaluated if the model predicts correctly an A3 event for the correct cell (that would have triggered the A3 event)

- If there's an event in ground-truth (positive case): True prediction means the cell predicted is the triggering neighbor (TP). False otherwise. If it predicted no event, it's false negative (FN). Otherwise, false positive (FP).

- If there's no event in ground-truth (negative case): True prediction means predicted no event (TN). False positive otherwise (FP)

Table 1 below summarizes the AI/ML prediction evaluation results.

| Model | Precision | Recall | F1 |
| --- | --- | --- | --- |
| Logistic Regression | 0.9214 | 0.9289 | 0.9251 |
| XGB | 0.9047 | 0.9837 | 0.9426 |
| RandomForest | 0.9041 | 0.9959 | 0.9478 |
| MLP | 0.9110 | 0.9573 | 0.9336 |
| CNN | 0.9241 | 0.8902 | 0.9068 |
| Transformer | 0.8971 | 0.9539 | 0.9247 |

Table 1: Evaluation Results

The first, and perhaps the most important observation one can immediately draw from these results is that direct measurement event prediction works with very high accuracy. Precision, Recall and F1-score above 0.9 are generally considered very good results. Second, no less important, observation is that there is almost no difference between a very simple model of logistic regression and fairly complex models such as CNN and Transformer – neither in terms of Precision, nor Recall, nor F1-score. This essentially indicates that direct measurement event prediction at the agreed simulation parameters is a very simple prediction task.

Table 2 below shows the model complexity.

| Model | Configuration | # of parameters (n) | Storage complexity | Computational complexity |
| --- | --- | --- | --- | --- |
| Logistic Regression | 9 input samples | O(W*NumNeighbors) | 120 B | ~60 FLOPs |
| XGB | 9 input samples Num estimators (t= 100) Max depth, d = 5 | O(t*2^d) | 25 KB | ~5k FLOPs |
| RandomForest | 9 input samples Num estimators ( t= 100) | O(2^d) | 25 KB | ~16k FLOPs |
| MLP | (128,64,3) | O(W*h1+h1*h2+h2) | 49 KB | ~19k FLOPs |
| CNN | 2 x Conv1D layers: Filters: 16, KernelSize 2 1 Dense layer: size 16x3 | O((W × KernelSize × NumNeighbors) | 3.5 KB | ~5.5k FLOPs |
| Transformer | 4 heads FFNN dim 64 | O((W x NumHeads × NumNeighbors) | 4.1 KB | ~1.7k FLOps |

Table 2: Model Complexity

The complexity calculations are essentially the same as what we’ve reported previously in [2] and [3] and so detailed explanation of how these calculations have been made are omitted for brevity. Naturally, the logistic regression model is by far the simplest one, which (as we’ve shown above) provides as good performance as a significantly more complex models such as CNN and Transformer.

## 2.2 	Key Observations

In this section we summarize the key findings from our performance evaluation submission with the intention of capturing these in the TR 38.744 [4].

First off, we’ve established that case A (and the second study goal of performance optimization) has benefits – at least for the scenario of measurement event prediction in FR1. As this is a fairly common scenario of interest, this finding needs to be captured in the TR (which otherwise currently emphasizes case B and the first study goal only)

Observation 0: case A (and the second study goal of performance optimization) has benefits for measurement event prediction.

As we’ve shown, not only direct prediction works, but it does so with very high precision, recall and accuracy. As a side note, we believe similar results would be achieved for indirect prediction as well – simply because, as it turns out, measurement event prediction (under the current study assumptions) is a fairly simple prediction task.

Observation 1: direct measurement prediction works well with very high (above 0.9) Precision, Recall and F1-score values.

Observation 2: even very simple models (such as logistic regression) provide good results at very low complexity.

We’ve established that OW of 800ms is the sweet spot and increasing OW beyond that doesn’t yield observable gains. This may not have direct impacts on the normative work, as the OW is likely to be left for implementation, but it may need to be documented in the TR 38.744 [4] nevertheless.

Observation 3: 800ms OW is the sweet spot, i.e. larger OW does not provide visible performance gains.

We’ve already established the benefits of the cluster approach in the RRM prediction case, and the measurement event prediction is no exception. Using measurements from multiple cells increases performances, but going above 3 cells is not necessary.

Observation 4: 3 cell input measurements is the sweet spot, i.e. input measurements from more cell don’t provide visible performance gains.

Contrary to intuition (but probably because this is a simple prediction model), the fact that we are operating with an unbalanced data set (i.e. much more “no event” samples and “event” samples), this poised no problems for the prediction algorithm. 50% probability threshold (which is usually recommended for balanced sets) worked well and methods such as down-sampling (to balance out the data set) which we’ve tried did not increase the accuracy (probably because the accuracy is very high anyway).

Observation 5: 50% probability threshold works well (for direct prediction).

Observation 6: no down sampling is needed (e.g. to balance the yes/no classification data).

We therefore propose to capture these observations in the TR 38.744 [4].

Proposal 1: we propose to capture the above observations in the TR 38.744, along with the conclusion that direct measurement event prediction works well and therefore should be considered for normative work.

Having established that measurement event prediction works with high accuracy and low complexity, we now proceed to discuss potential standards impacts to realize these gains.

## 2.3   	Potential Standards Impacts

Considering the most important observation from the previous section, that is – that UE-based event prediction works with reasonably high accuracy and low complexity, we would like to discuss the standards impact to define functionality exploiting such AI/ML UE capability.

As we have shown, a UE can rather easily predict a measurement event (e.g. A3) at least 400ms ahead of the event occurrence. In our evaluation, we focused on case A, where the intention is to improve handover performance. Admittedly, in the results submitted to this meeting we evaluated case A in FR1 – this is because in our view FR1 is the most important frequency range (in terms of practical deployments) by far. Regardless of whether the results are for FR1 or FR2, the point is that if prediction works at least in some (important) use cases, it is worth progressing the related functionality to the normative phase.

As a side note, we think that case B also has merits (in the context of event prediction, as it does in the context of RRM prediction), but in this contribution we focus on case A.

Observation 7: in this section we focus on standards impacts in support of case A with the task of HO performance optimization.

To analyze where performance gains can be realized, we refer to the following high level schematic call flow of a handover process and signalling in Figure x. We use Xn handover for illustration, with the understanding that the same applies to N2 handover as well – where, as we show below, the gains a potentially even bigger.

Figure 2: high level Xn HO call flow

This figure is well familiar and needs no explanation. We’d just note that it can be roughly divided into three parts:

- Measurements

- HO preparation (steps 2, 3 and 4)

- HO execution

In this section we focus on potential optimizations of the second part – HO preparation. According to field data, the delay between a UE reporting A3 till the UE gets ReconfigurationWithSync from the networks can be as high as 50ms. Granted, HO preparation doesn’t always takes 50ms – but in some cases it does. We have no visibility into the reasons for such large delay, it can be for example high latency backhaul or some other reason. Regardless of the cause for such delay, as we show below, AI/ML prediction can help to eliminate it entirely – with minimal cost to UEs and the network.

Observation 8: in real networks, HO preparation can take up as much as 50ms.

This wait time normally does not result in HO failure, as very few UEs move fast enough to go out of coverage pf the serving cell in 50ms, but even if the HO is successful, the relatively long wait in suboptimal radio conditions results in performance degradation, which is sometimes visible to the user.

Observation 9: 50ms wait in suboptimal radio conditions rarely results in HO failure but does affect performance.

With this in mind, we propose a simple HO enhancement (as illustrated by the Figure X below) which allows to eliminate the 50ms handover preparation wait time. The (minor) differences compared to the legacy flow are shown with “double arrows”.

Figure 3: AI/ML enhanced Xn HO

The enhancements are as follows:

- The UE performs measurements and also runs AI/ML measurement event prediction

- When an event is predicted, it is reported to the network (with the indication that it’s a predicted event)

- At this point the network (up to the network discretion) may start HO preparation (towards the predicted cell)

- The UE continues performing measurements and when the real measurement event is triggered (based on the real measurements) the UE reports it to the network (as in legacy)

- At this point, if the network has already done HO preparation towards the correct cell it can send ReconfigurationWithSync immediately

Now let’s consider the following scenarios:

- Successful measurement event prediction

- Wrong measurement event prediction

- Missed measurement event prediction

In the first case of the UE successfully predicting the correct A3 event (with the correct target cell), the HO preparation is triggered ahead of time (the network doesn’t need to do so 400ms in advance, as might be possible, but hopefully it would be somewhere between 100ms to 50ms). The HO then succeeds, and the UE spends 50ms less in suboptimal radio conditions.

Now let’s consider the rare case of a UE predicting a wrong A3. This may result in one unnecessary HO preparation which indeed would waste a tiny bit of network resources (in a single cell) for a very short period of time (of a few hundreds ms). After which the UE would detect the real A3 event based on real measurements and the legacy procedure would kick in.

Finally, if the UE prediction algorithm misses an event – nothing happens bad, as we simply fall back to the legacy procedure (i.e. HO based on real measurement event only without a prediction).

Proposal 2: consider measurement event reporting enhancements to allow the UE to report predicted events to the network, so that the network may start HO preparation ahead of time.

It is important stress that with this optimization the actual HO is always based on the actual measurement event – predicted measurement events are only used for HO preparation. In particular, this means that it doesn’t affect RAN4 requirements.

Observation 10: it is important to point out that in the proposed enhancement the HO is triggered based on real measurements (only the preparation is triggered by prediction) and therefore this does not affect RAN4 requirements and therefore does not need RAN4 TU.

Finally, on the topic of SLS, which is important to assess the gains of case B optimizations, for this particular optimization SLS (under the assumption that we don’t simulate traffic, which we’ve already agreed on) is not essential. This is because, as pointed out already, this optimization focuses on HO preparation (and not the HO decision itself) with the intention of optimizing performance (i.e. throughput and latency) during the HO preparation process.

# 3	Observations and Proposals

Observation 0: case A (and the second study goal of performance optimization) has benefits for measurement event prediction.

Observation 1: direct measurement prediction works well with very high (above 0.9) Precision, Recall and F1-score values.

Observation 2: even very simple models (such as logistic regression) provide good results at very low complexity.

Observation 3: 800ms OW is the sweet spot, i.e. larger OW does not provide visible performance gains.

Observation 4: 3 cell input measurements is the sweet spot, i.e. input measurements from more cell don’t provide visible performance gains.

Observation 5: 50% probability threshold works well (for direct prediction).

Observation 6: no down sampling is needed (e.g. to balance the yes/no classification data).

Proposal 1: we propose to capture the above observations in the TR 38.744, along with the conclusion that direct measurement event prediction works well and therefore should be considered for normative work.

Observation 7: in this section we focus on standards impacts in support of case A with the task of HO performance optimization.

Observation 8: in real networks, HO preparation can take up as much as 50ms.

Observation 9: 50ms wait in suboptimal radio conditions rarely results in HO failure but does affect performance.

Proposal 2: consider measurement event reporting enhancements to allow the UE to report predicted events to the network, so that the network may start HO preparation ahead of time.

Observation 10: it is important to point out that in the proposed enhancement the HO is triggered based on real measurements (only the preparation is triggered by prediction) and therefore this does not affect RAN4 requirements and therefore does not need RAN4 TU.

# 4	References

[1]				RAN2#128 (and previous) meeting minutes

[2]				R2-2409201, RRM measurement prediction results using field and simulated data, Apple

[3]				R2-2406664, RRM measurement prediction results for case B, Apple

[4]				TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR