3GPP TSG-RAN WG2 Meeting #129	R2-2500566

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.6.4

Source: 	China Telecom

Title:	Discussion on conditional intra-CU LTM

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

- Introduction

In last RAN2 meeting, the Rel-19 WI of Mobility enhancement was widely discussed and much progress has been made. Regarding conditional intra-CU LTM, the following agreements were reached.

Agreements on C-LTM:

- The triggering condition of conditional LTM can be based on L3 measurement.

- CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution.

- The L1 execution condition of a candidate cell is associated to only one triggering event.

- For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell.

- To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition:

- The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

- The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

- The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

- For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

- The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

- Candidate cell TA is maintained by a new timer.

- For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

In this contribution, we would like to share our opinions on remaining issues on conditional Intra-CU LTM in non-DC case.

- Discussion

UE evaluated conditions for executing a conditional LTM

In previous RAN2 meetings, the event-triggered L1 measurement reporting was widely discussed and much progress has been made, which can provide guidance to UE evaluated conditions for executing a conditional LTM.

In RAN2#126 meeting, the following beam-level events for L1 LTM measurement reporting were introduced.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

In RAN2#127bis meeting, it is agreed that Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. The detailed definition of events for conditional LTM has not been discussed.

For conditional LTM, it is common understanding that the triggering event shall be in beam-level as well. For conditional LTM cell switch, it is also very important to avoid “Ping Pong” or unnecessary cell switch, which may reduce the mobility performance and user experience. Therefore, not only the quality of the candidate beam but also the quality of the serving beam shall be considered when evaluating whether to execute a conditional LTM. Similar to CHO, the following conditional events could be introduced for conditional LTM.

- CondEvent LTM3: Beam of conditional candidate cell becomes amount of offset better than beam of serving cell;

- CondEvent LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of conditional candidate cell becomes better than another absolute threshold2.

Proposal 1: RAN2 to consider the following beam-level conditional events in order to support conditional LTM in non-DC case.

- CondEvent LTM3: Beam of conditional candidate cell becomes amount of offset better than beam of serving cell;

- CondEvent LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of conditional candidate cell becomes better than another absolute threshold2.

For event-triggered L1 measurement reporting, the following agreements were reached on beam of the serving cell and candidate cell used for event evaluation.

Agreements on L1 MR event evaluation

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

Considering the conditional LTM is a kind of cell switch procedure, which is not similar to L1 measurement reporting, to follow the same principle on event evaluation may lead to more frequent cell switch. Thus, the serving beam and candidate beam(s) used for event evaluation in above conditional events might need further discussion. Potential options are listed as below.

- Option 1: all the configured beams of the serving cell or candidate cell;

- Option 2: a certain number of beams of the serving cell or candidate cell, e.g. the best X beams or the worst Y beams;

- Option 3: a single beam of the serving cell or candidate cell, e.g. the best beam, the worst beam or any beam.

For Option 1, it seems too strict to trigger conditional LTM only when all the configured candidate beams are good enough while the serving beams are extremely bad. It may take a long time to meet the trigger conditions, which may lead to cell switch failure or reduce the user’s experience. For Option 3, only a single beam is used for evaluating the trigger conditions, which may lead to more frequent or unnecessary conditional LTM cell switch. Option 2 could be a trade-off solution, which can reduce the evaluation time before triggering conditional LTM as well as guarantee the UE to select a target cell with good enough beams. Only when a certain number of beams of the serving cell or conditional candidate cell meet the trigger conditions of conditional events, the conditional LTM execution could be triggered.

Proposal 2: Regarding evaluation for the execution of conditional LTM, a certain number of beams of the serving cell or conditional candidate cell, e.g. the best X beams or the worst Y beams, could be used.

RACH-less C-LTM

In RAN2#127bis meeting, it was agreed to support CG-based RACH-less C-LTM. In RACH-less C-LTM with CG, it is up to the UE to select a beam for first UL transmission on target cell. In order to guarantee the handover performance, it is meaningful to further discuss the beam selection issue in RACH-less C-LTM with CG. Firstly, upon the execution of C-LTM, there is at least one beam that satisfies the conditional events. The UE shall always select a beam that satisfies the conditions, otherwise the first UL transmission might be failed due to the selection of a beam not satisfying the execution conditions. Moreover, it was agreed to re-use the Candidate Cell TCI States Activation/Deactivation MAC CE for the early activation/deactivation of TCI state(s) of a C-LTM candidate configuration in last RAN2 meeting. If there are more than one beams satisfying the execution conditions for C-LTM, the UE shall select a beam associated with activated TCI state if any. If there are multiple beams satisfying the C-LTM execution conditions as well as associated with activated TCI state, it is up to UE implementation to select a beam.

Proposal 3: For beam selection in RACH-less C-LTM with CG, the UE shall always select a beam that satisfies the C-LTM execution conditions.

Proposal 4: If there are more than one beams satisfying the C-LTM execution conditions, the UE shall select a beam associated with activated TCI state if any.

Proposal 5: If there are multiple beams satisfying the C-LTM execution conditions as well as associated with activated TCI state, it is up to UE implementation to select a beam.

Another open issue on RACH-less C-LTM is whether to support RACH-less C-LTM with DG. From NW’s perspective, DG-based RACH-less C-LTM provides more flexibility on the scheduling of radio resources to the NW. However, unlike DG-based RACH-less LTM, the NW has no clear awareness of the beam selected by the UE. It is hard for the NW to schedule the UE dynamically if DG-based RACH-less C-LTM is supported without any enhancement. The UE may need to send a Bye message to the source cell upon execution of C-LTM, indicating the information of the selected target cell. The source cell then provides the received information to the target cell. Introducing a Bye message will bring additional complexity to both the UE and the NW. Besides, if the NW fails to receive the Bye message due to poor channel quality on the source cell, DG-based RACH-less C-LTM might be failed. Considering DG-based RACH-less C-LTM is a kind of enhancement feature, we suggest not support DG-based RACH-less C-LTM in this release and we can further discuss it in future release.

Proposal 6: RACH-less C-LTM with DG is not supported in Rel-19.

Co-existence of LTM and C-LTM

As known, LTM is introduced to reduce HO latency and C-LTM is introduced to increase HO robustness. It is meaningful to support the co-existence of LTM and C-LTM to guarantee the HO performance in different aspects.

Proposal 7: Co-existence of LTM and C-LTM is supported.

Regarding the configuration of candidate cells for LTM and C-LTM, the following options could be discussed.

- Option 1: Re-use the LTM candidate configuration for both LTM and C-LTM

- Option 2: Use separate candidate configurations for LTM and C-LTM

Both of the above options can work. The conditional LTM preparation is similar to LTM apart from the preparation of execution conditions for C-LTM. Re-using the LTM-config for C-LTM can reduce the spec impact, which could be used as the baseline. The stage-3 details can be further discussed in CR drafting.

Proposal 8: Re-use the LTM-Candidate IE for the configuration of C-LTM, and further discuss the details in stage-3 CR drafting.

- Conclusion

In this contribution, we discuss the remaining issues on conditional intra-CU LTM in non-DC case. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: RAN2 to consider the following beam-level conditional events in order to support conditional LTM in non-DC case.

- CondEvent LTM3: Beam of conditional candidate cell becomes amount of offset better than beam of serving cell;

- CondEvent LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of conditional candidate cell becomes better than another absolute threshold2.

Proposal 2: Regarding evaluation for the execution of conditional LTM, a certain number of beams of the serving cell or conditional candidate cell, e.g. the best X beams or the worst Y beams, could be used.

Proposal 3: For beam selection in RACH-less C-LTM with CG, the UE shall always select a beam that satisfies the C-LTM execution conditions.

Proposal 4: If there are more than one beams satisfying the C-LTM execution conditions, the UE shall select a beam associated with activated TCI state if any.

Proposal 5: If there are multiple beams satisfying the C-LTM execution conditions as well as associated with activated TCI state, it is up to UE implementation to select a beam.

Proposal 6: RACH-less C-LTM with DG is not supported in Rel-19.

Proposal 7: Co-existence of LTM and C-LTM is supported.

Proposal 8: Re-use the LTM-Candidate IE for the configuration of C-LTM, and further discuss the details in stage-3 CR drafting.

- Reference

RP-240299	Revised Work Item: NR mobility enhancements Phase 4

RP-24xxxxx	draft Report of 3GPP TSG RAN WG1 meeting #116bis

3GPP TS 38.331 18.0.0: "NR; Radio Resource Control (RRC) protocol specification".

R2-2403731	Report from session on V2X/SL, R19 NES and MOB