3GPP TSG-RAN WG2 Meeting #129	           R2-2500568

Athens, Greece, Feb 17th – Feb 21st, 2025

Agenda item:	8.1.2.2

Source:	China Telecom

Title:	LCM for UE-sided model for BM use case

Document for:	Discussion

- Introduction

During RAN2#127 meeting, RAN2 sent an LS to RAN1 to inform the agreements on applicable functionality reporting for beam management UE-sided model and ask RAN1 some questions related to applicable functionality reporting and activation. RAN1 has completed the reply LS R1-2410898[1] during RAN1#119 meeting. In this paper, we would like to further discuss applicable functionality reporting procedure and signaling based on the reply LS and data collection for the UE-sided model training.

- Discussion

2.1	Applicable functionality reporting

The general applicable functionality report procedure for BM cases identified by RAN2 in the LS as the figure 1 below.

Figure 1

At the RAN1#119 meeting, RAN1 has agreed to the following agreements regarding applicability reporting.

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related informatio - Set B related information - Report content related information / - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction

According to the configuration design in RAN1, the applicability report is based on one or more of CSI-ReportConfig and/or sets of inference related parameters. Considering that the above design may contain a large number of IEs, the level of signaling overheads may become an important dimension in applicability reporting design. Therefore, the delta design of applicability reporting, which is to report the functionalities from non-applicable to applicable and from applicable to non-applicable, can effectively reduce signaling overheads. The explicit non-applicable functionality report can be supported as a baseline. In addition, the explicit report can contain the reasons for non-applicable functionalities, which may be useful for NW to convert the status of a functionality from non-applicable to applicable.

Proposal 1: When there is a change of applicability, UE can support applicable functionality report with delta design, which reports non-applicable functionality and the reason explicitly.

When reporting applicable functionality, a prohibit timer should be set by NW to avoid frequent reports. In practical scenarios, there are some cases in which the state of the UE's applicable functionality changes rapidly; for example, when an application is started, the CPU occupancy and memory occupancy may increase significantly and then return to the average level when the loading of this application is completed. In those cases where the computational and storage resources of the UE may be suddenly high and suddenly low, if the prohibition timer is not activated, the UE may report the applicable functionalities frequently, wasting signaling resources.

Proposal 2: When reporting applicable functionality, a prohibit timer can be set by NW to avoid frequent reports.

2.2	Data collection for the UE-sided model training

At the RAN2#128 meeting [2], RAN2 has the following agreement regarding the request of data collection for the UE-sided model training.

Agreements: 7. For data collection configuration UE-side model training, the UE can send a request for data collection. FFS what the request contains.

Considering that UE needs to uniquely identify the functionalities/models for which data collection is desired, the request needs to contain the functionality/model ID. Furthermore, the corresponding radio resource configuration and associated ID may be carried. The radio resource configuration, such as reference signal configurations and Set A/B beam resource configurations, is used for UE measurements, and the associated ID for the UE-reported functionality/model, is used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model.

Proposal 3: When UE requests the data collection for UE-sided model training, the request may include functionality/model ID, radio resource configuration, and associated ID.

RAN2 has the following agreement regarding the network control of data collection for the UE-sided model training.

RAN2#127bis: 4. Data collection initiation and configuration for data collection is under network control. FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server) RAN2#128: 6. For BM use case for UE-side model, data collection related configuration(s) (e.g., measurement resources configuration) and associated ID(s) can be included in training data collection configuration. 9. The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection. - The network can decide whether to accept UE’s request for data collection. 10. FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration.

As indicated by the agreements, it is up to the network to configure the data collection for UE-sided model training. The data collection configuration may contain CSI-RS resource configuration for measurement for the UE-sided model training and the associated ID. It is possible that the UE can’t perform data collection based on the configuration from the network. The network may decide to stop the data collection or reconfigure the data collection for UE-sided model training based on the indication from the UE, in order to avoid the waste of radio resource or inefficiently model training.

Proposal 4: An indication from UE to network is needed when UE can’t perform data collection based on received configuration.

3	Conclusions

Based on the discussion above, we have the following proposals.

Proposal 1: When there is a change of applicability, UE can support applicable functionality report with delta design, which reports non-applicable functionality and the reason explicitly.

Proposal 2: When reporting applicable functionality, a prohibit timer can be set by NW to avoid frequent reports.

Proposal 3: When UE requests the data collection for UE-sided model training, the request may include functionality/model ID, radio resource configuration, and associated ID.

Proposal 4: An indication from UE to network is needed when UE can’t perform data collection based on received configuration.

4	References

- [1] R1-2410898, Reply LS on applicable functionality reporting for beam management UE-sided model, TSG RAN WG1.

- [2] Report of 3GPP TSG RAN WG2 meeting #128, Orlando, US.