3GPP TSG-RAN WG2 Meeting #129                               R2-2500573

Athens, Greece, Feb. 17th – 21st, 2025

Source:	China Telecom

Title:	Considerations on A-IoT data transmission and other general aspects

Agenda Item:	8.2.4

Document for:	Discussion and Decision

- Introduction

Based on the previous RAN2 progress, the related agreements about A-IoT functionalities and other general aspects were achieved in the following:

- RAN2 assumes that RRC layer is not necessary between the reader and the device.   RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC

- FFS whether further indication of device message size/status is needed

- At least the following information are considered useful to be visible to the reader from CN

- -The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful

- -targeted for one or more than one devices;

- -approximate number of target devices (if available).

- FFS on mandatory/optional

- RAN2 assumes that if “AS ID” is defined it is used at least for purpose of D2R scheduling and R2D reception.  Up to RAN1 to decide whether a “AS ID” is defined.

- RAN2 assumes this “AS ID” should be a short AS layer ID, rather than the full upper layer device ID.  FFS on the length.   FFS if AS ID can be based on partial upper layer device ID.

- From RAN2 perspective, following options are possible for “AS ID”:

- Option 1: a random ID if used in Msg1 can be reused;

- Option 2: reader assigns this “AS ID”. FFS via which R2D message.

- From RAN2 perspective, it is beneficial for the reader to know an estimate of expected D2R message size.  Two options can be captured: 1) from the CN and 2) from the device (simple message size indication).    Capture the advantages/disadvantages.

- The D2R message size would be beneficial but it is not essential.

- Ask SA2 if it is possible to provide the expected (e.g. approximate, estimate, exact, max) future response D2R message size.  Is it always available, sometimes, or never.

- RAN2 will study segmentation and RAN2 will need to give some guidance once there is more clarity on solution, on the minimum TB size.

- Study the functionality of segmentation in the MAC.   We focus on D2R and can consider the support for R2D later depending on SA2 and RAN1.   The solution should aim to be applicable to both direction (i.e. unified solution), if we decide to support on both directions.

- Do not support sequence number for A-IoT segmentation functionalities.

- Do not support segment number and the number of segments for A-IoT segmentation functionalities.

- Capture the following option in TR: An indication is used to indicate to reader whether the data is segmented and whether it is last segment.    FFS the details on how this is designed and number of bits (one or two).

- RAN2 assumes that for the device we will not support AS layer buffering for A-IoT segmentation functionalities, i.e., all buffered segment(s) are stored in upper layer(s).

- Capture the option that it is up to the reader whether to use the random ID as AS ID or assigns a new AS ID.   FFS what message is used

- Segmentation indication will be discussed in WI phase.

- If segmentation is supported, it is beneficial for the reader to be able to trigger a re-transmission of a segment.  FFS how this is done.

- The reader needs to know whether a response is expected and expected D2R message size.   FFS how the reader gets this information.  Wait for SA2 LS response on expected D2R message size to resolve the FFS.    FFS if command type is explicit, if it can be inferred from expected D2R message size if available.

According to the scopes of agenda, this contribution will focus on MAC PDU/signaling format,, segmentation for D2R, AS ID etc.

- Discussion

2.1 MAC PDU format design

For A-IoT, RAN2 assumes that there is no other L2 AS layer except for the A-IoT MAC layer. Regarding the A-IoT MAC layer, it needs to support the functionalities including paging, random access and data transmission. The legacy NR MAC PDU structure can be referenced as follows [1] and adapted with modifications and simplifications tailored for the A-IoT scenario.

TS 38.321

A MAC PDU consists of one or more MAC subPDUs. Each MAC subPDU consists of one of the following:

-	A MAC subheader only (including padding);

-	A MAC subheader and a MAC SDU;

-	A MAC subheader and a MAC CE;

-	A MAC subheader and padding.

The MAC SDUs are of variable sizes.

Each MAC subheader corresponds to either a MAC SDU, a MAC CE, or padding.

As for the A-IoT MAC PDU, it may consist of one or more MAC subPDUs. Considering that it does not support multiple A-IoT logical channels for upper layer data, there may be two basic structures for the MAC subPDU, namely MAC subheader and MAC message, or MAC massage only.  For the MAC subheader, it may consist of an L field to indicate the variable size of the MAC message. To differentiate the types of payload (e.g., paging, RAR, or feedback indication), an LCID-like field may be needed in the MAC subPDU. Moreover, the MAC message can be flexibly combined with A-IoT MAC CE and A-IoT MAC SDU. There are several feasible combination methods as follows: A-IoT MAC CE, or A-IoT MAC SDU, or A-IoT MAC CE+A-IoT MAC SDU, or A-IoT MAC CE1+A-IoT MAC CE2 etc. The detailed type of MAC message needs to be indicated in front of each MAC subPDU. If the MAC message is an A-IoT MAC CE and its length is fixed, the L field may not be required.

- A-IoT MAC subPDU structure can be either MAC subheader+MAC message or MAC message only.

- A-IoT MAC subheader may consist of following fields:

- LCID-like IE, to differentiate payload types (e.g., paging, RAR).

- L field, to indicate the variable size of MAC message.

- A-IoT MAC message may consist of A-IoT MAC SDU, or A-IoT MAC CE, or A-IoT MAC CE+A-IoT MAC SDU, or A-IoT MAC CE1+A-IoT MAC CE2.

- A message type indication is required in front of the MAC subPDU.

2.2 Segmentation

2.2.1 	Segmentation indication

As captured in the TR [2], “An indication is used to indicate to the reader on whether the data is segmented and whether the MAC PDU is the last segment”. The size of this indication (one or two bits) and the corresponding details require further discussion. Based on our understanding, it is possible to distinguish whether a message is segmented by the presence of a 1-bit indicator. If this indicator is present, it signifies that the message is segmented; otherwise, it is not. Furthermore, different values of this indicator can be used to differentiate whether the segmented message is the last segment. For instance, a value of 0 indicates that the message is segmented but not the last segment, while a value of 1 denotes that it is the last segment.

- 1-bit segmentation indication is supported and details are as follow:

- segment but not the last segment, if the indication is equal to ‘0’;

- last segment, if the indication is equal to ‘1’.

2.2.2 	Feedback procedure

Since the sequence number, the segment number and the number of segments are not supported, an acknowledgement feedback mechanism is required to ensure that segments are received in order and to prevent segments loss. Additionally, due to the A-IoT device will not support AS layer buffering, the content of acknowledgement may depend on the device’s implementation regarding how to determine the start position of the segment.

Solution 1: if the device uses two pointers to indicate the starting position of the current transmission and the estimated starting position of the next transmission, the feedback message can be an explicit ACK/NACK. The specific process is as follows:

- Initialization:

Pointers ptr_1 and ptr_2 both initially point to the starting position of the entire data to be transmitted.

- Sending the first segment:

The device sends the first segment of data with x bits;

Pointer ptr_1 remains unchanged, pointing to the starting position of the entire data.

Pointer ptr_2 points to the estimated starting position of the next transmission.

- Sending the remaining segment:

When the device receives a success indication (ACK) from reader:

The device sends the second segment of data starting from the position indicated by pointer ptr_2.

Pointer ptr_1 is updated to the starting position of the current transmission.

Pointer ptr_2 points to the estimated starting position of the next transmission.

When the device receives a failure indication (NACK) from reader:

The device re-transmits the first segment of data starting from the position indicated by pointer ptr_1.

Pointer ptr_1 remains unchanged.

Pointer ptr_2 points to the estimated starting position of the next transmission.

Figure 1 Segmentation Solution 1

Solution 2: if the device uses a single pointer to indicate the starting position of the current transmission, the feedback message needs to include the amount of data successfully transmitted in the previous transmission. Process is as follows:

- Initialization:

Pointer ptr initially points to the starting position of the entire data to be transmitted.

- Sending the first segment:

The device sends the first segment of data with x bits.

- Sending the remaining segment:

The device receives the amount of data successfully transmitted in the previous transmission, e.g., x bits. If x is equal to ‘0’, it means the previous transmission is failure.

The device calculates the new position of the current transmission based on the ‘x’. The pointer ptr updates to the new position.

The device transmits the current transmission with the position indicated by ptr.

Figure 2 Segmentation Solution 2

- Feedback mechanism is necessary to ensure in-order delivery without any missing segment. Following solutions can be considered:

- Solution 1: explicit ACK/NACK indication.

- Solution 2: implicit indication by indicating the amount of data successfully transmitted.

2.3 AS ID for A-IoT transmission

Based on the TR, it is shown that the AS ID is essential for D2R scheduling and R2D reception. How to allocate the AS ID needs to be further discussed during the WI phase. Currently, we think there are several options for us to discuss as follows:

Option 1: Reuse the 16 - bit random ID in Msg1 as the AS ID.

Option 2: The reader assigns an AS ID to the device. This method may occupy fewer bits.

Option 3: The index of the Msg1 time-frequency can be used as the AS ID, and the valid duration of the AS ID is within this access occasion.

In our opinion, the above methods are all feasible, and Option 2 may be more flexible. The detailed R2D message for allocating the AS ID needs to be further discussed.

- RAN2 needs to define the AS ID. Among various methods, the approach of the reader assigning the AS ID is more flexible.

- Conclusion

This contribution makes the following proposals:

- A-IoT MAC subPDU structure can be either MAC subheader+MAC message or MAC message only.

- A-IoT MAC subheader may consists of following fields:

- LCID-like IE, to differentiate payload types (e.g., paging, RAR).

- L field, to indicate the variable size of MAC message.

- A-IoT MAC message may consists of A-IoT MAC SDU, or A-IoT MAC CE, or A-IoT MAC CE+A-IoT MAC SDU, or A-IoT MAC CE1+A-IoT MAC CE2.

- A message type indication is required in front of the MAC subPDU.

- 1-bit segmentation indication is supported and details are as follow:

- segment but not the last segment, if the indication is equal to ‘0’;

- last segment, if the indication is equal to ‘1’.

- Proposal 6.  Feedback mechanism is necessary to ensure in-order delivery without any missing segment. Following solutions can be considered:

- Solution 1: explicit ACK/NACK indication.

- Solution 2: implicit indication by indicating the amount of data successfully transmitted.

- Proposal 7.  RAN2 needs to define the AS ID. Among various methods, the approach of the reader assigning the AS ID is more flexible.

# References

- 3GPP TS 38.321 Medium Access Control (MAC) protocol specification (Release 18).

- 3GPP TR 38.769 Study on solutions for ambient IoT (Internet of Things) (Release 19).