3GPP TSG-RAN WG2 #129	R2-2500611

Athens, GR, Feb 17th – 21st, 2025

Agenda item:	   8.1.2.2

Source:		Lenovo

Title:	LCM for UE-sided model in beam management

Document for:		Discussion and Decision

# Introduction

In this paper, we discuss LCM issues related to UE-sided model for beam management use case.

# Discussion

## Configuration for training data collection

RAN2#128 Agreement 6. For BM use case for UE-side model, data collection related configuration(s) (e.g., measurement resources configuration) and associated ID(s) can be included in training data collection configuration. 7. For data collection configuration UE-side model training, the UE can send a request for data collection. FFS what the request contains. 8. The network can provide the data collection configuration (at any point in time), with or without UE request. 9. The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration . - The network can configure whether UE is allowed to initiate request for data collection. 10. FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration

In the previous RAN2 meeting, it was agreed that UE can request NW for data collection configuration if allowed by the NW. One question worth RAN2 clarification is if all UEs or only limited UEs can request the data collection configuration, for instance:

- All UEs

- UEs that are AI capable (as indicated in UE capability report)

- UEs that are authorized by NW (as part of UE context)

RAN2 is suggested to clarify which of the following UEs can request the data collection configuration, if allowed by NW

  - All UEs

  - UEs that are AI capable (as indicated in UE capability report)

  - UEs that are authorized by NW (as part of UE context)

The next question is which signalling can be used by UE to request the data collection configuration. In our view, UAI would be the most straightforward option since this is triggered based on UE’s own demand and not urgent. When requesting the data collection configuration, UE will indicate the measurement resources configuration and associated ID of its interest. Correspondingly, NW can indicate in the OtherConfig if UE is allowed to request the data collection configuration.UE requests data collection configuration via UAI if allowed by gNB as configured in the OtherConfig.

In the UAI requesting data collection configuration, UE indicates the measurement resources configuration and associated ID of interest.

Following RAN1 discussion, if the CSI configuration framework can be used to configure both AI training data collection and AI inference, from RRC signalling design point of view, the purpose (i.e., if it is for AI training data collection or for AI inference) has to be clearly indicated to align the behaviour of UE and gNB. For example, if the set A/B beam resource configuration is for AI training data collection, UE will expect gNB to send reference signals on all set A beams for UE measurement.

If the CSI configuration framework is used for both training data collection configuration and inference configuration, in RRC configuration message design, it has to be clearly distinguished if the provided configuration is for training data collection or for inference.

## RAN1 LS (R1-2410898) related issues

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig - Any specification impact is a separate discussion

As replied in RAN1 LS, RAN1 considers the following two approaches for gNB to provide the applicable inference configuration, which can be summarized as following in our understanding

- A) in step 3, via RRCReconfiguration, gNB provides one or more of CSI-ReportConfig for inference configuration; in step 4, via RRCReconfigurationComplete, UE indicates the applicable CSI-ReportConfig; after step 4,

  - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW.

  - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

- B) in step 3, via RRCReconfigurationàOtherConfig, gNB provides candidate sets of inference related parameters; in step 4, via UAI, UE indicates the applicable ones among the candidate sets of parameters; in step 5, accordingly, via RRCReconfiguration, gNB will generate and provide the CSI-ReportConfig for inference configuration; after step 5,

  - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW.

  - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

Comparing Approach A) and Approach B), the fundamental difference seems to be in Approach B) gNB will receive the applicable parameters (in Approach B step 3-4) before providing the inference configuration to UE. The steps from CSI-ReportConfig provision until CSI-ReportConfig activation can be regarded as identical in Approach A) and B). Even though in Approach B) UE is not expected to be configured with a non-applicable CSI-ReportConfig for inference, it may not be reflected in the specification as it is eventually upon gNB implementation how CSI-ReportConfig for inference is generated.

Comparing Approach A) and Approach B), the fundamental difference seems to be in Approach B) gNB will receive the applicable parameters (in Approach B step 3-4) before providing the inference configuration to UE.

The steps from CSI-ReportConfig provision until CSI-ReportConfig activation can be regarded as identical in Approach A) and B).

Considering above analysis, a unified procedure (combining approach A and B) can be supported from specification point of view:

Step 3: (Optional) gNB configures the UE via OtherConfig with sets of parameters for inference configuration

Step 4: (Optional) UE reports in UAI the applicability of the sets of parameters for inference configuration

Step 5: gNB generates and provides inference configuration (CSI-ReportConfig) to UE in RRCReconfiguration

Step 6: UE implies the applicability of CSI-ReportConfig in RRCReconfigurationComplete. The periodic CSI report, if configured, will be activated if reported to be applicable.

Step 7: For aperiodic/semi-persistent CSI report, gNB may further activate via DCI/MAC CE.

It’s worth noting that step 3 and step 4 can be beneficial for gNB to reduce the possibility of providing a non-applicable inference configuration.

RAN2 is suggested to support the following unified procedure (combining approach A and B):

  - Step 3: (Optional) gNB configures the UE via OtherConfig with sets of parameters for inference configuration

  - Step 4: (Optional) UE reports in UAI the applicability of the sets of parameters for inference configuration

  - Step 5: gNB generates and provides inference configuration (CSI-ReportConfig) to UE in RRCReconfiguration

  - Step 6: UE implies the applicability of CSI-ReportConfig in RRCReconfigurationComplete. The periodic CSI report, if configured, will be activated if reported to be applicable.

  - Step 7: For aperiodic/semi-persistent CSI report, gNB may further activate via DCI/MAC CE.

## 2.3	Other issues related to UAI based applicability report

It is quite common understanding that UAI based framework will be used for UE to proactively report the applicability related info according to the gNB configuration (i.e., in OtherConfig). Given that gNB might only be interested in specific UE-sided AI functionalities based on the demand of the gNB, it is reasonable to support the configuration of the UE by the gNB (i.e., via OtherConfig) to report applicability related info pertaining to a subset of the supported UE-side AI functionalities (i.e., as indicated in UECapabilityInformation).

gNB can configure the UE (i.e., via OtherConfig) to report applicability information pertaining to particular UE-side AI functionalities among the supported ones (i.e., as indicated in UECapabilityInformation).

Another aspect related to the UAI configuration for applicability related info report is the concept of prohibit timer. In the legacy, prohibit timer is usually configured to avoid frequent report of UAI. Then the question is whether the concept of prohibit timer is needed for the applicability related info report as well, e.g., after the UAI is triggered in the following two cases, should UE starts a prohibit timer in case:

- Case#1: UAI triggered due to a non-applicable UE sided AI functionality becoming applicable

- Case#2: UAI triggered due to an applicable UE sided AI functionality becoming now non-applicable

In case#1, after sending the UAI, the UE sided AI functionality could be activated by NW. Then it is quite critical to not put any restriction (e.g. prohibit timer) on the next UAI report for UE to timely notify gNB about the non-applicability at any time to deactivate the UE sided AI functionality before too late. In this sense, it would be better not to restrict the report of non-applicability using a prohibit timer.

In case#2, after sending the UAI, the UE sided AI functionality could be deactivated and UE may use another UE sided AI functionality or fallback to non-AIML method. A prohibit timer can be used in this case to avoid UE immediately sending a new UAI indicating the deactivated AI functionality now becomes applicable again, which may lead to frequent or unnecessary UE sided AI functionality activation/deactivation. And it does not harm if the UE continues using the non-AIML method for a while.

UE can send UAI indicating a functionality/CSI-ReportConfig becomes non-applicable at any time without restriction (e.g., prohibit timer).

UE can be configured with a prohibit timer that after UE reports via UAI that a functionality/CSI-ReportConfig becomes non-applicable, UE will not send another UAI message for the same functionality/CSI-ReportConfig again within a time period.

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	Comparing Approach A) and Approach B), the fundamental difference seems to be in Approach B) gNB will receive the applicable parameters (in Approach B step 3-4) before providing the inference configuration to UE.

Observation 2	The steps from CSI-ReportConfig provision until CSI-ReportConfig activation can be regarded as identical in Approach A) and B).

Based on the discussion above, we propose:

Proposal 1	RAN2 is suggested to clarify which of the following UEs can request the data collection configuration, if allowed by NW

a.	All UEs

b.	UEs that are AI capable (as indicated in UE capability report)

c.	UEs that are authorized by NW (as part of UE context)

Proposal 2	In the UAI requesting data collection configuration, UE indicates the measurement resources configuration and associated ID of interest.

Proposal 3	If the CSI configuration framework is used for both training data collection configuration and inference configuration, in RRC configuration message design, it has to be clearly distinguished if the provided configuration is for training data collection or for inference.

Proposal 4	RAN2 is suggested to support the following unified procedure (combining approach A and B):

a.	Step 3: (Optional) gNB configures the UE via OtherConfig with sets of parameters for inference configuration

b.	Step 4: (Optional) UE reports in UAI the applicability of the sets of parameters for inference configuration

c.	Step 5: gNB generates and provides inference configuration (CSI-ReportConfig) to UE in RRCReconfiguration

d.	Step 6: UE implies the applicability of CSI-ReportConfig in RRCReconfigurationComplete. The periodic CSI report, if configured, will be activated if reported to be applicable.

e.	Step 7: For aperiodic/semi-persistent CSI report, gNB may further activate via DCI/MAC CE.

Proposal 5	gNB can configure the UE (i.e., via OtherConfig) to report applicability information pertaining to particular UE-side AI functionalities among the supported ones (i.e., as indicated in UECapabilityInformation).

Proposal 6	UE can send UAI indicating a functionality/CSI-ReportConfig becomes non-applicable at any time without restriction (e.g., prohibit timer).

Proposal 7	UE can be configured with a prohibit timer that after UE reports via UAI that a functionality/CSI-ReportConfig becomes non-applicable, UE will not send another UAI message for the same functionality/CSI-ReportConfig again within a time period.

# Reference