3GPP TSG-RAN WG2 #129	R2-2500614

Athens, GR, Feb 17th – 21st, 2025

Agenda item:	   8.1.5

Source:		Lenovo

Title:	Analysis on dataset and parameter transfer for two-sided model

Document for:		Discussion and Decision

# Introduction

In this paper, we discuss issues related to data and parameter transfer for two-sided model

# Discussion

R1-2410922 To fulfil the study objectives, RAN1 has been discussing various Options, Sub-Options, Directions for alleviating/resolving issues related to inter-vendor training collaboration. In particular, Direction A of RAN1’s discussion requires the following procedure: - NW-side shares model parameters and/or dataset to the UE or UE-side via standardized signaling, where the model parameters and/or dataset received at the UE or UE-side go through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., model (re-)training. RAN1 would like to ask RAN2’s feedback on the feasibility of a standardized signaling, via over-the-air and/or other approaches, for the dataset and/or parameter exchange in the above Option 4-1 and Option 3a-1, considering their overhead and any other aspects that may affect their feasibility. RAN2 may assume the following for the contents for the feasibility discussion. For Option 4-1: sharing {target CSI, CSI feedback} dataset: - Data size for target CSI depends on the format. There is no agreement on the format or necessary precision of the target CSI. Some examples based on companies’ evaluations are: eType-II format (up to ~1000 bits), eType-II-like format (~ a few 1000 bits), and float32 format (up to ~ 150K bits). The data size may also vary depending on the configuration, and the captured value indicates the order of magnitude of the typical data size per sample as a guideline. - There is no agreement on the CSI feedback size. Values in the order of eType II payload size may be assumed (up to ~ 1000 bits) for RAN2 discussion. - There is no agreement on the total number of samples N1 in the dataset. The range of the number of samples N1 used in RAN1 evaluations is 40K-1200K. As a representative number, RAN2 may assume that the total number of samples is around N1=600K. - Summing up the above, and assuming eType-II-like format of a size 2000 bits, the total dataset size for Option 4-1 may be roughly in the order of 600K * (2000 bits + 1000 bits) / (8bits/Byte) = 225 MB. The size may be smaller for eType-II format. The size may be larger if we consider the dataset consisting of supporting multiple configurations. For Option 3a-1 without target CSI: sharing encoder parameter: - The number of parameters in the encoder models used in RAN1 evaluations have a wide range, ranging from 36K parameters to 13M parameters, with the median value of 5.8M. - Based on the above range, the size of encoder parameters may range from 36K*(8/8)=36KB based on the smallest model size in RAN1 evaluations and the use of float8, to 13M*(32/8)=52M based on the largest model size in RAN1 evaluations and the use of float32. Based on the median size, RAN2 may assume the size of 5.8M*(16/8) = 11.6 MB based on the medium model size and the use of float16. For Option 3a-1 with target CSI: sharing encoder parameters, along with {target CSI} dataset: - The size of the encoder parameters is the same as in the Option 3a-1 without target CSI. The size of the dataset is smaller than the Option 4-1, because the dataset only contains the target CSI but not the CSI feedback. The number of samples N2 in the dataset may be the same or smaller than N1. The size becomes N2 * (2000 bits) / (8bits/Byte) + 11.6 MB. Regarding latency and frequency of the dataset and/or parameter sharing: As the purpose of the dataset and/or parameter sharing is for a UE-side offline training, the dataset and/or parameter sharing is expected to have relaxed latency (e.g., days/weeks) and be infrequent. Regarding how many of such dataset and/or parameter sharing is needed: RAN1 evaluations shows good generalization ability, so RAN2 may assume that the number of dataset and/or parameter sharing instances will be limited to a small number.

According to RAN1’s LS, we understand what RAN2 needs to discuss is if it is feasible to share parameters/dataset via Option 4-1 and Option 3-1 w/wo target CSI via

- Other the air interface from NW to UE in the DL

- The other approach from NW to UE-side OTT server

Note that it is not RAN2’s scope to justify if Option 4-1 or Option 3-1 w/wo target CSI is better for offline training of a CSI compression encoder.

To analyse above, the first question is which entity of NW holds the relevant parameter/dataset at the first place. In our view, compared to a single gNB, OAM or CN entity would be a more appropriate place considering:

- the parameter/dataset sharing has relaxed latency of days and weeks, implying UE will move across gNBs.

- the parameter/dataset is assumed to have good generalization ability, implying the original training data {target CSI, CSI feedback} is probably collected from multiple gNBs

RAN2 assumes it is OAM or CN entity that holds the parameter/dataset to be transferred to UE or UE-side OTT server.

Since the interface between OAM/CN entity and UE-side OTT server is out of RAN2 scope, and SA2 is discussing the issue in the meanwhile, RAN2 could further analyse if the parameter/dataset can be transferred from OAM/CN entity to UE over CP or UP.

As described in RAN1 LS, the size of the parameter/data set could range from 11.6MB to 225MB. Usually UP based solution is suitable for large size data transmission. On the other hand, since it could take days/weeks to finish the parameter/dataset transmission, it is also technically possible to transfer over CP piece by piece. The potential specification impacts would be different though in CP and UP based solution.

- In CP based solution, NAS message will be used to contain the parameter/dataset sent from OAM/CN entity to UE. Thus, enhancement to NAS message is expected.

- In UP based solution, UP connection between UE and the OAM/CN entity is needed, thus enhancement on the PDU session framework is probably needed.

RAN2 assumes both CP based solution (e.g. with enhanced NAS) and UP based solution (e.g., with enhanced PDU session framework) are possible for OAM/CN entity to transfer the parameter/dataset to UE.

# 3	Conclusion

Based on the discussion above, we observe:

No table of figures entries found.

Based on the discussion above, we propose:

Proposal 1	RAN2 assumes it is OAM or CN entity that holds the parameter/dataset to be transferred to UE or UE-side OTT server.

Proposal 2	RAN2 assumes both CP based solution (e.g. with enhanced NAS) and UP based solution (e.g., with enhanced PDU session framework) are possible for OAM/CN entity to transfer the parameter/dataset to UE.

# Reference