3GPP TSG-RAN WG2 Meeting #129	R2-2500646

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.5.2

Source:	HONOR

Title: 	Discussion on on-demand SSB configuration

Document for:	Discussion and decision

1	Introduction

In previous meeting, the following agreements were agreed [1]:

RAN2#127 Scenarios on OD-SSB: - RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B. OD-SSB transmission indication: - RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration. - New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A. Measurement on OD-SSB: - Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1). - For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated. - RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements. - RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration RAN2#127bis - No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required. - Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A. - NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

In this contribution, we will discuss the left issues of on-demand SSB.

- Discussion

It was agreed that NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE. In the existing specification, a cell group supports up to 31 SCells. New MAC-CE for OD-SSB transmission indication should support at least 31 SCells activation/deactivation. In earlier protocol versions (LTE), the maximum number of SCells is 7, so the current SCell Activation/Deactivation MAC CEs also have the fixed size of 1 octet (7 SCells), which can apply to the case of configured sCellIndex is less than 8. The similar method can be also applied for new MAC-CE for OD-SSB, such as 1 byte for highest sCellIndex is less than 8 and 4 bytes for highest sCellIndex is larger than or equal to 8.

Another aspect is how to indicate the SCell. A bitmap can be introduced to indicate the sCellIndex in order, which is simpler and shorter than Scell ID.

Proposal 1: The new MAC CE for OD-SSB transmission indication includes a bitmap to indicate up to 7 SCells or 31 SCells.

Compared with eLCID, there are not many reserved bits for LCID. Considering the one octet and four octet indications of Scell are needed for the new OD-SSB MAC CE, it is not suggested to use the LCID as two different LCID values will be occupied.

Proposal 2: The eLCID is used to identify the new MAC CE for OD-SSB transmission indication.

RAN1 has agreed that L1 and/or L3 measurement based on on-demand SSB is supported for NES cell. For the L3 measurement, there are the following options to use the OD-SSB to measure:

- Option 1: L3 measurement based on on-demand SSB pattern;

- Option 2: L3 measurement based on SMTC for on-demand SSB.

For the option 1, UE autonomously performing OD-SSB measurement will bring significant impact on specification. The current L3 measurement framework based on NW control uses the measId to link measurement Object with reporting configuration. Configuring each SSB pattern with reporting configuration needs to define a new framework in RAN2. Meanwhile, there are also some works on RAN1/RAN4 to support UE autonomously performing measurement. Therefore, the option 2 reusing existing L3 measurement framework is more preferred.

Proposal 3:  Use existing L3 measurement framework based on SMTC for OD-SSB measurement.

The current SSB configuration contains the following parameters:

-	ssb-Frequency

-	subcarrierSpacing

-	ssb-Periodicity

-	ssb-PositionsInBurst

-	ss-PBCH-BlockPower

RAN1 has agreed periodicity can be indicated in MAC CE, whether any other information can be included in MAC CE is FFS. Therefore, RAN2 can design the MAC CE when the detailed information is provided by RAN1.

Proposal 4: The detailed information of SSB configuration/SMTC configuration in the MAC CE for OD-SSB is dependent on RAN1.

3	Conclusions

In this contribution, we discussed several issues of on-demand SSB, and made the following proposals:

Proposal 1: The new MAC CE for OD-SSB transmission indication includes a bitmap to indicate up to 7 SCells or 31 SCells.

Proposal 2: The eLCID is used to identify the new MAC CE for OD-SSB transmission indication.

Proposal 3:  Use existing L3 measurement framework based on SMTC for OD-SSB measurement.

Proposal 4: The detailed information of SSB configuration/SMTC configuration in the MAC CE for OD-SSB is dependent on RAN1.

4	References

- RAN2 #127bis and #127 Chairman notes