3GPP RAN WG2 Meeting #129	R2-2500652

# Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.7.4.1

Source:	HONOR

Title:	Discussion on LCP enhancements

Document for:	Discussion and Decision

1. Introduction

In the previous meeting, the following agreements were agreed [1]:

For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - - Use additional priority configured to LCHs in case of these LCHs with delay-critical data RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way. RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR. As a baseline, additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round. Introduce an independent per-LCH remaining time threshold for applying delay-critical priority. For now, we will use “LCH priority-adjusted data” As a baseline, the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fallback to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round. As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP.

And there was an FFS left to discuss.

FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3)

As shown in above, the mainly agreements were achieved for LCP enhancements, we discuss the left issues and provide a text proposal for LCP enhancement in Annex.

2. Discussion

In the previous meeting, it was agreed to use additional priority for LCHs with priority-adjusted data. The following solutions to configure the additional priority has been raised:

- Option 1: priority over other LCHs.

- Option 2: configure via RRC signalling.

- Option 3: activate or deactivate over MAC CE.

The option 1 should be excluded due to it will impact on traffic from SRBs. In the previous meeting, it was agreed that RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. Therefore, the option 2 should be used to configure the additional priority of LCHs without introducing activation or deactivation via MAC CE. Another aspect is independent per-LCH remaining time threshold, which can be configured together with additional priority.

Proposal 1: Use the RRC signalling to configure the additional priority and remaining time threshold of LCHs, which can be included in the IE LogicalChannelConfig.

To avoid the problem that low-priority logical channels are always not served, each uplink logical channel is configured with a priority bit rate (PBR) for fairness. Considering the buffered time of data, the Bj based on PBR is used to maintain the size of bucket. The first round of resource allocation serves the selected LCH in a decreasing priority order whose Bj > 0, it restricts the LCH meeting PBR cannot be served first and limits the transmitted size of buffered data. When the Bj of LCH with priority-adjusted data is smaller than 0 or the LCH priority-adjusted data volume is larger than Bj, the priority-adjusted data may have no chance to transmit in the resource allocation stage. The agreed additional priority provides the possibility for LCH to be allocated resources in advance, which may not enough to solve the problem that priority-adjusted data cannot be filled to the resources due to Bj limitation.

Observation 1: The agreed additional priority is not enough to solve the problem that priority-adjusted data cannot be filled to the resources due to Bj limitation.

The effective way to solve the above issue is to temporarily raise Bj or set PBR to infinity. Generally, the PBR of LCH for SRBs can be set to infinity to guarantee the control signalling can be send timely. Therefore, setting the PBR to infinity is not fairness or suitable. One concern for raising Bj is how Bj can be raised and what value it should be raised to. A simple solution is to configure additional PBR to LCH and use the additional PBR in case of the LCH with delay-critical data. This allows the network to flexibly control the rate that may be increased when there is priority-adjusted data available for the LCH. Meanwhile, this method will not change the value of Bj in the resource allocation stage but can raise the Bj when setting the Bj value.

Proposal 2: RAN2 considers introducing additional PBR for the LCH to temporarily increase Bj.

In the intra-UE prioritization, the priority of an uplink grant is determined by the highest priority among priorities of the logical channels which contain retransmission data or the initial transmission data. If the additional prioprity is used for the logical channel when initiating resources allocation, this priority can be applied to intra-UE prioritization for priority transmission when the UL grant overlap.

Proposal 3: Additional priority can be applied to intra-UE prioritization.

A text proposal of TS38.321 v18.4.0 is provided for the LCP enhancements with considering the proposals in this document.

3. Summary

Proposal 1: Use the RRC signalling to configure the additional priority and remaining time threshold of LCHs, which can be included in the IE LogicalChannelConfig.

Observation 1: The agreed additional priority is not enough to solve the problem that priority-adjusted data cannot be filled to the resources due to Bj limitation.

Proposal 2: RAN2 considers introducing additional PBR for the LCH to temporarily increase Bj.

Proposal 3: Additional priority can be applied to intra-UE prioritization.

4. References

[1]	RAN2 #125bis-RAN2 #128bis Chairman notes

5. Annex

TS38.321 v18.4.0

### 5.4.3	Multiplexing and assembly

#### 5.4.3.1	Logical Channel Prioritization

##### 5.4.3.1.1	General

The Logical Channel Prioritization (LCP) procedure is applied whenever a new transmission is performed.

RRC controls the scheduling of uplink data by signalling for each logical channel per MAC entity:

-	priority where an increasing priority value indicates a lower priority level;

-	prioritisedBitRate which sets the Prioritized Bit Rate (PBR);

-	bucketSizeDuration which sets the Bucket Size Duration (BSD).

-	adjustedPriority where an additional priority value applies to logical channel with priority-adjusted data.

-	adjustedPriorityTimeThreshold which sets the threshold on remaining time for triggering to use adjustedPriority.

-	adjustedPrioritisedBitRate where an additional PBR value applies to logical channel with priority-adjusted data.

RRC additionally controls the LCP procedure by configuring mapping restrictions for each logical channel:

-	allowedSCS-List which sets the allowed Subcarrier Spacing(s) for transmission;

-	maxPUSCH-Duration which sets the maximum PUSCH duration allowed for transmission;

-	configuredGrantType1Allowed which sets whether a configured grant Type 1 can be used for transmission;

-	allowedServingCells which sets the allowed cell(s) for transmission;

-	allowedCG-List which sets the allowed configured grant(s) for transmission;

-	allowedPHY-PriorityIndex which sets the allowed PHY priority index(es) of a dynamic grant for transmission;

-	allowedHARQ-mode which sets the allowed UL HARQ mode for transmission.

The following UE variable is used for the Logical channel prioritization procedure:

-	Bj which is maintained for each logical channel j.

The MAC entity shall initialize Bj of the logical channel to zero when the logical channel is established.

For each logical channel j, the MAC entity shall:

1>	apply the priority and PBR to the logical channel configured with adjustedPriority as follows:

2>	if priority-adjusted data is buffered in the logical channel:

3> set the priority of the logical channel to adjustedPriority;

3> if adjustedPrioritisedBitRate is configured:

3> update PBR to the value of adjustedPrioritisedBitRate.

2> else:

3> set the priority of the logical channel to priority.

1>	increment Bj by the product PBR × T before every instance of the LCP procedure, where T is the time elapsed since Bj was last incremented;

1>	if the value of Bj is greater than the bucket size (i.e. PBR × BSD):

2>	set Bj to the bucket size.

- NOTE:	The exact moment(s) when the UE updates Bj between LCP procedures is up to UE implementation, as long as Bj is up to date at the time when a grant is processed by LCP.

##### 5.4.3.1.2	Selection of logical channels

The MAC entity shall, when a new transmission is performed:

1>	select the logical channels for each UL grant that satisfy all the following conditions:

2>	the set of allowed Subcarrier Spacing index values in allowedSCS-List, if configured, includes the Subcarrier Spacing index associated to the UL grant; and

2>	maxPUSCH-Duration, if configured, is larger than or equal to the PUSCH transmission duration associated to the UL grant; and

2>	configuredGrantType1Allowed, if configured, is set to true in case the UL grant is a Configured Grant Type 1; and

2>	allowedServingCells, if configured, includes the Cell information associated to the UL grant. Does not apply to logical channels associated with a DRB configured with PDCP duplication within the same MAC entity (i.e. CA duplication) when CA duplication is deactivated for this DRB in this MAC entity; and

2>	allowedCG-List, if configured, includes the configured grant index associated to the UL grant; and

2>	allowedPHY-PriorityIndex, if configured, includes the priority index (as specified in clause 9 of TS 38.213 [6]) associated to the dynamic UL grant; and

2>	allowedHARQ-mode, if configured, includes the allowed UL HARQ mode for the HARQ process associated to the UL grant.

- NOTE:	The Subcarrier Spacing index, PUSCH transmission duration, Cell information, and priority index are included in Uplink transmission information received from lower layers for the corresponding scheduled uplink transmission.

##### 5.4.3.1.3	Allocation of resources

Before the successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall not select the logical channel(s) corresponding to non-DAPS DRB(s) for the uplink grant received in a Random Access Response or the uplink grant for the transmission of the MSGA payload. The source MAC entity shall select only the logical channel(s) corresponding to DAPS DRB(s) during DAPS handover.

The MAC entity shall, when a new transmission is performed:

1>	allocate resources to the logical channels as follows:

2>	logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s);

2>	decrement Bj by the total size of MAC SDUs served to logical channel j above;

2>	if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally.

- NOTE 1:	The value of Bj can be negative.

NOTE 2:        If using adjustedPriority to the logical channel in the 1st resources allocation, it is up to UE implementation to fallback priority to the logical channel in the 2nd resources allocation when the all priority-adjusted data for the logical channel is multiplexed on the transmission in the 1st resources allocation.

If the MAC entity is requested to simultaneously transmit multiple MAC PDUs, or if the MAC entity receives the multiple UL grants within one or more coinciding PDCCH occasions (i.e. on different Serving Cells), it is up to UE implementation in which order the grants are processed.

/…omit…/