3GPP RAN WG2 Meeting #129	R2-2500653

# Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.7.4.2

Source:	HONOR

Title:	Discussion on DSR enhancements

Document for:	Discussion and Decision

1. Introduction

In RAN2#127bis meeting, the following agreements were agreed [1]:

We do not change the definition of delay-critical data For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s) Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - Buffer size of data volume in each portion - Shortest remaining time among PDCP SDUs buffered in each portion. There is no need to include PSI in the enhanced DSR MAC CE. A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

In RAN2#128 meeting, the following agreements were agreed [1]:

- Let the network configure the triggering and reporting thresholds without constraints. - RAN2 understanding is that the data that has been already reported in the DSR should not trigger another DSR - The existing cancelling and triggering of Rel-18 DSR is reused for the enhanced DSR. - The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

In this document, we discuss the left issues of DSR.

2 Discussion

2.1 The content of EDSR

Buffer size calculation

In the last meeting, it was agreed to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR. The type of non-delay critical data can be as following:

- Control PDU;

- For AM DRB, data PDU containing previously transmitted data;

- Low importance data ahead of delay critical data when the PSI-based discard is active.

If only the first two data are considered, the current delay-critical data volume calculation can be reused, and these two data can be indicated in the first portion with shortest remaining time. If all above data are considered, the UE may need to consider the volume of non-delay critical data into each portion, which is more complicated. So, RAN2 considers the first two data as non-delay critical data in the buffer size calculation for DSR.

Proposal 1: RAN2 considers the following data as the non-delay critical data in the buffer size calculation for DSR:

- Control PDU;

- For AM DRB, data PDU containing previously transmitted data.

One-bit indication

In the previous meeting, a one-bit indication indicated whether a certain/further pair of delay information is present in DSR for the associated LCG was introduced. Since there is no other information introduced in DSR MAC CE expect of delay information, it is straightforward that reusing the reserved bit in current DSR MAC CE to indicate whether a further pair of delay information is included for the LCG. The new DSR MAC CE may be as following:

The new F field indicates the presence of a further pair of delay information (i.e. the Remaining Time and Buffer Size fields) for an LCG. The F field set to 1 indicates that there is a further pair of delay information for the associated LCG. The F field set to 0 indicates that the last pair of delay information for the associated LCG is reported.

Proposal 2: The one-bit indication indicates whether a further pair of delay information (i.e. remaining time and buffer size) is present in the enhanced DSR MAC CE for the associated LCG.

Proposal 3: The one-bit indication set to 0 indicates the last pair of delay information for the associated LCG is reported.

2.2 PDU set related remaining time

In the current specification, if pdu-SetDiscard is configured, all SDUs belonging to a PDU set will be dropped at the same time. So, the delay critical data of PDU set is defined based on the entire PDU set. In the Rel-18, the all delay-critical data of PDU set is considered in buffer size of DSR. Since multiple portions is defined based on the multiple reporting time threshold levels, it is needed to discuss how to divide the volume of delay-critical data for PDU set into each portion. The critical issue for this is how to define the remaining time of all data for PDU set. Since all SDUs belonging to a PDU set will be dropped at the same time, the shortest remaining time among the data of PDU set needed to be used as the remaining time of all data of PDU set. Therefore, a simple solution is to consider this volume into the portion with shortest remaining time of PDU set.

Proposal 4: If pdu-SetDiscard is configured, all data of a PDU set belong to the same portion with the shortest remaining time among all data of the PDU set.

2.3 How to report old and new DSR

It has been agreed that network can configure multiple remaining time thresholds for each LCG, so there will be LCGs associated with reporting thresholds and LCGs associated only with triggering threshold. For the subsequent DSR triggering, the scenarios can be divided into the following cases:

- Case 1: only LCG with TT (triggering threshold) meets the triggering conditions of DSR.

- Case 2: only LCG with TT and RT (reporting threshold) meets the triggering conditions of DSR.

- Case 3: LCG with TT and RT, and LCG with TT meet the triggering conditions of DSR.

There is a unified solution for the above cases that enhanced DSR is always reported. For the case 1, if there is no other LCG meets the triggering conditions of DSR, the new DSR MAC CE will be used to report the delay information for LCG with TT. For the case 3, the pending new DSR is not cancelled when all the delay-critical data of LCG with TT and RT have been discarded. Same as case 1, the new DSR will be used to report the delay information of LCG only with TT. The enhanced DSR is introduced a new indication and may consider different buffer size calculation. If a LCG configured only with TT, it is not needed to use the new method to report its buffer size as the network does not configure RT for this LCG. So, it is preferred that the DSR MAC CE can be used if there is only LCG with TT needed to report delay information.

Proposal 5: When to construct the DSR, if at least one LCG with reporting threshold has delay information to be reported, the enhanced DSR MAC CE will be used to report the delay information; otherwise, the DSR MAC CE will be used to report the delay information.

3 Conclusions

Proposal 1: RAN2 considers the following data as the non-delay critical data in the buffer size calculation for DSR:

- Control PDU;

- For AM DRB, data PDU containing previously transmitted data.

Proposal 2: The one-bit indication indicates whether a further pair of delay information (i.e. remaining time and buffer size) is present in the enhanced DSR MAC CE for the associated LCG.

Proposal 3: The one-bit indication set to 0 indicates the last pair of delay information for the associated LCG is reported.

Proposal 4: If pdu-SetDiscard is configured, all data of a PDU set belong to the same portion with the shortest remaining time among all data of the PDU set.

Proposal 5: When to construct the DSR, if at least one LCG with reporting threshold has delay information to be reported, the enhanced DSR MAC CE will be used to report the delay information; otherwise, the DSR MAC CE will be used to report the delay information.

4. References

[1]	RAN2 #127bis Chairman notes

[2]	RAN2 #128 Chairman notes