3GPP TSG RAN WG2 #129          						R2-2500661

Athens, Greece, Feb 17th – 21st, 2025

Source:	Tejas Networks Ltd.

Title:	Discussion on A-IoT random access

Agenda item:	8.2.3

Document for:     Discussion and Decision

# Introduction

As discussed in TR38.769, the A-IoT devices receive Msg0 paging signal from reader or base station (BS), upon receiving Msg0, multiple A-IoT devices transmit Msg1 which carries a unique random ID of the device in response to Msg0. Transmission of Msg1 can be contention-based or contention-free random access. In case of contention-based random access, the devices transmit Msg1 using slotted ALOHA-based multi-access. In response to Msg1, the reader transmits Msg2 with scheduling information for the device to transmit the actual device ID and group ID. Based on the resource allocated to the devices, they send their data consists of device ID and group ID. Fig. 1 shows the timing diagram of A-IoT communication between the reader and a device. At every communication slot or occasion, the reader initiates the communication by sending a paging signal and the slot ends when the device received acknowledgement of msg3 or the command response.

Fig. 1: Timing diagram of A-IoT communication between reader and device.

# Contention-based and Contention-Free Random Access

Access occasion: An opportunity of time-frequency resource for A-IoT device(s) to perform access (e.g., transmitting the A-IoT Msg1 by the device). A set of access occasion(s) for different A-IoT device(s) is scheduled via the R2D message (referring to the "R2D transmission triggering random access" in clause 6.1.4) by the reader.

When the A-IoT device is selected to respond in accordance to the clause 6.3.3, the A-IoT device performs the following procedure:

-	Step 1: Random access type (i.e., contention-free or contention-based) and access occasion/resource determination:

-	The A-IoT device determines the random access type from the A-IoT paging message, in accordance to clause 6.3.3. The reader can configure either contention-free access or contention-based random access (and the corresponding configurations). It needs to be further discussed on whether it is explicitly or implicitly.

-	If the random access is contention-free access:

-	Selects the indicated D2R occasion/resource;

-	Skips the contention resolution in Step 2 and performs the data transmission in accordance to clause 6.3.5.

-	If the random access is contention-based random access:

-	Performs access occasion/resource selection: as the baseline for CBRA, at least for TDMA case, the device can randomly select one access occasion for A-IoT Msg1 within the access occasions provided/assigned by the reader. It needs to be further discussed if this is applicable to FDMA case. Further enhancement option(s) can be also considered after more physical layer detailed design on TDMA and FDMA;

-	Performs the Step 2 for contention resolution.

The following has been studied in TR 38.769, section 6.3.4 on random access type [1]:

According to TR 38.769, slotted-ALOHA is studied as baseline for A-IoT as contention-based access procedure, when multiple devices transmit Msg1 in response to the Msg0. In the slotted-ALOHA based access procedure, an ACK/NAK signal should be sent to the device based on successful/unsuccessful reception of the information.

The agreements made in RAN2 #127-bis [3] on TDMA and FDMA for Msg1 are as follows:

From RAN2 perspective, at least for TDMA the device can randomly select one access occasion for A-IoT Msg1 within access occasions provided/assigned by the reader, as the baseline for CBRA.  FFS if this is applicable to FDMA.  Further enhancement option(s) can be considered after more RAN1 progress on TDMA/FDMA.

The devices can be allocated few frequency domain resources along with time domain resources based on the transmission bandwidth of the devices and the total communication bandwidth of A-IoT.

Fig. 2a: Msg1 transmission occasions in slotted-ALOHA based random access.

Fig. 2b: Multiple Msg1 transmission from multiple devices at different time and frequency.

When multiple devices transmit Mgs1 at the same time in response to Msg0 from the reader, there is a chance of collision. To minimize collision and maximize throughput, the device can use slotted Aloha based random access method to transmit Msg1 frame. As shown in Fig. 2, the msg1 transmission duration is divided into multiple occasions. Each occasion consists of multiple equal time and frequency domain slots.

The D2R Msg1 transmission time is denoted as T1 which is bounded by [T1_min, T1_max]. T1_min is the Msg1 frame transmission time, which is the minimum time required to transmit Msg1. T1_max = X × T1_min, is the maximum time available to the device to transmit Msg1. After receiving the Msg0, the devices randomly select a time slot t between 1 to X and a frequency f between 1 to Y and transmit Msg1 at that time slot using that frequency. The signal propagation time Tp << T1_min. Thus, T1_min is considered as the slot duration of slotted Aloha. The device runs two timers to track T1 and T2. If the device does not get acknowledgement with time T2, it randomly chooses a time slot from the remaining available slots and retransmit Msg1. The flow diagram of slotted Aloha mechanism to transmit Msg1 is given in Fig. 4. If any device is not able to access the channel to send Msg1 within the time [T1_min, T1_max], it goes to sleep mode and reattempts in the next slot.

Fig. 3: Slotted Aloha-based multiaccess communication.

Observation 1: The maximum time duration for Msg1, considered as T1_max depends on the number of devices available, transmission bandwidth of device, and the receiver bandwidth of the reader which indicates the number of frequency domain resources available to the devices to transmit Msg1.

Proposal 1: RAN2 should specify the steps involved in the slotted-ALOHA based random access to transmit msg1 from multiple devices in A-IoT communication. T1_min = TD2R_min can be same as the Msg1 frame transmission time, which is the minimum time required to transmit Msg1. T1_max = X × T1_min = TD2R_max is the total msg1 transmission duration. T1_min is considered as the slot duration of slotted Aloha.

Proposal 2: The value of X (time domain resources) primarily depends on the total number of devices available in the vicinity of the reader. The value of Y (frequency domain resources) depends on the transmission bandwidth of devices and the receiver bandwidth of the reader.

Proposal 3: Device starts the timer with value T2 after sending msg1. The device waits for an acknowledge (msg2) before the expiry of timer T2. If no acknowledge is received before the expiry, device re-attempts msg1 within the same paging cycle.

-	Step 2: Contention resolution of contention-based random access:

-	There are three candidate solutions being studied for the contention resolution, as below (it needs to be further discussed on the down selection and/or the unified design):

-	Solution 1: A-IoT Msg1 without data

-	A-IoT Msg1: When the A-IoT device identifies the start of its own access occasion, it sends one 16-bit random ID generated by the A-IoT device to the reader.

-	A-IoT Msg2: The reader responds with the successfully received random ID.

If the A-IoT device receives the A-IoT Msg2 including a random ID, which is the same as the previously transmitted one in A-IoT Msg1, it considers the contention resolution as successful.

-	Solution 2: A-IoT Msg1 with data

-	A-IoT Msg1: When the A-IoT device identifies the start of its own access occasion, it sends the A-IoT Msg1 including the upper layer data, which can be the device ID and/or any other upper layer data, in addition to one 16-bit random ID generated by the A-IoT device to the reader.

-	A-IoT Msg2: The reader may respond with the successfully received random ID.

If the A-IoT device receives the A-IoT Msg2 including a random ID, which is the same as the previously transmitted one in A-IoT Msg1, it considers the contention resolution as successful. If the A-IoT Msg2 is not received by the device, the re-access is not autonomously performed i.e., the re-access is always controlled by the reader.

-	Solution 3: A-IoT Msg1 optionally includes data (unified solution supporting both Solution 1 and 2)

-	A-IoT Msg1: When the A-IoT device identifies the start of its own access occasion, it sends one 16-bit random ID generated by the A-IoT device to the reader. In addition, it is controlled by the reader on whether to also include the upper layer data, which can be the device ID and/or any other upper layer data.

-	A-IoT Msg2: The reader responds with the successfully received random ID.

If the A-IoT device receives the A-IoT Msg2 including a random ID, which is the same as the previously transmitted one in A-IoT Msg1, it considers the contention resolution as successful.

The following has been studied in TR 38.769, section 6.3.4 on contention resolution of contention-based random access [1]:

Observation 2: Solution 2 is related to 4-step CBRA and solution 1 is related to 2-step CBRA. Solution 3 optionally includes both solution 1 and solution 2.

Proposal 4: We propose to study solution 3 as it includes both solution 1 and solution 2.  Msg0 signal can carry one bit information to convey the 2-step or 4-step CBRA process to the device. Reader may initially choose 2-step CBRA at the first communication slot of any inventory cycle. For the subsequent slots, based on the number of msg1 signals received from the devices available at the vicinity, which is the response to msg0 paging signal, the reader can decide whether to shift to 4-step process or continue with the 2-step process.

# A-IoT Msg1 Content/Format

NOTE 3:	It needs to be further discussed if more information on command type (e.g. read/write/disable) is useful. The reader needs to know whether a D2R response is expected in D2R direction and, if so, the expected D2R message size. It needs to be further discussed on the details on how the reader gets that information (related to the "expected D2R message size" discussion in clause 6.3.5). It needs to be further discussed on whether the command type, if needed, is explicit or inferred from the "expected D2R message size" if available.

The following has been studied in TR 38.769, section 6.3.1 [1]:

Fig. 4: Device transmits Msg1 in response to Msg0 from the reader.

Once the device receives Msg0 from the reader, it sends a response to that Msg0 if the device has to send some information to the reader. A unified message structure of msg1 can be considered for both 2-step and 4-step process. The device generates a unique random ID. The expected D2R message size, which is the size of upper layer data, can be sent to the reader through Msg1 along with the unique random ID and the device energy status, which the reader can use for scheduling purpose.

The D2R information from layer 2 consists of the following:

- Unique random ID

- Message size (size of the upper layer data)

- Energy status of the device

- 1-bit data presence indicator

- Upper layer data

For 2-step process, the data presence indicator is set to ‘1’. The device sends the upper layer data, which is the actual information consists of device ID, group ID, etc. through msg1 only. For 4-step process, the data presence indicator is set to ‘1’. The device sends the upper layer data through msg3. In that case, the upper layer data field in msg1 is empty or no data present in that field.

Similar to Msg0, the D2R preamble bits consist of the start indicator part to indicate the starting of PDRCH signal and clock acquisition part as synchronization signal. Postamble indicated the ending of PDRCH signal.

Proposal 5: RAN2 to consider a unified msg1 structure for both 2-step and 4-step random access.

Proposal 6: For 2-step process, msg1 contains the device generated unique random ID, data size (size of upper layer data present in msg1), device energy status, 1-bit data presence indicator (set to ‘1’), and upper layer data.

Proposal 7: For 4-step process, msg1 contains the device generated unique random ID, data size (size of upper layer data to be conveyed through msg3), device energy status, 1-bit data presence indicator (set to ‘0’), data field is not present.

Proposal 8: If the energy available at the device is below certain threshold, reader may choose not to allocate resource to that device. The device can get more energy form the CW signal received in the next slot and try again to communicate with reader at that slot.

# Conclusion

This work includes the contention-based and contention-free random access, re-access of failure handling, and A-IoT msg1 content. We have made the following observations and proposals related to these aspects:

Observation 1: The maximum time duration for Msg1, considered as T1_max depends on the number of devices available, transmission bandwidth of device, and the receiver bandwidth of the reader which indicates the number of frequency domain resources available to the devices to transmit Msg1.

Proposal 1: RAN2 should specify the steps involved in the slotted-ALOHA based random access to transmit msg1 from multiple devices in A-IoT communication. T1_min = TD2R_min can be same as the Msg1 frame transmission time, which is the minimum time required to transmit Msg1. T1_max = X × T1_min = TD2R_max is the total msg1 transmission duration. T1_min is considered as the slot duration of slotted Aloha.

Proposal 2: The value of X (time domain resources) primarily depends on the total number of devices available in the vicinity of the reader. The value of Y (frequency domain resources) depends on the transmission bandwidth of devices and the receiver bandwidth of the reader.

Proposal 3: Device starts the timer with value T2 after sending msg1. The device waits for an acknowledge (msg2) before the expiry of timer T2. If no acknowledge is received before the expiry, device re-attempts msg1 within the same paging cycle.

Observation 2: Solution 2 is related to 4-step CBRA and solution 1 is related to 2-step CBRA. Solution 3 optionally includes both solution 1 and solution 2.

Proposal 4: We propose to study solution 3 as it includes both solution 1 and solution 2.  Msg0 signal can carry one bit information to convey the 2-step or 4-step CBRA process to the device. Reader may initially choose 2-step CBRA at the first communication slot of any inventory cycle. For the subsequent slots, based on the number of msg1 signals received from the devices available at the vicinity, which is the response to msg0 paging signal, the reader can decide whether to shift to 4-step process or continue with the 2-step process.

Proposal 5: RAN2 to consider a unified msg1 structure for both 2-step and 4-step random access.

Proposal 6: For 2-step process, msg1 contains the device generated unique random ID, data size (size of upper layer data present in msg1), device energy status, 1-bit data presence indicator (set to ‘1’), and upper layer data.

Proposal 7: For 4-step process, msg1 contains the device generated unique random ID, data size (size of upper layer data to be conveyed through msg3), device energy status, 1-bit data presence indicator (set to ‘0’), data field is not present.

Proposal 8: If the energy available at the device is below certain threshold, reader may choose not to allocate resource to that device. The device can get more energy form the CW signal received in the next slot and try again to communicate with reader at that slot.

# References

[1] TR 38.769, Study on solutions for ambient IoT (Internet of Things) in RAN, V2.0.0.

[4] 3GPP RAN2 Chairman’s Notes, 3GPP TSG RAN2 meeting #127-bis.