3GPP TSG-RAN WG2 Meeting #129 	R2-2500672

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.2.4

Source:	Panasonic

Title:	Discussion on AS ID, segmentation and control signaling format

Document for:	Discussion

# Introduction

Recent RAN plenary meeting has approved Rel-19 work item for A-IoT (RP-243326), according to which the RAN2 scope has been set as follows.

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

This paper is intended to discuss some remaining functionality aspects such as AS ID, segmentation and control  signaling format.

# Discussion

## AS ID

During the study item phase, it has been assumed that AS ID is used at least for purpose of D2R scheduling and R2D reception. This AS ID should be a short AS layer ID, rather than the full upper layer device ID.

The following issues need to be further discussed:

- Who generates AS ID (down-select from the 3 options)

- By which message if reader assigns the AS ID

- How long AS ID should be kept

Who generates AS ID

The following 3 options have been identified during the study item phase:

-	Option 1: a random ID (if used in first D2R message) can be reused;

-	Option 2: the reader assigns this "AS ID" to the device. It needs to be further discussed via which R2D message.

-	Option 3: It is up to the reader whether to reuse the random ID (if used in first D2R message) as the "AS ID" or to assign a new "AS ID". It needs to be further discussed via which R2D message.

It has been also concluded that further down-selection needs to consider one common design for all access procedures, if technically possible.

According to RAN2 scope defined in the WID (see section 1), all access procedures mean to include both contention-based random access (CBRA) and contention-free random access (CFRA). For CBRA, device generates a random ID and send to reader in Msg1 for contention resolution. If the device receives Msg2 including a random ID that is the same as the previously transmitted one in Msg1, the device considers the contention resolution as successful. Therefore, it seems natural to reuse this random ID as the AS ID for later R2D reception and D2R scheduling (i.e. option 1).

However, option 2 has following merits. Therefore, we propose to take option 2.

- Shorter length than random ID

It is possible to use a shorter length than random ID (=16 bits) if reader assigns the AS ID. As observed in [1], a network-assigned address would require 33% to 50% of the bits that a random generated address needs with the same number of competing devices. In other words, if reader assigns the AS ID, length can be reduced to 8 bits. One may argue that additional signaling overhead is needed for reader to provide such ID to the device. However, the provision of AS ID is only needed if there is follow-up command service after inventory where multiple R2D and D2R can be exchanged. Using a short AS ID (e.g. 8 bits) therefore certainly can save signaling overhead compared to using random ID (16 bits) every time even considering the ID provision overhead. For inventory-only use case, the access procedure ends by Msg3 where device reports its device ID to network. There is no need for AS ID assignment in this case.

- Unified design with CFRA

For CFRA there is no need for device to generate a random ID for contention resolution. The device would skip contention resolution steps (i.e. Msg1 and Msg2) and directly sends the upper layer data (e.g. device ID) in its very first D2R message after being triggered (by A-IoT paging). In this case, reader can allocate the AS ID for the device and send it to the device via the next R2D transmission (if there is follow-up command use case).

Proposal 1. Option 2 (reader assigns AS ID to device) is selected. The length of AS ID is shorter than random ID, e.g. 8-bit length can be considered for AS ID.

By which message if reader assigns the AS ID

As discussed above, AS ID may not be needed if the access procedure is for inventory only. Based on this, Msg2 is not suitable for assigning the AS ID as the device may not be using it anyway and the signaling overhead for assigning is wasted. If there is command after inventory, the R2D after Msg3 (i.e. Msg4) will schedule a D2R and will also provide the AS ID. Note that assigning AS ID should not be limited to Msg4 only. Whenever the reader needs to allocate a new AS ID to the device, reader can do so by including the new AS ID in the R2D message.

Proposal 2. Reader can assign AS ID via R2D message after Msg3.

How long AS ID should be kept

In general, it is preferable to keep the AS ID as long as possible to maximize the benefit. There was some security concern (e.g. unauthorized tracking) in the previous meeting if the AS ID is kept too long in the device. To address this, reader can always decide to assign a new AS ID whenever needed (see discussion on Proposal 2 above). There was another proposal in previous meeting to introduce a timer for the validity of AS ID. However, in our view, such scheme is not preferred because it further increases the complexity of AS ID maintenance in device and it is difficult to set the appropriate timer value.

On the other hand, it would be good for reader to know whether the assigned AS ID is still maintained by the device or not. This is related to how long the volatile memory contents are kept i.e. retained, is known to the reader. This is not only AS ID but general volatile memory content discussion. For example, it can be specified that all devices shall keep the volatile memory contents for a certain time duration.

Proposal 3. It is up to reader implementation to determine whether the assigned AS ID is maintained at device in Rel-19. The device shall keep volatile memory contents including AS ID for the certain duration.

## Segmentation for D2R

In the study item, it has been agreed that segmentation is supported at least for D2R. It has been also identified beneficial for the reader to be able to trigger re-transmission of a segment. We discuss further details on segment retransmission in this section.

Scheme 1: ACK/NACK based segment retransmission with 1-bit segment indication

As mentioned by some companies during the study item, each segment transmitted by device will be explicitly acknowledged by the reader by ACK/NACK indication. If an ACK is received by device, device will move forward to the next segment transmission. If a NACK is received by device, device will transmit the same segment again. The ACK/NACK indication can be provided together with a scheduling grant to schedule the next D2R transmission.

Since the segmentation is decided by the device, a segment indication needs to be used to indicate to the reader whether the data is segmented and whether it is last segment. It is still open how this works and number of bits (e.g. one or two).

Let us first consider the simplest design using one bit. In this case, the reader is able to know whether the current received segment is the last segment or not (including the case where there is no segmentation performed by the device).

Observation 1: The following scheme 1 (ACK/NACK-based retransmission with one-bit segment indication) is identified for D2R segmentation:

- ACK/NACK indication is used for reader to control whether device to transmit the next new segment or re-transmit previous segment.

- Device uses one-bit to indicate whether the data is segmented and whether it is the last segment.

Although being simple, the above solution cannot avoid error in segment delivery in some scenarios. If it occurs, neither reader or device can detect it due to lack of support of segment index and indication of total number of segments. One error case is illustrated in following Fig.1. As shown, after successfully receiving 1st segment, the reader sends a new grant with ACK to request the device to send 2nd segment. However, this new grant is missed by the device. Consequently, the reader will request the re-transmission by sending another grant with NACK. Since the device has missed previous grant, the device would interpret this grant with NACK as requesting the retransmission of 1st segment. As a result, the device will proceed to send 1st segment again. There is no way for reader to find out such misalignment. In the end, the duplicated segment will be stored by the reader as a new segment, resulting in packet assembly error.

Fig.1. Illustration of an error case in ACK/NACK-based retransmission with 1-bit segment indication

In our review of contributions submitted to previous meeting, the above issue has been also identified by [1] and [2], although no particular enhancement was suggested by these two contributions. Instead, it was proposed to rely on error checking mechanism of the whole upper layer packet. In our view, it is inefficient to rely on the retransmission of the whole packet if one segment is delivered in error. As will discussed later, such issue can be solved by marginal price as schemes 2 and 3.

Observation 2: If ACK is missed by the device, the scheme 1 will result in duplicated segment without awareness of the receiver.

Scheme 2: ACK/NACK based segment retransmission with 2-bit segment indication

If segmentation indication has two bits, the additional bit can used as a new segment indication (and the other bit is the end mark indication as in scheme 1). If the new segment bit is toggled (i.e. from 1 to 0, or from 0 to 1), it means a new segment is sent. On the other hand, if the new segment bit is the same as previous transmission, it means the previous segment is re-transmitted. In this way, the new segment indication from the device can act as a confirmation on “whether I am actually moving to the next segment”. With this additional information, the chance of misunderstanding of segment delivery between device and reader is reduced.

The following Fig.2 illustrates how scheme 2 can solve the identified issue on scheme 1.

Fig.2. Illustration of scheme 2 – two-bit segment indication from device

Observation 3: The following scheme 2 is identified for D2R segmentation:

- ACK/NACK indication is used for reader to control whether device to transmit the next new segment or re-transmit previous segment.

- Device uses two-bit segment indication:

  - New segment bit (toggle bit) to indicate whether a new segment is transmitted by device

  - End mark bit to indicate whether the data is segmented and whether it is the last segment

Observation 4: Scheme 2 is more robust than scheme 1 for segment delivery, with marginal price for additional signaling overhead.

Scheme 3: Differential encoded stop-go bit with 1-bit segment indication

In this scheme, the stop-go control bit is differential encoded and then feedback to the device. As known, differential encoding can make use of past state to provide the unambiguous state reception, which is exactly what we need to solve issue in Fig1.

For example, assuming xi is the stop-go bit intended to feedback to device and yi-1 was the previous state that has been transmitted, then the new state to be fed back for xi is yi = yi-1 ⊕ xi , where ⊕ is the binary addition.

The following table shows the example.

| yi-1 (previous state) | xi (current state) | yi (encoded state) |
| --- | --- | --- |
| 0 | 1 (go to a new segment) | 1 |
| 0 | 0 (retransmit previous segment) | 0 |
| 1 | 1 (go to a new segment) | 0 |
| 1 | 0 (retransmit previous segment) | 1 |

After the device receives the encoded state, it performs binary addition to the previously stored state to recover the intended state from the reader as xi = yi-1 ⊕ yi . In this way, the stop-go control is transmitted using the difference between symbol yi-1 and yi , rather than the fixed value of 1 or 0 (like the ACK/NACK bit in scheme 1). Therefore, the misunderstanding between reader and device as shown in Fig.1 can be avoided.

Note that another way to interpret the differential encoded stop-go bit is the even/odd segment indication as mentioned in [3]. Please see [3] for detailed discussion.

Observation 5: The following scheme 3 is identified for D2R segmentation:

- Differential encoded stop-go bit is used for reader control whether device to transmit the next new segment or re-transmit previous segment.

- Device uses one-bit to indicate whether the data is segmented and whether it is the last segment.

Observation 6: Scheme 3 is more robust than scheme 1 for segment delivery, with marginal price for additional one-bit storage in device memory for differential decoding.

Proposal 4. RAN2 confirms to support segment re-transmission for D2R segmentation.

Proposal 5. RAN2 down-select one of the following schemes for D2R segmentation:

Scheme 2: ACK/NACK based segment retransmission with 2-bit segment indication

Scheme 3: Differential encoded stop-go for segment retransmission with 1-bit segment indication

## Handling of L1/L2 control information

Generally speaking, L1/L2 control information is the information generated by PHY or MAC layer to control how upper layer message would be transmitted/received. We separate the discussion into R2D and D2R.

R2D control information

The following can be considered as R2D control information:

- Target ID intended for reception of R2D, e.g. AS ID, random ID, group ID

- TBS information or time duration information, based on which device decides on end of R2D

- Segmentation related information, e.g. ACK/NACK (or stop-go bit) to control D2R segment transmission, see above section 2.2

- Paging ID and reader ID to avoid duplicated responses (see our companion paper [4])

- D2R scheduling information (e.g. time/frequency domain resources, MCS-like information, chip duration, ID associated with device(s), repetition, midamble (if supported) related information).

The possible ways to handle the above R2D control information may include, via L1 control part of PRDCH (if supported), via MAC subheader, via MAC CE, or even via MAC SDU (in a similar way to handling upper layer data). Although it may be a matter of modeling preference to select one of the above ways, we think the following two design principles are important:

Principle 1: Target ID should be mapped into the beginning of PRDCH.

This is to facilitate the device to identify whether it is the target device or not even before receiving the whole PRDCH. The unintended device could skip the rest of PRDCH reception in order to save power.

One way to realize the above principle 1 is to define L1 control part of PRDCH to be located at the beginning of PRDCH and to carry target ID. See the following Fig. 3.

Fig.3. Target ID is located in front of MAC PDU

Alternatively, without introducing L1 control part, the MAC spec needs to specify that target ID is located at the beginning of MAC PDU (e.g. the first MAC subheader, or the first MAC CE). See the following Fig.4.

Fig.4. Target ID is located in the first MAC CE

Principle 2: Container for target ID should support variable size.

This is because a PRDCH can be addressed to random ID, AS ID, or group ID and those IDs may differ in size. In this sense, using a MAC CE seems to be a better option because variable size of MAC CE is already supported by NR and the method can be borrowed for A-IoT. If the container is the L1 control part, more discussion and specification efforts seem to be needed to support variable size.

Further considering the fact that whether or not to introduce L1 control part is an ongoing discussion in RAN1 that might take time to converge, we suggest RAN2 to consider the above two principles in the MAC PDU design. If later RAN1 agrees to introduce L1 control part, then the MAC CE containing target ID can be mapped into the L1 control part. In this way, two working groups can progress their discussion individually without blocking each other.

Proposal 6. RAN2 to discuss designing a MAC CE with variable size to carry target ID and this MAC CE should be placed at the beginning of MAC PDU.

D2R control information

The following can be considered as D2R control information:

- Segment indication to support D2R segmentation (see section 2.2)

- Energy status report (subject to further discussion)

- Data size indication (subject to further discussion)

Unlike in R2D, there seems no clear benefit for power saving regardless of where to locate the above D2R control information in PDRCH. In this sense, introducing a dedicated L1 control region in PDRCH is not motivated. Furthermore, the above D2R control information are related to MAC functionality. Therefore, we suggest that RAN2 considers to specify the format to transmit the above D2R control information.

Proposal 7. RAN2 to discuss the format to carry the following information in MAC:

- Segment indication to support D2R segmentation

- Energy status report (if supported)

- Data size indication (if supported)

# Conclusion

In this paper, we have discussed remaining issues for AS ID, segmentation and handling of control information. The proposals are as follows.

AS ID

Proposal 1. Option 2 (reader assigns AS ID to device) is selected. The length of AS ID is shorter than random ID, e.g. 8-bit length can be considered for AS ID.

Proposal 2. Reader can assign AS ID via R2D message after Msg3.

Proposal 3. It is up to reader implementation to determine whether the assigned AS ID is maintained at device in Rel-19. The device shall keep volatile memory contents including AS ID for the certain duration.

Segmentation for D2R

Observation 1: The following scheme 1 (ACK/NACK-based retransmission with one-bit segment indication) is identified for D2R segmentation:

- ACK/NACK indication is used for reader to control whether device to transmit the next new segment or re-transmit previous segment.

- Device uses one-bit to indicate whether the data is segmented and whether it is the last segment.

Observation 2: If ACK is missed by the device, the scheme 1 will result in duplicated segment without awareness of the receiver.

Observation 3: The following scheme 2 is identified for D2R segmentation:

- ACK/NACK indication is used for reader to control whether device to transmit the next new segment or re-transmit previous segment.

- Device uses two-bit segment indication:

  - New segment bit (toggle bit) to indicate whether a new segment is transmitted by device

  - End mark bit to indicate whether the data is segmented and whether it is the last segment

Observation 4: Scheme 2 is more robust than scheme 1 for segment delivery, with marginal price for additional signaling overhead.

Observation 5: The following scheme 3 is identified for D2R segmentation:

- Differential encoded stop-go bit is used for reader control whether device to transmit the next new segment or re-transmit previous segment.

- Device uses one-bit to indicate whether the data is segmented and whether it is the last segment.

Observation 6: Scheme 3 is more robust than scheme 1 for segment delivery, with marginal price for additional one-bit storage in device memory for differential decoding.

Proposal 4. RAN2 confirms to support segment re-transmission for D2R segmentation.

Proposal 5. RAN2 down-select one of the following schemes for D2R segmentation:

Scheme 2: ACK/NACK based segment retransmission with 2-bit segment indication

Scheme 3: Differential encoded stop-go for segment retransmission with 1-bit segment indication

Handling of control information

Proposal 6. RAN2 to discuss designing a MAC CE with variable size to carry target ID and this MAC CE should be placed at the beginning of MAC PDU.

Proposal 7. RAN2 to discuss the format to carry the following information in MAC:

- Segment indication to support D2R segmentation

- Energy status report (if supported)

- Data size indication (if supported)

# Reference

[1] R2-2410193, Functionality aspects for A-IoT, Ericsson

[2] R2-2410594, Discussion on functionality aspects of ambient IoT, LG Electronics Inc

[3] R2-2410300, Discussion on ambient IoT segmentation functionality, Panasonic

[4] R2-2500676, Discussion on A-IoT paging, Panasonic