3GPP TSG-RAN WG2 #129	R2-2500673

Athens, Greece, Feb. 17th – 21st, 2025

Source:		NEC

Agenda item:		8.6.4	Conditional intra-CU LTM

Title:	Discussion on conditional intra-CU LTM

Document for:		Discussion and decision

- Introduction

During last RAN2 meeting, there was an initial discussion on intra-CU LTM and the following agreements were made [1]:

Agreements on C-LTM:

- The triggering condition of conditional LTM can be based on L3 measurement.

- CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution.

- The L1 execution condition of a candidate cell is associated to only one triggering event.

- For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell.

- To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition:

- The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

- The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

- The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

- For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

- The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

- Candidate cell TA is maintained by a new timer.

- For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

In this paper, we provide our further consideration on the support of conditional intra-CU LTM.

- Discussion

2.1 TA maintenance for the candidate cells

It was agreed that TA associated with the candidate cell is signalled to the UE from the source cell via a new MAC CE, and TA associated with the candidate cell is maintained by a new time alignment timer. Currently, for TA maintenance of serving cells, the concept of timing advance group (TAG) is supported. The serving cells associated with the same TAG use the same timing reference cell and the same Timing Advance value. Each TAG is associated with one time alignment timer. And at most 4 TAGs can be configured for one cell group. For TA maintenance of candidate cells, if the UE maintains TA and time alignment timer for each of LTM candidate cells, it means that the UE needs to at most maintain 8 additional TA and running 8 additional time alignment timers for one cell group, even if some of the LTM candidate cells are located in the same DU or TRP. Therefore, we think it is beneficial to apply the concept of TAG to the TA maintenance of LTM candidate cells.

Proposal 1: Support per TAG TA maintenance for the LTM candidate cells.

Before UE switches to the candidate cell, the UE may maintain TA associated with LTM candidate cells by running the new time alignment timer associated with candidate cells. If the UE switches to one of the candidate cells, i.e., one of the candidate cells becomes the serving cell of the UE, whether the UE stops or continues TA maintenance for the target candidate cell should be further discussed.

Proposal 2: To discuss on the following options for the TA maintenance of the target candidate cell upon/after LTM cell switch

- Option 1. UE continues TA maintenance for the target candidate cell, i.e., continue the running of time alignment timer associated with the target candidate cell

- Option 2. UE stops TA maintenance for the target candidate cell

2.2 CLTM cell switch execution

During last meeting, there was a short discussion on whether the event evaluation for CLTM is based on one or multiple beams. Proponent of multiple beams think that it can ensure that the triggering of cell switch is based on more representative quality of a concerned candidate cell event evaluation based on multiple beams, rather than one single beam. But we think this condition would be too restrictive to trigger LTM, as beam of the serving cell is always the current beam, which are mostly the best beam. Supporting multiple beams of the candidate cell means that multiple beams of the candidate cell need to be offset better than the best beam of the serving cell, and we think that there is a risk that the LTM triggering will be too late and results in performance degradation. Therefore, we think the same as L1 event triggered measurement reporting, LTM cell switch procedure is triggered when at least one beam fulfils the associated execution condition is sufficient.

Proposal 3: The LTM cell switch procedure is triggered when at least one beam of the candidate cell fulfils the associated execution condition.

Upon initiation of LTM cell switch execution, the MAC layer shall determine whether RACH-less LTM can be performed. And one factor for initiating RACH-less LTM is that the UE has a valid TA for the target LTM candidate cell. To determine whether the TA for the target candidate cell is valid, at least the running of time alignment timer associated with the LTM candidate cell should be taken into account, i.e., if the time alignment timer expires, the UE shall consider that there is no valid TA for the LTM candidate cell.

For Rel-19 LTM, to avoid large signalling overhead, the network may not have measurement report from the UE to tigger anther early UL sync for the LTM candidate cell in time. For example, the UE is only configured with event based L1 measurement report for one LTM candidate cell, and if the L1 report indicates that the candidate cell’s radio condition is good enough, the source cell triggers PDCCP order for early TA acquisitor for the UE. However, when the channel condition between the UE and candidate cell changes, the source cell may be not able to trigger another PDCCH ordered early UL sync, because the event triggered the L1 measurement report is not fulfilled. Therefore, it is beneficial for the UE to also take the change of measurement result for the candidate cell into account when determining the TA validity, i.e., if the measurement result (e.g., RSRP) of the LTM candidate cell changes above a threshold compared to the measurement result when the TA is received, the TA is considered as invalid, similar as it is supported in CG-SDT.

Proposal 4: UE determines whether there is valid TA for RACH-less conditional LTM by taking both the running of time alignment timer and the change of measurement result for the candidate cell into account.

In Rel-18 LTM cell switch, the time alignment timer of PTAG is started/restarted upon initiation of RACH-less LTM cell switch procedure, and the network configuration ensures that the time alignment timer will not expire during the execution of RACH-less LTM cell switch procedure. However, in case of CLTM, the time alignment timer associated with the candidate cell is already running when the LTM cell switch execution is initiated, and if the UE starts/restarts the time alignment timer for PTAG upon initiation of RACH-less LTM, it means that validity time of the TA received from the source cell is extended, which results in LTM failure as the TA associated with the target candidate cell is already invalid. Therefore, we propose RAN2 to discuss the following options.

Proposal 5: To discuss on the following options for PTAG handling:

- Option1. Upon initiation of RACH-less CLTM, the UE applies the TA associated with the target candidate cell for PTAG, and starts/restarts the time alignment timer associated with the PTAG

- Option 2. The UE performs RACH-less CLTM based on the TA and time alignment timer associated with the target candidate cell, and upon completion of the RACH-less CLTM, the UE applies the TA associated with the target candidate cell for PTAG, and starts/restarts the time alignment timer associated with the PTAG

At previous meeting, RAN2 agreed to support CG-based RACH-less LTM, and FFS on whether to support DG based LTM. For CLTM, since the network does not have information of when the LTM cell switch is triggered, and which beam or TCI state will be selected or used by the UE, it cannot provide UL grant for dynamic scheduling of the UE properly. And a “bye message” from the UE to the network would introduce additional latency and complexity, therefore we think there is no need to support DG-based RACH-less conditional LTM.

Proposal 6: Do not support DG-based first UL transmission for RACH-less conditional LTM.

It was agreed to support CG-based RACH-less LTM. And for CG-based RACH-less LTM, the UE needs to perform a beam selection to further determine valid CG resource for first PUSCH transmission. And how to select the beam for CG-based RACH-less is one remaining issue. Since the L1 event for execution condition is one or more beams associated with the target candidate cell fulfils a condition, which means that the beam fulfilling the condition are stable good enough, therefore the selected beam should be at least one beam fulfilling the conditions of L1 event.

Proposal 7: For RACH-less conditional LTM, UE selects a beam of the target cell which fulfils the L1 event of the execution condition.

To faster the delay caused by the DL synchronization, it is also beneficial that if the selected beam is associated with the activated TCI state. For example, there are multiple beams fulfilling the L1 event, and s of the multiple beams is associated with activated TCI state, the UE should select one beam which is also associated with activated state.

Proposal 7a: For RACH-less conditional LTM, UE selects a beam of the target cell which fulfils the L1 event of the execution condition and is associated with an activated TCI state, if any.

2.3 LTM recovery

In Rel-18, LTM recovery is only RACH-based, even if the UE has measured TA for the target cell of the LTM recovery. For conditional LTM, the UE may have TA value of the candidate cell provided by the network, or measured by itself, and RACH-less conditional LTM can be performed. For LTM recovery, one question is whether the LTM recovery to a candidate cell of conditional LTM can be also RACH-less or RACH-based only. In our understanding, except the triggering conditions and beam selection, there is no other difference between the initial LTM cell switch and the LTM recovery for RACH-less based conditional LTM. And RACH-less based LTM recovery can be easily supported in Rel-19 to further reduce the latency of LTM recovery procedure.

Proposal 8: RAN2 to discuss whether to support RACH-less based LTM recovery in case that there is valid TA maintained at the UE side for the selected CLTM candidate cell.

2.4 Co-existence of LTM and CLTM

In the end, we would like to discuss whether co-existence between LTM and C-LTM can be supported in rel-19. In our understanding, there is no specific spec impact or difficulty to support the co-existence of LTM and CLTM. So it is up to network implementation to support it.

Proposal 9: Co-existence of LTM and CLTM can be supported based on network implementation.

# 3. Conclusion

In this paper, we provide our initial consideration for the support of CLTM, and have the following proposals:

TA maintenance for the candidate cells

Proposal 1: Support per TAG TA maintenance for the LTM candidate cells.

Proposal 2: To discuss on the following options for the TA maintenance of the target candidate cell upon/after LTM cell switch

- Option 1. UE continues TA maintenance for the target candidate cell, i.e., continue the running of time alignment timer associated with the target candidate cell

- Option 2. UE stops TA maintenance for the target candidate cell

CLTM cell switch execution

Proposal 3: The LTM cell switch procedure is triggered when at least one beam of the candidate cell fulfils the associated execution condition.

Proposal 4: UE determines whether there is valid TA for RACH-less conditional LTM by taking both the running of time alignment timer and the change of measurement result for the candidate cell into account.

Proposal 5: To discuss on the following options for PTAG handling:

- Option1. Upon initiation of RACH-less CLTM, the UE applies the TA associated with the target candidate cell for PTAG, and starts/restarts the time alignment timer associated with the PTAG

- Option 2. The UE performs RACH-less CLTM based on the TA and time alignment timer associated with the target candidate cell, and upon completion of the RACH-less CLTM, the UE applies the TA associated with the target candidate cell for PTAG, and starts/restarts the time alignment timer associated with the PTAG

Proposal 6: Do not support DG-based first UL transmission for RACH-less conditional LTM.

Proposal 7: For RACH-less conditional LTM, UE selects a beam of the target cell which fulfils the L1 event of the execution condition.

Proposal 7a: For RACH-less conditional LTM, UE selects a beam of the target cell which fulfils the L1 event of the execution condition and is associated with an activated TCI state, if any.

LTM recovery

Proposal 8: RAN2 to discuss whether to support RACH-less based LTM recovery in case that there is valid TA maintained at the UE side for the selected CLTM candidate cell.

Co-existence of LTM and CLTM

Proposal 9: Co-existence of LTM and CLTM can be supported based on network implementation.

# Reference

[1] meeting report of RAN2 #128.