3GPP TSG-RAN WG2 #129 	R2-2500684

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.7.5  RLC AM Enhancements

Source:	NEC

Title:	RLC AM enhancements

Document for:	Discussion, Decision

# 1.Introduction

Following are latest RAN2 agreements on how to enhance RLC AM mode to avoid unnecessary retransmission and how to retransmit data timely:

Agreements on timely retransmissions - Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them. Agreements on avoiding unnecessary retransmissions - RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU - RX side abandons the SDU based on a local timer - Special handling to avoid PDCP control PDU discard is not needed. - A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs. The timer starts when the gap is detected at RLC layer. - The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report. - FFS if some indication is sent from Tx to Rx. The assumption is this is not a full status report, but something simple (if needed)

In this document, we further discuss the remaining details of the combined approach to avoid unnecessary retransmission and discuss details of timely retransmissions solution

# 2 Avoiding unnecessary retransmissions (combined)

## 2.1 Details of window operation

To avoid unnecessary retransmissions, we agreed the combined approach which includes two basic aspects: 1) TX side stops transmissions of an outdated SDU and 2) RX side abandons the SDUs based on a local timer. The window operation at TX and RX side are relatively independent, hereafter, we looked into details:

- TX side stops transmissions of an outdated SDU. An SDU become outdated when its PDB runs out. Moreover PDB is reflected in discard timer running at PDCP layer, hence we believe that discard indication from PDCP layer can be reused to determine that a SDU is outdated at TX side. In legacy, TX side will stop a SDU’s retransmission once it is ACKed, additionally, TX shall stop retransmission of an outdated/discarded SDU. This should be further taken into account to move the TX window as below, i.e., lower edge of TX window shall move up to the smallest SN which has not been ACKed and has not been discarded.

TX window operation

- RX side abandons the SDUs based on a local timer. as many companies proposed, we can reuse RLC UM RX operation, where RLC UM RX side determines that an SDU should be abandoned based on a local timer t-Reassembly and variable RX_Timer_Trigger. The timer t-Reassembly is initiated when missing RLC SDU(s) are detected (i.e., gap appears between received RLC SDUs), RX_Timer_Trigger variable holds the value of the SN following the SN which triggered t-Reassembly. when the timer t-Reassembly expires, RX side no longer waits for the missed RLC PDU(s) and move the reassembly window forward. The only changes would include 1) give new names to the local timer and the variable, because the t-Reassembly timer is currently used for STATUS report purposes in RLC AM; 2)when the local timer expires, RX side will move the lower edge of RLC AM window forward, instead of reassembly window in RLC UM. The RX window operation is shown in below

RX window operation

Here we summarize the TX and RX side window operation of combined approach as below for agreement, example of specification change details can be seen in provided TP:

Proposal 1a: For Tx side operation, a SDU become outdated upon receiving the discard indication from PDCP, and the lower edge of Tx window shall move to smallest SN of SDUs not yet ACKed and not discarded

Proposal 1b: For Rx side operation(same as RLC UM RX), we introduce a new variable in addition to the agreed new timer, the new variable holds the SN value at which the RX window should move forward upon the new timer expiry

## 2.2 RX to TX feedback

It is agreed “The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report” .We look into the existing specification, A STATUS PDU is constructed as shown in below:

When constructing a STATUS PDU, the AM RLC entity shall: - for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer: - for an RLC SDU for which no byte segments have been received yet: - include in the STATUS PDU a NACK_SN which is set to the SN of the RLC SDU. - for a continuous sequence of byte segments of a partly received RLC SDU that have not been received yet: - include in the STATUS PDU a set of NACK_SN, SOstart and SOend. - for a continuous sequence of RLC SDUs that have not been received yet: - include in the STATUS PDU a set of NACK_SN and NACK range; - include in the STATUS PDU, if required, a pair of SOstart and SOend. - set the ACK_SN to the SN of the next not received RLC SDU which is not indicated as missing in the resulting STATUS PDU.

As above, in a STAUS PDU, Rx side will list all RLC SDUs which has not been completely received, start from RX-NEXT till ACK_SN.

After introducing the “combined” approach for avoiding unnecessary RLC retransmissions, upon determining to abandon one or multiple RLC SDUs, the variable RX-NEXT which indicate the lower edge of the receiving window will be updated to a higher value (i.e., to advance the receive window) then the SN of the abandoned SDUs. Hence the abandoned RLC SDUs will be moved out of the Rx window and not included in STATUS PDU, which will be interpreted as ACKed at Tx side. Additionally, to ensure more timely synchronization of the Tx and Rx windows, a new trigger for STATUS report may be introduced, for example, when one or more RLC SDUs or PDUs are discarded by the receiving side.

Proposal 2: RAN2 confirm that STATUS PDU will ACK abandoned SDUs without specification change.

Proposal 3: RAN2 consider additional STATUS reporting trigger when one or more RLC SDUs are abandoned at Rx side for faster synchronization.

## 2.3 Capability and configuration

It is obvious the feature of terminating retransmission of an outdated SDU is only needed for RLC AM DRB with very short latency requirements, and a UE usually can be configured with multiple RLC AM DRBs. So the feature should be enabled per AM DRB under NW control. To enable this feature, NW side has to configure the new timer for Rx side operation. Beside RLC AM entity is a bi-direction entity, so configurating the new timer can implicitly enable the feature in both uplink and downlink direction at the same time.

Proposal 4: Enabled the feature “terminating retransmission of outdated SDU” implicitly per AM DRB via configuring the new timer etc.

To enable NW to configure this feature, UE capability is necessary:

Proposal 5: Introduce a UE capability of support the feature “terminating retransmission of outdated SDU”

## 2.4 Other Spec Impact

Because legacy RLC AM never give up retransmission unless it is ACKed or RLF is triggered, as shown below, when discard indication of a SDU is received from PDCP, the transmitting side of an AM RLC cannot discard the SDU if the RLC SDU or its segment has not been submitted to lower layer, i.e., its transmission has been attempted. However, if terminating retransmission of outdated SDU is enable for RLC AM DRB, this condition shall be lifted.

## 5.4 SDU discard procedures When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.

Proposal 6: Update the section “SDU discard procedures” to allow the transmitting side of an AM RLC entity to discard the indicated RLC SDU and corresponding RLC PDU (consequently it is not pending for retransmission), even if the RLC SDU or a segment there of has been submitted to the lower layers.

With the understanding that RLC AM will not retransmit the discarded/outdated RLC PDU, These are not SDUs pending for retransmission, and will not be taken into account for data volume calculation purpose, “Data volume calculation” section can be keep as it

Proposal 7: if P6 is agreeable, RAN2 confirm that discarded RLC PDU is not ones pending for retransmission, and will not be taken into account for Data volume calculation, i.e., Data volume calculation section can be keep as it.

# 3 Timely retransmissions

To achieve timely RLC retransmission for XR, RAN2 agreed:

Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them.

## 3.1 Autonomous retransmission

Autonomous retransmission could be triggered in following conditions:

- When the remaining time of a RLC PDU drop to a configured threshold

- After a PDU has failed a configured number of ARQ retransmissions

- After a PDU has failed a configured number of HARQ retransmissions

Solution1) sounds more simple and reasonable to us and it is our preference. Advantage of this solution is to provide fastest retransmission action and fulfil the need of timely retransmission in most straight forward way. Because the solution bring extra retransmission overhead, and SDU with short remaining time may or may not already be received by RX, so we should limit to one time autonomous retransmission. And the TX window could move forward without waiting for any further feedback.

Solution 2) does not solve the issue fully. A given number of ARQ retransmission could last longer or shorter , which depends on the variable round trip delay, consequently it cannot guarantee timely retransmission.

Solution 3), it is true that scheduler may stop HARQ retransmissions after a configured number of HARQ retransmissions, then it would be a good time to trigger ARQ retransmission. However, we think we can leave to network implementation to carefully configure a proper remaining time threshold ,which is after the time needed for maximum number of HARQ retransmissions but before the PDB run out.

In general autonomous retransmission is suitable when system has enough capacity and so it should be enabled and disabled by network as we have been agreed

Proposal 8: if enabled, one time of autonomous retransmission is triggered based on remaining time

## 3.2 Polling enhancement (TBU)

There are different flavours to enhance polling for timing retransmission:

- trigger polling when the remaining time of a SDU less than a threshold,

it should be clarified that this should only apply to SDUs in the buffer which are waiting for acknowledgment. For SDU which has been ACKed or NACKed but waiting for the retransmission opportunity, there is no need to trigger polling. The specification change could be as follows

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall: - increment PDU_WITHOUT_POLL by one; - increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU; - if PDU_WITHOUT_POLL >= pollPDU; or - if BYTE_WITHOUT_POLL >= pollByte: - include a poll in the AMD PDU as described below. Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU; or - if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling); or - if [remainingtimeforpolling] is configured and there are RLC SDU or an RLC SDU segment waiting for acknowledgment and with remaining time less then [remainingtimeforpolling] - include a poll in the AMD PDU as described below.

- Use shorter values for polling parameters e.g., pollPDU, pollByte, t-PollRetransmit, when there is delay critical data in buffer

With this solution, network potentially need to configure a second set of polling parameters. Polling parameters including poollPDU, PollByte and t-PollRetransmit are not running per RLC SDUs but running per RLC entity, so it is not possible to apply different value for delay-critical SDUs and non-delay- critical SDUs. The only possible way is to switch to shorter values upon there is one or multiple delay-critical SDUs.

Our preference is solution a, which is more straightforward and can trigger polling timely.

Proposal 9: include a poll when any RLC SDU or RLC SDU segment waiting for acknowledgment has  remaining time less a configured threshold.

# 4 Conclusion

In this contribution, we first discussed the remaining details of the combined approach for avoiding unnecessary retransmission, we have following proposal and an example TP is also provided in section 5: :

Proposal 1a: For Tx side operation, a SDU become outdated upon receiving the discard indication from PDCP, and the lower edge of Tx window shall move to smallest SN of SDUs not yet ACKed and not discarded

Proposal 1b: For Rx side operation(same as RLC UM RX), we introduce a new variable in addition to the agreed new timer, the new variable holds the SN value at which the RX window should move forward upon the new timer expiry

Proposal 2: RAN2 confirm that STATUS PDU will ACK abandoned SDUs without specification change.

Proposal 3: RAN2 consider additional STATUS reporting trigger when one or more RLC SDUs are abandoned at Rx side for faster synchronization.

Proposal 4: Enabled the feature “terminating retransmission of outdated SDU” implicitly per AM DRB via configuring the new timer etc.

Proposal 5: Introduce a UE capability of support the feature “terminating retransmission of outdated SDU”

Proposal 6: Update the section “SDU discard procedures” to allow the transmitting side of an AM RLC entity to discard the indicated RLC SDU and corresponding RLC PDU (consequently it is not pending for retransmission), even if the RLC SDU or a segment there of has been submitted to the lower layers.

Proposal 7: if P6 is agreeable, RAN2 confirm that discarded RLC PDU is not ones pending for retransmission, and will not be taken into account for Data volume calculation, i.e., Data volume calculation section can be keep as it.

Secondly we discussed details of autonomous retransmission and polling enhancement for timely retransmission:

Proposal 8: if enabled, one time of autonomous retransmission is triggered based on remaining time

Proposal 9: include a poll when any RLC SDU or RLC SDU segment waiting for acknowledgment has  remaining time less a configured threshold.

# 5 TP of the combined approach

### 5.2.3	AM data transfer

#### 5.2.3.1	Transmit operations

##### 5.2.3.1.1	General

The transmitting side of an AM RLC entity shall prioritize transmission of RLC control PDUs over AMD PDUs. The transmitting side of an AM RLC entity shall prioritize transmission of AMD PDUs containing previously transmitted RLC SDUs or RLC SDU segments over transmission of AMD PDUs containing not previously transmitted RLC SDUs or RLC SDU segments.

The transmitting side of an AM RLC entity shall maintain a transmitting window according to the state variable TX_Next_Ack as follows:

-	a SN falls within the transmitting window if TX_Next_Ack <= SN < TX_Next_Ack + AM_Window_Size;

-	a SN falls outside of the transmitting window otherwise.

The transmitting side of an AM RLC entity shall not submit to lower layer any AMD PDU whose SN falls outside of the transmitting window.

For each RLC SDU received from the upper layer, the AM RLC entity shall:

-	associate a SN with the RLC SDU equal to TX_Next and construct an AMD PDU by setting the SN of the AMD PDU to TX_Next;

-	increment TX_Next by one.

When submitting an AMD PDU that contains a segment of an RLC SDU, to lower layer, the transmitting side of an AM RLC entity shall:

-	set the SN of the AMD PDU to the SN of the corresponding RLC SDU.

The transmitting side of an AM RLC entity can receive a positive acknowledgement (confirmation of successful reception by its peer AM RLC entity) for an RLC SDU by the following:

-	STATUS PDU from its peer AM RLC entity.

If [t-Giveup] is not configured, when receiving a positive acknowledgement for an RLC SDU with SN = x, the transmitting side of an AM RLC entity shall:

-	send an indication to the upper layers of successful delivery of the RLC SDU;

-	set TX_Next_Ack equal to the SN of the RLC SDU with the smallest SN, whose SN falls within the range TX_Next_Ack <= SN <= TX_Next and for which a positive acknowledgment has not been received yet.

Else if [t-Giveup] is configured, when receiving a positive acknowledgement from its peer AM RLC entity or receiving a discard indication from its upper layer for an RLC SDU with SN = x, the transmitting side of an AM RLC entity shall:

-	send an indication to the upper layers of successful delivery of the RLC SDU if a positive acknowledgement is received;

-	set TX_Next_Ack equal to the SN of the RLC SDU with the smallest SN, whose SN falls within the range TX_Next_Ack <= SN <= TX_Next and for which a positive acknowledgment has not been received yet and it has not been indicated as discarded from upper layer.

#### 5.2.3.2	Receive operations

##### 5.2.3.2.1	General

The receiving side of an AM RLC entity shall maintain a receiving window according to the state variable RX_Next as follows:

-	a SN falls within the receiving window if RX_Next <= SN < RX_Next + AM_Window_Size;

-	a SN falls outside of the receiving window otherwise.

When receiving an AMD PDU from lower layer, the receiving side of an AM RLC entity shall:

-	either discard the received AMD PDU or place it in the reception buffer (see clause 5.2.3.2.2);

-	if the received AMD PDU was placed in the reception buffer:

-	update state variables, reassemble and deliver RLC SDUs to upper layer and start/stop t-Reassembly as needed (see clause 5.2.3.2.3).

When t-Reassembly expires, the receiving side of an AM RLC entity shall:

-	update state variables and start t-Reassembly as needed (see clause 5.2.3.2.4).

##### 5.2.3.2.2	Actions when an AMD PDU is received from lower layer

When an AMD PDU is received from lower layer, where the AMD PDU contains byte segment numbers y to z of an RLC SDU with SN = x, the receiving side of an AM RLC entity shall:

-	if x falls outside of the receiving window; or

-	if byte segment numbers y to z of the RLC SDU with SN = x have been received before:

-	discard the received AMD PDU.

-	else:

-	place the received AMD PDU in the reception buffer;

-	if some byte segments of the RLC SDU contained in the AMD PDU have been received before:

-	discard the duplicate byte segments.

##### 5.2.3.2.3	Actions when an AMD PDU is placed in the reception buffer

When an AMD PDU with SN = x is placed in the reception buffer, the receiving side of an AM RLC entity shall:

-	if x >= RX_Next_Highest:

-	update RX_Next_Highest to x+ 1.

-	if all bytes of the RLC SDU with SN = x are received:

-	reassemble the RLC SDU from AMD PDU(s) with SN = x, remove RLC headers when doing so and deliver the reassembled RLC SDU to upper layer;

-	if x = RX_Highest_Status:

-	update RX_Highest_Status to the SN of the first RLC SDU with SN > current RX_Highest_Status for which not all bytes have been received.

-	if x = RX_Next:

-	update RX_Next to the SN of the first RLC SDU with SN > current RX_Next for which not all bytes have been received.

-	if t-Reassembly is running:

-	if RX_Next_Status_Trigger = RX_Next; or

-	if RX_Next_Status_Trigger = RX_Next + 1 and there is no missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU; or

-	if RX_Next_Status_Trigger falls outside of the receiving window and RX_Next_Status_Trigger is not equal to RX_Next + AM_Window_Size:

-	stop and reset t-Reassembly.

-	if t-Reassembly is not running (includes the case t-Reassembly is stopped due to actions above):

-	if RX_Next_Highest> RX_Next +1; or

-	if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU:

-	start t-Reassembly;

-	set RX_Next_Status_Trigger to RX_Next_Highest.

-	if t-Giveup is configured:

-	if t-Giveup is running:

-	if RX_Giveup_Trigger <= RX_Next; or

-	if RX_Giveup_Trigger falls outside of the receiving window and RX_Timer_Trigger is not equal to RX_Next_Highest; or

-	if RX_Next_Highest = RX_Next + 1 and there is no missing byte segment of the RLC SDU associated with SN = RX_Next before the last byte of all received segments of this RLC SDU:

-	stop and reset t-Giveup.

-	if t-Giveup is not running (includes the case when t-Giveup is stopped due to actions above):

-	if RX_Next_Highest > RX_Next + 1; or

-	if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the RLC SDU associated with SN = RX_Next before the last byte of all received segments of this RLC SDU:

-	start t-Giveup;

-	set RX_Giveup_Trigger to RX_Next_Highest.

##### 5.2.3.2.4	Actions when t-Reassembly expires

When t-Reassembly expires, the receiving side of an AM RLC entity shall:

-	update RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_Next_Status_Trigger for which not all bytes have been received;

-	if RX_Next_Highest> RX_Highest_Status +1: or

-	if RX_Next_Highest = RX_Highest_Status + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Highest_Status before the last byte of all received segments of this SDU:

-	start t-Reassembly;

-	set RX_Next_Status_Trigger to RX_Next_Highest.

##### 5.2.3.2.x	Actions when t-Giveup expires

When t-Giveup expires, the receiving side of an AM RLC entity shall:

-	update RX_Next to the SN of the first SN >= RX_Timer_Trigger that has not been received;

-	discard all segments with SN < updated RX_Next;

-	if RX_Next_Highest > RX_Next+ 1; or

-	if RX_Next_Highest = RX_Next+ 1 and there is at least one missing byte segment of the RLC SDU associated with SN = RX_Next before the last byte of all received segments of this RLC SDU:

-	start t-Giveup;

-	set RX_Giveup_Trigger to RX_Next_Highest.

## 5.3	ARQ procedures

### 5.3.1	General

ARQ procedures are only performed by an AM RLC entity.

### 5.3.2	Retransmission

The transmitting side of an AM RLC entity can receive a negative acknowledgement (notification of reception failure by its peer AM RLC entity) for an RLC SDU or an RLC SDU segment by the following:

-	STATUS PDU from its peer AM RLC entity.

When receiving a negative acknowledgement for an RLC SDU or an RLC SDU segment by a STATUS PDU from its peer AM RLC entity, the transmitting side of the AM RLC entity shall:

-	if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer:

-	consider the RLC SDU or the RLC SDU segment for which a negative acknowledgement was received for retransmission.

When an RLC SDU or an RLC SDU segment is considered for retransmission, the transmitting side of the AM RLC entity shall:

-	if the RLC SDU or RLC SDU segment is considered for retransmission for the first time:

-	set the RETX_COUNT associated with the RLC SDU to zero.

-	else, if it (the RLC SDU or the RLC SDU segment that is considered for retransmission) is not pending for retransmission already and the RETX_COUNT associated with the RLC SDU has not been incremented due to another negative acknowledgment in the same STATUS PDU:

-	increment the RETX_COUNT.

-	if RETX_COUNT = maxRetxThreshold:

-	indicate to upper layers that max retransmission has been reached.

When retransmitting an RLC SDU or an RLC SDU segment, the transmitting side of an AM RLC entity shall:

-	if needed, segment the RLC SDU or the RLC SDU segment;

-	form a new AMD PDU which will fit within the total size of AMD PDU(s) indicated by lower layer at the particular transmission opportunity;

-	submit the new AMD PDU to lower layer.

When forming a new AMD PDU, the transmitting side of an AM RLC entity shall:

-	only map the original RLC SDU or RLC SDU segment to the Data field of the new AMD PDU;

-	modify the header of the new AMD PDU in accordance with the description in clause 6.2.2.4;

-	set the P field according to clause 5.3.3.

### 5.3.3	Polling

#### 5.3.3.1	General

An AM RLC entity can poll its peer AM RLC entity in order to trigger STATUS reporting at the peer AM RLC entity.

#### 5.3.3.2	Transmission of a AMD PDU

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall:

-	increment PDU_WITHOUT_POLL by one;

-	increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU;

-	if PDU_WITHOUT_POLL >= pollPDU; or

-	if BYTE_WITHOUT_POLL >= pollByte:

-	include a poll in the AMD PDU as described below.

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall:

-	if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU; or

-	if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling);

-	include a poll in the AMD PDU as described below.

NOTE:	Empty RLC buffer (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) should not lead to unnecessary polling when data awaits in the upper layer. Details are left up to UE implementation.

To include a poll in an AMD PDU, the transmitting side of an AM RLC entity shall:

-	set the P field of the AMD PDU to "1";

-	set PDU_WITHOUT_POLL to 0;

-	set BYTE_WITHOUT_POLL to 0.

Upon submission of an AMD PDU including a poll to lower layer, the transmitting side of an AM RLC entity shall:

-	set POLL_SN to the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer;

-	if t-PollRetransmit is not running:

-	start t-PollRetransmit.

-	else:

-	restart t-PollRetransmit.

#### 5.3.3.3	Reception of a STATUS report

Upon reception of a STATUS report from the receiving RLC AM entity the transmitting side of an AM RLC entity shall:

-	if the STATUS report comprises a positive or negative acknowledgement for the RLC SDU with sequence number equal to POLL_SN:

-	if t-PollRetransmit is running:

-	stop and reset t-PollRetransmit.

#### 5.3.3.4	Expiry of t-PollRetransmit

Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall:

-	if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements); or

-	if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling):

-	consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission; or

-	consider any RLC SDU which has not been positively acknowledged for retransmission.

-	include a poll in an AMD PDU as described in clause 5.3.3.2.

### 5.3.4	Status reporting

An AM RLC entity sends STATUS PDUs to its peer AM RLC entity in order to provide positive and/or negative acknowledgements of RLC SDUs (or portions of them).

Triggers to initiate STATUS reporting include:

-	Polling from its peer AM RLC entity:

-	When an AMD PDU with SN = x and the P field set to "1" is received from lower layer, the receiving side of an AM RLC entity shall:

-	if the AMD PDU is to be discarded as specified in clause 5.2.3.2.2; or

-	if x < RX_Highest_Status or x >= RX_Next + AM_Window_Size:

-	trigger a STATUS report.

-	else:

-	delay triggering the STATUS report until x < RX_Highest_Status or x >= RX_Next + AM_Window_Size.

NOTE 1:	This ensures that the RLC Status report is transmitted after HARQ reordering.

-	Detection of reception failure of an AMD PDU

-	The receiving side of an AM RLC entity shall trigger a STATUS report when t-Reassembly expires.

NOTE 2:	The expiry of t-Reassembly triggers both RX_Highest_Status to be updated and a STATUS report to be triggered, but the STATUS report shall be triggered after RX_Highest_Status is updated.

When STATUS reporting has been triggered, the receiving side of an AM RLC entity shall:

-	if t-StatusProhibit is not running:

-	at the first transmission opportunity indicated by lower layer, construct a STATUS PDU and submit it to lower layer.

-	else:

-	at the first transmission opportunity indicated by lower layer after t-StatusProhibit expires, construct a single STATUS PDU even if status reporting was triggered several times while t-StatusProhibit was running and submit it to lower layer.

When a STATUS PDU has been submitted to lower layer, the receiving side of an AM RLC entity shall:

-	start t-StatusProhibit.

When constructing a STATUS PDU, the AM RLC entity shall:

-	for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer:

-	for an RLC SDU for which no byte segments have been received yet:

-	include in the STATUS PDU a NACK_SN which is set to the SN of the RLC SDU.

-	for a continuous sequence of byte segments of a partly received RLC SDU that have not been received yet:

-	include in the STATUS PDU a set of NACK_SN, SOstart and SOend.

-	for a continuous sequence of RLC SDUs that have not been received yet:

-	include in the STATUS PDU a set of NACK_SN and NACK range;

-	include in the STATUS PDU, if required, a pair of SOstart and SOend.

-	set the ACK_SN to the SN of the next not received RLC SDU which is not indicated as missing in the resulting STATUS PDU.

## 5.4	SDU discard procedures

When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall:

If t-Giveup is not configured:

-	 discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.

Else if t-Giveup is configured

- discard the indicated RLC SDU (i.e., neither the RLC SDU nor a segment thereof are pending for initial transmission or retransmission)

## 5.5	Data volume calculation

For the purpose of MAC buffer status reporting, the UE shall consider the following as RLC data volume:

-	RLC SDUs and RLC SDU segments that have not yet been included in an RLC data PDU;

-	RLC data PDUs that are pending for initial transmission;

-	RLC data PDUs that are pending for retransmission (RLC AM).

For the purpose of MAC delay status reporting, the UE shall consider the following as delay-critical RLC data volume:

-	delay-critical RLC SDUs and delay-critical RLC SDU segments that have not yet been included in an RLC data PDU;

-	RLC data PDUs pending for initial transmission, and containing a delay-critical RLC SDU or a delay-critical RLC SDU segment;

-	RLC data PDUs that are pending for retransmission (RLC AM).

In addition, if a STATUS PDU has been triggered and t-StatusProhibit is not running or has expired, the UE shall estimate the size of the STATUS PDU that will be transmitted in the next transmission opportunity, and consider this as part of RLC data volume for MAC buffer status reporting and as part of delay-critical RLC data volume for MAC delay status reporting.