3GPP TSG-RAN WG2#129												R2-2500685

Athens, Greece, Feb. 17th – 21st 2025

Agenda Item:	8.8.2 Downlink Coverage Enhancement

Source: 	NEC

Title:	        Downlink coverage enhancement

Document for:	 Discussion

# Introduction

During last RANP, the WID has been updated that the system level coverage enhancement with beam hopping will focus on SSB periodicity extension up to 160ms (i.e., 320ms and longer periodicity are excluded). On link level coverage enhancement, Msg4 PDSCH repetition is being discussed in RAN1. This document will address RAN2 remaining issues on SSB extension and msg4 PDSCH repetition.

# Discussion

## Access control

As below, RAN2 agreed during RAN2 #128 meeting to introduce a new barring bit, in order to bar pre-Rel19 and Rel-19 UEs not supporting DL-CE, but we have not confirmed yet if it is needed.

If we need to bar UEs not supporting DL-CE (via legacy mechanism), we introduce a new barring in SIB1 to be able to selectively bar “UEs supporting DL-CE” (FFS if we will finally refer to “UEs supporting extended SSB periodicity” instead). FFS on the details (e.g. how many bits, etc.)

Since it is confirmed in RANP that the SSB periodicity is extended only up to 160ms, as we agreed in RAN2 #127 meeting, there is no RAN2 impact regarding SSB configuration:

if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration; If the SSB periodicity is larger than 160ms, several parameters may need to be extended.

The new barring bit if needed will be added in SIB1 of an cell with extended SSB periodicity. if an legacy UE already detected the cell and able to read its SIB1to check the new barring bit, it also be able to read SIB1 including the SSB periodicity configuration up to 160ms. In conclusion, we see no need to kick legacy UE or UE not support extended SSB periodicity, out of the cell at the stage of SIB1 reading:

no need of new barring bit in SIB1 due to extended SSB periodicity.

## Cell reselection enhancement

It was discussed whether to prevent UE not supporting DL CE to cell reselect to a DL CE cell:

(also depending on the details of the RAN1 solution) we can further consider methods to allow UEs not supporting DL CE to down-prioritize or prevent re-selection to cells operating with DL CE.

more specifically, the solution is to include DL CE cells in the legacy excluded cell lists , and at the same introduce a new cell list to indicate these cells to UE capable DL CE feature. This will allow DL CE capable UEs cell-reselecting to these cells, but prevent DL CE incapable UEs from cell-reselecting to these cell at the same time.

However, with the same logic as same for not introducing new barring bit. We do not see the necessity to prevent legacy UEs to cell reselect to a cell with extended SSB periodicity, if the legacy UE by chance can detect the cell based on neighbouring measurement procedure.

no need to enhance cell reselection due to extended SSB periodicity.

## Possible SMTC impact

With a typical cellular network model as below, one serving cell has 6 neighbouring cells. With beam hopping, it is possible that SSB transmission of every neighbouring cell will be on different time. In this extreme case, maximum 4 SMTC configuration seems not enough.

Different solutions has been proposed:

Option1: deploy neighbour cells using different frequencies:  SMTC is configured per frequency, 4 SMTC per frequency configuration would mean maximum n*4 SMTCs for n frequencies. It is up to network implementation to make sure that maximum 4 neighbouring cell are deployed with same frequency if their SSB transmission are all in different time window and need separated SMTC configuration.

Option2: place some(not necessary all) neighbouring cells in the same beam hopping cell group, i.e., with same SSB transmission timing. in this case, one SMTC configuration could cover neighbouring cells in the same beaming hopping group. and less than 6 SMTCs are needed. it is up to network implementation to make sure all (intra-frequency)neighbouring cell are not in more than 4 beam hopping groups.

Option 3: SSB transmission time of some/all neighbouring cells are evenly spaced. In principle, it should be possible that NW transmission can hop from one cell group to another cell group with regular pace. for example, NW transmits SSB every 20ms, but hop between 4 cell groups with same pace, then SSB periodicity of each beam hopping group could be 80ms. And s single SMTC configuration for 20ms periodicity SSB could cover all these neighbouring cells measurement.

Option4: Increase the number of configured SMTCs, UE down selects a subset of the configured SMTCs based on its location.

With more SMTCs are configured, UE will need to spend time and energy to scan neighbouring cells during all configured SMTCs. According to RAN4 specification,  UE may choose to sequentially perform the measurement over the configured SMTCs, this will result in longer measurement periodicity over each neighbouring cell and reduce the measurement performance:

#### 4.2C.2.3 Measurements of intra-frequency NR cells […] The requirements in this clause apply provided that the number of SMTCs for intra-frequency carrier does not exceed the parallelSMTC-r17, otherwise UE may select one or subset of all the configured SMTCs sequentially for performing the measurements until all of the SMTCs can be measured. The selection of SMTCs to be used is up to UE implementation, and in this case, measurement period longer than the corresponding measurement period specified in Table 4.2C.2.3-1 and Table 4.2C.2.3-2 is expected.

To avoid this, furthermore, proponent companies propose to allow UE selecting a subset of the configured SMTCs to measure based on UE location, e.g., UE only need to measure SMTC corresponding neighbouring cell which are closer to it. However this will require UE know the linkage between configured SMTC and satellites/cell.

Considering the specification effort, and the extra issue brought up by option 4, we prefer NW implementation solutions other than option4.

Not to increase the maximum number of configurable SMTCs.

## Msg4 PDSCH repetition

In RAN1 #119 [2], PDSCH repetition for Msg4 can be supported and the corresponding agreements are below.

Agreement For PDSCH with Msg4 Link level enhancement: - Support PDSCH repetition - FFS: signalling design including number of repetitions - FFS: impact on UE capability - Note: the target coverage enhancement to bridge the gap with respect to single Msg4 transmission is 2.8 dB - Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level. Agreement For PDCCH CSS (except Type-3) link level enhancements, support only PDCCH repetition for NTN. FFS: intra-slot and/or inter-slot

As agreed above, Msg4 PDSCH repetition can be supported. Then in the following, we will discuss how to support Msg4 PDSCH repetition. The first question is how to determine whether Msg4 PDSCH repetition is needed. If it is determined by the UE, a threshold for judgment should be configured by the network similar to PUCCH repetition of Msg4 HARQ-ACK. When a threshold is configured and the RSRP of the DL pathloss reference is less than the configured threshold, Msg4 PDSCH repetition is needed. Then the UE will request Msg4 PDSCH repetition.

RAN2 to discuss whether threshold for Msg4 PDSCH repetition is needed.

When the UE determines Msg4 PDSCH repetition is needed based on the configured threshold and measured result, it will request Msg4 PDSCH repetition to the network. If Msg4 PDSCH repetition is determined by the network, the UE also needs to inform the UE capability of supporting Msg4 PDSCH repetition to the network. In other words, regardless of who determines Msg4 PDSCH repetition, the UE capability of supporting Msg4 PDSCH repetition or a request for Msg4 PDSCH repetition can be needed. For the UE capability indication or request indication, the UE may inform the network by one of the following options:

- Option 1: RACH resource

  - Spare bit in the feature combination of RACH resource configuration can be used to indicate Msg4 PDSCH repetition

- Option 2: LCID for CCCH message of Msg3

  - Considering (e)RedCap UEs, 6 LCIDs will be needed

For option 1, a spare bit is used for Msg4 PDSCH repetition, which is very simple. If Msg4 PDCCH repetition is also needed, the other spare bit will be used. In other words, if Msg4 PDCCH repetition and Msg4 PDSCH repetition are also needed, two spare bits are enough. But this option can also introduce more RACH resource fragmentation.

If option 2 is adopted, for Msg4 PDSCH repetition, 6 LCIDs will be needed. If Msg4 PDCCH repetition also needs to be informed, 6 LCIDs are needed. Besides, if both Msg4 PDCCH repetition and Msg4 PDSCH repetition are also informed, the combined LCIDs are also needed. In summary, if Msg4 PDCCH repetition and Msg4 PDSCH repetition are also needed, 18 LCIDs are needed, which will consume too many LCIDs.

RAN2 to discuss the following two options of indicating UE request/UE capability of Msg4 PDSCH repetition:

- Option 1: spare bit in the feature combination of RACH resource

- Option 2: LCID for CCCH message of Msg3

Besides, the repetition number needs to be indicated to the UE. Similar to PUCCH repetition of Msg4 HARQ-ACK, firstly, a repetition number list should be configured to the UE by SIB1. Secondly, which repetition number can be used will be indicated to the UE by DCI 1_0. Regarding which field of DCI 1_0 can be used is up to RAN1 determination.

One repetition number list for Msg4 PDSCH repetition can be configured by SIB1.

Repetition number value can be indicated to the UE by DCI 1_0, and which bit field of DCI 1_0 is used is up to RAN1 determination.

# Conclusion

In summary, we would like to share our view on downlink coverage enhancement with following proposals:

Proposal 1:	no need of new barring bit in SIB1 due to extended SSB periodicity.

Proposal 2:	no need to enhance cell reselection due to extended SSB periodicity.

Proposal 3:	Not to increase the maximum number of configurable SMTCs.

Proposal 4:	RAN2 to discuss whether threshold for Msg4 PDSCH repetition is needed.

Proposal 5:	RAN2 to discuss the following two options of indicating UE request/UE capability of Msg4 PDSCH repetition:

-	Option 1: spare bit in the feature combination of RACH resource

-	Option 2: LCID for CCCH message of Msg3

Proposal 6:	One repetition number list for Msg4 PDSCH repetition can be configured by SIB1.

Proposal 7:	Repetition number value can be indicated to the UE by DCI 1_0, and which bit field of DCI 1_0 is used is up to RAN1 determination.

# References

[1] Draft_RAN2_128_Meeting_Report_v1.

[2] Chair notes RAN1#119.