3GPP TSG-RAN WG2 Meeting #129	R2-2500691

Athens, Greece, 17 - 21 Feb, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on on-demand SSB SCell operation for NES

Agenda Item:	8.5.2

Document for:	Discussion and decision

Introduction

In this contribution, we discuss the remaining issues of on-demand SSB SCell operation, including MAC CE design and L3 measurements.

Discussion

2.1 MAC CE design

In RAN2#127bis, RAN2 has agreed that the new MAC CE (for indicating OD-SSB transmission) is not used for SCell activation/deactivation, and that the MAC CE should be able to indicate the OD-SSB transmission for multiple SCells:

Agreements on OD-SSB SCell

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

On MAC CE design, there are several issues to be discussed:

Issue 1: Which SCells to be included? Only the SCells supporting OD-SSB or all the SCells?

Issue 2: How to identify an SCell, using explicit SCell indexes or the SCells are listed in an implicit order?

Issue 3: How to indicate SSB periodicity? By absolute value or by configuration index?

Including all the required information for OD-SSB transmission triggering of up to 31 SCells can make the new MAC CE very big (e.g. including OD-SSB periodicity per SCell and other possible information introduced later by RAN1). To save signalling overhead, we think only the SCells supporting OD-SSB should be included in the new MAC CE for OD-SSB transmission. Regarding how to inform the UEs of which SCells supporting OD-SSB, we think it can be indicated in the SCell configuration, or implicitly indicated by the initial status of OD-SSB transmission for SCell (regardless of whether the initial status of OD-SSB is transmitted or not transmitted, the presence of the field indicates the SCell supports OD-SSB).

Proposal 1: Only the SCells supporting OD-SSB will be included in the OD-SSB transmission MAC CE.

The Issue 2 above implies two different ways of MAC CE design. Considering RAN2 has agreed that the new MAC CE should be able to indicate the OD-SSB transmission for multiple SCells, the new MAC CE is of variable size. If we use the explicit serving cell ID, each serving cell ID will take up 5 bits, and is less efficient compared with implicit SCell index. Therefore, we prefer to use implicit SCell index in the MAC CE.

On Issue 3, the existing SSB periodicity contains 6 values (5ms, 10ms, 20ms, 40ms, 80ms, 160ms), and 3 bits can be used to indicate the SSB periodicity.

In the MAC CE design, we include some reserved bit in case other parameters are agreed to be included in the MAC CE (e.g. gaps between bursts, time instance A and time instance B, etc)

Example: MAC CE includes implicit SCell index and SSB periodicities (variable size)

RRC signalling can be used to configure an additional index for the SCells supporting OD-SSB (i.e. enabling the OD-SSB feature from UE perspective), and the assumption is that there will not be more than 8 SCells supporting OD-SSB simultaneously. To be specific, a new field od-SSB-SCellIndex can be added to SCellConfig with the value range “INTEGER (1..8)”. And the MAC CE can be designed as follows:

-	Oi: This field indicates the activation/deactivation status of the on-demand SSB transmission of the SCell with Od-SSB-SCellIndex i as specified in TS 38.331 [5]. The Oi field is set to 1 to indicate that the on-demand SSB transmission of the corresponding SCell is activated. The Oi field is set to 0 to indicate that the on-demand SSB transmission of the corresponding SCell is deactivated;

-	Periodicity: This field indicates the periodicity of the on-demand SSB of the SCell for which on-demand SSB transmission is activated. The length of the field is 3 bits. The Periodicy fields are included in ascending order based on the Oi set to 1.

Figure 1: On-demand SSB indication MAC CE

Table 1: SSB periodicity for On-demand SSB indication MAC CE

| Periodicity value | On-demand SSB periodicity |
| --- | --- |
| 000 | 5 ms |
| 001 | 10 ms |
| 010 | 20 ms |
| 011 | 40 ms |
| 100 | 80 ms |
| 101 | 160 ms |
| 110 | Spare |
| 111 | Spare |

Proposal 2: RRC indicates whether an SCell supports OD-SSB when configuring the SCell, MAC CE indicates the activation/deactivation status sequentially.

The above periodicity field can also be replaced by an index, in case more parameters are agreed by RAN1 to be dynamically indicated in MAC CE (pre-configure multiple sets of parameters in RRC and use MAC CE to dynamically select one of them). The issue is still open in RAN1, there is an active proposal in the FL summary [3]:

### Proposal #5-3 (RRC+MAC-CE indication): For a cell supporting on-demand SSB SCell operation, in addition to periodicity of the on-demand SSB, for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - SSB positions within an on-demand SSB burst - The number of on-demand SSB bursts to be transmitted after on-demand SSB is indicated[, if supported]

Proposal 3: In addition to OD-SSB periodicity, if RAN1 agrees on other parameters to be pre-configured by RRC and dynamically indicated by MAC CE, RAN2 considers to use configuration index in MAC CE.

For Case #1 (no always-on SSB), the above design is straightforward. For Case #2 (always-on SSB), the above design is based on the understanding that Case #2 means having a periodic SSB transmission as a baseline and using on-demand SSB for triggering additional SSB transmission, and MAC CE is only used to activate and indicate the SSB periodicity of the OD-SSB, it has nothing to do with the always-on SSB.

Proposal 4: For Case #2 (always-on SSB), the new MAC CE is only used to activate and indicate periodicity of OD-SSB, it is not used to modify/activate/deactivate the always-on SSB.

The new MAC CE requires an LCID or eLCID. Currently we only have 12 reserved values for LCID (values 35-46), while there are plenty of reserved values for one-octet eLCID (values 0 to 215) for DL-SCH. It is preferred to use eLCID for the OD-SSB transmission MAC CE.

Proposal 5: Use an eLCID for the OD-SSB transmission MAC CE.

2.2 L3 measurements

There are several open issues on L3 measurements:

Issue 1: Whether OD-SSB (on-demand SSB) is on the same frequency with AO-SSB (always-on SSB) or not

Issue 2: Whether OD-SSB measurement is based on SMTC or OD-SSB pattern

Issue 3: For Case #2 (AO-SSB), whether AO-SSB is also used to generate serving cell measurement results

On Issue 1, RAN4 has discussed the issue and identified the following scenarios:

Agreement: - In OD-SSB Case 2, RAN4 to first discuss the deactivated SCell requirement for Scenario 1. RAN4 to further discuss whether to define the requirement for the scenario 2 and 3 if scenario 2 and 3 are supported in RAN1 based on RAN1’s agreement. - Scenario 1: OD-SSB and always-on SSB are with same SSB carrier frequency, same offset but different periodicities. - Scenario 2: OD-SSB and always-on SSB are with same SSB carrier frequency, different offsets and periodicities. - Scenario 3: OD-SSB and always-on SSB are within different SSB carrier frequencies. - Send the above information to RAN1, further discuss the text including the questions to RAN1 in the LS. - RAN4 will inform RAN1 if any further agreement.

The final decision on whether OD-SSB and AO-SSB can have different frequency (Scenario 3) is up to RAN1, and RAN4 discussion will start from Scenario 1. RAN1 has agreed in RAN1#119 that at least for the case where AO-SSB is not CD-SSB, the OD-SSB and AO-SSB are on the same frequency, and will further discuss the case where AO-SSB is CD-SSB:

Agreement Response to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?) of Obj.1: - The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB. Agreement Down-select at least one of the following alternatives. - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - Alt 2: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB - Alt 3: Do not support the case where always-on SSB is CD-SSB on a synchronization raster. Down-select at least one of the following alternatives. - Alt A: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB can be same or different from the frequency location of always-on SSB, subject to its configuration. - Alt B: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB - Alt C: Do not support the case where always-on SSB is CD-SSB and not on a synchronization raster.

From RAN2 perspective, we would also like to prioritize Scenario 1, because Scenario 3 would bring considerable RAN2 impacts:

1) Impacts on servingCellMO configuration: In existing spec, each serving cell can be configured with a servingCellMO for deriving serving cell measurements, and only one SSB frequency is configured per MO. If both OD-SSB and AO-SSB are used for serving cell measurements, either two servingCellMOs are considered, or two SSB frequencies are configured in the single servingCellMO. Also, the serving cell CSI-RS measurement is based on an associated SSB, it is also unclear whether it is based on OD-SSB or AO-SSB.

2) Impacts on neighbour cell measurements: In existing spec, whether an SSB based measurement is considered as intra-frequency or inter-frequency measurement depends on whether the neighbour cell SSB is of the same frequency with the SSB of serving cell, and whether the SCS is the same as serving cell SSB. If both OD-SSB and AO-SSB are configured for serving cell measurements, RAN2 needs to clarify whether OD-SSB or AO-SSB will be used as baseline to identify intra-frequency/inter-frequency measurements.

3) Impacts on gap configuration: In existing spec, other than the cases where UE reports gap requirement (e.g. via NeedForGapsInfoNR reporting), if any of the configured BWPs do not contain serving cell SSB (which refers to the SSB of servingCellMO) and are not configured with NCD-SSB for serving cell measurement (which was introduced in RedCap), the network should configure gaps to the UE for intra-frequency measurements. However, if the SCell can be configured with AO-SSB and an inter-frequency OD-SSB, it is unclear whether gaps are needed if a BWP contains AO-SSB but not OD-SSB.

- For SSB based intra-frequency measurement, if the measurement gap requirement information is reported by the UE, a measurement gap configuration may be provided according to the information. Otherwise, a measurement gap configuration is always provided in the following case: - Other than the initial BWP, if any of the UE configured BWPs do not contain the frequency domain resources of the SSB associated to the initial DL BWP, and are not configured with NCD-SSB for serving cell measurement.

Proposal 6: RAN2 starts discussion on L3 measurement based on Scenario 1 (OD-SSB and always-on SSB are with same SSB carrier frequency, same offset but different periodicities).

Proposal 7: If Scenario 3 (OD-SSB and always-on SSB are within different SSB carrier frequencies) is considered, the following issues needs to be addressed:

- Impacts on servingCellMO configuration (whether both OD-SSB and always-on SSB frequencies are configured)

- Impacts on neighbour cell measurements (whether intra-frequency/inter-frequency determination can be based on OD-SSB)

- Impacts on gap configuration for intra-frequency measurements (whether gaps are needed if a BWP contains always-on SSB but not OD-SSB)

On Issue 2 (Whether OD-SSB measurement is based on SMTC or OD-SSB pattern):

In legacy mechanism, L3 measurement is based on the measConfig, which uses measIds to link measObjects (which provide SMTC for UE to monitor SSB) and reportConfigs, and UE measurement reporting is also based on measIds. For OD-SSB, at least the OD-SSB pattern of the serving cell will be informed to the UE. Therefore, RAN2 needs to determine whether the L3 measurement is still based on SMTC or it is based on the OD-SSB pattern or both.

For both Case #1 (no always-on SSB) and Case #2 (always-on SSB), as long as the OD-SSB pattern is informed to the UE, UE can perform serving cell measurements. There is no need to modify the legacy SMTC or make up a new dedicated SMTC for serving cell measurement purpose. If the specification forces the NW implementation to configure an SMTC for OD-SSB measurements, this configuration is anyway aligned with the OD-SSB pattern, which is already known to the UE, and an unnecessary step is defined at the NW side.

It was raised by some companies that reusing the existing SMTC framework minimizes RAN4 impacts. From our perspective, it is up to RAN4 to decide whether the spec impact is acceptable, for instance RAN4 is also discussing replacing the SMTC periodicity in the existing requirements with OD-SSB periodicity, so the measurements of OD-SSB can be based on OD-SSB pattern rather than SMTC. Besides, even if the OD-SSB measurement is based on SMTC, the existing framework also needs to be modified, considering the following open issues:

- If AO-SSB needs to be measured as well, the SMTC may not cover both OD-SSB and AO-SSB;

- If the configured SMTC window does not fit OD-SSB, it needs to be updated/reconfigured, additional signalling design is needed;

- The measurement reporting procedure anyway needs to be modified, as the OD-SSB can be denser than normal SSB and it is not appropriate to use legacy events like A3/A5 to compare the measurement results with neighbour cells with normal SSB.

Observation: The existing measurements framework needs to be modified anyway, regardless of whether OD-SSB pattern or SMTC is used for OD-SSB measurements.

Proposal 8: For both Case #1 (no always-on SSB) and Case #2 (always-on SSB), the measurements of OD-SSB is based on OD-SSB pattern rather than SMTC.

Another question is, for Case #2 (always-on SSB), there is always-on SSB within the legacy SMTC, it is unclear whether the UE should ignore these SSBs or the UE needs to measure both always-on SSB and OD-SSB and combine them to generate measurement results, as illustrated in the below picture:

Figure 3: L3 measurements on OD-SSB

Understanding 1 is simpler: since the serving cell can use OD-SSB for measurements, the NW can make sure the OD-SSB is transmitted with enough periods/occasions to satisfy the measurement need, and UE does not need to refer to the SMTC for serving cell measurements.

Understanding 2 is more flexible: the UE can utilize both OD-SSB and always-on SSB to generate measurement results, but the complexity to combine the measurement results is increased. There is parallel discussion in RAN4 and the following has been agreed in RAN4#113:

Issue 1-4-1: The known cell condition – measurement reporting Agreement: - In case 1, known/unknown condition should be defined based on only OD-SSB. - In case 2, known/unknown condition should be defined based on either always-on SSB or OD-SSB. - Note 1: whether the always-on SSB and OD-SSB can be combined is discussed in another issue. - Note 2: the agreement is valid at least for the scenario with the same frequency for always-on SSB and OD-SSB. - Note 3: RAN4 to further clarify the relation for the SSB measured for known cell condition and the SSB used in SCell activation procedure.

RAN1 has agreed to introduce a more compact SSB pattern for OD-SSB, making it even more difficult to conclude on whether AO-SSB and OD-SSB can be combined to generate a measurement result.

Proposal 9: For Case #2 (always-on SSB), postpone the discussion on whether the always-on SSB is also used to generate serving cell measurement results until RAN4 concludes on whether the always-on SSB and OD-SSB can be combined.

The next issue is how to obtain OD-SSB pattern for serving cell measurements. The necessary parameter includes SSB start offset, duration, periodicity, which are quite similar with the SMTC parameters. Besides, a bitmap indicating the transmitted SSB indexes (similar to ssb-ToMeasure or ssb-PositionsInBurst) is needed.

RAN1 discussion used the terminology “time instance A” and “time instance B”, which describe the whole transmission duration for OD-SSB, but it does not illustrate the individual SSB burst duration.

--Cited from RAN1#116 agreements - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A. - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B. - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted. - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.

Time instance A can be viewed as the start offset of the OD-SSB pattern. The SSB duration can be determined by the UE after detecting the SSB index. Therefore, at least the SSB periodicity and the SSB bitmap need to be informed to the UE.

Proposal 10: The SSB periodicity and the ssb-PositionsInBurst of the OD-SSB need to be indicated to the UE. RAN2 further discuss whether to use RRC signalling or MAC CE for such information.

For neighbour cells measurements, we think on-demand SSB status of neighbour cells should be transparent to the UE (otherwise large signalling overhead in Uu interface and Xn interface are incurred), and it is up to gNB implementation how to handle the reported neighbour cell measurement results which could be degraded based on the intermittent SSB transmission status of neighbour cell.

Proposal 11: On L3 measurements towards neighbour cells, legacy measurement and reporting procedures are used, and the OD-SSB transmission status of neighbour cells is transparent to the UE.

In RAN4#112bis, RAN4 agreed to introduce fast L3 measurements for deactivated SCell:

Agreements: Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. - After the time window, UE can perform deactivated SCell measurement with a slow mode. - Option 2-1: fallback legacy requirement - Option 2-2: stop measurement - Option 2-3: No requirement - Time window means the time duration starting from receiving OD-SSB activation indication - FFS the time window configuration or pre-defined - FFS the fast L3 measurement requirement - FFS the relation between time window and OD-SSB deactivation command in the time domain When UE receives OD-SSB deactivation, UE is not required to measure the deactivated SCell.

In RAN4#113, RAN4 further agreed that in fast mode the measurement periodicity is based on OD-SSB periodicity rather than measCycleSCell:

Issue 1-2-1: OD-SSB based deactivated SCell measurement – measurement periodicity in fast mode measurement Agreement: For the case of CSSF = 1: - OD-SSB periodicity regardless of measCycleSCell and DRX cycle. - FFS whether a lower bound will be specified. FFS for the other cases of CSSF.

However, the fast measurement of deactivated SCell cannot be guaranteed, if the UE is configured with multiple MOs. In legacy spec (earlier than R18), it is up to UE implementation which MO if measured first if multiple MOs are configured. In R18, the field measSequence was introduced to solve the unpredictable sequence of inter-frequency measurement.

To satisfy RAN4’s need of fast deactivated SCell measurement, the SCell’s servingCellMO needs to be prioritized. But setting the sequence as 1 in measSequence of the servingCellMO may not work. First, there may be multiple MOs configured with the same meaSequence (e.g. set to 1, the highest priority), it is still up to UE implementation to measure which one. Second, the measSequence is configured in the MO, while the MO of the deactivated may not always be prioritized (e.g. only prioritized when OD-SSB is triggered).

Proposal 12: RAN2 to discuss enabling the UE to prioritize deactivated SCell measurements when OD-SSB of that SCell is triggered.

2.3 Measurement reporting

In the existing spec, the report types of RRM measurements include the following:

ReportConfigNR ::=                          SEQUENCE {

reportType                                  CHOICE {

periodical                                  PeriodicalReportConfig,

eventTriggered                              EventTriggerConfig,

...,

reportCGI                                   ReportCGI,

reportSFTD                                  ReportSFTD-NR,

condTriggerConfig-r16                       CondTriggerConfig-r16,

cli-Periodical-r16                          CLI-PeriodicalReportConfig-r16,

cli-EventTriggered-r16                      CLI-EventTriggerConfig-r16,

rxTxPeriodical-r17                          RxTxPeriodical-r17,

reportOnScellActivation-r18                 ReportOnScellActivation-r18

}

}

Event triggered reporting and periodical reporting are widely used to trigger serving/neighbour cell measurement reports.

For OD-SSB based SCell measurements, considering OD-SSB is transmitted in a limited period (e.g. from time instance A to time instance B), and there could be no intra-frequency neighbour cell SSBs during the period or there are fewer SSB occasions of neighbour cells compared with serving cell OD-SSB, it is difficult to define a proper report interval.

Looking at the existing RRM measurement events, event A1 and A2 only concern serving cell, and can be used for triggering OD-SSB measurement reporting. For event A3 and A5, the neighbour cell measurements need to be compared with serving cell measurements to determine whether the event is triggered. Considering the different characteristics of serving cell OD-SSB (which is denser, in a confined period of time) and neighbour cell always-on SSB (which is sparser), it is unclear whether A3 and A5 can be triggered properly.

Proposal 13: RAN2 to discuss how periodical reporting and/or event-triggered reporting is applied to OD-SSB measurements.

Conclusion

In this contribution, we discussed on-demand SSB for intra/inter-band CA, and propose the following:

Proposal 1: Only the SCells supporting OD-SSB will be included in the OD-SSB transmission MAC CE.

Proposal 2: RRC indicates whether an SCell supports OD-SSB when configuring the SCell, MAC CE indicates the activation/deactivation status sequentially.

Proposal 3: In addition to OD-SSB periodicity, if RAN1 agrees on other parameters to be pre-configured by RRC and dynamically indicated by MAC CE, RAN2 considers to use configuration index in MAC CE.

Proposal 4: For Case #2 (always-on SSB), the new MAC CE is only used to activate and indicate periodicity of OD-SSB, it is not used to modify/activate/deactivate the always-on SSB.

Proposal 5: Use an eLCID for the OD-SSB transmission MAC CE.

Proposal 6: RAN2 starts discussion on L3 measurement based on Scenario 1 (OD-SSB and always-on SSB are with same SSB carrier frequency, same offset but different periodicities).

Proposal 7: If Scenario 3 (OD-SSB and always-on SSB are within different SSB carrier frequencies) is considered, the following issues needs to be addressed:

- Impacts on servingCellMO configuration (whether both OD-SSB and always-on SSB frequencies are configured)

- Impacts on neighbour cell measurements (whether intra-frequency/inter-frequency determination can be based on OD-SSB)

- Impacts on gap configuration for intra-frequency measurements (whether gaps are needed if a BWP contains always-on SSB but not OD-SSB)

Observation: The existing measurements framework needs to be modified anyway, regardless of whether OD-SSB pattern or SMTC is used for OD-SSB measurements.

Proposal 8: For both Case #1 (no always-on SSB) and Case #2 (always-on SSB), the measurements of OD-SSB is based on OD-SSB pattern rather than SMTC.

Proposal 9: For Case #2 (always-on SSB), postpone the discussion on whether the always-on SSB is also used to generate serving cell measurement results until RAN4 concludes on whether the always-on SSB and OD-SSB can be combined.

Proposal 10: The SSB periodicity and the ssb-PositionsInBurst of the OD-SSB need to be indicated to the UE. RAN2 further discuss whether to use RRC signalling or MAC CE for such information.

Proposal 11: On L3 measurements towards neighbour cells, legacy measurement and reporting procedures are used, and the OD-SSB transmission status of neighbour cells is transparent to the UE.

Proposal 12: RAN2 to discuss enabling the UE to prioritize deactivated SCell measurements when OD-SSB of that SCell is triggered.

Proposal 13: RAN2 to discuss how periodical reporting and/or event-triggered reporting is applied to OD-SSB measurements.

References

- RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson

- R4-2416916, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson

- R1-2410782, Summary #4 of on-demand SSB for NES, LG Electronics