3GPP TSG-RAN WG2 Meeting #129	R2-2500699

Athens, Greece, Feb 17th – 21st, 2024

Agenda item:		7.0.2.14

Source:		Samsung

Title:			On multicast MCCH information acquisition

Document for:	Discussion & Decision

# 1 Introduction

In this paper, we discuss a potential issue for the UE for acquiring multicast MCCH information upon group paging reception.

# 2 Discussion

In the present spec, the UE behaviour for multicast MCCH information acquisition upon reception of group paging is specified as follows:

#### 5.10.2.3 Multicast MCCH information acquisition by the UE A UE configured to receive an MBS multicast service in RRC_INACTIVE shall: 1> if the procedure is triggered by a multicast MCCH information change notification: 2> start acquiring the MBSMulticastConfiguration message on multicast MCCH in the concerned cell from the slot in which the change notification was received; - if the UE moves to a different cell providing SIB24; or 1> if the procedure is triggered by the reception of GroupPaging in the paging message in 5.3.2.3; or 1> if the UE receives RRCRelease configuring the UE to receive MBS multicast in RRC_INACTIVE which does not include PTM configuration for at least one multicast session for which the UE is not indicated to stop monitoring the G-RNTI: 2> acquire the MBSMulticastConfiguration message on multicast MCCH in the concerned cell at the next repetition period.

Observation 1:  Upon receiving group paging, UE acquires the multicast MCCH information at the next repetition period.

Further, it can be noted that MCCH modification period has a much larger range of configurable periodicities and network can choose a most appropriate value that can cater to provisioned services requirements. However, it can not be restricted to exactly same periodicity as that of Paging cycle used in the specific cell. Moreover, range of paging cycle lengths do not match with that for MCCH.

mcch-ModificationPeriod-r17 ENUMERATED {rf2, rf4, rf8, rf16, rf32, rf64, rf128, rf256, rf512, rf1024, rf2048, rf4096, rf8192, rf16384, rf32768, rf65536} PagingCycle ::= ENUMERATED {rf32, rf64, rf128, rf256}

Observation 2:  MCCH modification period has a much larger range of configurable periodicities to cater to diverse set of service requirements and therefore, it can not be restricted to exactly same periodicity as that of Paging cycle used in the specific cell. Otherwise, it leads to drastic impact to network deployment flexibility and service requirements.

Figure 1: Group paging and multicast MCCH acquisition

Figure 1 depicts a scenario for group paging reception and multicast MCCH acquisition. It is clear that as there are many UEs receiving group paging on their POs and in accordance with the present RRC spec, these UEs would attempt to acquire multicast MCCH at their immediate next repetition period. With all the possibilities considered, some of the UEs would definitely fail to acquire updated multicast MCCH information, if the immediate next repetition period does not carry updated MCCH information. Consequently, these UEs would see an absence of PTM configuration on the acquired multicast MCCH message corresponding to the group-paged MBS sessions and in this situation, would be forced to initiate RRC connection resumption. This is drastic and defeats the purpose.

Observation 3:  In certain scenarios, there are definitely some UEs which would fail to acquire updated multicast MCCH information upon group paging, if the immediate next repetition period does not carry updated MCCH information.

It is to be noted that multicast MCCH change notification cannot help in this case, as the change notification is generic for any configuration change and UE cannot deduce anything about the specific PTM configuration update/inclusion.

It can also be noted that discussed problem is not an issue for non-group paging cases (i.e., UE moves to a different cell providing SIB24 or UE receives RRCRelease configuring the UE to receive MBS multicast in RRC_INACTIVE which does not include PTM configuration for at least one multicast session for which the UE is not indicated to stop monitoring the G-RNTI) as there is no paging cycle consideration involved and network can suitably provide updated MCCH information for UE’s reception.

As this is a definite issue faced by UEs upon group paging reception, the following options should be considered to resolve this:

- UE acquires multicast MCCH in next modification period: It is guaranteed that UE would get updated multicast MCCH information in the next modification period after receiving group paging. However, it may add to latency in some cases, when previous modification period could also carried updated information.

- Network ensures to page UE only after multicast MCCH is updated with PTM configuration: This approach ensures UE definitely receives updated multicast MCCH information when it is group-paged. However, there is additional latency and network requirements on the timing of the group paging.

- UE acquires multicast MCCH in immediate next repetition period and if PTM configuration for group-paged session is not included, ensure to acquire multicast MCCH in next modification period: This approach ensures UE attemps to avail updated multicast MCCH information (i.e. PTM configuration for the sessions that are group-paged) until the next modification period when it is not provided in the immediate next repetition period.

In our understanding, option 3 seems more reasonable and reliable for UE behaviour. However, RAN2 should discuss and clarify the behaviour with selecting one of them.

Proposal 1: RAN2 to discuss and select one of the options:

Option 1: Upon receiving group paging, UE acquires multicast MCCH in immediate next modification period. (Adopt TP1)

Option 2: Network ensures to page UE only after multicast MCCH is updated with PTM configuration. (Adopt TP2)

Option 3: Upon receiving group paging, UE acquires multicast MCCH in immediate next repetition period and if PTM configuration for group-paged session is not included, ensure to acquire multicast MCCH in next modification period. (Adopt TP3)

# 3 Conclusion

Based on the discussion above, RAN2 is requested to consider the following observations and proposals:

Observation 1:  Upon receiving group paging, UE acquires the multicast MCCH information at the next repetition period.

Observation 2:  MCCH modification period has a much larger range of configurable periodicities to cater to diverse set of service requirements and therefore, it can not be restricted to exactly same periodicity as that of Paging cycle used in the specific cell. Otherwise, it leads to drastic impact to network deployment flexibility and service requirements.

Observation 3:  In certain scenarios, there are definitely some UEs which would fail to acquire updated multicast MCCH information upon group paging, if the immediate next repetition period does not carry updated MCCH information.

Proposal 1: RAN2 to discuss and select one of the options:

Option 1: Upon receiving group paging, UE acquires multicast MCCH in immediate next modification period. (Adopt TP1)

Option 2: Network ensures to page UE only after multicast MCCH is updated with PTM configuration. (Adopt TP2)

Option 3: Upon receiving group paging, UE acquires multicast MCCH in immediate next repetition period and if PTM configuration for group-paged session is not included, ensure to acquire multicast MCCH in next modification period. (Adopt TP3)

# 4 Annex

4.1 Option 1 – TP1

#### 5.10.2.3 Multicast MCCH information acquisition by the UE A UE configured to receive an MBS multicast service in RRC_INACTIVE shall: 1> if the procedure is triggered by a multicast MCCH information change notification: 2> start acquiring the MBSMulticastConfiguration message on multicast MCCH in the concerned cell from the slot in which the change notification was received; - if the UE moves to a different cell providing SIB24; or 1> if the UE receives RRCRelease configuring the UE to receive MBS multicast in RRC_INACTIVE which does not include PTM configuration for at least one multicast session for which the UE is not indicated to stop monitoring the G-RNTI: 2> acquire the MBSMulticastConfiguration message on multicast MCCH in the concerned cell at the next repetition period. 1> if the procedure is triggered by the reception of GroupPaging in the paging message in 5.3.2.3: 2> acquire the MBSMulticastConfiguration message on multicast MCCH in the concerned cell at the next mcch-ModificationPeriod.

4.2 Option 2 – TP2

#### 5.10.2.3 Multicast MCCH information acquisition by the UE A UE configured to receive an MBS multicast service in RRC_INACTIVE shall: 1> if the procedure is triggered by a multicast MCCH information change notification: 2> start acquiring the MBSMulticastConfiguration message on multicast MCCH in the concerned cell from the slot in which the change notification was received; - if the UE moves to a different cell providing SIB24; or 1> if the procedure is triggered by the reception of GroupPaging in the paging message in 5.3.2.3; or 1> if the UE receives RRCRelease configuring the UE to receive MBS multicast in RRC_INACTIVE which does not include PTM configuration for at least one multicast session for which the UE is not indicated to stop monitoring the G-RNTI: 2> acquire the MBSMulticastConfiguration message on multicast MCCH in the concerned cell at the next repetition period. Note: Network ensures to page UE only after multicast MCCH is updated with PTM configuration.

4.3 Option 3 – TP3

#### 5.10.2.3 Multicast MCCH information acquisition by the UE A UE configured to receive an MBS multicast service in RRC_INACTIVE shall: 1> if the procedure is triggered by a multicast MCCH information change notification: 2> start acquiring the MBSMulticastConfiguration message on multicast MCCH in the concerned cell from the slot in which the change notification was received; - if the UE moves to a different cell providing SIB24; or 1> if the UE receives RRCRelease configuring the UE to receive MBS multicast in RRC_INACTIVE which does not include PTM configuration for at least one multicast session for which the UE is not indicated to stop monitoring the G-RNTI: 2> acquire the MBSMulticastConfiguration message on multicast MCCH in the concerned cell at the next repetition period. 1> if the procedure is triggered by the reception of GroupPaging in the paging message in 5.3.2.3: 2> acquire the MBSMulticastConfiguration message on multicast MCCH in the concerned cell at the next repetition period and if PTM configuration is not included for multicast session(s) for which TMGI(s) is indicated in pagingGroupList, acquire the MBSMulticastConfiguration message on multicast MCCH in the concerned cell at the next mcch-ModificationPeriod.