3GPP TSG-RAN WG2 Meeting #129	R2-2500700

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.13.2

Source: 	Huawei, HiSilicon

Title: 	Relay discovery and (re)selection for Multi hop Relay

Document for:	Discussion and Decision

# Introduction

In RAN #105 meeting, the new work item on NR sidelink multi-hop relay [1] was approved.

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] 1. Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. A. Relay discovery and (re)selection [RAN2] B. Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] C. Impact on SRAP and QoS handling for multi-hop [RAN2] D. Control plane procedures [RAN2, RAN3] 2. Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: A. Intra-gNB multi-hop indirect to direct path switching using existing framework B. Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: C. Intra-gNB direct to multi-hop indirect path switching D. Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In RAN 2# 128 meeting some agreements were made for overall design and relay discovery and (re)selection  for NR sidelink multi-hop relay which are listed as below.

Agreements for relay discovery and (re)selection: - A lower bound of Uu RSRP is not required for the UE to operate as an intermediate Relay UE. - If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE (i.e., minimise number of hops when possible). FFS if operation as an intermediate relay UE towards a different cell would be supported. - The network can configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE. If the upper bound is not configured, there is no threshold, but this does not override the previous agreement. FFS if there would be additional restrictions in the case of no upper bound. - The following cases are supported for L2 multihop relay: - One last Relay UE can have two connections with one intermediate Relay UE and one Remote UE (the intermediate Relay UE and Remote UE are physically different UE). - Two physically different Remote UE(s) can have each indirect path via the same intermediate Relay UE and the same last Relay UE. FFS if the last relay UE can use the same L2 ID for both of the connections in either case. - Cases with two indirect paths to the gNB for the same remote UE are not supported. - An Intermediate Relay UE can serve multiple multi-hop indirect paths of different Remote UEs. - If the intermediate Relay UE also is acting as a Remote UE, it cannot support different indirect paths to the gNB with same/different last/U2N/parent intermediate Relay UE(s) based on different PC5 unicast links. - RAN2 sends LS to ask SA2 to clarify whether an intermediate relay needs to establish PC5 link with the relay from which the discovery announcement message is received before sending/forwarding the discovery announcement message.

In this paper, some discussion on the relay discovery and relay (re)selection aspects are provided.

# Discussion

Rel-19 Multi-hop relay can further extend the coverage by supporting the sidelink based multiple hop relay on top of Rel-17 U2N relay. Figure 1 shows the scenario where the remote UE connects with the network through two relay UEs, i.e., remote UE -> first relay UE -> last relay UE -> gNB.

Figure 1. Multi-hop relay with two relaying hops

In this paper, we use the terminology agreed in RAN2#127 meeting for the convenience of discussion.

- last relay UE: the U2N relay UE directly connected to the gNB

- intermediate relay UE: the relay UEs except for the last relay UE

- first relay UE: the intermediate relay UE directly connected to the remote UE

To support the multi-hop relay, two candidate solutions have been discussed up to RAN2#128 meeting,.

- Approach 1: The network would directly control each of the relay UEs (first, intermediate, last) via Uu RRC.

- Approach 2: Only the last relay UE would require control by the network via Uu RRC.

## Topology design

In RAN2#128 meeting, the following agreement were reached for last relay UE, and a remaining issue is whether last relay UE can operate as an intermediate relay UE.

If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE (i.e., minimise number of hops when possible). FFS if operation as an intermediate relay UE towards a different cell would be supported.

The scenarios for the above FFS are shown in Figure 2, which can include (a) the intra-gNB case (i.e., the intermediate relay UE connects to a different cell in the same gNB) and (b) the inter-gNB case (i.e., the intermediate relay UE connects to a different cell in a different gNB).

(a)

(b)

Figure 2. Scenarios last relay UE also operation as an intermediate relay UE towards a different cell

From the figure, it is clear that the last relay UE must  support the multi-path relay if the above FFS scenarios is to be supported. One path is connected directly with a gNB via Uu, while another path is connected with the same or a different gNB through  either single-hop or multi-hop relaying. According to the WID shown above, the multi-hop relay for a single indirect path via SL relay UEs should be specified “to support up to two additional hops relays on top of Rel-17 U2N relay”, meaning the multi-path relay is not supported. Considering the TU limit, the focus should on a basic version of the single path multi-hop relay support.

Additionally, there appears to be no strong motivation to support the scenarios in figure 2. The remote UE can connect via the path “remote UE <->first relay UE<->last relay UE<->gNB”, without any clear advantage of adding an additional hop in the path “remote UE <->first relay UE<->last relay UE<->last relay UE1<->gNB/gNB1”

Proposal 1: RAN 2 agrees not to support scenarios where the last relay UE also acts as the intermediate relay UE towards a different cell or gNB.

Furthermore, in our understanding the multi-hop relays should have tree-like topology for the following reasons.

First, the Rel-17 U2N relay supports the tree-like topology. In Rel-17, the relay UE connects to one gNB (and to one serving cell) and can serve multiple remote UEs, while each remote UE only connects to one relay UE. As stated in the WID, the primary goal of multi-hop relay is to address the range limitation of a single hop sidelink relay with the enhanced mechanism considered on top of Rel-17 relay. Therefore, it makes sense to adopt the tree-like topology design used in Rel-17, as it enables maximum reuse of the Rel-17 relay mechanism.

Secondly, the control plane aspects (like the local ID allocation, QoS split, RRC configuration) can be simpler as the gNB can control all the UEs within a tree-like topology. Given the limited TUs allocated for the multi-hop relay topic, RAN 2 should only agree to adopt the tree-like topology avoid further discussion related to the topology.

Based on the tree-like topology, the intermediate relay UE connects to only one parent relay UE, and adopts the parent relay UE’s cell as its own serving cell.  In this architecture, the gNB can control all the UEs, including the remote UE and the relay UEs. In [2],  our view is expressed that the approach 1 is a natural extension of the Rel-17 U2N relay mechanism which can be further extended to more hops.  This approach offers several benefits such as simplified local ID allocation, QoS handling, RRC State management.

Proposal 2: Given the limited TUs allocated for the multi-hop relay, RAN2 confirms that the multi-hop relay will operate exclusively in a tree-like topology, which involves hop extension based on the Rel-17 U2N relay mechanism. Consequently Rel-17 U2N relay mechanisms can be reused to the greatest extent possible.

## Conditions for performing Relay discovery

In RAN2#128 meeting, the following agreements for the intermediate relay UE to perform discovery were reached.

• A lower bound of Uu RSRP is not required for the UE to operate as an intermediate Relay UE. • The network can configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE. If the upper bound is not configured, there is no threshold, but this does not override the previous agreement. FFS if there would be additional restrictions in the case of no upper bound.

We believe the above agreements were only applicable  to the approach 2, where the intermediate relay UE can connect through Uu even though there is an indirect path. The question is, as  illustrated in section 2.1, why would the remote UE need to connect with the gNB through an indirect path of the intermediate relay UE when intermediate relay UE has a direct Uu link?. This could introduce an additional hop in the previous topology

Approach 1 should only support the tree-like topology, as we proposed in Proposal 2. In approach 1, we identify two possible cases for the intermediate relay UE.

Case 1: The intermediate relay UE has already established the PC5 connection with its parent relay UE, acting as a remote UE. (e.g., for its own data communication before serving as the relay UE).

Case 2: The intermediate relay UE has not yet established the PC5 connection with a parent relay UE.

In case 1, the intermediate relay UE can perform the discovery if it is capable of providing relaying service for multiple remote UEs. No AS condition is required since there is an existing  RRC connection with the network. Furthermore, according to the legacy Rel-17 relay mechanism, if a UE connects through an indirect link, it will suspend the Uu RLM, meaning no quality measurements will be taken for the Uu link.

Case 2 occurs  when the intermediate relay UE has just performed the relay (re)selection. In this scenario, we believe the intermediate relay UE can first perform the discovery as a remote UE. checking the Uu RSRP (whether below a threshold or not) to determine whether to function as a remote UE. Once the intermediate relay UE connects to a parent relay UE, it will then perform the discovery as a relay UE. In this case, the Uu RSRP is only used when the intermediate relay UE is functioning as a remote UE.

In above mentioned case 1 and case 2, the Rel-17 relay discovery procedure can be reused for the intermediate relay UE to search for  a parent relay UE and for the remote UE to search for  the first relay UE. This allows for efficient discovery without requiring major changes to the existing Rel-17 mechanism.

Observation 1. In Approach 1, the intermediate relay UE functions as a remote UE, when discovering and selecting a parent relay UE, allowing the legacy Uu RSRP threshold to be reused. Similar to Rel-17 relay, when the intermediate relay UE connects through the indirect path, it will suspend the Uu RLM.

Proposal 3: Reuse the Rel-17 direct discovery procedures for the intermediate relay UE to first search for a parent relay UE followed by the remote UEs searching for  the first relay UE in multi-hop scenario.

Proposal 4. In approach 1, no Uu RSRP threshold is needed for the intermediate relay UE.

Technically, we must ensure that the indirect link between the intermediate relay UE and the network should be in good condition. As a minimum, the intermediate relay UE should verify that the PC5 link between itself and its parent relay UE is in good condition. If the link quality between the intermediate relay UE and its parent relay UE is poor, the intermediate relay UE can trigger a relay reselection. Therefore, we believe no AS condition need to be checked by the intermediate relay UE before performing the discovery message transmissions.

Proposal 5: The intermediate relay UE performs the discovery procedure as a relay UE after establishing the PC5 connection with its parent relay UE. In this case it does not need to check the AS condition.

## Multiple connections for L2 multi-hop relay

In RAN2#128 meeting, the following agreements for supporting cases of multiple connections were reached, and we need to discuss whether the last relay UE can use the same L2 ID for both of the connections in either case.

The following cases are supported for L2 multihop relay: - One last Relay UE can have two connections with one intermediate Relay UE and one Remote UE (the intermediate Relay UE and Remote UE are physically different UE). - Two physically different Remote UE(s) can have each indirect path via the same intermediate Relay UE and the same last Relay UE. FFS if the last relay UE can use the same L2 ID for both of the connections in either case.

(a)  Last Relay UE having  two connections, one with intermediate Relay UE and one with Remote UE

(b) Two physically different Remote UE(s) each having indirect path via the same intermediate Relay UE and the same last Relay UE

Figure 3. Topology of the cases supported for L2 multi-hop relay

The FFS in the agreement is discussed for the above two cases in the following section:

For the first case, where the last Relay UE have two PC5 links, sidelink 1 (SL1) to connect with the intermediate relay UE and sidelink 2 (SL2) to connect with the remote UE1, as shown in Figure 3a. The decision of whether the last relay UE could use the same or  different L2 ID for these two sidelink can follow, the legacy mechanism in Rel-16 V2X. Specifically, it is up to UE implementation on whether to use the same or different L2 ID over different sidelinks.

For  the second case, as an example, when the remote UE1 and remote UE2 connects to the same intermediate relay UE and the same last relay UE as shown in Figure 3b. In this case, there is no need to establish/use two PC5 links between the last Relay UE and the intermediate Relay UE , which means remote UE1 and remote UE2 will share the same PC5 link. Therefore,  the same L2 ID could be used.

Proposal 6: If the different connections share the same PC5 link, the same L2 ID could  be used. If they do not share the PC5 link, it is up to the last Relay UE implementation to decide whether to use the same or different L2 ID over different PC5 links.

## Relay selection and reselection

Relay selection and reselection of the remote UE:

For Relay selection and reselection procedures, the legacy PC5 radio measurements mechanism can be reused as follows:

- The U2N Remote UE uses the SD-RSRP measurements to evaluate the link quality if there is no PC5 connection between the U2N Remote UE and the candidate Intermediate Relay UE.

- The U2N Remote UE uses the SL-RSRP measurements to evaluate the link quality between the Remote UE and the serving Intermediate Relay UE. The serving Intermediate Relay UE is the one  providing the relaying service for the U2N Remote UE.

- It is left to the Remote UE implementation to decide whether to use SL-RSRP or SD-RSRP or both for measurement  if there is no data transmission from serving Intermediate Relay UE to U2N Remote UE.

Proposal 7: Release-17 PC5 link measurement and evaluation principles for relay selection and reselection can be reused for multi-hop relay selection and reselection.

Similar to Release 17, the U2N Remote UE can trigger U2N relay selection in following cases .

- The RSRP measurement of the cell on which U2N Remote UE is camped goes below a configured threshold

- Indicated by upper layer of the U2N Remote UE

Hence, we propose the following

Proposal 8: The Relay selection for multi-hop relay can follow the Release 17 design that U2N Remote UE triggers U2N Relay selection in following cases:

- The RSRP measurement of the cell on which U2N Remote UE is camped goes below a configured threshold

- Indicated by upper layer of the U2N Remote UE.

The Remote UE can trigger relay reselection in the following cases:

- PC5 link quality between the current serving Intermediate Relay UE  and the remote UE  is below a configured threshold.

- Upon receiving the notification message from the serving Intermediate Relay UE. The serving Intermediate Relay UE sends the notification message when the cell selection, relay reselection, handover, PC5 RLF, Uu RRC connection establishment/resume failure happens.

- Upon receiving the notification message from the serving Intermediate Relay UE, which happens when the serving Intermediate Relay UE receives the notification message indicating the Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure of the U2N Last Relay UE.

- The U2N Remote UE receive the PC5 link release indication from the serving Intermediate Relay UE.

- The U2N Remote UE detects the PC5 RLF

- Indicated by upper layer of the U2N Remote UE.

Proposal 9: The Remote UE can trigger relay reselection in the following cases:

- PC5 link quality between the current serving Intermediate Relay UE and remote UE is below a configured threshold.

- Upon receiving the notification message from the serving Intermediate Relay UE.

- The U2N Remote UE receive the PC5 link release indication from the serving Intermediate Relay UE.

- The U2N Remote UE detects the PC5 RLF

- Indicated by upper layer of the U2N Remote UE.

Proposal 10: The serving Intermediate Relay UE sends the notification message to the U2N Remote UE when the cell selection, relay reselection, handover, PC5 RLF, Uu RRC connection establishment/resume failure happens.  Additionally, the Intermediate Relay UE sends a notification upon receiving the notification message from the U2N last Relay indicating the Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure of the U2N Relay UE

Content of the discovery message:

There may  be several candidate U2N Relay UE(s) or candidate Intermediate Relay UE(s) near the U2N Remote UE during selection and reselection. According to the current spec, the only information available to the Remote UE regarding  these candidate relay UE(s) is the PC5 link quality between the remote UE and candidate relay UE. For multi-hop relay, this information may not be sufficient to select a good relay UE.

Considering the case where  the Remote UE discovers the following candidate relay UE(s).

- Relay UE 1: The SD-RSRP is value 1, and the relay UE 1 is connected to the gNB through a U2N Relay UE. Value 1 satisfies the relay selection criterion, as it is above the configured threshold.

- Relay UE 2:  The SD-RSRP is value 2, and the relay UE 2 is connected to the gNB through a U2N Relay UE and an Intermediate Relay UE. Value 2 satisfies the relay selection criterion, and is larger than value 1.

Based on the PC5 link quality, the Remote UE is likely  to select the relay UE 2, as the PC5 link quality between the Remote UE and the relay UE2 is better. However, from the QoS perspective, the Relay UE 1 may be better since the hop number between the relay UE 1 and the network is smaller. Therefore, we propose that the hop number of each candidate Intermediate Relay UE be indicated to the U2N Remote UE during the discovery procedure.  This additional information could help the Remote UE to select a better relay UE.

Proposal 11: RAN2 should discuss whether/how to support indicating the hop number of the candidate Intermediate Relay UE to the U2N Remote UE during the discovery procedure.

Relay selection and reselection of the intermediate relay UE:

In the email discussion [Post127][401][Relay] [3], we also discussed the relay selection and reselection. Based on the tree-like topology, the intermediate relay UE performs relay selection and reselection as a remote UE. If the intermediate relay UE connects to a last relay UE, it performs the relay selection and reselection as a remote UE in Rel-17. If the intermediate relay UE connects to a parent intermediate relay UE, if performs the relay selection and reselection as a remote UE in multi-hop relay scenario, i.e., as proposed in proposal 7 to proposal 10.

Proposal 12: In general, the intermediate relay UE can perform the relay selection and reselection as a remote UE. If the intermediate relay UE connects to a last relay UE, it performs the relay selection and reselection as a remote UE in Rel-17. If the intermediate relay UE connects to a parent intermediate relay UE, if performs the relay selection and reselection as a remote UE in multi-hop relay scenario.

# Conclusion

In this paper, we discuss the relay discovery and relay (re)selection aspects and provide the following observations and proposals.

Topology design

Proposal 1: RAN 2 agrees not to support scenarios where the last relay UE also acts as the intermediate relay UE towards a different cell or gNB.

Proposal 2: Given the limited TUs allocated for the multi-hop relay, RAN2 confirms that the multi-hop relay will operate exclusively in a tree-like topology, which involves hop extension based on the Rel-17 U2N relay mechanism. Consequently Rel-17 U2N relay mechanisms can be reused to the greatest extent possible.

Conditions for performing Relay discovery

Observation 1. In Approach 1, the intermediate relay UE functions as a remote UE, when discovering and selecting a parent relay UE, allowing the legacy Uu RSRP threshold to be reused. Similar to Rel-17 relay, when the intermediate relay UE connects through the indirect path, it will suspend the Uu RLM.

Proposal 3: Reuse the Rel-17 direct discovery procedures for the intermediate relay UE to first search for  a parent relay UE followed by the remote UEs searching for  the first relay UE in multi-hop scenario.

Proposal 4. In approach 1, no Uu RSRP threshold is needed for the intermediate relay UE.

Proposal 5: The intermediate relay UE performs the discovery procedure as a relay UE after establishing the PC5 connection with its parent relay UE. In this case it does not need to check the AS condition.

Multiple connections for L2 multi-hop relay

Proposal 6: If the different connections share the same PC5 link, the same L2 ID could  be used. If they do not share the PC5 link, it is up to the last Relay UE implementation to decide whether to use the same or different L2 ID over different PC5 links.

Relay selection and reselection of the remote UE

Proposal 7: Release-17 PC5 link measurement and evaluation principles for relay selection and reselection can be reused for multi-hop relay selection and reselection.

Proposal 8: The Relay selection for multi-hop relay can follow the Release 17 design that U2N Remote UE triggers U2N Relay selection in following cases:

- The RSRP measurement of the cell on which U2N Remote UE is camped goes below a configured threshold

- Indicated by upper layer of the U2N Remote UE.

Proposal 9: The Remote UE can trigger relay reselection in the following cases:

- PC5 link quality between the current serving Intermediate Relay UE and remote UE is below a configured threshold.

- Upon receiving the notification message from the serving Intermediate Relay UE.

- The U2N Remote UE receive the PC5 link release indication from the serving Intermediate Relay UE.

- The U2N Remote UE detects the PC5 RLF

- Indicated by upper layer of the U2N Remote UE.

Proposal 10: The serving Intermediate Relay UE sends the notification message to the U2N Remote UE when the cell selection, relay reselection, handover, PC5 RLF, Uu RRC connection establishment/resume failure happens.  Additionally, the Intermediate Relay UE sends a notification upon receiving the notification message from the U2N last Relay indicating the Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure of the U2N Relay UE

Content of the discovery message:

Proposal 11: RAN2 should discuss whether/how to support indicating the hop number of the candidate Intermediate Relay UE to the U2N Remote UE during the discovery procedure.

Relay selection and reselection of the intermediate relay UE:

Proposal 12: In general, the intermediate relay UE can perform the relay selection and reselection as a remote UE. If the intermediate relay UE connects to a last relay UE, it performs the relay selection and reselection as a remote UE in Rel-17. If the intermediate relay UE connects to a parent intermediate relay UE, if performs the relay selection and reselection as a remote UE in multi-hop relay scenario.

# References

RP-242349, New WID on NR sidelink multi-hop relay

R2-2500701	Control plane procedures for multi-hop relay

R2-2410305, Report of [Post127][401][Relay] Multi-hop relay discovery and (re)selection

R2-2410006, Report of [POST127][402][Relay] Multi-hop relay control plane.