3GPP TSG-RAN WG2 Meeting #129	R2-2500718

Athens, Greece, 17 – 21 February, 2025

Agenda item:	8.7.4.2

Source:	InterDigital

Title:	DSR enhancements for UL scheduling

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

RAN2#127-bis made the following agreements wrt. the DSR enhancement:

We do not change the definition of delay-critical data For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s) Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: • Buffer size of data volume in each portion • Shortest remaining time among PDCP SDUs buffered in each portion. There is no need to include PSI in the enhanced DSR MAC CE. A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

RAN2-128 in turns made the following further agreements:

Let the network configure the triggering and reporting thresholds without constraints. RAN2 understanding is that the data that has been already reported in the DSR should not trigger another DSR The existing cancelling and triggering of Rel-18 DSR is reused for the enhanced DSR. The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

# 2	Discussion

## 2.1	Reporting vs. triggering threshold(s)

Since RAN2 agreed that there is no constraints for the network in configuring the triggering and reporting thresholds for Enhanced DSR, it seems worth agreeing that the triggering threshold (ie., remainingTimeThreshold) is still used as one of the reporting thresholds.

Proposal 1: The DSR triggering threshold (ie., remainingTimeThreshold) is also used as one of the reporting thresholds.

## 2.2	Enhanced DSR MAC CE

Based on the agreement and the discussion in above sections, in the below Figure 1, an illustration of constructing the Enhanced DSR MAC CE for an LCG is provided:

-	For reporting threshold 1: the Buffer Size of the red packets is reported with Remaining Time being set to the shortest remaining time of the red packets;

-	For reporting threshold 2: the Buffer size of the lavender packets is reported with Remaining Time being set to the shortest remaining time of the lavender packets;

-	For remaining time threshold: reporting is omitted since there is no packet with remaining time more than reportingThreshold2 but less than remainingTimeThreshold;

-	For reporting threshold 3:

-	The Buffer size of orange packets is reported – including the packets being regarded as delay critical data due to being part of a PDU set with a PDCP SDU having remaining time till discardTimer expiry less than remainingTimeThreshold.

-	Remaining Time being set to the shortest remaining time of the orange packets.

Figure 1. Illustration of constructing Enhanced DSR MAC CE for an LCG.

Hence, in the above example, for LCGX, three pairs of remaining time and buffer size is reported in the Enhanced DSR MAC CE (regardless of UE being configured with 4 thresholds). NW knows based on the reported Remaining Time to which “portion” and, hence, to which reporting threshold the reported Buffer Size is related to. This being the case, simplest approach for constructing the Enhanced DSR MAC CE is to take the legacy DSR MAC CE as baseline and use the available R bit in the octet of Remaining Time to indicate if further pair of Remaining Time/Buffer Size will follow for the LCG. Since the R bit is in an already used octet, no additional byte needs to be added (e.g., as with bitmap solution). Bitmap solution would require additional byte for each LCG while it would not provide any additional information compared to the allocating the available R bit as extension bit (E in the below figure) Hence, the Enhanced DSR MAC CE format should be the following:

Figure 2. Enhanced DSR MAC CE format.

Proposal 2: The R bit in the octet of Remaining Time in the Rel-18 DSR MAC CE is replaced with extension bit which indicates if further pair of Remaining Time/Buffer Size follows for the same LCG or not.

## 2.3	DSR truncation

In the RAN2#127-Bis meeting, it was discussed whether UE should be allowed/required to send the legacy Rel-18 DSR MAC CE in case the Enhanced DSR MAC CE does not fit to the available grant size scheduled by the NW – the following FFS was captured:

FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

Such optimization would work only for the special case that the legacy DSR MAC CE fits to the available grant and the Enhanced DSR MAC CE does not. This assumption would only hold for a case where at least one LCG would otherwise report multiple pairs of Remaining Time/Buffer Size thresholds – otherwise, the Enhanced DSR MAC CE also fits to the grant being the exact same size as the legacy DSR MAC CE. Such small optimization seems unnecessary and we should rather discuss whether we create new truncated Enhanced DSR MAC CE format (which could be optimized better and give more information to NW than the legacy format), or we assume NW knows there could be available DSR report (and the maximum size of the DSR) and schedules the UE accordingly.

Proposal 3: Legacy DSR MAC CE is not used when at least one LCG is configured with multiple reporting thresholds.

Proposal 4: If DSR truncation is supported, create a new Truncated DSR MAC CE format for the purpose.

# 3	Conclusion

This document has made the following proposals:

Proposal 1: The DSR triggering threshold (ie., remainingTimeThreshold) is also used as one of the reporting thresholds.

Proposal 2: The R bit in the octet of Remaining Time in the Rel-18 DSR MAC CE is replaced with extension bit which indicates if further pair of Remaining Time/Buffer Size follows for the same LCG or not.

Proposal 3: Legacy DSR MAC CE is not used when at least one LCG is configured with multiple reporting thresholds.

Proposal 4: If DSR truncation is supported, create a new Truncated DSR MAC CE format for the purpose.