3GPP TSG-RAN WG2 Meeting #129                         R2-2500758

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.2.3

Source: 	Transsion Holdings

Title:  	Discussion on Random Access for A-IoT

Document for:	Discussion and Decision

- Introduction

In the last RAN#106 meeting, a new work item for Ambient IoT is approved with following objective[1]:

  - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission:

    - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs.

Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

    - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129).

    - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R.

    - Only MAC layer is included

In this contribution, we provide our views on Random access to proceed the work item

- Discussion

2.1 A-IoT contention-free access procedure

In last meeting the procedure of CFRA for AIoT device was discussed with which the device sends Device ID and/or any other upper layer data in MSG1, the reader may echo some information from MSG1 in MSG2. For the contention-free access procedure RAN2 will study single and multi-device case. For single and multi-device case, as the number of devices is already known by the reader, when trigger the device to access, the corresponding access configuration can also be assigned to each device to increase efficiency, the access configuration can include time and frequency configuration.

Proposal 1: The device id list and corresponding access configuration list can be included in the initial trigger message.

In last meeting, it has been agreed that for 2step and 3step CBRA, the A-IoT device performs re-access in another opportunity provided by the reader (i.e. retry the random access) at least in case of contention resolution failure. For contention-free access, although there is no contention between devices, the device may failed to access due to the poor signal. To increase the access efficiency, the re-access configuration can also be provided by the reader for contention-free access, the device can use the re-access configurtion to re-access if contention-free access failed without the reader re-initiated trigger.

- Proposal 2: For contention-free access, the devices can re-access in another opportunity provided by the reader without re-trigger.

- 3 Conclusions

In this contribution we discussed issues related to A-IoT Random Access and made the following proposals:

Proposal 1: The device id list and corresponding access configuration list can be included in the initial trigger message.

Proposal 2: For contention-free access, the devices can re-access in another opportunity provided by the reader without the reader re-trigger..

- 4 References

- “RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”